# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Automatic control and data acquisition system for public lighting

- Publication reference: 1317/Public Enterprise Varos/97213/28.11.2013-16

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Item name: Supervisory control and data acquisition IT system (SCADA) Quantity: 1 |  |  |  |
|  | Administration - Administration of the system components (remote control / modification of parameters of measuring devices, switching on and off public lighting and communications modems, with the appropriate administrative rights). - Provide access control to system components as well as periodic archiving (back up) of important data |  |  |  |
|  | Data collection and archiving Automatically collecting information from remote control and data acquisition units to be stored in the database. The data collected automatically at a predetermined schedule are: - Currents and voltages in phases - Instantaneous power per phase - Total expenditure for electricity metering point (monthly) - Timing of turning on / off lights |  |  |  |
|  | Reporting - Reports of the status and alarms - Statements relating to the management and parameterization - Communication status reports - Control reports |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Item name: Server Quantity: 1 |  |  |  |
|  | HARDWARE CONFIGURATION: minimum required for offered SCADA system to works with extended capacity of at least 100 Remote control and data acquisition units |  |  |  |
|  | SOFTWARE: installed operative system and all necessary software required for offered SCADA system to works with extended capacity of at least 100 Remote control and data acquisition units |  |  |  |
|  | MONITOR (LCD/LED TFT display; at least 27 inch; at least 300 cd/mp; contrast ratio at least 40.000:1; at least 5 ms response time) |  |  |  |
|  | AUXILARY EQUIPMENT: Depending on offered ITsolution all necessary auxiliary equipment should be delivered with server in order for offered SCADA system to work including communication equipment if necessary. |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Item name: Remote control and data acquisition unit Quantity: 34 |  |  |  |
|  | Support standard communication protocols using GSM / GPRS technology for transmitting data (such as TCP / IP over GPRS interface) for sending data, receiving commands and configuration parameters up to 200 public lights . |  |  |  |
|  | Historical data log that covers min 3 weeks. |  |  |  |
|  | The planned timing of turning on / off according to the astronomical calendar (time of sunset / sunrise each day according to geo position of transformer area for public lighting) or user request. |  |  |  |
|  | Switching device (NO contact) for turning on / off lights within the area of public lighting. |  |  |  |
|  | Measuring part - provides the necessary data for basic system functions (currents up to 60A, voltages, powermeter consumption). By collecting and analyzing these data, Supervisory control and data acquisition IT system provide a constant overview of the state of public lighting system. |  |  |  |
|  | Communication GSM/GPRS modem: provides communication between system components passing a measured data to acquisition system, as well as modification of system parameters. Communication with the server is initiated in following situations: - Scheduled for automatic answer - Alarm condition - Upon request from the server |  |  |  |