# VOLUME 3

# TECHNICAL SPECIFICATIONS

CONCISE TECHNICAL DESCRIPTION

А1 		Technical characteristic of object

Existing parcel  is located in Kusic, on Square of Martyrs nr.4. Kusic cad. parc. nr. 165, K.O. Kusic 1.

The object consist of space settled on two levels. It is placed as freely existing object. Ground floor contents rooms make to meet the needs to performing cultural manifestations and also an apartment for janitor. Ground floor contents room .In. the attic there is a  room where is equipment for cinema projector

А2	Technical description of solution

Object is an older date of build. During the exploitation of object is doing regular maintenance, but, during the passing time and growing old materials, it comes a need to investment maintenance. In someplace on the object, there is a lot of humidity  and  very big surface of deteriorate of the floor.  Also, object  hasn't  need characteristic according with existing approach of energy efficiency.

Works on investment maintenance are anticipated:

- Processing existing roof cover whit replacing deteriorate tiles and cleaning the tiles.

- Replacing deteriorate parts of roof construction and necessary reinforcement of some parts of roof,

- Replacing gutters and other sheet-metal part of the roof.

- Replacing carpentry on object.

- Replacing deteriorate of floor and processing a thermo isolation floor.

- Replacing deteriorate sanitary equipment.

- Replacing and production lowered ceilings.

- Reparation seats in show room.

- Processing interior and exterior walls.

- Replacing deteriorate  of electrical installation.

- Replacing deteriorate mirror.

- Rehabilitation of humidity.

With these works, object is ready for absolute functional condition and is ready for  undisturbed use.

1.6.1 Review surface of object

| LEGEND AREA ground floor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| br. | room |  | П(m2) |  | kind of floor |
| 1 | entrance hall |  | 31.84 |  | terrazzo |
| 2 | wardrobe |  | 8.16 |  | terrazzo |
| 3 | passage to hall |  | 6.35 |  | wooden floor |
| 4 | ticket office |  | 3.15 |  | wooden floor |
| 5 | memorial hall |  | 36.21 |  | wooden floor |
| 6 | corridor |  | 13.26 |  | terrazzo |
| 7 | office |  | 14.52 |  | woodenfloor |
| 8 | office |  | 12.94 |  | wooden floor |
| 9 | hall for meetings |  | 47.71 |  | wfloor |
| 10 | stairs to engine room |  | 8.52 |  | wooden floor |
| 11 | entrance in toillet |  | 9.63 |  | ceramics |
| 12 | room whith kitchen and dining room |  | 35.62 |  | wooden floor |
| 13 | manifestation hall |  | 226.78 |  | wooden floor |
| 14 | pantry |  | 8.37 |  | wooden floor |
| 15 | pantry |  | 8.38 |  | floor |
| 16 | hall |  | 12.25 |  | ceramics |
| 17 | anteroom |  | 8.62 |  | ceramics |
| 18 | anteroom |  | 8.64 |  | ceramics |
| 19 | toilet for people whith disabilities |  | 3.18 |  | ceramics |
| 20 | toilet |  | 1.32 |  | ceramics |
| 21 | toilet |  | 1.32 |  | ceramics |
| 22 | toilet for people whith disabilities |  | 3.28 |  | ceramics |
| 23 | toiletт |  | 1.32 |  | ceramics |
| 24 | toilet |  | 1.32 |  | ceramics |
|  | ΣP net = |  | 512.69 |  |  |
|  | Z P gross |  | 604.82 |  |  |
| LEGEND AREA- attic |  |  |  |  |  |
| br. | room | П(m2) |  | Kind of floor |  |
| 1 | Hall for cinema projector | 9.94 |  | wooden floor |  |
|  | ΣP net= | 9.94 |  |  |  |
|  | ΣP gross= | 14.12 |  |  |  |