Republic of Serbia

Municipality of Nova Crnja

Annex to Terms of Reference

for Services of preparation of

Technical Documentation

for construction of bicycle paths along state road IB-12

on section Nova Crnja – Srpska Crnja

Section 01222: Vojvoda Stepa –SRB/ROU Border (Srpska Crnja), length 10.005 km

Section 01223: Nova Crnja - Vojvoda Stepa, length 1.865 km

Chainage: from KM 264+967 to KM 276+837

Length: 11.87 km

Nova Crnja, October 2017

C O N T E N T S

1	BACKGROUND	2

1.1	Area and Scope	4

2	Objective	5

3	Scope of Services and Tasks	5

3.1	General Requirements	5

3.2	Design Basis	6

3.2.1	Preparation of cadastral-topographic maps	6

3.2.2	Study on Engineering, Geological and Geotechnical Surveys	6

3.3	Pavement Solution	6

3.4	Detailed construction design of bicycle path	7

3.4.1	Bill of Quantities (BoQ)	7

3.5	Design of Traffic Signage and Road Furniture	8

3.6	Design for Temporary Traffic Signage and Road Furniture for Traffic Management during Works Execution	9

4	Contents, Processing and Delivery of Technical Documentation	10

4.1	Contents of Technical Documentation	10

4.2	Processing and Submission of Technical Documentation	11

# BACKGROUND

The road network of AP Vojvodina consists of network of state roads, the framework of which are the pan-European corridors E-75 (border with Hungary – Novi Sad – Belgrade) and E-70 (border with Croatia – Belgrade – Vršac – border with Romania). The main role of these corridors on sections through Vojvodina is to provide a high-capacity and high-level service for expected traffic flows. In addition to these corridors, the developed network of public roads allows the functioning and future development of Vojvodina. One of the most important cross and circular links in the network is the state road IB number 12, which is one of the main roads in Vojvodina. This route connects the major economic regions and a number of urban centers in the country, such as Subotica, Sombor, Bačka Palanka, Novi Sad and Zrenjanin. At the same time, this road connects the Republic of Serbia with the countries in the region, and is most used road link between Croatia and Romania.

The municipality of Nova Crnja encompasses the border crossing Srpska Crnja. For this border crossing significantly higher crossing of trucks and buses is recorded compared to the border crossings Nakovo and Vrbica; there is also a higher number of crossings made by travelers and significantly greater exchange of goods between Serbia and Romania.

Picture 1-1 Position of the road IB-12 (blue color) and of sections covered by the ToR (pink color) in the network of the AP Vojvodina

The Municipality of Nova Crnja, in function of tourism development, primarily bike-tourism, and raising the level of traffic safety for cyclists and other road users, and taking into account the significant number of cyclists on the section between Nova Crnja and Srpska Crnja and especially the position of the European bicycle route Euro Velo 13 that passes through the municipality and village Srpska Crnja until the border crossing, plans to build bike paths and the Regional Centre for cycling. European bicycle routes Euro Velo 13, was laid along the national road IB number 12, from the intersection with state road IIA Order No. 104 to the border crossing Srpska Crnja. This cycling route, also known as the Iron Curtain Trail, follows an extraordinary idea of offering the cyclists a ride along the entire length of the former Iron Curtain (border between the former Eastern and Western blocs). From Nordkapp in Norway to the Black Sea, in a total length of about 8000km, the travelers not only ride their bikes and go through different landscapes, but also pass through the living history of our continent, introducing themselves to the artifacts that marked former European ominous division as well as to the values of present-day European cooperation and erasing of borders between peoples and countries. The length of EuroVelo 13 route through Serbia is around 900km and runs along the border between the former Yugoslavia and Eastern Bloc: from Subotica via Kikinda to Vršac and Bela Crkva (on this section the route runs through the territory of Romania, while the alternative route is in Serbia), through Đerdap Gorge to Kladovo and Negotin, proceeding to Zaječar, Knjaževac and Pirot, and finally via Stara Planina mountain to Dimitrovgrad and exit to Bulgaria. In 2014, within the Project of sustainable mobility in South-East Europe, and with the help and coordination of the CF, Slovakia, Slovenia, Hungary, Croatia, Serbia, Romania and Bulgaria worked together on the creation of all the necessary documents and studies required for the practical development of this route. For our country, among other things, the following documents were created:

- National Status Report, defining (after a thorough fieldwork) the final route of the trail and presenting the state of the roads through which it runs;

- National Action Plan – a list of measures required to eliminate the identified deficiencies in infrastructure, organization and tourist offer;

- The Feasibility Study, which precisely defines technical measures and the necessary funds to improve infrastructure, public transport of cyclists, sustainable tourist offer, etc.;

- National Marketing Plan – a set of measures and activities for the promotion of the route in the country and abroad

These documents make the basis for further development of EV13 in Serbia. In addition, thanks to the financial help of the Ministry of Trade, Tourism and Telecommunications, the Design for Traffic Signage for EV13 in Serbia has been made.

In order to improve traffic conditions, attracting of a large number of bike tourists from developed European countries, which will use the route EV13 in future and in order to develop cycling tourism in the Republic of Serbia, as well as to create conditions for safe traffic on the section of the state road IB-12 between the village of Nova Crnja and border crossing, the Municipality of Nova Crnja in cooperation with the Regional Centre for Social and Economic Development Banat and the Romanian municipality Jimbolia jointly applied for participation in the IPA Program of cross-border cooperation Romania – Serbia (IPA CBC RU - SRB) in order to receive funds for realization of the planned activities. For the needs of submission of the application, the Municipality of Nova Crnja prepared the Feasibility Study for construction of bicycle paths along the IB-12 State Road between Nova Crnja and Border Crossing Point Srpska Crnja in February, 2016.

In addition to construction of bike paths, the Municipality of Nova Crnja plans also reconstruction and adaptation of existing auxiliary building located on municipal land near the border crossing Srpska Crnja on the cadastral plot number 7212/1 K.O. Srpska Crnja.

After completion of works on reconstruction and adaptation existing auxiliary, it will be transformed into the Regional Centre for cycling which will make the conditions for receiving a large number of cyclists primarily bike-tourists using the European cycling routes Euro Velo 13. Plan of the Municipality of Nova Crnja is to provide the best conditions for rest along the route for the future tourists and to become a center of cycling tourism in the southern part of the cycling route Euro Velo 13.

Upon a final decision of the IPA Program of cross-border cooperation between Romania and Serbia (IPA CBC RU - SRB), the municipality of Srpska Crnja was granted the use of requested funds.

At this stage of realization of the project, it is necessary to produce the technical documentation for construction of the bicycle path. In addition, the municipality of Nova Crnja shall engage also the technical control performer who will be obliged to check the level of completeness and validity of design, level of responsiveness to the requirements of the ToR and compliance with legal and technical regulations.

## Area and Scope

The starting point of the state road IB number 12 is located in the intersection with Subotica bypass road (Y branch) and goes along the route Bajmok – Sombor – Odžaci – Bačka Palanka – Novi Sad – Zrenjanin – Žitište – Nova Crnja – Srpska Crnja border crossing (state border with Romania). The starting chainage km 0+000 of the road is in the intersection with Subotica bypass road (Y branch), while the ending point is at the chainage km 276+837 at Srpska Crnja border crossing.

The subject ToR determines the area to be covered by the technical documentation, and includes the following sections of the state road IB-12:

- Section 01222: Vojvoda Stepa –SRB/ROU border (Srpska Crnja), length 10.005 km

- Section 01223: Nova Crnja - Vojvoda Stepa, length 1.865 km

In accordance with the valid Reference System of State Road Network in the Republic of Serbia, the starting point covered by this design is at the exit of Nova Crnja settlement, at chainage km 264+967 and it ends at the node 1222 on the chainage km 276+837, which implies the total length of the section where the bicycle path is going to be constructed of 11.87 km. The exact start and end of the bicycle path shall be determined by the design.

A solution for the bicycle path should be prepared within the limits of the land plot belonging to the state road IB-12, which is in the ownership of the state, so to have property legal relations clearly defined. The procedure for the project realization shall follow the provisions of the Law on Planning and Construction of the Republic of Serbia.

Picture 1-2 Sections of the road IB-12 (blue color) covered by the ToR

# Objective

Objective of the ToR is preparation of the technical documentation which will cover the solution for the bicycle path, which will create conditions for better traffic flow for vehicles and improvement of traffic safety conditions for all traffic participants on the section included in the design.

# Scope of Services and Tasks

The scope and contents of the technical documentation shall be harmonized with the Rulebook on the content, method and procedure for technical control of technical documentation according to the class and purpose of the structure ("Official Gazette of RS", No. 23/2015). The subject ToR covers the preparation of the following:

- Preliminary solution;

- Preliminary solution with the feasibility study;

- Detailed design for a building permit.

## General Requirements

While preparing the technical documentation, the Designer shall:

- obey the valid laws and regulations, standards and quality requirements for this type of the structure,

- collect the existing planning documentation and make the analysis of its impact on the design solution and inform the Client on the results of such analysis in advance to commencing the work on preparation of the technical documentation,

- perform geodetic measurements, surveys and laboratory testing needed for establishment of the design basis,

- comply with the remarks of the Technical Control in a manner and within the deadlines stipulated in the ToR and Contract.

## Design Basis

### Preparation of cadastral-topographic maps

The Designer shall be obliged to perform the geodetic surveys and prepare a relevant digital layout map with the topographic base (CTB) in a dwg file format which will be used for the design preparation. Cross-sections shall be surveyed at distances of max. 20 meters where the obligation shall be to survey pavement at least at three points on the cross-section. The information on property of the lots along the route of the bicycle path shall be taken from the cadaster. All expenses for performing the geodetic work and preparation of design basis shall be covered by the Designer.

### Study on Engineering, Geological and Geotechnical Surveys

As part of the surveys, the Designer shall perform relevant geo-mechanical surveys. It is necessary to have at least one trial pit excavated at every 500 meters. It is necessary to have the following soil mechanics testing for the materials taken from the trial pits:

- determination of grading composition of materials;

- determination of moisture content in the natural state;

- determination of maximum bulk density and optimum moisture per standard and modified Proctor's experiment;

- determination of consistency limits of materials;

- determination of soil bearing ratio (CBR).

Based on the geo-technical surveys, the relevant study shall be prepared. The study should include all results of the geological and geo-technical surveys presented in form of tables, graphic charts and diagrams. The relevant attachments in the layout maps shall include the information on all locations and types of the completed surveys.

## Pavement Solution

Depending on the results of soil mechanics, it shall be necessary to have a relevant solution for pavement on the bicycle path prepared. The Designer shall propose at least three options for pavement which shall be presented to the Client. The Client shall issue an approval for the accepted solution. Pavement solution shall be included as part of the structural design (construction part of the design).

## Detailed construction design of bicycle path

Based on the Terms of Reference, geodetic works and pavement solution, detailed construction design for the bicycle path shall stipulate the following:

- elements of layout, longitudinal profiles and cross-sections (radii of horizontal and vertical curves, deviation angles, longitudinal and transverse slopes etc.)

- solution for bicycle path draining;

- levelling solutions of the existing supporting facilities (accesses of other roads, bus stops, gas stations and parking lots) within the right of way of the designed section

All solutions shall be given within the existing right of way to avoid expropriation of new land.

The Designer shall identify the installations that cannot be kept at the current location, i.e. that need to be relocated due to the execution of the designed works. The Designer shall inform the Client on presence of such installations in writing immediately upon their identification.

Within the detailed design, the Designer shall identify and describe the construction technology of designed works, as well as all other aspects typical for safe execution of works, preservation of structures and environmental protection during the execution of works.

When deciding on requirements for quality of construction materials, special attention shall be paid primarily to quality of the final product, but also to the properties of materials from the domestic market, and a locality, whenever possible, under condition that quality of designed solution is not endangered.

While deciding on technology, special attention should be paid to the minimum of technical criteria needed for appliance of a certain technology.

When selecting the technology for execution of works, particular attention shall be paid to the fact that the works are to be executed under traffic flow on the subject section.

### Bill of Quantities (BoQ)

Bill of Quantities should provide all necessary information for comprehending the scope of works and enable efficient and accurate preparation of a bid for execution of the works.

The BoQ shall be prepared in a form of table.

The items of works within the BoQ should be organized in a way to provide a clear division to particular groups and subgroups of works according to the location /section. By its form and content, the Bill of Quantities must be clear, comprehensible and simple.

All works encompassed within the design must be contained in the items of works in the BoQ.

The BoQ should be provided separately for three stages of the works execution. For the first phase, it is necessary to plan the construction of the bicycle path on the section 01223 from the intersection with the state road IIA number 104 to the entrance to Srpska Crnja settlement. For the second stage, it is necessary to plan construction of the bicycle path on the section number 01223 from the entrance to Srpska Crnja settlement to the border crossing. The third stage should include the construction of the bicycle path on the section 01222 from the exit from Nova Crnja settlement to the intersection with the state road IIA number 104.

## Design of Traffic Signage and Road Furniture

Based on the field inspection of the subject section of the state road, the existing condition of spatial and physical structure in the right of way should be determined, as well as other elements significant for traffic safety.

Designer of traffic signage and road furniture shall take over from the designer of the detailed construction design all completed plans and shall perform a check of the solutions which would refer to the following:

- driving-dynamic and visual characteristics of the section,

- maximum speed of vehicles in curves,

- internal conformity and dynamic homogeneity of elements of road alignment,

- traffic safety for all participants,

- determination of required stopping visibility, available and overtaking visibility etc.

While designing, special consideration shall be given to all interventions stated by the construction design, as well as to solutions resulting from the traffic safety analysis.

As part of the design for traffic signage and road furniture, the Designer shall anticipate and include all necessary solutions for horizontal and vertical signage (traffic signs, sign girders, sign support poles and calculation of foundation) and road furniture.

Proposed traffic solution and proposed traffic signage and furniture must be in agreement with the category of the subject section. Dimensions, quality of materials and durability of traffic signage and road furniture shall also comply with the category of the subject road and shall be based on the SRPS-EN standards and valid Rulebook on traffic signage ("Official Gazette RS" No. 134/2014).

Plan of horizontal and vertical signage and road furniture within the detailed design shall be presented through graphical attachments that include chainages and levels needed for positioning and placement/incorporation of signage and road furniture in the field. Chainage of traffic signs must be in compliance with the construction part of the detailed design.

The Designer is obligated to justify specific solutions separately within the Technical Report.

The Technical Report shall include:

- description of the location, the most important features and description of the proposed traffic solution,

- description of proposed elements of traffic signage and road furniture,

- other data and justifications of the Designer that are of relevance for elaborated and proposed traffic solution.

Detailed design for traffic signage and road furniture shall mandatorily include BoQ. Items for dismantling the existing elements of traffic signs and road furniture which are in disagreement with the designed solution and which need to be removed shall be presented as part of BoQ. While preparing the priced BoQ, the actual market unit prices shall be used.

For elements of traffic signs that are standardized, marking stated by Serbian standards shall be applied.

All descriptions of the items shall be mentioned in the Specifications and must be precisely defined and harmonized with details on drawings. Same item shall not be described differently on different places in the design, i.e. data in the quantity of the Works, Specifications and on drawings must be identical.

As part of the attachment in details of the detailed design, contents that support a precise and quality incorporation of the proposed traffic signage and road furniture should be presented. Graphical drawings and detail drawings should be presented as magnified, in the appropriate scale for that type of presentation.

All relevant technical details and descriptions that enable ordering of traffic signage and road furniture must be presented in the Specifications, without favoring of particular suppliers and their products.

## Design for Temporary Traffic Signage and Road Furniture for Traffic Management during Works Execution

A design for temporary traffic signage and road furniture shall provide for specific organization and traffic management on road sections where disturbances occur in normal traffic flow as a result of the construction works.

The proposed solutions for traffic signage and road furniture should be in agreement with category of the road and should ensure undisturbed and safe traffic flow on the road section where the Works are being performed. These should also guarantee full safety of all traffic participants and workers in the area of the Works performance. The designed solution for traffic management during the Works execution shall take into consideration the traffic loading and spatial and temporal distribution of traffic flows on the subject section.

Special attention during preparation of the design for traffic signage during works execution shall be paid to vulnerable traffic participants and to accessibility to the facilities alongside the road.

The prepared traffic management plan shall contain a layout of traffic signage on the subject road section in typical form of appropriate scale with the analysis of impact of the road works on reduction of the road capacity and increase of travel time due to narrowing or partial closure of the section. Layouts shall present the existing traffic signage and manner of their treatment in accordance with the designed solution.

Depending on technology of the Works execution and proposed designed solution, appropriate quantities of traffic signage and road furniture during the Works execution should be planned and designed.

For marking of the road works, especially during night and under conditions of reduced visibility (fog, dust, rain etc.), the design should include blinkers and strobe lights. On sub-sections where due to the applied work technology (milling of asphalt), the horizontal signage is temporarily missing, the design should include temporary horizontal signage for safe traffic management.

Designed solutions should be prepared in accordance with valid standards and "Technical guidelines for marking the work zones in maintenance of state roads in the Republic of Serbia" Belgrade 2016. The use this source and consultations with the Client do not release the Designer from full responsibility for quality of designed solutions.

Characteristics of elements of vertical signage shall be applied in accordance with valid SRPS-EN standards. Materials for manufacturing of traffic signs shall possess reflective properties required by the Rulebook on traffic signage ("Official Gazette RS" No. 134/2014) and shall correspond to the class of the road.

In addition to description of the problem, limitations, description of proposed solution, traffic-technical calculation and expected effects on the section, the technical report should also include description of locations that require a special treatment from the traffic safety aspect (school zones, bus stops, locations with the increased intensity of pedestrian movement etc.).

Contents that are in accordance with the defined manner of placement and that support quality and precise construction of the proposed signage should be presented in the attachment in Details. Graphical presentations and drawings of details should be treated in an increased scale appropriate for that type of presentation.

As part of the attachment on measures for safety at work, it is also necessary to define precisely the current and potential risks that may appear during the Work execution on placement of traffic signage and road furniture and road use. Based on the findings, appropriate measures for avoiding the identified dangers should be proposed.

# Contents, Processing and Delivery of Technical Documentation

## Contents of Technical Documentation

Scope and contents of the technical documentation shall be harmonized with the Rulebook on the content, method and procedure for technical control of technical documentation according to the class and purpose of the structure ("Official Gazette of RS", No. 23/2015), and depending on the type of the technical documentation (preliminary solution, preliminary solution with a feasibility study or detailed design).

## Processing and Submission of Technical Documentation

The technical documentation shall be prepared, bounded and submitted to the Client in hard and soft copies and shall contain the relevant textual, graphical and numerical attachments.

The technical documentation shall be prepared in Serbian and in English.

Technical documentation shall be delivered to the Client in 3 (three) copies with the cover letter, which, in addition to basic data on the design and Designer, shall include an identification of the contract. A copy implies a printed version of the design and its digital version on a CD of appropriate capacity.

While processing the technical documents, the following shall be met:

- e-form of technical documentation shall include all source files (unprotected e-documents in form of program used for preparation and processing) and unprotected pdf form. Both e-forms shall be identical;

- e-form of technical documentation shall be fully compliant with the printed copy;

- text of technical documentation shall be written by UNICODE font. No YUTTF fonts are allowed;

- document processing (text, graphics, vector, grids) shall imply use of generally accepted and available commercial programs (such as applications from package MS Office, Auto CAD, etc.). Forms of these documents shall be compatible with older versions of those programs;

- if the Designer, for processing of documents, uses non-standard programs which are not available to the Client, the Designer shall, with previous agreement with the Client, convert all documents into formats available to the Client before the final processing of the technical documentation;

- e-documents are named in accordance with the contents of design, logically and in full title or associatively if use of the full title requires the use of big number of characters. The title of the documents shall be written in Latin letters. The title shall not include special characters and letters characteristic for Serbian letters. It is not allowed to specify the file title in Cyrillic;

- documents including contents in English shall bear the title in English, while documents containing contents in Serbian shall have the titles in Serbian. Bilingual documents shall have a Serbian title with a unique postfix mark SE to indicate bilingual form;

- cover of a CD that contains the e-documents shall bear the name of Designer, title of technical documentation and date, where the date represents the month of verification of design by the technical control. The text on the cover shall be printed electronically. Also, the disc shall bear legible title of the technical documentation and the Designer;

- textual parts of design shall be processed, printed and packed in hard covers in A4 format;

- graphical attachments shall be processed and printed in the appropriate format A1/A3 and bound in A4 format; and

- printed copies shall be bound in hard covers, verified and sealed in accordance with the valid regulations.