# VOLUME 3

# TECHNICAL SPECIFICATIONS

GENERAL DATA ON THE STRUCTURE AND LOCATION

| Type of structure: | Freestanding bulding |  |
| --- | --- | --- |
| Type of works: | Adaptation |  |
| Building category: | V |  |
| Classification of particular parts of the building: | Part in the total surface of the object (%): | Classification mark: Meeting halls and multi-purpose halls which are mostly used for cultural and artistic activities and fun |
|  | 100% | 126102 |
| Name of the spatial, i.e. urbanism plan: | Spatial plan of the Municipality of Coka (“Official Gazette of the Municipality of Coka” No. 11/13) |  |
| Place: | Banatski Monostor |  |
| Number of the cadastral plot/ list of cadastral plots and the cadastral municipality | Cadastral plot1518, Cadastral Municipality Crna Bara |  |
| Number of the cadastral plot/ list of cadastral plots and the cadastral municipalities that the infrastructure connections cross | - |  |
| Number of the cadastral plot/ list of cadastral plots and the cadastral municipalities which the public transportation routes cross | - |  |

| INFRASTRUCTURE CONNECTIONS: |  |
| --- | --- |
| Connection to electric power: | Designed |
| Connection to the water works: | Designed |
| Connection to the sewer: | Designed |

BASIC DATA ON THE BUILDING AND LOCATION:

| Dimensions: | Total surface of the plot/the plot: | 1087 m2 |
| --- | --- | --- |
|  | Total gross building area above ground level | 197.35 m2 |
|  | Total gross surface | 197.35 m2 |
|  | Total net surface | 152.70 m2 |
|  | Surface area of the ground level | 152.70 m2 |
|  | Surface of the land beneath the building/occupation: | 197.35 m2 |
|  | Floors (above and below ground) | Ground level |
|  | Height of the building (range, ridge, back floor, etc.) according to locational conditions: | 8.36 m |
|  | Absolute altitude angle (range, ridge, back floor, etc.) according to locational conditions. | - |
|  | Floor height: | 3.90 m |
|  | Number of functional units/No. of apartments | 1 |
|  | Number of parking spaces | - |
| Materialisation of the building: | Facade material | Plastered – painted |
|  | Ridge orientation: | West – east |
|  | Roof slant | 45 degrees |
|  | Roof material | Pepper tile |
| Percentage of green surfaces: |  | 81.85% |
| Occupation index: |  | 18.15% |
| Building index: |  | 0.18% |
| Other characteristics of the building: |  |  |
| Estimated value of the building | 2,815,653.50 RSD (No TAV) |  |

0.8. CONCISE TECHNICAL DESCRIPTION

Location:

There is a ground level building of the Community Centre in Banatski Monostor, Petefi Sandor Street, No. 33, on cadastral plot No. 1518, cadastral municipality Crna Bara as shown in the situation drawing. The dimensions of the existing building are 25.53x7.73 m. The main entrance is located on the front side of the building with an additionally built eaves.

Functional solution:

Based on the design assignment of the investor, the existing functional layout of the rooms will stay unchanged. By forming the new sanitary block in the front area of the office the building comfort will be raised to a contemporary, high level. The surface of the basic functional units of the Community Centre is shown in tabular form.

Totalusablenet surface:

- Performance hall			53.60 m2

- Theatre stage			18.12 m2

- Kitchen				29.00 m2

- Open hallway			 9.08 m2

- Front area and toilette		 8.15 m2

- Office				16.88 m2

- Summer kitchen			11.27 m2

- Warehouse				 6.45 m2

_____________________________________

Total:					152.55 m2

Existing state of the construction:

The construction of the object is classical, the foundation is a ribbon one out of full brick, with the assumption that the angle of foundation is -0.8 m in relation to the angle of the terrain. The building was built in 1908 and has been reconstructed multiple times.

The outside, bearing walls are load walls, partly made of brick with hydro-isolation at the floor level. The width of the bearing walls with plaster on both sides is d=55 cm. All walls and ceilings are plastered. The kitchen walls are covered with ceramic tiles. The outside wall plaster is falling off both outside and inside at the height of approximately 0.80 m as a result of humidity in the poorly made hydro-isolation, i.e. the flow of atmospheric water into the foundation.

The interior height of the rooms totals between 3.60 – 3.70 m.

The existing attic construction is wooden (karatavan form), which leans on the outside walls. The attic shows slight consequences of leakage.

The roof construction is a two way, wooden construction, covered in pepper tile. The roof slant is approximately 40 degrees. The roof also contains a horizontal gutter made out of galvanized sheet metal.

Construction and processing:

A refurbishing of the roof construction is in plan after removing the roof cover and the roof laths in places where the roof sagged. All rotten and deformed parts of the construction should be replaced by new, 1st class fir build. The removed roof tiles is to be cleaned and re-set on the roof, with the damaged and broken tiles being replaced.

The annexed roof structure over the summer kitchen and the warehouse is thermally 
isolated with mineral wool, d = 10 cm, between the rafters and lining up 1st class fir baseboards, which are then glaze painted three times.

The places where cracks and leakage are found in the ceiling will beinspected and the wooden structure will be searched and strengthened at the joists as well as other structural elements above the large building hall.

The setting of a lowered ceiling is projected to be done beneath the existing ceiling of the large hall that is deformed – bent by setting drywall on a metal sub-construction. It will all be done in accordance with the manufacturer’s instructions. The ceiling will be painted over twice after the bandaging and smoothing using dispersive paint.

The wall isolation should be approached after the plaster is taken down from all the walls on the inside and outside at the place where moisture reached an average height of 0.80 m. The large performance hall is projected to have the baseboards and plaster removed from the full length of the inside walls.

The hydro-isolation of the inside and outside walls, d=50 cm and 35 cm, should be done in a classical manner by underbuilding using brick. The Fimflex 4 mm strips with appropriate overlaps at the seams should be used for the hydro-isolation of the walls and floors.

Building of the new internal wall of the sanitary block is projected to be full brick, plastered using whitewash R = 1:3 in two layers, and the smoothed and painted using dispersive paint.

The toilette walls and floor are to be covered using whitewash tiles. The choice of ceramic tiles and layout should be done in accordance with the designs.

Plaster from all inside walls of the large hall should be removed and again plastered using whitewash R = 1:3 in two layers. All newly plastered surfaces are smoothed and painted using dispersive paint. Old walls and ceilings are smoothed and painted after the removal of the old paint. The outside walls are painted using dispersive façade paint twice in a colour chosen by the designer.

The large hall walls are to be covered using 1st class fir in the height of h = 130 cm mounted on the wooden sub-construction. The covering is then painted three times using glaze paint.

The decking is painted using oil based paint and is lacquered using semi-matte lacquer in a colour chosen by the designer.

The existing windows in the large hall and the entrance are replaced by ne wooden windows and doors that have the same dimensions and shape as the existing ones. The window panes will be thermo-pan glass 4+16+4 with the standard mounting for this type of joinery. The joinery specifications are provided in the graphic details.

The entrance and windows are made of 1st class laminated fir build that was ultimately painted with a glaze paint three times in accordance with the joinery schematics.

The existing windows on the remaining rooms are painted three times using glaze paint.

The inside door of the sanitation block are made out of double plywood and finally treated.

Water-works and sewage:

The internal water-works network of the new toilette is projected to be done using FLUIDTERM water pipes, manufactured by Pestan, Arandjelovac, PN 20 Bara, and D20 with the needed additional pieces, fixed and slide holders and valves in the recesses of the walls. Everything should be done in accordance with the manufacturers’ rules and instructions. The connection should be done at the closest known location of the water-works.

The sewage is projected to be made of PVC vinidurit sewage pipes with all the needed additional pieces. The building should be connected to the closest sewage shaft (septic tank) of the outhouse. The diameter of the attached pipe should be 160.

The sanitation elements are projected to be 1st class white ceramics in accordance with the SRPS standards chosen by the designer.

A low placed 10 l boiler for the purpose of supplying the sanitation block with warm water.

All works should be performed in accordance with the acting technical regulations and standards for the purpose of conducting this type of works, using quality material and a professional work force.

1-ARCHITECTURE

GENERAL DOCUMENTATION

1.2. CONTENT

| 1.1 | Main page of the Main book |
| --- | --- |
| 1.2 | Content of the Main Book |
| 1.3 | Decision on appointment of the Chief Designer |
| 1.4 | Chief Designer Statement |
| 1.5 | Textual documentation |
| 1.6 | Numerical documentation |
| 1.7 | Graphical documentation |

TEXTUAL DOCUMENTATION

Investor:	Municipality of Coka

Building:	Adaptation of the Community Centre – Banatski Monostor

Place of work:	Banatski Monostor, Petefi Sandor Street, No. 33.

TECHNICAL DESCRIPTION

Location:

There is a ground level building of the Community Centre inn Banatski Monostor, Petefi Sandor Street, No. 33, on cadastral plot No. 1518, cadastral municipality Crna Bara as shown in the situation drawing. The dimensions of the existing building are 25.53x7.73 m. The main entrance is located on the front side of the building with an additionally built eaves.

Functional solution:

Based on the design assignment of the investor, the existing functional layout of the rooms will stay unchanged. By forming the new sanitary block in the front area of the office the building comfort will be raised to a contemporary, high level. The surface of the basic functional units of the Community Centre is shown in tabular form.

Total usable net surface:

- Performance hall			53.60 m2

- Theatre stage			18.12 m2

- Kitchen				29.00 m2

- Open hallway			 9.08 m2

- Front area and toilette		 8.15 m2

- Office				16.88 m2

- Summer kitchen			11.27 m2

- Warehouse				 6.45 m2

_____________________________________

Total:					152.55 m2

Existing state of the construction:

The construction of the object is classical, the foundation is a ribbon one out of full brick, with the assumption that the angle of foundation is -0.8 m in relation to the angle of the terrain. The building was built in 1908 and has been reconstructed multiple times.

The outside, bearing walls are load walls, partly made of brick with hydro-isolation at the floor level. The width of the bearing walls with plaster on both sides is d=55 cm. All walls and ceilings are plastered. The kitchen walls are covered with ceramic tiles. The outside wall plaster is falling off both outside and inside at the height of approximately 0.80 m as a result of humidity in the poorly made hydro-isolation, i.e. the flow of atmospheric water into the foundation.

The interior height of the rooms totals between 3.60 – 3.70 m.

The existing attic construction is wooden (karatavan form), which leans on the outside walls. The attic shows slight consequences of leakage.

The roof construction is a two way, wooden construction, covered in pepper tile. The roof slant is approximately 40 degrees. The roof also contains a horizontal gutter made out of galvanized sheet metal.

Construction and processing:

A refurbishing of the roof construction is in plan after removing the roof cover and the roof laths in places where the roof sagged. All rotten and deformed parts of the construction should be replaced by new, 1st class fir build. The removed roof tiles is to be cleaned and re-set on the roof, with the damaged and broken tiles being replaced.

The annexed roof structure over the summer kitchen and the warehouse is thermally 
isolated with mineral wool, d = 10 cm, between the rafters and lining up 1st class fir baseboards, which are then glaze painted three times.

The places where cracks and leakage are found in the ceiling will beinspected and the wooden structure will be searched and strengthened at the joists as well as other structural elements above the large building hall.

The setting of a lowered ceiling is projected to be done beneath the existing ceiling of the large hall that is deformed – bent by setting drywall on a metal sub-construction. It will all be done in accordance with the manufacturer’s instructions. The ceiling will be painted over twice after the bandaging and smoothing using dispersive paint.

The wall isolation should be approached after the plaster is taken down from all the walls on the inside and outside at the place where moisture reached an average height of 0.80 m. The large performance hall is projected to have the baseboards and plaster removed from the full length of the inside walls.

The hydro-isolation of the inside and outside walls, d=50 cm and 35 cm, should be done in a classical manner by underbuilding using brick. The Fimflex 4 mm strips with appropriate overlaps at the seams should be used for the hydro-isolation of the walls and floors.

Building of the new internal wall of the sanitary block is projected to be full brick, plastered using whitewash R = 1:3 in two layers, and the smoothed and painted using dispersive paint.

The toilette walls and floor are to be covered using whitewash tiles. The choice of ceramic tiles and layout should be done in accordance with the designs.

Plaster from all inside walls of the large hall should be removed and again plastered using whitewash R = 1:3 in two layers. All newly plastered surfaces are smoothed and painted using dispersive paint. Old walls and ceilings are smoothed and painted after the removal of the old paint. The outside walls are painted using dispersive façade paint twice in a colour chosen by the designer.

The large hall walls are to be covered using 1st class fir in the height of h = 130 cm mounted on the wooden sub-construction. The covering is then painted three times using glaze paint.

The decking is painted using oil based paint and is lacquered using semi-matte lacquer in a colour chosen by the designer.

The existing windows in the large hall and the entrance are replaced by ne wooden windows and doors that have the same dimensions and shape as the existing ones. The window panes will be thermo-pan glass 4+16+4 with the standard mounting for this type of joinery. The joinery specifications are provided in the graphic details.

The entrance and windows are made of 1st class laminated fir build that was ultimately painted with a glaze paint three times in accordance with the joinery schematics.

The existing windows on the remaining rooms are painted three times using glaze paint.

The inside door of the sanitation block are made out of double plywood and finally treated.

| NUMERICAL DOCUMENTATION |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Community Centre – Banatski Monostor |  |  |  |
| No. | Name | m2 | Floor |  |
| 1. | Hall | 53.60 | Decking |  |
| 2. | Stage | 18.12 | Decking |  |
| 3. | Kitchen | 29.00 | Ceramic tile |  |
| 4. | Hallway | 9.08 | Concrete |  |
| 5. | Front area and toilette | 8.15 | Ceramic tile |  |
| 6. | Office | 16.88 | Ceramic tile |  |
| 7. | Summer kitchen | 11.27 | Concrete |  |
| 8. | Warehouse | 6.45 | Concrete |  |
|  | Total: | 152.55 |  |  |

Bill of Quantities

Object: Cultural Centre – Banatski Monostor

Investor: Municipality of Coka

The price should include all the necessary working conditions foreseen by the construction standards. General note: this subject encompasses the entirety of labour, first class material, waste, amortization, small auxiliary materials.

|  |  |  |  |  |  |  |  |  |  | Measure |  |  |  | Quantity |  | Unit price din. |  | Unit price € |  | Total € |  | Total din. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | A. DEMOLITION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A 01 |  | Extraction of joinery with loading and shipping to a place determined by the investor, up to 5 km. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Entrance - dimensions 155/230 cm |  |  |  |  |  |  |  | piece |  |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  | Window - dimensions 130/140+30 cm |  |  |  |  |  |  |  | piece |  |  |  | 4 |  |  |  |  |  |  |  |  |
| A 02 |  | Removal of existing pepper tiles and conduits from the roof of the object in places where there is damage. The price should include cleaning and storage until repurposed. |  |  |  |  |  |  |  | Square metres |  |  |  | 150 |  |  |  |  |  |  |  |  |
| A 03 |  | Removal of roof battens in damaged places. Price should include loading and shipping of construction materials to a place determined by the investor, up to 5 km. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Batten |  |  |  |  |  |  |  | Square metres |  |  |  | 150 |  |  |  |  |  |  |  |  |
| A 04 |  | Swingling of plaster of the inside and outside walls of the object in damp places in the average height of h-80 cm in reference to the floor. The plaster in the performance hall is to be swingled fully. The price includes loading and shipping if waste to a landfill up to 5 km away |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Inside |  |  |  |  |  |  |  | m2 |  |  |  | 200 |  |  |  |  |  |  |  |  |
|  |  | Outside |  |  |  |  |  |  |  | m2 |  |  |  | 50 |  |  |  |  |  |  |  |  |
| A 05 |  | Dismantling the lining of the panelling from the inside walls of the object in the performance hall. The price includes the loading and shipping to a place determined by the investor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | m2 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| A 06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | In the wall 50 cm diameter |  |  |  |  |  |  |  | m3 |  |  |  | 0.4 |  |  |  |  |  |  |  |  |
| Total demolition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | B MASONRY WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B 00 | The construction must be clean and with proper connections. The built sections must be vertical. Glue or adhesive is applied to the entire surface. All works above 3.00 m are to be done using scaffolds |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B 01 | Building of the partition wall by using bricks in cement mortar R=1:3:9 d-12 cm. The price must include a ring beam in the height of the transom with dimensions 12x20 cm made of reinforced concrete MB20 |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 6 |  |  |  |  |  |  |  |
| B 02 | Underpinning of interior and exterior walls d-55 cm and d-35 cm with full brick, in cement mortar R=1:3:9 up to a height of cca. 50 cm in lamella of 1 m with the setting of hydro isolation made of film flex strips d- 4 mm with regulated overlaps. The price includes shipping of waste to a landfill up to 5 km away |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d- 55 cm |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 13.5 |  |  |  |  |  |  |  |
| B 03 | Plastering interior brick walls with lime mortar R=1:3 in two layers d-2cm with previously spraying the concrete parts with grout. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 12 |  |  |  |  |  |  |  |
| B 04 | Plastering of existing interior compacted walls where plaster was previously swingled from and in the performance hall with lime mortar R=1:3 in two layers d-2 cm. The price should include the placing of copper wire mesh. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 200 |  |  |  |  |  |  |  |
| B 05 | Plastering of exterior walls where mortar was previously swingled from with extended mortar R=1:3:9 in two layers d-2 cm. The price should include the placement of copper wire mesh. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 50 |  |  |  |  |  |  |  |
| B 06 | Repair of plastered exterior walls where damaged as well as around the newly installed windows and doors. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 15 |  |  |  |  |  |  |  |
| B 07 | Procurement and installation of pre-fabricated transom made of reinforced concrete MB-30, width 12 cm, height 20 cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Length 110 cm |  |  |  |  |  |  |  |  |  |  |  | piece |  | 4 |  |  |  |  |  |  |  |
| B 08 | Concreting the floor of the toilet with concrete MB20. d-10 cm. The price should include buffering sand and PVC foil for hydro isolation. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 2.5 |  |  |  |  |  |  |  |
|  | Covering the walls with I class ceramic tiles by gluing them on a prepared surface up to 2 metres height. The price includes grouting. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 12 |  |  |  |  |  |  |  |
|  | Covering the floors with I class ceramic tiles by gluing them on a prepared surface up to 2 metres height. The price includes grouting. |  |  |  |  |  |  |  |  |  |  |  | m2 |  | 2.5 |  |  |  |  |  |  |  |
|  |  |  |  |  | Total masonry works |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Measure | Quantity | Unit price din. | Unit price € | Total € | Total din. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C CARPENTRY WORK |  |  |  |  |  |  |  |
| C 01 | Search of the gabled roof structure with the substitution of deformed and obsolete elements. The price should include the substitution of obsolete construction elements made from I class cut fir build cca. 20% |  |  |  |  |  |  |  |  |
|  | Calculation per horizontal projection |  |  | m2 | 193 |  |  |  |  |
| C 02 | Battening of the roof for covering with pepper tiles in areas where there is damage |  |  |  |  |  |  |  |  |
|  | Dimensions of the roof batten 2.5/5 cm |  |  | m2 | 150 |  |  |  |  |
| C 03 | Search of the wooden garret construction with the substitution and reinforcement of rotten and deformed parts with hewed fir build. |  |  | Lump sum | 1 |  |  |  |  |
| C 04 | Procurement and installation of first class wooden fir carpentry on the performance hall wall up to a height of h=130 cm on the necessary sub-construction (wooden planks) as well as the necessary upper trim, painted with enamel paint three times. |  |  | m2 | 40 |  |  |  |  |
| Total carpentry works |  |  |  |  |  |  |  |  | hh4 |

|  |  |  |  | Measure | Quantity | Unit price din. | Unit price € | Total € |  | Total din. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D ROOFING WORKS |  |  |  |  |  |  |  |  |  |  |
| D 01 | Roofing with old, scrubbed pepper tile |  |  | m2 | 135 |  |  |  |  |  |  |  |
| D 02 | Procurement of old pepper tiles (identical to the existing ones) and roofing |  |  | m2 | 15 |  |  |  |  |  |  |  |
| D 03 | Roofing of the ridge with old, cleaned ridge tiles where damaged |  |  | m2 | 24 |  |  |  |  |  |  |  |
| Total roofing works |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Measure | Quantity | Unit price din. | Unit price € | Total € | Total din. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | E CONSTRUCTION JOINERY |  |  |  |  |  |  |  |
| E 00 | All joinery should be made by an experienced work force according to specs, i.e. the joinery patterns, and according to measures taken from the location with all necessary and additional material |  |  |  |  |  |  |  |  |
| E 01 | Manufacturing and installation of double wing windows made from l. kl. Enamelled fir, with double pane glass filled with argon 4+16+4 mm together with a window board. The windows come equipped with an Eslinger blinds from thermo isolated aluminium profiles. The price should foresee a final paint job with enamel three times. In everything according to joinery position. |  |  |  |  |  |  |  |  |
|  | Pos 1-130/140+30 cm - double wing |  |  | piece | 4 |  |  |  |  |
|  | Pos 2-60/60 cm - with a mosquito netting |  |  | piece | 1 |  |  |  |  |
| E 02 | Manufacturing and installation of double wing doors made from 1st class enamelled fir with covers in the fields, a jamb and the necessary edging. |  |  |  |  |  |  |  |  |
|  | Pos 5 - 155/230 cm - double wing, width of the jamb 50 cm |  |  | piece | 1 |  |  |  |  |
| E 03 | Manufacturing and installation of one wing, double-plied doors with a jamb and made from 1st class Fir wood. The price should include the final coating with polyurethane lac, semi matt glow. All according to joinery position. |  |  |  |  |  |  |  |  |
|  | Dimension 80/210 cm - full, width of the jamb 12 cm |  |  | piece | 1 |  |  |  |  |
| Total construction joinery |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Measure | Quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F PAINTING WORKS |  |  |  |  |  |  |  |
| F 00 | Work must be done consciously and superbly. The painted surfaces cannot show brush strokes, the tone must be even. For all high works mobile scaffolding, must be prepared. |  |  |  |  |  |  |  |  |
| F 01 | The painting of interior mortar walls with a semi-dispersive paint twice with smoothing and all necessary prep work. The price includes removal of old paint. Colour to be chosen by the investor. |  |  |  |  |  |  |  |  |
|  | Newly plastered walls |  |  | m2 | 255 |  |  |  |  |
|  | Existing walls |  |  | m2 | 135 |  |  |  |  |
| F 02 | Painting of exterior walls with dispersive paint two times with all necessary prep work. Colour to be chosen by the investor |  |  | m2 | 280 |  |  |  |  |
| F 03 | Painting of the gypsum cardboard ceiling with a semi-dispersive paint two times with smoothing and all necessary prep work. Colour to be chosen by the investor. |  |  | m2 | 73 |  |  |  |  |
| F 04 | Painting of the existing ceiling with a semi-dispersive paint two times with all necessary prep work. The price includes the removal of old paint. Colour to be chosen by the investor. |  |  | m2 | 56 |  |  |  |  |
| F 05 | Painting of the existing decking with oil paint and finishing lacquer with a semi-matt lac with all necessary prep work. Colour to be chosen by the investor. |  |  | m2 | 73 |  |  |  |  |
| F 06 | Painting of wooden columns and the roof of the entrance covered with baseboards and facing boards with enamel paint three times with all necessary prep work. Colour to be chosen by the investor. |  |  | m2 | 25 |  |  |  |  |
| F 07 | Painting of existing wooden windows with enamel paint three times with all necessary prep work. |  |  |  |  |  |  |  |  |
|  | Dimensions 105/140 cm |  |  | pieces | 2 |  |  |  |  |
|  | Dimensions 105/140 cm |  |  | pieces | 2 |  |  |  |  |
|  | Dimensions 105/140 cm |  |  | pieces | 1 |  |  |  |  |
|  | Dimensions 105/140 cm |  |  | pieces | 1 |  |  |  |  |
| Total painting works |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Measure | Quantity | Unit price din. | Unit price € | Total € | Total din. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G Plaster works |  |  |  |  |  |  |  |
| G 00 | Plaster works are to be done by an expert work force. The price will include all necessary material, substructure, joint tapes and miscellaneous expendable material. Everything to be done as advised by the manufacturer Knauf or Rigips. |  |  |  |  |  |  |  |  |
| G 01 | Veneer of the slanted ceiling with l.kl. fir baseboards upon the needed substructure. . The price will include final painting of the baseboards with enamel products three times, the setting up of a PVC steam block and thermo isolation made from semi-hard rock wool in panels of d-100 mm. |  |  | m2 | 26 |  |  |  |  |
| G 02 | Procurement and installation of gypsum cardboard panels d-12.5 mm below the existing ceiling using a factory substructure made from zinc steel profiles. The lowered ceiling is installed at a height of +3.50 m in reference to the floor. The price will include joint tapes and smoothing of the joints and all to be done as advised by the manufacturer Knauf and Rigips. |  |  | m2 | 73 |  |  |  |  |
| Total plaster works |  |  |  |  |  |  |  |  |  |

|  |  | SUMMARY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Demolition |  |  |  |  |  |  |  |
| B | Masonry work |  |  |  |  |  |  |  |
| C | Carpentry work |  |  |  |  |  |  |  |
| D | Roofing works |  |  |  |  |  |  |  |
| E | Construction joinery |  |  |  |  |  |  |  |
| F | Painting works |  |  |  |  |  |  |  |
| G | Plaster works |  |  |  |  |  |  |  |
| I | Unplanned works |  |  |  |  |  |  |  |

GRAPHICAL DOCUMENTATION

3-HYDROENGINEERING INSTALLATIONS

General Documentation

TEXTUAL DOCUMENTATION

Investor: Municipality of Coka

Structure: Adaptation of the Community Centre – Banatski Monostor

Place of build: Banatski Monostor Petefi Sandor street no. 33

Technical description

Internal water network of the new toilets is projected from fluidterm water pipes manufacturer PEŠTAN Arandelovac PN 20 Bar, D20 with the necessary fittings, fixed and sliding brackets, valves together in the image of the walls. In accordance with the instructions and manufacturer's instructions. Connection should be performed at the nearest place of the existing water supply system.

Sewage is projected vinidurit PVC sewer pipes with all necessary fittings together. The building is to be connected to the closest manhole (septic tank) outhouse toilet. Smoke of the connecting pipes should be Ø160.

Sanitary fittings are predicted from white ceramics in everything according to SRPS standards optional investors.

Under the sink is envisaged to have a low mounted electrical water heater 10 lit. to supply the toilet with warm water.

All work should be carried out according to the applicable technical regulations and standards for this kind of work with quality materials and skilled labour.

NUMERICAL DOCUMENTATION

Bll of quantities – water supply and sewer

Object: Culture Club – BanatskiMonostor

Investor: Municipality of Coka

|  |  |  |  |  | Measure | Quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B EARTH WORKS |  |  |  |  |  |  |  |
| B 01 B 02 B 03 | Excavation of III class land with manual beautification for duct trenches with expulsion of land within 1 m from the edge of the excavation. Trench width is 40 cm. Procurement, delivery and deposition of sand to the bottom of the trench, below, next to and above the pipe. While depositing, the sand will be manually compacted into a layer of 30 cm. Total will be calculated per m2 of deposited sand. Depositing of earth into the canal trenches with compacting in layers of 20 cm, loading and transportation of excess earth to the landfill up to 5 km. Depositing Excess earth |  |  |  | m2 m2 m2 m2 | 7.00 2.50 4.50 2.50 |  |  |  |  |
|  | TOTAL LAND WORKS |  |  |  |  |  |  |  |  |  |
|  | R. WATERWORKS |  |  |  |  | Quantity | Unit price din. | Unit price € | Total € | Total din. |
| R 01 R 02 R 03 R 04 | Procurement of the necessary material, delivery and manufacturing of the water network from FLUIDTERM water pipes PN 20, made from raw PPR-Random, type 3 (DIN 8077-8078 and DIN 16962) Manufacturer: PESTAN Arandjelovac with all the necessary additional materials and jointing material. To be calculated by square meter upon completion. Cutting of pipes and waste material are not paid separately. To be calculated by square meter of piping. Cold water ᴓ15 Procurement, delivery and installation of the pass valves with nickel caps. To be calculated by piece upon completion ᴓ15 Connector to the existing water network. Testing using the starting water pressure of 8-10 bar, irrigation and disinfection of the water network. Lump sum to be calculated upon completion. The record and reports to be handed over to the investor upon technical acceptance. |  |  |  | m2 piece piece Lump sum | 13.00 1.00 1.00 1.00 |  |  |  |  |
|  | TOTAL WATERWORKS |  |  |  |  |  |  |  |  |  |
|  | S. SEWER |  |  |  |  | Quantity |  |  |  |  |
| S 01 S 02 | Procurement, delivery and installation of a PVC channel network with a drop of 15% (SRPS - EN 1401, SRPS-EN 13476), pipe type SDR 41 SN4 by "Pestan" Arandjelovac, along with all necessary amenities and sealing material, calculated by square meter upon completion. Horizontal distribution is maintained in the channel trenches, the pipes are laid in sand. The sewer vertical in installed in the wall grooves. ᴓ50 ᴓ75 ᴓ110 ᴓ160 Procurement, delivery and installation of floor drains with water smell blockage. Vertical drain and metal bar, type HL3100. ᴓ75 |  |  |  | m2 m2 m2 m2 piece | 1.00 2.00 1.00 15.00 1.00 |  |  |  |  |
| S03 | Connection of sewer to the existing septic tank |  |  |  | Lump sum | 1 |  |  |  |  |
| S 04 | Testing of the installation of the sewer on the tightness in accordance with the technical standards. |  |  |  | Lump sum | 1 |  |  |  |  |
|  | TOTAL SEWER |  |  |  |  |  |  |  |  |  |
|  | V. SANITARY WARE |  |  |  |  | Quantity |  |  |  |  |
| V 01 | Procurement and installation of the sink, domestic production I class ceramic, dimensions 60x40 cm. The sink is to be fastened to the wall using appropriate fixings and brass screws over rubber pads. The sink is to be connected with the drain by a chrome siphon, diameter 5/4" with a rosette, plug and chain. White colour. |  |  |  |  |  |  |  |  |  |
|  | Place a mirror next to the sink, dimensions 50x60 cm, liquid soap holder, paper towel holder. The sink and equipment to be ordered according to the investor’s choice. |  |  |  |  |  |  |  |  |  |
|  | Sink |  |  |  | piece | 1 |  |  |  |  |
|  | Mirror |  |  |  | piece | 1 |  |  |  |  |
|  | Paper towel holder |  |  |  | piece | 1 |  |  |  |  |
|  | Liquid soap holder |  |  |  | piece | 1 |  |  |  |  |
| V 02 | Procurement, delivery and installation of a standalone toilet, domestic production I class ceramic with lower outburst. The connection of the toilette with the sewer line is to be done with the appropriate putty so the seal is 100% closed. The toilette seat is to be fastened using rubber pads and brass screws. |  |  |  |  |  |  |  |  |  |
|  | Place a low standing water tank Geberit 3+6 litres. Connect to the water line through a chrome valve and quality hose and using a pipe and rubber cufflink to connect to the toilette. Place the cover made from powder-coated MDF or hard plastic. The bowl and equipment to be ordered according to the investor’s choice. |  |  |  |  |  |  |  |  |  |
|  | Price will include a cold water valve ᴓ15 |  |  |  |  |  |  |  |  |  |
|  | To be ordered according to the investors choice |  |  |  | piece | 1 |  |  |  |  |
| V 03 | Procurement, delivery and installation of a one hand, standalone sink mixer with flexible connectors and a faucet manufactured by Armal or alike. The sink mixers are to be delivered along with accompanying additional material. The price will include two corner valves for hot and cold water ᴓ15. |  |  |  |  |  |  |  |  |  |
|  | Sink mixers to be ordered according to the investors choice |  |  |  | piece | 1 |  |  |  |  |
| V 04 | Procurement and installation of an electric water heater, volume 10 l. A safety valve is to be delivered and installed along with the water heater as well as chrome tube connectors. Install the water heater and connect to a power source. Price will include a cold water valve ᴓ15. |  |  |  |  |  |  |  |  |  |
|  | Low standing 10 litres |  |  |  | piece | 1 |  |  |  |  |
|  | TOTAL SANITARY WARE |  |  |  |  |  |  |  |  |  |

| Z REMAINING WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Z 01 | Additional and unforeseen works 10% of the total value of works. |  |  |  |  |
| Total remaining works |  |  |  |  |  |

|  |  | SUMMARY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B | EARTH WORKS |  |  |  |  |  |  |
| R | WATERWORKS |  |  |  |  |  |  |
| S | SEWER |  |  |  |  |  |  |
| V | SANITARY WARE |  |  |  |  |  |  |
| Z | REMAINING WORKS |  |  |  |  |  |  |
|  | TOTAL |  |  |  |  |  |  |