# VOLUME 3

# TECHNICAL SPECIFICATIONS

TECHNICAL DESCRIPTION

# GENERAL INFORMATION ABOUT THE OBJECT:

The building scheduled for a Visitor centre is located in the open space of the entrance plateau next to the main pedestrian routes of the Big fortress. This facility should occur at the site where once there used to be the arsenal and barracks. In accordance with the overall concept of reconstruction silhouettes of Fetislam, which implies the need to renew those authentic historic buildings that have been destroyed within a long period of time, it is necessary to restitute buildings 1 and 2, which were demolished due to the construction of HPP Djeradap II. These were once arsenal and barracks

Disposition of the Visitor centre

Authentical look of the arsenal and the barracks in Fetislam

# CURRENT CONDITION

One of these buildings (No. 1), rectangular in shape, measuring 35.60 x 15.40 m, was built with quarry stone. In the longitudinal axis of the building there are two opposing inputs. The portal is 2.54 m wide, and the outer sides arc-shaped. There are two windows in massive outer walls on the shorter sides, and five windows in two rows on the longer ones. The total height of the building was about 7.50 m. An extremely nice roof construction is carried by 32 wooden pillars arranged in four rows, placed at the 4.20 m. The roof is gabled with a covering of hollow tiles. Based on the architectural features of the building it was used as arsenal and gunsmith of tophan.

The authentical look of the arsenal (building 1)

Authentic arsenal section (building 1)

Building No. 2 was partly well preserved, on the basis of which the relevant data was collected for the modern reconstruction. The dimension is 22.90 x 11.50 m, and it was also built of broken stone. Its entrance was done in the the same way as a portal of the building No.1. The roof structure is supported by asimetrically placed wooden poles. The roof is four-gabled and it’s also covered with tiles. Based on the preserved parts of the ceiling structure, made of sedges, it is concluded that it was a residential building. It is assumed that it dates from the same period as building No.1, primarily by means of construction, materials used and processing of architectural details.

# CONSERVATION OF RECONSTRUCTION AND REVITALIZATION OF OBJECT

Reconstruction of the destroyed buildings contributes to restoring of the authentic appearance, architectural and urban look of Fetislam. Because of its significant size and position that relies on the inner space, it is appropriate for the creation of the Visitor centre. It is a modern variant, similar to any world city hall in which wider social life ofthe city would be affirmed. According to its content it would be multifunctional, because the conditions for the conciliar gathering of people and the operation of various programs were created.

By combining these two facilities into one unit, the functions for these purposes would be provided. In accordance with the preserved documentation it is necessary to restore these buildings, at the authentic position, which was recorded within the cadastral base. In such a historical area without additional solutions, it is necessary to create the possibility for conducting of various topics on the experiences of the world city halls: the promotions of social, economic, cultural and other commercial and non-profitable activities. Primary activities of a such organized multifunctional space are:

- various mass programs as a part of activities at the main city square

- business promotions

- banquets

- various donor celebrations

- Film projections

- celebrating the New Year and Christmas Eve

- various chamber performances

- exhibitions, concerts, literary and poetry evenings, book fair

Productive’programme activities are facing a wide variety of target groups, primarily urban and regional needs, but also the wider area. These objects can be used for carrying out rich multimedia activities, business conferences, seminars, fairs, etc, animating a wide range of visitors. The impression would be even greater if their stay is connected to the housing at a national restaurant, in direct contact with these objects. Spacely observed, they a form part of a whole with the various objects at the central square, with which thea are compatible by the programme. In spatial terms they close the amphitheater form.

# Functional organization of the visitor centre

The Visitor Center consists of two historic buildings the arsenal and the barracks, which are connected by a new conjuncting element. The conjuncting part represents the entrance hall from which it enters into two multifunctional spaces. The halls also enter the basement, where a sanitary block is located. The main access is from the central fortress square. Entrance to the lobby is through wide doors. On the opposite side there is a passage that leads into the area of ​​the fortress wall. Object 1 is a universal hall with an auditorium. The floor is tilted and it is intended for programs of scenic and multimedia character. In addition to this, at the bottom there is a stage with all the requirements for installing more complex equipment since the technical level has been created under the stage space. In addition to the entrance to this space through the halls, there are two more projects that directly come from the market to the square or ramparts. Object 2 is a universal hall with a flat floor. Its form enables the maintenance of various perfomances, fairs, catering and anything that does not require specific scenic conditions.

The entrance hall is, by a rule, a communication space. However, with its surface and position it allows independent programs to be held in it as well as those that will complement the programs in the main spaces. It also comes down from the hall to the basement where the sanitary facilities are located. Through the connecting element, the public toilet is located in the basement of the building.

# CONSTRUSTIVE AND BUILDING PROCESSING OF THE OBJECT

The built parts of the building / construction

The lower walls of the ground floor are 65 cm thick and are constructed with a constructive wall, d= 30cm, of the 'YTONG' thermal insulation block or equivalent, TB30, 62,5x30x20 cm, Z7-Z9 and Z10-Z11 in mortar R 1:3:9, or in a special YTONG white thinlayered mortar for building or equivalent, under the system of block connections. Before building, clean the blocks well and sprinkle with water so that the mortar does not blaze. This should be carried out in accordance with the manufacturer's instructions and norms. Possible cuts for the serklingers enter the price of the building. Plastering of the interior surfaces of the walls of YTONG or equivalent and concrete is carried out with the extension mortar conditions of R=1:2:6 in two layers with the previously taken sprinkling with the thin cemented mortar of R=1:2.

# THERMICAL PROTECTION OF THE OBJECT:

Thermical protection of the walls of the building is provided with a basic wall from Ytong or equivalent through which stone slabs and stone blocks of different thickness are laid. Concrete basement walls are to be  covered with styrofoam and protected with concrete blocks. The construction of thermical protection of the attic space and the roof level is done by laying the hard pressed stone wool d =10,5cm (class A1). On the floor of the attic space, over the thermical insulation of the styrofoam d=10cm, the cement screed d =4cm is spread.

# Hydro isolation of the object :

The hydro isolation of the building is carried out within several different architectural constructions:

- The construction of horizontal and vertical insulation and the isolation of the basement and basement walls, is carried out over a concrete substrate which is formed just to make it sag. Hydroisolation is made of PVC membrane 'Sikaplan TU 9.6 or equivalent.' (unarmed plastified (unarmed plasticised polyvinyl chloride), d = 1.5 mm, or the product of another manufacturer of similar characteristics. The tape is installed by welding the joints in a sandwich machine made of two felt from geotextile (300-500gr / m2), welded with the protective-slip layer function

- Making horizontal rigid hydo isolation of floors in wet nodes, over a glossy concrete substrate. Hydro isolation - 'SikaTopSeal107' or equivalent - a two component, waterproof, polymer modified cement mortar, or a product of another manufacturer of similar characteristics. Apply with brush or glitter in 2-3 layers, in administrative directions

Inner processing:

Walls and ceilings in all rooms are plastered with an extension mortar and finally plastered with fine Danube sand and are coloured in the tone as selected by the designer. The ceilings of the sanitary unit are to be lowered partly, to mask the sewer pipes of the ventilation basement. The suspended layer of the ceiling is with a horizontal continuous view, with a covered grid made of galvanized steel profiles (CD 60/27) as prefabricated and bearing profiles, molded Knauf or equivalent anchors with beams with a wire for a.b. construction (fixing a wire with an elongated steel anchor nail for a.b. ceiling). Lowering height approx. 20 cm. The coating is from one layer of GKBI (impregnated) plates d = 12.5cm. The connection of the ceiling joint to the wall is to be taqken with a split tape. Processing of GK compounds in quality Q2. Cover staircase arms with slabs of belvedere sandstone in cement mortar. Size 1. Tile plates are of thickness d = 3cm, with falcated finish. Alongside the walls, make a tile of stone slabs at a height of 10cm. All visible surfaces must be smoothly milled. Cross section of stairs 17 / 30cm. The staircase fence is made of wooden profiles. From this material are poles and filling interstitial spaces. Protect the wooden construction of the enclosure with a shade of color. Make the handrail of the fence from massive oak boards 40/70 mm, protect it with a stain coating, and finish with a lacquer finish, everything according to design details.

Constructions covered with ceramic tiles

Inner walls and floors that are in contact with water, economic spaces, sanitary and production areas and the other alike ones are usually covered with ceramic tiles. Depending on the purpose of the space, the laying method and the type of coating are adapted to this

- Walls of the sanitary facilities of the staff and guests in the common spaces (wc). Walls are to be  covered with ceramic tiles I class, up to the wall height, according to the interior design. Tiling the tile on the joint using a layer of ceramic adhesive.

- Floors of the sanitary facilities are to be tiled with ceramic tiles I class with mat glazer. The types, sizes and colors of tiles are to be used according to the interior design. Apply ceramic tiles over a layer of ceramic tile adhesive, a joint on the joint.

Constructions of the floors

All floors are done as the swimming/floating ones. Through the reinforced concrete structure, take the layer of the floor of extruded polystyrene (styrofoam) 6cm of thickness, on the floor in the part of an elastic layer of floating floor. Styrofoam should be lifted up to the walls at the floor level in the same, or thickness of 1cm. The polystyrene must be fitted with a 'knap' plate to the plate without the smallest air space between the plates. Through the styrofoam, carry out easyarmed cement screed as a substrate for laying of floor coverings made of stone slabs. Place the stone with a minimum height of 10cm in the volume of the rooms.

Floors of ground floor, porch, and steps from stone slabs   :

In the entrance hall the use of slabs of belvedere sandstone is to be used. Place the sheds with the Belovodski sandstone, which is machined to a thickness of 2.5cm. The type and colors of the sandstone are to be used according to the designer's choice. Processing of panels is according to the project and details. Place the plates in a cement mortar, clean the joints and grooves. The same material should be covered with stair treads. The floors of the ground floor should also be made of slabs of  'Belovodski' sands or equivalent, dimensions 30x30x3 cm, according to the designer's choice. Place the plates in a cement mortar, scale 1:3, groove and clean, in the style according to the design and instructions of the designer.

Interior and exterior joinery

Exterior joinery is made in a new, authentic form, using modern standards, adapted to modern technology. Carpentry is made of wooden solid-laminated profile, composed of the most suitable parts of fir or spruce of the building. The obligation is to produce a historical secondary decorative profiling of the elements according to the saved detail indicated in the schemes. The final surface treatment is glazed with the emphasis on the structure of the wood. The built-in glass surfaces must be thermally insulated with a W / m2K coefficient of 1.5 or less. The manufacturer determines the method of posting this coefficient (the number of layers of glass, gas loading, etc.). Sound insulation must be about 40 dB. Swivel-tilt-fitting 'ROTO' or equivalent. Dry installation method over the substructure (6x2) from the boom.

The interior carpentry is made new and, according to the research, upon the original and is made by new Solid Spruce or oak-protected wood. Since it is a public building, the doors to the technological and sanitary facilities must be soundproof with a coefficient of Rw 40 dB. The doors in the sanitary space and the storage room are made from single-leaf PVC doors. Doors made of high-resistant hard PVC with multi-chamber profile system and reinforced steel stainless steel profiles, sealing and sealing with EPDM rubber, according to woodwork and details. Fittings, two-key lock, three hinges and door colors, according to the designer's choice.

Exterior processing of the object

The facade walls of the ground floor are mostly covered with stone slabs of natural stone, which should reflect the structure of quarried stone thickness of different thicknesses of 10-30 cm and 15 cm, to be laid over Ytong or equivalent. The corners of the erker, the console and the front of the terrace are to be laid with planks of coniferous I class. Wrap width - 20 and 40cm. Paint coated with a protective coating in toning and color according to the choice of designer in three coatings. Work in everything according to design details. Tiling of the partera-pavement around the building and the connecting plateau towards the terrace with stone slabs from a bayonet sandstone of a lighter color on a cement mortar.

INSTALATIONS:

The object facility has designed and foreseen the installation of water supply and sewerage, hydrant networks, and electrical installations.

TECHNICAL DESCRIPTION  ALONG WITH THE MAIN PROJECT INSTALLATION OF WATER AND CHANNELIZATION CENTER IN FETISLAM, KLADOV K.P. no.3922 k.o. Kladovo - Town

##### GENERAL

On a building plot KP 3922 K.O. Kladovo - town, in Kladovo, the Visitor Centre was designed in the fortress 'Fetislam'. The object is a rectangular, scatterred-shaped, located on the southeast side of the complex. The building contains a ground floor where exhibition areas with multipurpose hall are planned, and the basement provides wet nodes for visitors of the facility and complex. In accordance with the project task, this main project defines the technical solutions of hydrotechnical installations which will, in terms of capacity ensure the functional use of facility which can satisfy the wishes and requirements of the investor.

The main project for the installation of water supply and sewerage for the facility is done in accordance with the main architectural construction project, project task, conditions and data from the water and sewage project of the complex, while respecting the relevant international and domestic standards and the applicable technical regulations and norms for installations of this type.

This main project includes the following hydrotechnical installations with associated equipment, devices and accessories:

1.Internal water supply network with connection to the water supply network of the complex.

2. Inner sewerage network for collection and evacuation of sanitary waste water from the facility,

3. Sanitary devices and accessories

4. Inner fire hydrant water supply network.

## PLUMBING SYSTEM

This project defines the technical solution of the internal water supply network that ensures proper supply of sanitary water to all planned consumers in the facility and yard, as well as fire protection of the facility with internal hydrants. The external hydrant network of the complex is processed by the water supply project of the complex and is not the subject of this study.

The supply of the facility with the required quantity of sanitary water is carried out from the city water supply network to which the water supply of the complex is connected. The building is connected to the water supply network of the complex, which is provided by PE water pipe, diameter D160mm. The connecting water supply pipe is predominated from PE diameter D75mm. In the water meter shaft, two water meters are provided for one DN50mm for the fire network and another diameter D25mm for the sanitary water supply network.

This main project, in accordance with the requirements and provisions of the TECHNICAL NORMATIVES FOR HYDRAULIC FIRE EXTINGUISHING NETWORKS, defines a technical solution for the protection of the object against fire using an external and internal hydrant network, which ensures reliable and effective fire extinguishing in case of its occurrence in any part of the building.

With this elaborate in the facility, for the needs of fire protection, it is foreseen to extinguish any possible fire with two internal hydrants with a water volume of 2.5 l / sec per hydrant, while simultaneously operating two external hydrants (not subject to this project) with a water volume of 5.0 l / sec per hydrant with a total ammount of 15 l / sec.

Based on the analysis of the required quantities, the total flow rate needed to settle the sanitary consumption in the facility is Q 0,95 l / s. The designs of the external water supply network have been harmonized with all other outdoor installations in the yard. The dimensioning of the water supply network of the facility was made on the basis of the analysis of the required relevant quantities sanitary and / or fire-fighting water, in accordance with the applicable standards and applicable technical regulations.

As the investor did not have the information on the available pressure, the calculation required the minimum necessary pressure in the water supply network of the complex at the point of connection of the water supply network of the facility for the proper functioning of the sanitary and fire protection network. If it turns out that the water supply network of the complex due to the possible low pressure in the city water supply network does not have the necessary pressure, the solution can be provided by the construction of a single hydro power station (Buster) for the entire complex

Plumbing PEHD pipes made of high density polyethylene, D75 and D40 mm for working pressures up to 10 bar, type PE-NP10, class S-5 (SDR-11), with permanently water-resistant compounds were adopted as pipe material for the outdoor water network. fusion welding, or appropriate polyethylene couplings. Newly projected outdoor water network of the building is shown on the situation and on the ground floor of the building. The internal divisions of sanitary and fire water in the building that are located below the floor of the ground as well as all branches to some sanitary facilities are provided by steel galvanized water pipes and fittings with a thread connection .

The water diversion diameters are defined according to the hydraulic budget, in accordance with the provisions and guidelines of the relevant EU norms, ie some of the internationally recognized standards (DIN, NORM, etc.). In the internal divorces of the sanitary water there is a required number of central and permissible valves in order to ensure their proper functioning and maintenance and proper supply of all consumers in regular and aquarious conditions. The preparation of hot water for sanitary consumption in the facility is done individually. The project includes electric water heaters of 80 l / 2000W.

##### INTERNAL FIREWORK HYDRANT NETWORK

The fire protection hydrant network in the building is designed from steel galvanized pipes Ø65 and ∅50 mm. The hydrant network of the building is designed in accordance with the regulations on fire protection. On the internal network, wall fire protection hydrants of diameter Ø 50 mm have been designed in accordance with the Rules on technical norms for external and internal hydrant fire extinguishing network / ¶L.list SFRJ ¸ ¹ no.30 / 91. The internal hydrant network is constantly under the pressure of water, so that at the highest hydrant in the building there is a pressure of not less than 2.5 bar. For the internal hydrant network, the pipes of the smallest inner diameter of 50 mm, ie the hydrant connection of the smallest inner diameter of 50 mm, type C, according to the standard SRPS M.B6, are used. 673.

A fire hose of a nominal diameter of 50 mm is placed in the hydrant cabinet, with a nozzle of 12 mm in diameter. The valve in the hydrant cabinet is placed at a height of 1.50 meters from the floor and the hydrant cabinet is designated as a hydrant mark ('H'). The entire hydrant water supply network of the building is designed as independent with a special water meter for measuring water consumption, located in a water window.

##### CANALIZATION

This study deals with the internal sewage network of the faecal sewerage of the facility with connection to the sewage collector of the complex. The drainage water from the roof freely flows to the surrounding terrain. Acceptance of all rainwater in the complex will be especially addressed, by the design of the space of the complex. The internal and internal sewer of the building is designed in accordance with the applicable technical regulations and norms for this type of installation. External sewer lines are harmonized with other outdoor installations of the facility.

The dimensioning of the sewage system has been carried out according to the applicable technical regulations, based on the relevant quantities of sanitary wastewaters that are evacuated through the internal sanitary diversion, as well as the corresponding amounts of rain water. Total outflow of sanitary water from the building, to which the main drainage channel of the outer sanitary sewerage of the building and its connection to the city sewage system amounts to Q s 2.36 / s. The entire sewerage system is designed from sewage pipes and fittings of hard PVC, DN160 mm, load class SN4, in accordance with the provisions of DIN 1852 and DIN 4102, with screw connection with integrated rubber sealing ring.

On all horizontal and vertical fractures of the external sewage there are provided prefabricated AB revision desks with kinet or revisions and climbers. The modification of LG duct covers DN625 mm with frame, class load D250, according to EU standard EN124, is foreseen via prefabricated AB rings Ø1067 / 850 mm, 16 cm thick. The entire sewer of the building is shown on the situation and on the ground floor of the building. The internal diversion is designed to ensure the acceptance of sanitary waste water from all planned sanitary facilities in the facility and its efficient evacuation, through the outer sewer of the building, to the network of city sewage . Inner sewer installation installations underneath the floor of the basement, as well as branches to individual sanitary devices are provided with PVC for domestic sewage with a connection to the head with integrated rubber sealing ring.

The dimensioning of internal sanitary divisions is done according to DIN regulations, based on connection values ​​(AWs) or outflows from individual sanitary devices, the sum of which yields Qs, to which the sanitary lines are dimensioned. The ventilation of primary divisions is provided by bringing the sewage branches to the revision silo RS3 and RS4 which have only this purpose, this is the avoidance of ventilation pipes through the ground and roof space.

##### SANITARY DEVICES AND ACCESSORIES

All designed sanitary appliances, fittings and accessories, in wet nodes in the building, are class I, white and foreign products, which can be found on our market. The type of sanitary devices, as well as the type of fittings and accessories will be adopted in accordance with the requirements of the Investor and the Interior Designer and with the relevant, applicable technical regulations and norms. Urinal rinse aid in sanitary nodes is electronic with sensor and power supply.

All planned sanitary ware and fancy goods are domestic or foreign production and can be found on the domestic market, during the design of the project for the design of sanitary devices connections the projector used the following sanitary devices:

• Blok WC cup product of the company 'Keramika' Mladenovac or equivalent

• Wash basin product of the company 'Keramika' Mladenovac or equivalent dimensions according to the architectural project,

• Pisarov 'Keramika' Leskovac or equivalent,

• Sanitary haberdashery selected by the interior designer

The contractor is obliged to obtain all sanitary devices and accessories only on the basis of the submitted and by the supervisory authority of the approved reputable copies. All sanitary installations should be professionally and most carefully installed and connected to the water supply and sewerage network, without any damage and according to the instructions and details of the manufacturer, designer or supervisors. All damaged items and accessories must be removed and installed by the contractor at their own expense.

ELECTRICAL INSTALATIONS - TECHNICAL DESCRIPTION

- General technical conditions

The main project for electrical and lightning installations was made on the basis of the project task, technological project and other phase projects. In addition to these descriptions and technical conditions, the project includes all the necessary calculations of construction works and materials, elaborate on protection measures, as well as necessary graphic documentation. The project solution also elaborates additional measures for uninterrupted installation of installation works, assembly and connection of equipment. During the execution of the works, certain deviations from the project can be made, in order to facilitate the completion of the works, especially in the choice of the route of the installation lines, the performance details related to the assembly, with the aim of adjusting the technology of production of the equipment of the manufacturer. After performing the works, the contractor is obliged to perform all functional tests and control measurements, which should be done by a technical report.

b) Main power supply

The power supply of the entire building is carried out by an underground aluminum NN cable 2x (PP00-A-4x150mm, 1kV, Al). The power cables start directly from the newly designed TS-10 / 0,4kV substation, 2x1000kVA, 'TVRĐAVA' - which is foreseen within the 'FETISLAM' fort. The power cables come to (K.P.K.) and the separated measuring cabinet (I.M.O.1.) Placed on the facade of the building. On the route from the new projected tafostanica TS-10/0,4kV up to (KPK) and (IMO1.) - cabinet, the cables are installed in PVC pipes Ø110mm and in a rope-0,8m below the surface of the earth, as given in attachment of graphic documentation. The power supply cable of the building is not the subject of this project because it is processed in the main project of the fortress infrastructure, which represents a special whole.

c) Mainly separated measuring-distribution cabinet (I.M.O.1.) and distribution potable (R.O.P, 1,2,3.)

The separate main measuring and distribution measuring cabinet (IMO1.), Which serves to accommodate the meter for measuring the electrical energy of both buildings, is a metal construction of the type 'DMS' 'IP55' of basic dimensions (500x850x250) mm, mounted at the designated site on the facade of the building, this is attached to the graphic documentation. It contains all the necessary elements for the undisturbed measurement of the electrical energy of the entire building. It must be equipped with a single-pole or three-pin scheme for connecting measuring equipment. It contains 1 (one) three-phase two-tariff meter of electricity, which has the role of a semi-indirect measuring group similar to the type 'DMG2-US-MAX-OC-4T', (5A), 3x230V / 400V, 50Hz, production of 'ENEL'-Belgrade or equivalent.

The semi-indirect three-phase measuring group has the function of remote reading, maxigraph, data storage up to twelve months and switching hours. In addition to the meters, the main measuring circuit breaker (IMO1) contains a main power switch similar to the type 'KS1-250A', production 'Energoinvest' or equivalent -Free, three current measuring transformers similar to type 'STEM 081-150/5A', class- 1.5, FMT production – Zajecar or equivalent, and the required number of high-quality fuses similar to type 'NV00- / 100A', production 'EP' –Aranđelovac or equivalent. Distribution cabinets (R.O.P.) is a tin construction of basic dimensions (600x800x220) mm, is placed in a designated place in the basement premises of the building, as given in the graphic documentation. The distribution cabinet (RCO.KV.) for ventilation and air conditioning is optionally given in this project because it will be subsequently processed by a special project. The distribution cabinet (R.O.K.) is also given optionally and will be subsequently processed by a special project when the heating mode is selected. The diversified potable (R.T.1,2,3.) For the installation in all three separate parts of the building is a PVC-three-sided type with a protective cover and are mounted according to the schedule given in the accompanying graphic documentation. Power cables for all distribution cabinets are 'halogen free' NHXHXFE90-5x16mm or equivalent, 1kV, Cu and NHXHXFE90-5x10mm, 1kV, Cu and directly from (IMO1.) - cabinet, are placed partially in cable racks or under mortar according to schedule which is attached to the graphic documentation. A complete project of electrical installations and main power facilities must be harmonized with the Technical Terms of the Electricity Distribution Kladovo when they are obtained.

d) Instalation of light

Lighting is intended mainly as direct, with luminaires with fluorescent, compact or incandescent bulbs in all rooms, as already stated in the graphic documentation. Luminaires for illumination of the main gallery rooms-1,2,3 of the building are similar type 'PRIMA DLP-1x54W / DT / T16 / G5' 'IP65', production 'BUCK' – Belgrade or equivalent, mounted on load bearing wooden beams in room no. 1 and rooms No.2,3 are mounted on the ceiling according to the schedule given in the graphic documentation. The lighting of other premises of the building is also a superstructure type 'BCN-LANOS-2x26W / DT / G24', 'IP44', production 'BUCK' -Beograd or equivalent, mounted on the ceiling throughout the facility according to the schedule given in the accompanying graphic documentation.. Luminaires for special decorative illumination of supporting wooden structures above the gallery of facility No.1 are reflector type 'DARYL ts-70W / MH / Rx7s / G53', 'IP40', production 'BUCK' -Beograd or equivalent, mounted above the supporting wooden beams galleries according to the schedule given in the attached graphic documentation. Lights for lighting of all toilets and auxiliary facilities in the whole building are similar to the type 'BIN-45 / 100W' 'IP65', production 'BUCK' – Belgrade or equivalent, which are mounted on the ceiling according to the schedule given in the accompanying graphic documentation. Luminaires for illumination of space in front of the entrance to the building are 'BSO 1617 FLOOR-70W / MH / Rx7s', 'IP67', production 'BUCK' - Belgrade or equivalent, which are mounted in special concrete shafts in the country according to the manufacturer's recommendation according to the schedule attached to the graphic documentation. The lights for illumination of the main entrances to the building are similar to the type 'BIN-44 / 100W' 'IP65' production 'BUCK' – Belgrade or equivalent, which are mounted above all entrance doors of the building according to the schedule given in the graphic documentation. The lighting for all rooms of the building is similar to the type 'BPN-PRATICA-1x8W' 'IP55', production 'BUCK' – Belgrade or equivalent, mounted on the ceiling or wall in all entrance hallways of the building according to the schedule given in the accompanying graphic documentation. The lights for illumination of the main entrances to the building are similar to the type 'BIN-44 / 100W' 'IP65' production 'BUCK' – Belgrade or equivalent, which are mounted above all entrance doors of the building according to the schedule given in the graphic documentation. The lighting for all rooms of the building is similar to the type 'BPN-PRATICA-1x8W' 'IP55', production 'BUCK' – Belgrade or equivalent, mounted on the ceiling or wall in all entrance hallways of the building according to the schedule given in the accompanying graphic documentation. Anti-theft lights have an ac battery that is permanently plugged in, and in the event of a power failure they can light up for at least 3 hours in a non-volatile state. Installation lines of the complete illumination of the object are 'halogen free' type NHXHXFE90-3x1.5mm, 1kV, Cu and are placed under mortar in cable racks, PVC ducts in all premises of the building. Switches are similar to 'MOSAIC' -Legrand type or equivalent and are placed in the wall in all rooms of the facility according to the schedule given in the graphics documentation.

e) Installation of connectors

For general use and connection of mobile consumers in the whole facility, the required number of monophase and three-phase connectors with protective contact for wall or wall mounting is envisaged. The installation lines are 'halgen free' type NHXHXFE90-3x2,5mm, 1kV, Cu and NHXHXFE90-5x2,5mm, 1kV, Cu and are placed partially beneath the mortar in cable racks and PVC channels, and are short-circuit protected by automatic fuses in special distribution cabinets or pots. The final arrangement of all sockets and installation lines is given on certain drawings attached to the graphic documentation.

f) Instalation of telephones

Telephone connection of the building is carried out from a special PTT connection located in the 'LAGUMA' room - underground telephone cable TK DSL (30) 59-20x2x0,4mm, GM and for multimedia power supply with fiber optic cable type FO-12xSM-9/125, which are placed in PEHD tubes Ø75mm and in a rope 0.8m below the ground surface on the route from the point of connection to the facility and (ITO-LII) closet.  All telephone sockets in the building are similar to the type 'MOSAIC' –Legrand or equivalent and for wall mounting according to the layout and the requirements of the premises. Installation guides are type IY (St) Y-2x2x0.8mm, and they are laid in PVC-moving hose beneath plaster or in cable racks or PVC-channels. The final arrangement of telephone sockets and telephone cables is provided in the accompanying graphic documentation. The complete project of telephone installation and telephone connection must be harmonized with the Technical Terms, PTT 'TELEKOM' of Serbia, when they are obtained.

g) Instalation of the connected computers

Depending on the purpose and the requirements of the premises throughout the facility, the installation for networking of the computer, which is performed with FTP cables of class 6, is installed in a PVC-moving hose, under mortar or in cable racks and PVC channels. In the office of the entrance part of the facility there is a REK-frame that is not the subject of this project. It will be subsequently processed when networking of computers and special requests of the investor. Computer networking jacks are similar to the 'MOSAIC' -Legrand type or equivalent and are set according to the schedule given in the accompanying graphics documentation.

h) Instalation of the antifire protection

The entire facility also provides manual fire protection and signaling of the PDO. In the large gallery No.1, a PDA was installed, with a certain number of sound sirens-24V with at least 30 minutes of operation in the fire state and the required number of fire detectors. The QVADEL-NIS Fire Protection Center of the type QVADEL-NIS or equivalent to the independent power supply and autonomy of 72 hours operation in a low-voltage state and at least 30 minutes of operation in a fire state, has been installed at an affordable location according to the schedule given in the accompanying graphic documentation. The cables for the ZOP installation are of type IY (St) Y-2x0,8mm, and are installed in PVC-moving hose and under mortar or in cable racks and PVC channels.

i) High voltage protection system

As an over-voltage protection system, the system 'TN-C-S' has been named, after which the protective conductor for equalizing the potential has been conveyed throughout the installation. As an additional measure of protection against overconsumption, additional potential equalization of all metal parts that can come under voltage with a special conductor P-4mm or P-6mm, which is necessarily connected to a special grounding device, was made. This phase was also applied to all distribution cabinets and distribution boards of metal construction, as already given in the enclosed graphic documentation.

j) Equipment approval, measurement and testing

Upon completion of all works, the contractor is obliged to provide the investor with all the necessary attests of the installed equipment as well as the elaborate of measurement and functional testing of complete equipment and installations issued by an authorized institute. The contractor is also obliged, together with the supervisory body, to prepare a record of the handover of all installed installations and equipment to the investor during the official commissioning of the facility.

ELECTRICAL INSTALATIONS - TECHNICAL CONDITIONS

- Perform all the works on electrical installations according to the project documentation, professionally and in a quality manner in accordance with the applicable Technical regulations and standards.

- When carrying out works, the contractor should study in detail the project documentation, the conditions at the place of works, as well as its compliance with the projects of the other phases, and in case of need before carrying out the works, make the necessary changes. It is necessary to study in detail the necessary changes for the purpose of rational execution and request the designer's consent to modify the same

- Changing the projected materials, equipment and the manner of carrying out the works can be done in principle with the consent of the designer or supervisory body, if the changes achieve certain savings, shortening the deadline for the execution of the works without significant influence on the quality and technical correctness of the design solutions.

- After executing the works, the contractor is obliged to do the documentation of the derived state, with possible changes and subsequent works carried out with the prior consent of the designer or supervisory body.

- The Contractor is obliged to constantly monitor the quality of the works during the execution of the works, and after the completion of the works, complete examination and measurement, in accordance with Article 193, shall be performed. 'Rulebook on Technical Norms for Low Voltage Electrical Installations'.

- Electric equipment must correspond to the designed current, that is, the effective value of the alternating current during normal operation, and the same must be subjected to dynamic and thermal stresses and in extraordinary conditions.

- The sea protection device shall be positioned and labeled so that it is easily recognizable by its associated current circuitry and must be located in a switch block (distribution cabinet, panel, counter or similar).

- Conducted cables and cables must be protected against mechanical, thermal and chemical damage by the appropriate type of electrical divider, way of installation, position and lining. Especially in vulnerable areas, additional protective measures must be provided, such as placement in protective tubes, cable ducts and alike.

- Colored conductors and cables must not be continued in the installation ducts

- The minimum permissible distance between electric distribution and other installations is 30mm, with measures of protection against harmful interactions

- Cables and conductors similar to the cable in the wet rooms, fix by means of the hooks at the distance between them:

a) 30 cm for a cross section of 1.5 mm,

b) 40 cm for a cross section of 2.5 mm to 4 mm,

c) 50 cm for an intersection greater than 6 mm2.

- Place office switches on 0.9 m from the floor, and switches in corridors and common areas at 1.5 m from the floor. Mount connections in offices to 0.6 m from the floor, and in workshops and production halls at a height of 1.2 m from the floor

- Cables and installation lines are laid horizontally and vertically. Horizontal laying is carried out at a distance of 30cm to 110cm from the floor and 200cm from the floor to the ceiling. For vertical laying, the distance from the edge of the window and door is at least 15 cm.

- The placing of the cables to the wall is permitted if the cable has insulation from thermoplastic material with a filler and cover, and the laying is carried out on the clips, with the execution of mechanical protection at a height of 2 m from the floor

- When laying cables into the ground, the depth of burial must not be less than 0.6 m from the upper surface of the earth, or 0.8 m below the road surface. Cables intended for laying in the ground may be laid in concrete cables and underground insulation tubes with cables or lead sheath provided that the tube is mechanically solid, protected against penetration of liquids and that the cable variability is provided

- Upon completion of all works, the contractor is obliged to inspect the entire installation, to draft the Technical Report on the correctness and to give consent for the use of the same

- General test must be performed according to the following schedule:

a) Continuity of the protective conductor and main and additional for equalizing the potential.

b) Resistance of insulation of the electrical installation and loop to the currents of the three-pole and single-pole short-circuit or earth faults.

c) Protection by electric discharge of electric circuits.

d) Resistance to floors, walls and grounding.

e) Functionality of all elements and entire installation

- The insulation resistance depending on the voltage of the circuitry should be for

a) Power supply circuit with safety low voltage over the transformer for separation-more than 0,25 MΩ, and the test voltage is 250V .

b) Rated current circuits up to and including 500V more than 0, 5 MΩ, and the test voltage is 500V.

c) For a voltage current circuitry above 500V, more than 1MΩ, and the test voltage is 1000V.

- TECHNICAL DESCRIPTION WITH A STATIC INVOICE

DESCRIPTION OF THE LOCATION AND THE OBJECT

The visitor center is composed of three lamellas, objects 1 and 2 representing the reconstruction of old buildings, while the central link part is a completely sarem building in a constructive and architectonic view.

Building 2 is a concert hall of bright height about 7m, with a stage and a spectacular view to the stage. Roof construction is a wooden grille that imitates the former roof structure. Facade walls of height of 7,3 m from the outside are covered with gamma while the interior is plastered, the supporting part is made of masonry construction with horizontal and vertical silks and filling of 'Ytong' blocks on glue

Building 1 is an exhibition hall 4.6m of height. In a constructive sense, the object is a reinforced concrete skeleton with circular columns and a full ab plate. Facade walls are of the same construction as in the building 1

The connecting part represents the entrance hall for both objects, with a wet core in the basement. In a constructive sense, the object is also a reinforced concrete skeleton with circular columns and canvas around the perimeter. The façade is a glass wall curtain

- CONSTRUCTIVE CONCEPTION OF A NEWLY PROJECTED OBJECT

The main features of the construction of the new project are:

Object 1 - exhibition hall

- The building is a reinforced concrete skeleton with circular pillars of 25 cm in diameter, ab plate with thickness d 20 cm and reinforced concrete beams, in both orthogonal directions, above the pillars measuring 20/40 cm.

- Skeletal reinforcement elements are masonry facade walls made of 'Ytong' blocks on adhesives

- Above the concrete slab is a four-convex roof of wooden structure of coniferous II class with classical roof joints in a double chair system

- Roof cover is a filling crep

- Temporary construction is designed as a barbecue from reinforced concrete cones, with a dimension of 100 / 40cm

- Facade walls are coated with stone on the outside.

Object 2 – concert hall

•	The main structural elements are façade walls of thickening thickness 35-30-25cm, made of 'Ytong' blocks on adhesive, on the outside covered with stone thickness d 20cm.

• Vertical grooves in the façade wall are below the roof ties at a distance of 4.2 meters and vary in dimensions, at the bottom they are 35x30cm, then 30x30cm and on the wall of the wall are 25x30cm. Reinforced reinforced with reinforcement according to the static calculation

• Horizontal serkling divides the wall into three equal fields and varies according to the width of the pillar, from 35 to 30cm, 30x30cm and 25x40cm. Reinforced reinforcement with reinforcement according to the static calculation

• Roof binders are wooden bars of L 14.5m range, of sliced ​​second-class material, grates are at a distance of 4.2m, corneas come over, horns and planks and battens and contralets • Roof the covering is a falconed Mediterranean crep

• Foundations of reinforced concrete bands under facade walls measuring 150 / 40cm.

Connecting building

•   The object  is designed as a reinforced concrete skeleton with circular columns with a diameter of 25cm and ab canvas for encircling a thickness of 15cm

• Ceiling construction is full of mushroom reinforced concrete slab thickness d 20cm

• Beyond the board comes a wooden substructure and a cover made of sandwich profiled sheet with minimal inclination

• Under the larger part of the building there is a basement with a wet node. The peripheral walls of the cellar are made of reinforced concrete thickness d 20 cm while the interior are made of slate blocks of the same thickness.

• On the ground floor there are no walls other than a canvas for stiffening. The façade is made of glass that has its own metal substructure.

• The foundation of the building is armored bars with different depth of foundation depending on the basement.

CALCULLATION MODEL

The considered loads for the newly constructed structure are their own weight of construction, snow and wind as movable loads. The project is constructed in accordance with the Preventnik on technical norms for design and construction of high-rise buildings in seismic areas. The underground part of the building is also the active pressure of the country. constructive concrete elements are counted as individual elements in a level in accordance with the above burdens. All that is not given by this stethic budget is to do the harm to the applicable standards and regulations for this type of work.

TECHNICAL CONDITIONS FOR PERFORMING OF THE WORKS

CONTENT

1.     PREPARING WORKS AT THE BUILDING SITE

2.     GROUND WORKS

3.     BUILDING WORKS

4.     CONCRETE, REINFORCED CONCRETE WORKS

5.     REINFORCING CONCRETE WORKS

6.   CARPENTRY WORKS

GENERAL CONDITIONS

All paragraphs of the calculation and calculation shall mean the performance of each work in accordance with the plans, technical description, calculation and calculation of works, static account, details and further details of the designer, applicable technical regulations, JUS standards and instructions of the Supervisory Authority and Designer, unconditionally, professionally and precisely.

All works and materials listed in the description of individual items of this projection must be included in the bidder's bid price. The prices included in the projection are the Contractor's sales prices and include all labor costs, materials with the usual spread, work scaffold, water, lighting, propulsion material and energy for machines, cutting, execution, social contributions, all state and municipal taxes , the Contractor's earnings, as well as all other expenses conditioned by the existing regulations for the development of the selling price of building materials, including all expenses arising from special working conditions that provide for the average standards in construction

The Contractor shall not be entitled to request an additional fee for the prices entered in the budget of the works, unless it is stated in the same position of this proforma invoice that the particular work is paid separately in another position. Calculation and classification of performed works will be done according to the average norms in construction, which is obligatory for the Investor. The description of works from the average standards in construction is obligatory for the Contractor, if the description for individual points is not supplemented. General description for one type of work and material, obliges the Contractor to perform all such works in certain positions according to this description, regardless of whether the position is called in the general description, unless otherwise specified in the position in question.

In all construction and craft works, the use of quality materials is conditioned, which must correspond to the existing technical conditions, JUS standards and description of the appropriate position of the works in the calculation of works. Embed the material only for the first-class, foreseen type, unless the position of the calculation specifies its quality of lesser value. All the materials that the Investor finds that it does not correspond to a similar calculation and conditions, the Contractor is obliged to immediately remove from the construction site and the Investor to suspend the work if the Contractor tries to use it. In disputed cases, the material is sent to the Institute for Material Testing, which is found to be applicable to both Investor and Contractor. If the Contractor embeds material in addition to negative findings, all material damage from the ordered demolition shall be borne by the Contractor without the right to complain and the objection to demolition issued by the Investor, and the Contractor shall without objection accept the Investor's decision

The site manager, as the representative of the Contractor, is obliged, prior to the start of each work, to promptly inform the Investor's representative about the place where the works will be carried out, and only after the instruction, through the Construction Diary, to perform the order and start the business. If the work is done contrary to the orders through Građ. logs, details, Palestinian plans, the Contractor does not have the right to complain. If the Contractor performs certain works contrary to the foregoing description, the Palestinian plans and details, no justification will be considered, but the Contractor is obliged to remove and at all costs, regardless of the quantity of the completed work, at all costs, at his own expense , bring it as planned, description and details provided, unless such changes are made through the Constructing logs by the representatives of the Investor approved. If the Contractor works to work better and more expensive than foreseen, he / she does not have the right to request an additional fee if he or she did this without the approval or order of the representative of the Investor through the Construction Diary.

The objects and the whole construction site must be maintained by the Contractor, and at the end of the works, before the objects are handed over, all the holes, the public pit, the scaffolding holes and the like. to pile up, the entire surface leveled and everything well and solidly (that after the absence of sagging) do. The contractor must clean the whole building well from the shots, scaffolds and dust, the yard is arranged and the entire facility is prepared for surrender in order and cleanliness

Any damage caused by the Contractor during the construction of a building within the construction site shall be eliminated and brought to its original condition at its own expense. In case of constructive changes, increase or omission of individual works from the budget, in whole or in part, and consequently the costs incurred (surpluses or deficits), the Contractor is obliged to adopt without remarks, limitations and claims for compensation, with the surplus or deficit calculated at the affected prices. In the event that there is a need for works that do not have a contractual price in this pro forma invoice, the Contractor is obliged to determine the price for them, with the approval of the Investor Representative, and hereby introduce them in the Construction Diary

The investor has the right to request special works (roof insulation, VDP penetrates, new materials, etc.) that the firm that performs such work issues a written guarantee that the work it performs will be durable and solid. The contractor is obliged to harmonize the work of certain companies that perform certain types of works on a regular basis or independently, so that one would not harm other works, and if they do so, they will immediately remove the damage done to the other company at their own expense. Otherwise, the Contractor will bear the costs for removing these damages. The same is the case with possible obstruction in the performance of the job.

The supervisory authority has the right to request from the Contractor to submit for the new materials a few samples (attested) for the selection. Procurement of these samples is not paid separately

In addition to all temporary structures that are necessary for the Contractor to perform the works, the Contractor is obliged to provide a room for the office of the Supervising Authority and to keep it in order during the execution of the project, in addition to ensuring the necessary lighting, heating, cleaning and necessary office furniture.

If the Contractor needs to take, in order to store the material, even the adjacent yard, landing or pavement, the Contractor will obtain the approval of the competent authority or the owner for this use, except that the expenses for this use can not be specifically charged.

The Contractor is obliged to submit to the Investor a confirmation that the building was placed according to a certain location (Certificate of location check) as well as a receipt for paid water and electricity during construction of the building.

After handing over the finished object to the Investor, the Contractor shall be responsible for all of it within the warranty period, according to the legal regulations, and in the event of any damage or malfunction it is obligated to bring everything into proper condition at its own expense.

building book and the construction diary will be guided on the basis of the existing legal regulations, every day by entering the necessary data, which the representatives of the Contractor and the Investor review and verify on their own pages with their signatures on each side.

In addition to the general conditions, the 'General Conditions for Construction of Buildings and Works' are also an integral part of the contract (Official Gazette of SFRY 13/57) and the existing provisions on construction, as well as the Draft and Budget.

1.           PREPARING WORKS AT THE SITE

Prior to compiling the bid with which to participate in the auction, the Contractor should go through the terrain in order to get acquainted with the local climatic and other conditions, the characteristics of the terrain, the possibilities of construction and other elements essential for the formation of unit prices. It is necessary for the Contractor to become acquainted with local conditions, regulations, access roads, possible landfills and other elements that may affect the smooth running of works.

If in the technical documentation provided to the Contractor for use, there are no detailed situational plans, before the start of the works, the site is to be shot by the Investor or by a competent authority and to make appropriate substrates with sufficient angles, permanent points, profiles and other elements of the essential for future works on buildings. The investor is obliged to determine with the Contractor the most suitable places for raising temporary - helping objects, such as shacks, warehouses, silos and others. After the completion of the construction, these objects are demolished and the material is removed, which will not be accounted for separately.

The investor should intervene with the local authorities in order to solve certain administrative tasks related to construction in a timely manner. Timely introduction of the Contractor into the terrain where the facilities and the delivery of the designated routes, the list of benchmarks and other geodetic points will be built and the locations of the geodetic points in the field will be presented, timely submission of the technical documentation and parts of the documentation necessary for the execution of the works, is the obligation of the Investor.

The contractor is obliged to supply the materials, tools, machinery and everything else that is necessary for the construction to start on time and finish within the agreed time. The contractor must have sufficient number of qualified and qualified staff at the site.

2.           GROOUND WORKS

| 2.1. | EXSCAVATIONS |
| --- | --- |
| 2.1.1. | Exscavations for the objects |

GENERAL

Excavations for objects include all excavations of basic pits, foundations, trenches, canals and all other excavations that have to be performed in order to build the objects that are the subject of this Project. The excavation will be performed in a wide range of digging with the possibility of preparing all types of digging machinery.

The contractor is obliged to carry out all the excavations to the limits, slopes and angles shown in the drawings. Excavations performed directly with derived objects or their parts must be carried out with great care in order to protect these objects from damage. The Contractor shall make any damage incurred at his own expense.

The Contractor is obliged to propose the acquisition in all cases when he determines or believes that the excavation, which has been carried out according to the slopes, angles and dimensions shown in the drawings or determined by the Supervisory Body, is not stable or does not correspond to the project requirements of the Funding. The digging will be performed only if the Authority determines that the Contractor's proposal is justified and approved. Such approved and completed excavation will be taken into account and paid to the Contractor at the agreed unit prices of the relevant excavation site during the measurement for the payment of the excavation.

If the Contractor performs the excavation beyond the boundaries, the heel and the angles indicated on the drawings or determined by the Supervisory Authority, he will be obliged to complete the flooding in the manner authorized by the Supervisory Body, and at his own expense.

The contractor is obliged to drain the construction pit and drain the workplaces as needed by portable pumps from atmospheric water.

2.1.1.2. Payment Measurement

Measuring the amount of the excavation for payment shall be made to the slopes, lines and angles shown in the drawings for each class as defined in Article 1.3. of this chapter.

2.1.1.3.  Payment of the excavations

The excavation will be paid at the agreed unit prices per m3 of excavation.

Unit prices include:

- excavation costs

- rejection outside the workspace (up to 50 m) with disassembling

- dismantling and other security of the excavation surface in the manner and in the extent that guarantees the stability and safe operation (if applicable);

- maintenance of excavation slopes throughout the construction course

- draining and disposing of atmospheric waters by secondary transmission pumps, in accordance with Article 4 of this chapter

- all other costs that may be incurred to make excavations performed and maintained in full compliance with the provisions of these Technical Terms.

The Contractor will not be granted any requests for additional payment above the agreed unit prices on account of the excavation being carried out in wet or wet material.

2.1.2. Digging trenches

The investor is obliged to deliver to the Contractor, if not otherwise agreed, the feed route and the sketch of the insurance. Excavation should be carried out with proper cutting of the sides and adjusting the bottom at the projected angles. The excavated ground should be thrown to the edge of the trench at least 1 m, so that it does not collapse into the trenches and that the work in the hollow is 100% secure. The pipe joints shall be able to excavate a niche of such dimensions in order to ensure smooth operation during assembly. 'And a description given for wide excavation.

Calculation per m3 of excavation of the self-propelled land includes all works as a price included and disturbance from decommissioning and underground installations.

2.1.3.     Classification of the excavations

Before the beginning of any excavation or when moving from one dig of material to another, the Contractor and the supervisory authority will jointly perform the necessary geodetic survey of the site. If during the excavation a material class changes occur, the Surveillance Authority and the Contractor will jointly execute the classification and geodetic record the boundaries of the change class of materials.

2.1.4.     Excavation in the soft materials

Excavation in soft material includes all excavations in materials in which the excavation can be done manually or mechanically without the use of explosives. The soft material means that material that a bulldozer type D-8 with 'riper' can expose.

2.1.5.     Excavations in rocks

Rock excavation involves all excavations in the material for which the digging is necessary to use an explosive. In addition, this excavation involves the excavation of solids whose volume exceeds 1,0 m3.

2.1.5.1.  Explosive and mining

The contractor is obliged to develop a mining project for all objects where mining excavation is foreseen that will detail the drilling and mining methods including location, depths and mining areas, diameter, layout, depth and inclination of boreholes. Also, the type, strength, quantity and arrangement of explosives and detonators per well, as well as mining schemes, should be shown with appropriate budget and explanations. This project should also order works on the construction of surface protection where the excavation is completed in relation to the order and progress of the mining.

If mining takes place in the vicinity of objects, the Contractor is obliged to submit to the Supervising Authority for approval a mining plan in which will be given drawings of the position of the building and wells, the sequence of mining and a measure of safety measure.

Mining is not permitted in the zone of facilities where the mining site is less than 30 meters away from the concrete or injection sites that are not older than seven days in permanent buildings, or two days at temporary facilities. The contractor is obliged, in accordance with the Regulations on protection measures for (Official Gazette of SFRY No. 9/1967, Corrections 35/1967, Supplement No. 35/1972) shall prepare and submit to the Supervisory Authority for approval the organization of public security security during mining.

The organization should include all security details such as visual and audible signalization of the start and end of the mining, guard schedule and other types of markings that will fully enable safe blasting. The Contractor's duty is to store and handle explosives and other materials, as well as for mining , also recruit the responsible person for this type of work.

Notwithstanding the approval obtained from the Supervisory Authority, the Contractor is only responsible for carrying out any repair of damage resulting from mining at his own expense. For each specific case, the Contractor will submit a proposal for repair of the damage to the Supervisory Body's approval. The Contractor assumes full responsibility for all unfortunate cases arising as a result of mining.

The cost of implementing the above measures will not be paid in particular, but will be covered by unit excavation prices.

2.1.5.2.  Explosive and storing

The Contractor is obliged, in accordance with the Rulebook on Measures of Protection in Explosive and Mining Mining ('Official Gazette of SFRY' No. 9/1967, Corrections 35/1967, Supplement 35/1972) to develop a project of warehouse with all security measures and written warnings that will include the manner of storing, transporting and manipulating explosives and other materials needed for mining, and to submit it to the Supervisory Authority and the competent authority of the interior for approval.

The contractor will, in cooperation with the Supervisory Authority, determine the location of the explosives warehouse. The contractor is obliged to ensure the continuous storage of the warehouse, and to conduct an official investigation of the stored and issued quantities of explosives and other materials.

The cost of storing explosives and other materials needed for mining and all other costs related to this item will not be paid in particular, but will be covered by unit excavation prices.

2.1.5.3.  Mining

In order to obtain the excavation to the lines, slopes and dimensions of the given projects or ordered by the Supervisory Authority and reduce the runoff and damage of the wall outside these lines, the Contractor may apply one of the following controlled mines:

• Pre-tearing

• Smooth blasting

In addition to these, the Contractor may submit proposals and other mining techniques to the Supervisory Body, which satisfy the above requirements. By adopting the proposed mining technique, the Contractor is not entitled to change the contracted unit price of the excavation.

2.1.5.4.  Pre-tearing

The pre-tearing technique of the wall will be applied to the formation of external excavations and to the formation of excavations to which concrete is directly attached. This technique consists of a single row of boreholes, a light continuous filling with explosives of lesser frets that fall simultaneously. In order to create the previous crack along a definite digging line, the first completion of this mining and then mining the wall mass in front of the line of previous splitting.

2.1.5.5.  Smooth blasting

The smooth blasting technique consists of a well-drilling system at a suitable interval. The wells are in a row at a slight distance, light continuous filling with explosives of weaker insight. One-line ignition is at the same time. In the mining scheme, the peripheral line of the well is burning the last. No matter which blasting technique is applied immediately after the sinking, the Contractor is obliged to remove all loose and loose pieces of the wall on the newly excavated surfaces.

It is the responsibility of the Contractor to perform tests of various drilling and mining techniques in order to find the best spacing and depth of boreholes, row spacing, height of the floor, length of the stocks, type and amount of explosives, method and quantity of filling, type of detonator and mining scheme. During the entire excavation period, the Contractor is obliged to make the necessary adjustments to the drilling and mining methods that will minimize the flow outside the required lines and slopes.

2.1.5.6. Payment Measurement

No measurements of surfaces obtained by controlled blasting or any other mining elements will be performed separately.

2.1.5.7.  Payment

Regardless of the mining technique applied, all costs of the blown blasting technique and test mines will not be paid separately but will be covered by the agreed unit cost of excavation in the wall of the respective position.

2.2.        TRANSPORT OF MATERIALS

2.2.1.     General

Under the transport of excavated materials, it is understood that:

• the transport of materials from the excavation to the landfill

• the transport of materials from the landfill to the place of installation

2.2.2.     Measure payment

The excavated material will be partially permanently deposited with the dismantling, and partially deposited in the landfill from which the building will be filled. The average transport of the excavated material is 50,0 m, and the return of the material for the filling is 25,0 m. It is not intended to transport the material to a higher length than the indicated ones.

2.2.3. Payment

Transport of excavated material up to 50.0 m is covered with a unit price of 1 m3 of self-made soil, and the recovery of material from the filling site at a mean distance of 25.0 m is covered by the unit price for filling. Larger lengths of transport are not foreseen.

2.3. DEPOSITION

2.3.1. General conditions

Landfill excavation will be carried out on temporary and / or permanent landfills whose locations have been approved by the Supervising Authority. Cleaning of the terrain at the sites of the dumpsites will be done only to the extent necessary for the undisturbed disposal. The deions must have stable, uniform sloping slopes, designed to are easily used, and the surfaces of permanent landfills exposed to the view are arranged according to the instructions of the Supervisory Authority.

2.3.2. Measurements for payments

No measurements are made in relation to works on landfilling, arrangement and maintenance of the landfill.

2.4. WATER PROTECTION

2.4.1. General Provisions

In the case of water protection, the drainage of the pit is handled by hand pumping or portable pumps. The contractor is obliged to submit a project of drainage and drainage of workplaces as part of the Conceptual Design Project. The solution should show the pumping points of the transmission pumps. When making the solution, use a gravitational drainage of water wherever possible.

The contractor is obliged to submit to the Supervisory Authority a detailed solution for the drainage of workplaces within the framework of the Main Project of the construction organization after the contracting.

2.4.2. Water Pumping by Secondary Pumps

Water Pumping by Secondary Transfer Pumps will take place at places where relatively short-term water extraction is required due to atmospheric influences or the like.

2.4.3. Payment Measurement

Measurements for the payment of water pumping to secondary transmission pumps will not be carried out.

2.5. FILLING

2.5.1. General Provisions

In the case of filling, all impregnations of foundation pits, trenches and all other detentions, sanding of a layer of sand or sink / sand material at the bottom of the foundation pond or trench (substrate replacement) prior to concreting or laying of pipes, filling over and above objects and all other fillings foreseen in the lists of positions of works with quantities including flooding. The material for the pouring must be adequate, without organic matter of the carrots and other inadequate substances, which will be decided by the Supervisory Body.

Material for sandy and pebbly tampon layers (replacement of materials) will be natural sand or natural sand and gravel mixes with a maximum grain of 10 cm. This material will be obtained from the nearest locations approved by the Supervisory Authority. The bulkhead or compactor must not contain pieces of gravel or stone larger than 10 cm. All filling will be carried out to the lines, angles and slopes shown in the drawings, ie those which is determined by the Supervisory Body.

The material for filling around the building will primarily be obtained from the excavation. The constructor is obliged to submit to the Supervisory Body, at the latest 15 days before the beginning of the filling works, more detailed information on the piling machinery, in order to obtain the consent for its use. The inspection body will determine which material will be used for filling, the method of filling and protection measures of constructed objects during filling, which involves manual loading and pouring in places where required by the Supervisory Authority.

2.5.2. Compilation of materials

In places where the fill is required, the material will be compacted according to the following requirements.

2.5.2.1. Stacking of incoherent material

Filling with sand or sandy pebbly material can begin on prepared foundation foundations approved by the Supervisory Authority. The non-coherent material of sand or gravel sand, approved by the Authority, will be slow in horizontal layers of thickness after compaction of up to 30 cm, which will be determined by the Supervisory Body depending from the type of mechanization with which the material falls. The material will be added to the water in the amount needed to achieve the required compaction. The compression will generally be performed by a 4-pass roller with intensive wetting with water. Compaction and wettability will be carried out in the presence of the Supervisory Authority.

By compacting the material it is necessary to achieve a relative compaction of Rd 0.70 which is defined by the term:

γ d max (γ d ugr . − γ d min)

Rd = -------------------------------------- x 100 %

γ dugr . (γ d max - γ d min)

Where the γd is dry bulk density, determined on the field by the method with water in a plastic film or calibrated peskommaxγd maximum dry bulk density obtained in the laboratory minγd minimum dry bulk density determined in the laboratory.  maxγd maximum dry bulk density obtained in the laboratory minγd minimum dry bulk density determined in the laboratory.

In the zone of denivelation of the sand or gravel tampon, for the purpose of correcting the boundary zone between the two levels (slopes), the expansion of the filling of the work with a higher angle will be carried out, and after the compacting, the removal of the unbounded part and the formed slope will be done according to the percentage.

The supervisory authority may require that a quality control be carried out of the material. Regulations that control the quality of the built-in material

• JUS U.B1.010 – Sampling

• JUS U.B1.016 - Determination of the volume weight of the soil

• JUS U.B1.018 - determination of the granulometric composition

• JUS U.B1. 024 - the content of combustible and organic materials.

In the zone next to the concrete structures in order to protect them from damage by heavy machinery, the embankment of the embankment tape directly next to the facility in the width of 1 m will be done by hand mechanical ramps (vibrating plates, etc.) in layers of thickness up to 20 cm in unbound state. The required compaction is the same as on the other parts of the embankment.

2.5.2.2. Collection of coherent material

The granting and placing of materials on the prepared ground floor or on the already built embankment layer can begin after taking the lower layers by the Supervisory Authority. Each individual layer must be disintegrated in the longitudinal direction horizontally or at most in the tilt of 5%. In cross-sense, each individual layer must have a double or one-sided slope of 2-5%. This slope is necessary for the removal of atmospheric water, which makes the surface of the coat during the installation of coherent earth materials must be degraded and immediately compacted (daily).

Each individual layer must be embedded against the projected cross-section. When acclimatising the transport means, they must be as evenly distributed throughout the plateau's width.

The height (thickness) of a single degraded layer must be in accordance with the compaction effect by which the depth of the compaction agent is used and the type of  bulk material (max. 20 cm). Each layer of embankment must be charged in full width with the appropriate mechanical means, where the compaction should in principle be carried out from the edge to the center.

Any unsuitable places for machinery or places where the use of heavy pouring equipment for other reasons would be inappropriate (collapse of objects) should be borne by other suitable means or methods, the use of which will be approved by the Authority.

Each layer of the embankment must be moistened or dried to humidity, which is 2% above or below the optimum humidity, before starting to pour. If after pouring and quality control it does not proceed immediately with the filling of the next layer, it is continued with pouring after a longer period of time, under different weather conditions, the filling quality should be controlled again before filling. In this case, the accumulation can begin only when the quality of compaction is proven by testing.

When the risk of rain is threatened during the day, the Authority will, if necessary, determine the suspension of further work on the embankments, without reimbursement. On the dike of coherent material, it is necessary to plan and roll the upper surface of the layer with a light smooth roller (3-5 tons), so that the surface is inclined from 2 to 5% on one side, to be smooth and untouched in which the atmospheric water. Prior to filling the new layer, this soiled surface should be roughened to achieve the best possible connection between the layers. This also applies to other major interruptions in the construction of embankments (due to the termination of the construction season, etc.).

The filling must be carried out so that the layers in the longitudinal sense are preferably horizontal and so as to avoid sudden height crossings between the layers of varying heights, and they are carried out under a slope under which further compaction can still be carried out. Work on the filling will be interrupted at any time when it is not possible to achieve satisfying results, especially due to rain or some other atmospheric disasters. On this basis, the Contractor is not entitled to any compensation.

The embankment material must not be installed on frozen surfaces, nor be fitted on snow or ice. Controlled areas

• JUS U.B1.010 - sampling

• JUS U.B1.012 - determination of soil moisture

• JUS U.B1.016 - Determination of volume weight of soil

• JUS I.B1.046 - Determination of the modulus of compressibility with a circular plate.

2.5.2.3. Compensation checks

The supervisory authority may require quality control of the compaction, and in this case, the Contractor is obliged to provide control over the constraints of the embedded layers. The control will be performed by the Contractor, and the sampling points and their number will be determined by the Supervising Authority, which will also control the results of the control.

The humidity of the material should be about 2% above or below the standard Proctor's optimum, to be uniform in each layer. The required minimum bulk density of the embedded material in dry condition is 95% of the laboratory compaction according to the standard Proctor.

2.5.3. Measure for payment

The measure for payment of the executed filling per m3 will be made up to the line, slopes and angles indicated on the drawings or those specified by the Supervisory Authority. Measurement shall not be performed for the payment of the filling of faults caused by the Contractor's error.

2.5.4. Payment

The placement of the coated material will be done at the agreed unit prices per m3 for bulkheading.

3.           BUILDING WORKS

3.1. MATERIAL

The building material must be of high quality and made professionally. The brick must be machine-made, well baked, regular in shape, with flat edges and surfaces, without cracks, lime or scallops, properly porous.

The positions will determine the type of brick

• full brick clay JUS B.D1.011,

• hollow facade bricks and blocks of clay JUS B.D1.014,

• hollow bricks and blocks of clay JUS B.D1.015,

• hollow plates from clay JUS B.D1.022,

• full façade bricks of clay JUS B.D1.013,

• hollow blocks of clay, fillings for the intermediate structure JUS B.D1.030,

• semi-contact interconnecting bearing structure JUS U.N1.050 ,

• hollow concrete blocks JUS U.N1.100.

GAS CONCRETE elements are products of quartz sand, lime, cement and water with additives that give it a porous structure

• wall blocks JUS U.N1.308,

• insulating plates JUS U.N1.306,

• reinforced wall and partition plates JUS U.N1 .304 and

• Intermediate and roof reinforced sheets JUS U.N1.302.

The SAND must be clean, sharp, free of organic matter and sludge, according to PTP-7 (Sl.l.FNRJbr.32 / 47).

The LIME must be baked, properly extinguished and restrained for 30 days for building and 8 weeks for plastering. It must be clean, clear without any kind of sludge and organic ingredients.

CEMENT should comply with the regulations JUS B.C.1011 and 1012.MALTER to be made exactly on the required scale, mixed well, cleaned from the chest, and for the plaster, and sow through the appropriate sieve.

All the required material should be brought in a timely manner to the building site.

3.2.        BUILDING WITH BRICKS AND BLOCKS

3.2.1. General Provisions

Expose cleanly in horizontal lines with regular connections and with raised surfaces and flat and vertical edges underneath. The joints must be well grouted with mortar and the mortar in the joints must not be thicker than 1.0 cm. The outside of the joints should leave an empty 1.5 - 2.0 cm for bonding the mortar when plastering the walls. When building hot and masonry in cement mortar, brick and blocks are to be spilled with water. Particular attention should be paid to building the ending of the edges and walls between the pillars. When building masonry, leave openings according to the masonry measures and take into account the acquisition of certain construction elements.

The price of the building includes the production of openings and grooves for installations of water supply, sewage, electricity and hot water. Closing and running of these grooves and holes, whether brick, either by hand or otherwise, is not paid separately. The price for building is also the purchase and use of various packages necessary for fixing carpentry or locksmith frame and for sheet metal and will not be paid separately.

For partition walls of 1/2 brick, release the connection from solid walls in each row of 1/2 brick. For bulkhead walls, leave in groove walls in 1/2 brick in each other row of bulkhead walls. For walls with a length of up to 4,0 m, it is anticipated to be serkling. On the walls of the brick brick, the outer, visible part of the wall is made with serkling which is not visible, so that the outer appearance of this wall throughout its surface is made of fused brick. For longer lenghts of 4.0 m ', insert a stiffening pillar. If the wall is made of fused brick, the pillars must be tied to the construction and the concrete in the wall. In seismic areas (Official Gazette of the Socialist Federal Republic of Yugoslavia No. 39/64), which refers to the connection of 7 and 12 cm partition walls with constructive walls, the joint is carried out by reinforcing each other joint, in the length of 50 cm.

In these areas, the partition walls are to be carried out in an extended mortar. The walls of the brick brick are made of brick of the same length and height, and if it is done with two coated faces and the same width, with joints of equal thickness. The brick must be full and have all the edges of a sharp, undamaged and must be of the same color. In the case of brick-brick walls, special attention must be paid to the complete correct connection without the use of smaller pieces of 1/4 brick. Vertical and horizontal joints must be of exactly the same width over the entire length, and the mortar in the couplings is pulled 2 cm from the outer wall level. When masonry, use a square iron rod whose thickness corresponds to the thickness of the predetermined layer of mortar and the width, the depth of the plastering. The arrangement of rows on the couplings is marked with auxiliary strips, which are fixed at the ends of the walls and serve as guides for checking the horizontal rows. The mortar should not be sprayed at least in a certain depth in the joints, nor the brick may be sprayed with mortar. After completion of the building, do the grooming cem. mortar 1 2. During operation and after finishing the masonry, the walls of the brick brick should be protected from dirt and damage, either naturally with paper or yarn. Damaged and paved wall surfaces of fused bricks will not be recognized.

3.2.2. Payment

The calculation for masonry will be done per m2 or m3, which will be indicated for each position. The offered unit prices include work, material, movable and facade scaffolds and all other direct and indirect costs related to building.

3.3. MALTERISATION

3.3.1. General conditions

Do plastering / malterisation only when the walls are dried and the building collapses. The walls must be clean and the joints loose before plastering, so that the mortar can be well received. Before plastering, the walls must be wrinkled, especially at cem. plaster. If the shaft is released on the walls, it must be cleaned with the brush (with the consent of the Supervising Authority) and washed with a solution of acetic acid in water, in the ratio of 1 10, and at the expense of the Contractor. Plastering is done in two layers, rough and fine. The fine coating is applied when the first, rough, well-dried. Concrete walls are plastered with an extension or cement mortar, but the surface is pre-cleaned from mortar and dust, washed with water and dispersed. mortar.

Plastering of the façade should not be performed before the internal plastering  is finished and dried. Maltered surfaces must be flat, without waves, and the accessories are precise, sharp and straight. It is forbidden to reuse plastered mortar. When plastering at higher altitudes, as well as in plastering the facade, the Supervisory Authority receives the scaffold.

The plastered surfaces that have been improperly executed, cracked or unblocked from the substrate (sobac) will be demolished and overlaid by the Contractor at their expense. The surfaces in the continuation of the plastering must be clean, smooth, flat and so designed to act as a whole. The surface finish is made with a steel jetter , cement mortar, free from cement dust. Pay special attention to the proper care of the patted and cem. mortars, suitable and permitted temperatures, the quality of the used material as well as the prescribed handling.

It is recommended that the cement for large surface surfaces be ordered in particular by consulting with the manufacturer's laboratory, as when working with standard portland cement, microfisures, mesh spraying of oversized surfaces - which will not be tolerated.

Addition of additives to mortar - cement or concrete, in order to improve the required properties can be allowed only by submitting the necessary attestations, references and results of previous developments by authorized institutes, investors' consent, but in principle it can not affect changes in unit prices.

Pay particular attention to the correct cleaning of the glue cement mortar, the temperatures suitable and allowed for work, the quality of construction and the used material as well as the proper handling.

The surfaces of concrete and mortar walls, which are plastered, need to be cleaned, cleaned of loose and other particles and moistened by spraying up to 5 mm below the surface of the wall. After wetting, on the wall surfaces, spraying is rare. mortar with sharp 'unit' sand, covering the whole surface in a layer of about 3 mm thick. After 24 hours, the first layer of the second mortar is applied, the thickness for the outer walls is 15-20 mm, and the inner layer is about 10 mm. Since this layer is well bonded, a finishing layer of fine mortar is applied in a thickness of about 5 mm, with the permeability of visible surfaces with a wooden or filched perdash. It is desirable to add one of the certified hydrophobic agents to the final layer of external plaster or use a pre-prepared mortar with hydrophobic properties.

3.3.2. Payment

The plastering calculation will be done per m2. Unit prices include all necessary materials and work for the performance of individual positions, working and facade scaffolds, hand or motor cranes, plaster pumps, tools, installation of all packages, pavement and anchor for carpentry, locksmiths or sanitary ware, all openings and dies for installations , shrinking, plastering and plastering ceramic tiles with mortar part of the wall above, regular collection and removal of shots outside the construction site during the course of the work.

4. CONCRETE, CONCRETE AND ARMORATED-CONCRETE WORKS

4.1.FULL PART

4.1.1.General conditions

Concrete works in the sense of these Technical Terms mean the selection, preparation and handling of materials that form components of concrete, mixing, transport, installation, protection and care of concrete, delivery, installation and removal of scaffolding and formwork, fabrication and processing of joints, preparation of concrete surfaces, installation of all parts that go into concrete, as well as all previous and control tests of the components and the concrete itself.

All concrete works will be performed according to shapes and dimensions, as well as according to the phases or order that are given in the performance drawings or ordered or approved by the Supervisory Authority. If the Contractor proposes a change in the performance of the works, the proposal shall be submitted to the Supervising Authority for approval at least (30) days prior to the start of the construction with the explanation and the necessary evidence of non-standard conditions and deformations.

4.1.2. Regulations and Standards

In addition to the provisions of these Technical Terms, the Contractor is obliged, when performing all phases of concrete works, to follow the relevant provisions of the applicable Regulations and Rules as well as all standards that are directly or indirectly related to concrete works, and

• the Rulebook on Technical Measures and Conditions for Concrete and Reinforced concrete (hereinafter: BAB); • Yugoslav standards (hereinafter JUS).

4.1.3. Composition of Concrete

Concrete consists of cement, small and large aggregates, water and mandatory additive that creates air bubbles in fresh concrete. All the materials of which concrete is made must comply with the provisions of these Technical Terms or the corresponding Regulations and Standards.

4.1.4. Previous and control tests of concrete and its components

The contractor is obliged to obtain information on preliminary concrete tests. The Inspector shall perform the control tests of concrete and its components during the execution of works by the sub-control of the Supervising Authority in accordance with the provisions of these Technical Requirements.  All inquiries will be carried out for the Contractor's account by an authorized specialized organization to be contracted by the Contractor for this purpose.

All costs of controlling the concrete and its components will not be paid to the Contractor in particular, but will be covered by unit prices of concrete. During the performance of concrete works, the Contractor is obliged to enable the Supervising Authority to take samples for the control tests of all components of concrete and concrete itself in order to prove the prescribed quality of concrete as well as taking samples for storage.

In all cases where the inspection of components of concrete or fresh concrete does not give the required results in quality, the Contractor is obliged to promptly and urgently perform the necessary interventions and corrections in the work process in order to ensure the required quality, as evidenced by new control tests. If the Contractor fails to obtain the required quality with these urgent interventions and corrections, or if the Supervisory Body considers that the quality can not be repaired with urgent interventions and corrections, the initiated concreting will be interrupted, and the Supervisory Body is the sole decision maker to decide on the manner of interruption and the treatment of the already installed concrete (removal, possible remediation, etc.).

In all cases where the test trials of test blocks or other control tests carried out after a certain period of time do not give the required results on the quality of the embedded concrete, the Contractor is obliged to take samples from hardened concrete at places designated by the Supervisory Body and submit them to the Authority for examination . Examination of these samples will be carried out by the specializations of the organizations referred to in point 1.4. these conditions, and under the control of the Supervisory Authority. If these additional control tests give unsatisfactory results, the Supervisory Authority will decide on further treatment of the block (removal, cure repair, etc.).

The Contractor is obliged to execute, at his own expense, all subsequent works according to the Supervisory Body's order, on remediation, removal and re-concreting and adjacent concrete blocks and elements where the results of the control and additional control tests have shown that the embedded concrete does not meet the requirements of these Technical Requirements . Also, the Contractor will not be entitled to compensation for any costs and damages due to possible delays in work that are caused by these subsequent works.

The costs of all control tests that produce unsatisfactory results will be borne by the contractor.

4.1.5. Concrete Laboratory

The contractor is obliged to provide testing of concrete and its components during the performance of works in a specialized organization, laboratories. The laboratory keeps a record with accurate records of all samples taken and examinations carried out. The laboratory keeps records of all other preliminary and control tests and certificates of concrete components.

4.1.6. The concrete works logbook

The contractor is obliged to keep a log of concrete works from the beginning to the end of the concreting. All the data necessary for determining the entire concreting process and the conditions under which it has been carried out is entered into this diary.  As a mandatory, the entry and the concrete block, the beginning and the end of the concrete, the type of concrete, the origin and quality of cement, aggregates and additives, the composition of the mixture and the amount of cement and watercourse factor, the method of handling, transport and installation of concrete with thickness and the number of layers, all changes in these data, if they occur during concreting, the eventual standstill, their patterns and the duration of the same, all eventual events of crucial importance that occur during the concreting, the humidity of the air at the beginning and the end of the concreting, that is, every six hours, if the concreting takes longer, data on the conducted control tests and taking samples for the control tests and all other data at the request of the Supervisory Authority.

4.1.7. Organization of concrete works and concreting program

In accordance with the provisions of the Construction Agreement, the Contractor is obliged to do the appropriate project of the entire organization of works. Within this, the Contractor will develop the organization of concrete works and general technology for the transport and installation of concrete. In addition to this Contractor, at least thirty (30) days prior to the commencement of the concreting of each facility, or each specific part of the facility, it shall submit to the Supervisory Authority no approval a detailed concrete program specially to process the method of transport and transfer of concrete and the method of incorporation, including order of blocks, number and thicknesses of individual layers, as well as projects of appropriate scaffolding and formwork (in accordance with the provisions of item 14).

The supervisory authority shall, at the latest fifteen (15) days after receiving the Contractor's detailed concrete project, give its consent, or submit remarks on that project. In this case, the Contractor will deliver a corrected or refined concrete program for re-approval within the next ten (10) days. If, through its concrete program or project, it is prepared and paid, the Contractor foresees the execution of appropriate preparatory works, he shall be responsible for the time necessary for their execution and submit the concreting project to the Supervisory Authority for approval in advance.

The Contractor will not be entitled to any compensation for possible costs and damages due to the delay of the start of concreting, if the same is caused by the untimely delivery of the concreting project, as well as in all cases where the Authority does not approve the concreting program for any reason, but returns it to the Contractor with comments on its modification and re-approval.

During the construction of the concrete program, the Contractor is obliged to take maximum care of the harmonization of his works with the works of other participants under construction.

4.1.8. Concession Approval

The Contractor shall not begin to concretion any block before obtaining approval from the Supervising Authority by entering in the Construction Diary and the corresponding form.

This applies to the approval of all elements of concrete and reinforcing works, and the basic joints, work joints, formwork, scaffolding, reinforcement, all parts that are embedded in concrete. Prior to making a call to the Concession Approval Authority, the Contractor is obliged to perform all the works in accordance with the provisions of these Technical Terms and in accordance with the approved concreting program. The Contractor is obliged to submit a call for control and approval to the Supervising Authority at least three (3) hours before the deadline in which the Supervisory Body wants to start control, with the request being submitted during business hours. The start of the control can not be less than one hour before the end of working hours, unless the Contractor has previously received the consent for the continuous work (three shifts).

If, during this approval control, the Control authority, in determining the approval of the Contractor, did not act in the essential elements and prepared the concrete block according to the provisions of these Technical Terms and according to the approved concreting program, the Supervisory Body has the right to determine the new start of control by assessing the time that the Contractor can the elements are brought into the required state.

Control and approval for concreting will be performed and issued partially for individual elements in all cases when the Authority considers it necessary, but the concreting can not begin before the final approval of all elements of the block. The contractor will not be entitled to any compensation for possible costs and damages due to untimely request for concreting approval due to the duration of the control, or for postponing approval and re-control in the foregoing.

4.2. COMPONENTS OF CONCRETE

4.2.1. Cement

4.2.1.1. General conditions

Cement, in terms of these Technical Terms, means a binding agent for making concrete.

4.2.1.2. Cement quality

For all concrete work, Portland Cement for hirdotechnical structures 15Z PC 35 will be applied at 35 kN / cm2 after 28 days. If during the construction there is a need for special types of cement, the change must be approved by the Supervisory Authority.

The contractor is obliged to provide data to the Authority on the previous cement tests. The contractor is obliged to carry out control tests of cement for the entire duration of the concrete work in the manner prescribed by the relevant standards.

4.2.1.3. Payment

The costs of purchasing, transporting and storing cement will not be paid separately but will be included in the unit price of embedded concrete.

4.2.2. Agregat

4.2.2.1. General

Under the terms of these Technical Terms, aggregate means granular sand and gravel or broken stone.

4.2.2.2. Aggregate quality

For the construction of concrete, only an aggregate that meets the quality requirements determined by the provisions of the Rules BAB and JUS B.B3.100 according to the requirements of mineralogical-petrographic composition, grain shape and quality, sulphate and sulphide and fine particles.

4.2.2.3. The fraction of the aggregate

Agregate for the preparation of concrete must be divided and dosed in fractions. The concrete of the brand 30 is not allowed to be removed from the natural mixture of the aggregate. The optimum granulometric composition of the aggregate mixture will be determined experimentally within the previous tests.

4.2.2.4. Previous aggregate tests

The Contractor is obliged to submit to the Authority information on the preliminary testing of the aggregate used for making concrete.

4.2.2.5. Control aggregate tests

The control tests of the aggregate will be performed continuously during the performance of the works in order to prove that the used aggregate meets the requirements of the BAB Rulebook and that the composition of the material does not differ from the composition established by the previous tests to the extent that it could affect the quality of the concrete. If the control tests show that the characteristics of the tested materials do not meet the prescribed requirements, the Contractor will make an adequate correction.

4.2.2.6. Payment

All costs related to the aggregate will not be paid separately, but the unit price of the embedded concrete will also be included.

4.2.3. Water

Under water, in terms of these Technical Terms, water is used for the preparation and care of concrete.

4.2.3.2. Water quality

When concrete is used, only water for which it has been confirmed by appropriate previous tests that it does not damage concrete, or that it has the prescribed properties according to JUS U.M1.058, will be used. The use of water which does not damage the concrete in direct contact with the concrete surface is allowed for concrete care, that is, it has the prescribed properties according to the BAB Rulebook according to parameters of the content of sulphates and organic ingredients.

4.2.3.3. Preliminary and control tests of water

The preliminary and control tests of water will be carried out in a manner and in accordance with the appropriate JUS.

4.2.3.4. Payment

All costs of supply, distribution, heating, etc. and other water-related costs will not be paid separately but will be included in the unit price of embedded concrete.

4.2.4. Additions to concrete

4.2.4.1. General conditions

Under the additions to the concrete, in the sense of these Technical Terms, substances are defined as substances that are added to concrete prior to or during mixing in very small quantities in order to reach specific persons of fresh or hardened concrete. The used accessories must satisfy the quality requirements according to JUS U.M1.035.  Prior to making concrete using the additives, it must be checked whether the admixture to the concrete corresponds to the projected concrete mixture according to JUS U.M1.037.

4.2.4.2. Required concrete additives

For all concrete, air bubbles will be created in fresh concrete, unless otherwise specified by the Authority. The creation of air bubbles is ensured by adding a suitable admixture to the concrete to the JUS U.M1.034 drum mixer. This additive to concrete will be dosed in an amount that ensures that the fresh air content of the air is not less than 3.5% nor more than 6% relative to the volume of the concrete.

4.2.4.3. Optional additions to concrete

The use of other types of additives to concrete is permitted on the proposal of the Contractor and with the approval of the Supervisory Authority, only in cases when the Contractor proves their necessity in terms of achieving the required fluidity of fresh concrete or regulating the required cementitious time in concrete, and concurrent evidence of previous concrete tests and by appropriate approvals, that their use does not weaken the basic properties of concrete and reinforcement.

4.2.4.4. Examination of the characteristics of the additives to the concrete

The contractor will not perform the concrete testing of the concrete additives, but is obligated to obtain appropriate approvals from the manufacturer with each new quantity of supplied accessories and to provide it to the Inspectorate and keep it in the technical documentation for the evidence of the quality of the executed.

The contractor is obliged to carry out the storage, storage and use of the additives to the concrete strictly according to the manufacturer's instructions. It is not allowed to use concrete additives without an attestation and after expiry of the shelf life.

Upon the request of the Supervising Authority, the Contractor is obliged to carry out a control examination of additives to concrete which due to improper or other storage, although the expiration date did not expire, could change the production properties. The costs of these tests will not be paid in particular.

4.2.4.5. Payment

All costs associated with additions to the concrete will not be paid separately but will be included in the unit price of the embedded concrete.

4.3.1. Types and qualities of concrete

The types and qualities of concrete are defined on the appropriate drawings of the objects and in the description of the position of the works. For the execution of concrete works and the framework of these Technical Terms, the classes of concrete with the following characteristics:

- Hydrotechnical concrete of concrete brand MB 30, waterproofing V-4 and resistance to frost M-100, in structures in contact with water.

- MB-banner brand that will be used in structures where waterproofing is not required.

- Concrete brand MB 15 or MB 20 to be used for filling, leveling layer and the like.

For all the mentioned concrete classes an aggregate with a maximum grain size of up to 32 mm will be used. The amount of cement and the water-cement factor for each type of concrete will be approved by the Authority on the basis of the results of the preliminary testing of concrete considering the conditions for the transport and installation of concrete.

4.3.2.     Previous tests

Previous tests of concrete shall be carried out in the manner described in item 4.1.4. of these Technical Terms. Preliminary concrete tests will determine and demonstrate the optimal composition of the concrete mix, including the granulometric composition, ie the percentage participation of individual fractions, the water-cement factor, the amount of cement and the amount of additives for the formation of air bubbles in concrete, while achieving all the required concrete characteristics specified in point 4.3.1. of these Technical Requirements ie. an appropriate mark, tensile strength at bending, watertightness and frost resistance, and the required consistency of fresh concrete, which ensures good embedment in the given conditions of transport and installation. Ukoliko se prethodnim ispitivanjima zaključi da su potrebni i drugi dodaci betonu, utvrdiće se i optimalna količina tih dodataka.

Ovim Tehničkim uslovima ostavlja se mogućnost smanjivanja količina cementa poboljšanjem granulometrijskog sastava mešavine, kvaliteta izrade i uniformnosti vodocementnog faktora.  Based on the results of previous surveys, the Authority will subsequently prescribe the conditions for the consistency of fresh concrete on the concrete plant and at the installation site to be controlled by the Slump test.

4.3.3. Control tests on concrete

Concrete testing of concrete shall be carried out in the manner described in item 4.1.4. These technical conditions. Control tests of concrete shall be carried out in order to prove the required properties of concrete mixing and embedded concrete. In all cases, when the required properties have not been proven, the Contractor is obliged at all costs to carry out all the work on bringing the quality of concrete to the required level, including the corrections of the composition of the concrete, the manner of harvesting, transport and installation, as well as the rehabilitation of already hardened concrete, and at the request of the Supervisory Authority removing and replacing it.

At least once a week, and at the request of the Supervising Authority, more often a concrete concrete mixer must be taken and the composition of the concrete mix should be tested to determine the degree of homogeneity, the granulometric composition of the aggregate mixture and the amount of water and cement in the concrete. The consistency of fresh slump will be controlled at least every 2 (two) hours according toJUS U.M8.050.

The amount of air in fresh concrete is measured at least once in the shift work according to JUS U.M1.031. The temperature of the fresh concrete after the exit from the mixer is measured at least once in a shift, and at outside air temperatures lower than -5 ° C and higher than 25 ° C at least once every 2 (two) hours. The maximum temperature of the concrete at the time of incorporation must not exceed 30oC nor be lower than 5oC. The control of the transport of the concrete mixture consists in checking at time intervals determined by the Supervisory Authority, the homogeneity, the consistency and the temperature of the concrete mixture in in relation to the pure characteristics of the exit from the concrete mixer, where the changes must be within the limits prescribed by the Authority on the basis of the results of the preliminary tests and the approved mode of transport.

The control of embedded concrete consists in proving the quality of concrete hardened at its place of production and under laboratory conditions and in proving the quality of hardened concrete embedded in the building. The quality control of the produced concrete is carried out by determining its compressive strength, according to the JUS UM1.020 tensile strength at bending , according to JUS U.M1.010, waterproof according to JUS U.M1.015 as well as resistance to frost effect according to JUS U.M1.016 for parts of the objects for which it is required. In each of these tests, the concrete weight of the concrete must also be determined by measuring the body.

For the purpose of controlling the strength of the produced concrete, the required number of test bodies collected from individual mixtures is at least 3 (three) for each 500 m3 of embedded concrete in case of continuous concreting or from each block, the block, the part of the construction is concreted with interruptions. When several types of concrete are installed on the construction site, then the required number of inspection bodies should be adjusted according to the front requirements. The supervisory body may determine that, in addition to real concrete bodies to be tested in a time interval of shorter or longer than 28 days. The tightness control of bending when bending the concrete produced requires a number of test bodies at least 3 for each 500 - 700 m3 of embedded concrete.

For the purpose of controlling the waterproofing of concrete, it is necessary to conduct at least 1 series of 6 (six) test bodies for each 500 m3 of embedded concrete, for each type of concrete separately or for the series from the construction for which waterproof concrete is required, and contains less than 500 m3 concrete .

The supervisory authority will subsequently prescribe for which parts of the facility and in what time interval the determination of resistance to the effect of frost will be carried out. The results of strength and pressure testing must be processed statistically and at the same time fulfill the determined quality criterion. If for examined test bodies not tested for quality parts prescribed by the inspection bodies, the Supervisory Body may request that a core of 15 cm in diameter be removed by rotary drilling places and depths it deems appropriate, which will be examined in a laboratory designated by him. If laboratory tests or subsequent tests show that the quality of the concrete is unsatisfactory, the Authority will decide on the further treatment of the concrete concerned. The costs of these tests shall be borne by the Contractor.

4.3.4. Definitions

4.3.4.1. Reinforced concrete

Under reinforced concrete, in terms of these Technical Terms, it is understood that concrete contains a reinforcement, a minimum quantity of 20 kg / m3 of concrete.

4.3.4.2. Nearmixed concrete

Under unarmed bertones, in the sense of these Technical Terms, it is understood that concrete contains no reinforcement, or the quantity of aramt is less than 20 kg / m3 of concrete.

4.3.5. Testing the achieved quality of hardened concrete

4.3.5.1. General

The following tests are carried out by controlling the quality of hardened concrete:

• compressive strength after 7, 28 and 90 days

• tensile strength after 7, 28 and 90 days

• waterproof after 28 and 90 days

• frost resistance after 28 days

The supervisory authority may require quality control for individual tests even after 180 days. The quality of concrete will be assessed according to standardized criteria.

4.3.5.2. Trial load

The test load is carried out in accordance with the regulations on technical measures and conditions for concrete and reinforced concrete, ie the requirements of the project (test filling) .

4.3.5.3. Destructive Testing Methods

Read more reliable and accurate quality testing of embedded concrete The supervisory authority may require the extraction of samples from the finished construction. In addition, the samples will be disassembled in all cases where the quality of manufactured or embedded concrete is suspected, and based on a control test. Examination of the samples will be carried out on the basis of the program to be defined for each particular case by the Supervisory Authority.

The samples should, as a rule, be removed from concrete of at least 28 days old, provided that the curing is carried out under normal conditions. For the assessment of the strength of the concrete at the depth of the massif, taking samples is done using a Krelius drill. In this case, the equipment with which the core can be removed is at least 15 cm. From the core, the cores are shaped like cylinders whose height is equal to their diameter. For each section of concrete in which drilling is performed, 3 or 6 test bodies should be made, which will be tested for pressure. The strength obtained during the testing of cylindrical test bodies must be calculated for the strength of the cube 20 x 20 x 20 cm.

This also applies to cubic-shaped cubes when the size of their edges is less than or greater than 20 cm. When taking samples of concrete with a drill, in addition to testing the cores for strength, the number of cores per length of the well must be determined on certain concrete sections, and the same along the length of the wells on certain concrete sections, and also the exterior appearance of the concrete that has been extracted and the characteristics of its structure must be described (gravel, sandy, compacted, porous, spongy, etc.). The test procedure is the same as in the test An analogue concrete test body. The constructor is obliged to perform destructive tests in all cases, which are defined by the program, or a written order issued to him by the Supervisory Authority. The contractor is obliged in this regard to a specialized organization.

4.3.5.4. Non-destructive testing methods

The following non-destructive methods of for the evaluation of the elastic properties of concrete can be used:

- ultrasonic impulse methods (dynamic modulus of elasticity), determination of defects that occurred during the manufacture, determination of the thickness of the layer in which physical damage was caused (fire, frost) or chemical samples (corrosion) as well as hardened concrete;

- mechanical methods for determining the concrete strength of the concrete (shaft method, method of prints, method of punching and extraction of wedges).

The above-mentioned methods can be combined with the following:

- ultra-sound velocity and velocity or impression velocity;

- velocity of ultra-sound propagation and weakening.

Non-destructive testing will be carried out at the request of the Supervisory Authority. The contractor is obliged to engage a specialized organization for these tests.

4.5.3.5. Payment

The quality testing costs of hardened concrete will not be paid separately. They are included in the unit price of concrete.

4.4. CONCRETE WORKS

4.4.1. Concrete preparation

4.4.1.1. General

The manufacturer is obliged to keep in operation and maintain a concrete factory that will be able to produce the concrete of the required quality and sufficient capacity to meet the requirements arising from the construction dynamics, with a reasonable safety factor to cover eventual discharge from the drive, reducing the effect on cold weather and similar patterns. The Contractor shall submit the description of the concrete factory with all the characteristics to the Supervisory Body for approval with the offer at the level of the Preliminary Design of Works.

The concrete plant should be in operation enough time before the planned start of concreting to provide evidence of the prior testing of concrete. Only an automatic concrete plant is allowed.

4.4.1.2. Dosing of concrete components

Dosing of concrete components is carried out in the concrete factory manually or automatically. No cumulative aggregate measurement is allowed, and therefore each fraction of the aggregate should have a specific weight. Cement is measured and dosed separately from the aggregate. Dosage controls should be adjusted so that the new dosing cycle can not be started until all fungal doses are empty.

4.4.1.3. Mixing the concrete

The mixing time should be determined experimentally in order to achieve the optimal homogeneity of the composition of the concrete mix, or to adopt the recommendation of the concrete mixer manufacturer regarding the mixing time. The permissible limit of non-homogeneity is that the cement-sand ratio in the water-cement factor is less than 10% between the samples of fresh concrete taken from the front end, the back end and the middle of the mixer or taken at the beginning, in the middle and at the end of extrusion of the concrete from the mixer. it must consist of two mixers of the same or approximately the same capacity. It is not allowed to move the concrete that partially tied it.

4.4.1.4. Payments

All costs related to concrete will not be paid separately but will be included in the unit price of embedded concrete.

4.4.2. Concrete transport

4.4.2.1. GENERAL

Under the transport of concrete in terms of these Technical Terms, it is understood that the concrete is transported from the concrete factory to the unloading site and from the unloading site to the place of installation.

4.4.2.2. Transport from the concrete factory to the building

The transport of concrete from the concrete factory to the facility must be carried out as quickly as possible, in an approved manner that prevents segregation, component losses, an improper rise in the temperature of the concrete mixture or loss of water.

4.4.2.3. Transport from the unloading point to the place of installation

Transportation to the unloading site to the place of installation must be carried out in such a way as to prevent the segregation and losses of concrete components. The use of sloping ramps is forbidden, except for those whose length is less than 3.0 m of the minimum slope 1 2, to which concrete is directly extruded and from which the concrete is directly installed.

| Temperature of the mixture in oC | The maximum mixing interval to the beginning of the concrete installation | Maximal mixing interval to completion of concrete installation |
| --- | --- | --- |
| 5 - 10 | 1 h 30 min. | 3 h |
| 10 - 15 | 1 h 15 min. | 2 h 30 min. |
| 15 - 20 | 45 min. | 2 h 15 min. |

The contractor will use the funnel, the use of which will be previously approved by the Investor. All means of transporting concrete must be maintained under satisfactory conditions. The hindering of hardened concrete must not be allowed. The start or completion of the concrete installation in the working concrete layer must be carried out at the following time intervals, counting from the moment of release of the concrete mix from the mixer. For the temperature of the concrete mixture from 20-30oC intervals will be prescribed by the Supervisory Authority.

If cement is used, the beginning of the binding is 1 hour and 30 minutes. the specified values ​​of the interval are not valid and the Supervisory Authority will prescribe the new ones. The timing intervals from the previous table can be extended during transport from the concrete factory to the facility if during the transport the slow transfer of the concrete mixture is carried out, but the extension of the time interval can not be moved 60 min. At high and low air temperatures (above 30 ° C and below 5 ° C), the concrete mixture must be adequately protected during transport.

4.4.2.4. Payment

The costs of transport and concrete installation will not be paid separately but will be included in the unit price of concrete.

4.4.3. Preparation of concrete surfaces

4.4.3.1. General

Betoning shall be carried out on the surfaces of a previously excavated excavation, without or with excavation protection, on the surfaces of the previous concrete, on surfaces which are clad with the appropriate formwork and on the surface of the hydromechanical, machine and other equipment that is deposited.

Under the surfaces, in the foregoing sense, means all surfaces to which and on which fresh concrete is installed, regardless of the angle that they cover with the horizon. The constructor is obliged to prepare all concrete surfaces in accordance with the provisions of these Technical Requirements and the Supervisory Authority will carry out control and acceptance of them within the framework of the concession approval (point 4.1.8.).

4.4.3.2. Preparation of the basic coupling

Under the basic coupling, in the meaning of these Technical Terms, all contact surfaces between the concrete and the floor are understood. The contractor is obliged to prepare the basic coupling in accordance with the project's decision.

4.4.3.3. Payment

The costs of preparing the concrete surfaces will not be paid separately but will be included in the unit price of the embedded concrete.

4.4.4. Concrete incorporation

4.4.4.1. General

The contractor will timely submit to the Supervisory Authority for approval, a concrete installation program from which the position of the blocks, the sequence of concreting and the respective concreting dates will be perceived.

4.4.4.2.  Installation of concrete

Prior to the start of the concrete installation, the Contractor shall submit to the Supervising Authority for approval a standard form signed by the responsible persons on the readiness of the formwork, reinforcement, reinforcement, elements that permanently remain in the concrete, the preparation of the surface as well as the readiness of the concrete factory and means for transport, installation and care The construction will be done according to a block project approved by the Supervisory Authority. Installation of concrete will begin only after obtaining a written approval by the Inspectorate for each block.

Concrete incorporation will not be approved if, in the opinion of the Supervisory Body, the weather conditions do not correspond to this. The start or completion of the installation will be in accordance with the provisions of item 4.4.2.3. The installation will be performed as a continuous operation until the end of the block. The constructor will determine such an organization of concrete installation to change shifts, meals, etc. do not interfere with the continuity of concreting.

4.4.4.3. Vibrating of the concrete

Beton will be compacted exclusively by machine, using pervibrator. The size of the pervibrator must be adapted to the type of concrete, the size of the aggregate, the dimensions of the work unit and the thickness of the layers. When compaction of each layer of the needle of the pervibrator it must enter in the previous layer about 5 cm. Compaction is done until the cement milk appears on the surface of the layer, or on the joints of the formwork. The contractor is obliged to pay special attention to the installation and compacting of concrete around all installed elements, equipment, seals and formwork. In special cases, the Contractor may, upon his proposal submitted with the project of concrete, approve the use of formwork vibrators for concreting concrete.

The thickness of the individual layers will be determined by the concrete project, depending on the capacity of the concrete factory and the dimensions of the block, with a minimum layer thickness of 20 cm and a maximum thickness of 50 cm. Concreting around the installed equipment and pipelines will be done with special care in the thin layers and evenly so that no movement is made.

4.4.4.4. Interruptions in Concrete

If during concreting of concrete, due to force majeure, the concreting is interrupted, and if the block is not completed, the Contractor will form a flat, flat horizontal coupling or will be formed as the Authority decides at that moment.

Before extending the concrete, this coupling must be treated in the manner described in item 4.4.5.

4.4.4.5. Payment

The cost of embedding concrete will not be paid separately but will be included in the unit price of the embedded concrete.

4.4.5. Working joints of concrete

4.4.5.1. General

Under working couplings, in the sense of these Technical Terms, are meant all contact surfaces between pre-conceived and hardened concrete and a new, fresh concrete forming a single unit and along which a monolithic bond must be achieved.

For the purposes of these Technical Terms, two types of working joints are distinguished: freewheel joints (horizontal or gently sloping in which the surface of pre-conceived and hardened concrete is free and accessible) and working joints with cladding - vertical and steep surfaces formed by the plating for precast concrete.

4.4.5.2. Jointing process

All work joints must be properly roughened, cleaned, dried and washed before the start of concreting. The blasting will be done by removing the cement scaffold from the workpiece surface, with the free working joints being also washed with a mixture of air and water under pressure, and in the vertical and steep working joints, the same is done after removing and removing the formwork, by handwriting or pickhamers, or otherwise approved. For the freezing of free working joints, the Authority will subsequently prescribe the time when the cement cycle can be started, depending on the season, taking into account the efficiency of the same and preventing the removal of large grains from the surface layer of concrete. If for any reason the Contractor fails to do the washing of free working joints by washing, he must do the same by punching, with Pickhammer being able to apply the earliest 24 (twenty-four) hours after completion of concrete bonding.

Prior to concreting, the Authority will, within the framework of the concession approval, perform the control and reception of the working couplings. Immediately before applying the fresh concrete, the work joint will be moistened sufficiently to prevent moisture withdrawal from fresh concrete.

Horizontal and vertical couplings in the outer walls of the reservoir (pool) have sealing strips of 150 mm, which are processed in appropriate detail.

4.4.5.3. Payment

The cost of manufacturing work joints and the installation of sealing tape will not be paid separately but will be included in the unit price of the embedded concrete.

4.4.6. Dilatation couplings

With this project, the dilatation couplings are foreseen in swimming pools and will be reported in detail according to the details from the project.

4.4.7. Upholstered parts

4.4.7.1. General conditions

The parts that are embedded shall mean all parts which, in accordance with the detailed drawings, or according to the orders and instructions of the Supervisory Authority, are installed and installed entirely or partially in fresh concrete, including parts that are placed in pre-prefabricated niches, and then subsequently assembled with secondary concrete . Any parts that may be embedded may be parts supplied by the Contractor and parts supplied by other participants in the construction of facilities. The Contractor is not responsible for the functionality and quality of the embedded parts delivered from other participants in the construction if it is evident that it was not damaged and treated with them and installed them in everything according to the appropriate instructions.

4.4.8. Taking care of concrete

4.4.8.1. General

Concrete performance in terms of these Technical Terms represents the protection of free surfaces of concrete from moisture losses at least 14 days after concreting. The inspection body may reduce or extend this period, as permitted or required by such conditions, and depending on the climatic conditions, the type of cement used and the additive concrete and humidity in underground facilities.

Equipment and concrete care products will be ready at every place of concreting before the concreting begins. All exposed concrete surfaces will be protected from direct sunlight for at least 3 (three) days from the day of concreting.

4.4.8.2. Concrete care by moistering / soaking

Concrete care by soaking will be done by pouring water, or by spraying it, by making the concrete of the pool full of water, covering with cords or wet overlays that moisture, etc. The water used for caring in everything must satisfy the requirements given in item 4. of these Technical Conditions. If the formwork is wooden, it must be maintained during the whole period of the care in a wet state. In the case of an earlier removal of the formwork, the exposed surfaces of the concrete shall be deemed to be free in the sense of the first paragraph of this item and maintained in a wet state until the specified deadlines for maintaining the concrete.

Concrete, especially when it is still fresh, must be protected from accidental damage by padding or otherwise, during work, in particular if it is exposed to exposed surfaces.

4.4.8.3. Membrane protection of concrete

If the Contractor wishes to use membrane protection of concrete, he will submit a proposal with a detailed explanation on the type of membrane protection, location, method and condition of application. The membrane protection of concrete will only be applied with the written approval of the Supervisory Authority for each concrete case.

4.4.8.4. Payment

Consumption costs for concrete will not be paid separately but will be included in the unit price of embedded concrete.

4.4.9. Concrete production at low temperatures

The handling, transport, installation and protection of concrete at low air temperatures lower than 5oC to -10oC must therefore be carried out to ensure the monolithicity of the concrete and the design requirements for the strength and watertightness.

The curing temperature must ensure at least 50% of the concrete strengths and strengths at the pressure before the concrete is exposed to the cold. When filling the concrete, the aggregate must not be frozen lumps, grains, or pieces of ice. At an outdoor air temperature of 5 ° C to -10 ° C temperature concrete mixing must be at least 6oC to 9oC. In order to achieve the required temperature of the concrete mixture, its components are heated. At an outside air temperature of -5 ° C, it is sufficient to heat the water (not more than 80 ° C), and at lower outside air temperatures, a small aggregate and even fraction up to 30 mm (not over 70 ° C) must be heated.

They must also be prevented and taken the necessary measures to ensure that during transport of concrete mixture there is no significant decrease in its temperature. Bottling can only be done over the surface of a previously installed concrete with a temperature above 0oC. If this is not the case, artificial heating requires a temperature of at least 0 ° C to a depth of at least 30 cm. The actual heating temperature must not be higher than 90oC. The preparation of each block or campsite that is concreted at low temperatures is carried out by commission. The Contractor is obliged to repair damaged premises according to special instructions of the Supervising Authority.

4.4.10. Surface treatment of concrete

4.4.10.1. General

The Contractor shall perform the work on the surface on the manner specified by these Technical Conditions, depending on the position and role of the concrete surface in the exploitation of the object. All the works that are the main project are intended as a special treatment of concrete surfaces (painting, bitumen and epoxy resin coatings, plastering, isolation, etc.) are not considered as surface treatment in terms of these Technical Terms, but are treated as special works and calculated and paid on the manner determined by these Technical Terms for these types of works.

4.4.10.2. Division of concrete surfaces

According to the position and role of the concrete surfaces in the exploitation of the object, all surfaces are divided into

- clogged surfaces, ie surfaces that will be permanently buried or impregnated

- visible surfaces ie surfaces that will permanently remain free and visible on objects

- dried surfaces ie surfaces which are in contact with water.

In all designated surfaces, they differ in the way they are made, the free surfaces that are made without formwork and the formwork surface.

4.4.10.3. Free surface treatment - no formwork

Free surface freezing (horizontal and subhorizontal surfaces) will be carried out successively with concreting until the concrete on the surface has finished bonding. On free surfaces to be filled, fresh concrete will only be leveled according to the required dimensions.

For free surfaces, which will be visible, but treated as external visible surfaces, fresh concrete will be smooth. For free surfaces that will be visible and treated as inner visible surfaces, fresh concrete will be smoothened and settled to give a completely smooth surface without traces of ironing. If this surface serves as a substrate for the installation of various types of flooring or asphalt coverings, it will be processed in the manner of requiring the processing of the substrate for these types of works. In the free surfaces to be wet, fresh concrete will be installed manually or mechanically until it is obtained completely smooth surface with no traces of ironing.

The cost of processing the surface without formwork is calculated in the price of 1 m3 of concrete and will not be calculated in particular.

4.4.10.4. Treatment of surfaces under the cladding.

The surfacing of the surfaces that are carried out under the formwork will be done after removal and removal of the formwork. Surface treatment will only be done on quality surfaces, while improperly applied surfaces will be repaired in accordance with the provisions of item 4.4.13. of these Technical Terms.

For surfaces under the cladding that will be clogged in the outer visible surfaces, no processing is performed. In the areas under the cladding that will be visible and regarded as internal surfaces, processing will also not be performed if the presence of the bubble on the clad surface is insignificant and smaller dimensions so that the bubbles are not noticeably noticeable when the surface of the concrete is viewed from a distance of 3.0 m. If this condition is not fulfilled, the Contractor is obliged to perform the processing of such visible inner surfaces by filling and smoothing the bubbles using an epoxy resin (sica top, etc.). It is only the Supervisory Authority that is authorized to decide whether it is necessary to process such areas or not in the sense of the above provisions.

If the visible inner surfaces of the concrete under the cladding serve as a basis for placing various types of wall claddings (tiles, plastering, dyeing, etc.), they will be processed in a way that requires processing of the substrate for these types of works. The constructor is obliged to process the surfaces under the formwork so that all bubbles and even other local unevenness will be leveled and polished using an epoxy resin based preparation (sieve top, etc.).

4.4.10.5. Payment

The cost of surface treatment of concrete will not be paid separately but will be included in the unit price of the embedded concrete.

4.4.11. Permitted deviations from the projected dimensions of concrete structures

4.4.11.1. Unevenness

All concrete surfaces, which do not have a working joint connection, are obliged to perform the Contract exactly in accordance with the shapes and dimensions given in the respective drawings. Possible unbalances must be within the limits of these Technical Requirements for Tolerances. In all cases where the unevenness is higher than prescribed, it shall be considered that the concrete is rendered inferior and the Contractor is obliged to perform repairs of such uneven surfaces of concrete in accordance with the provisions of item 4.4.13. These technical conditions will not be applied. The concrete surfaces will be measured by flat surfaces, ie the corresponding patterns for the curved surfaces. Measured lengths will be 1.5 m for surfaces that are paved with cladding, ie 3.0 m for surfaces that are concreted without the use of formwork.

4.4.11.2. Dimensional tolerance

For all concrete and reinforced concrete structures, the following deviations of the dimensions of the cross sections of the pillars, beams, foundations and thickness of slabs and walls are permitted.

- for surfaces that are filled from 12 mm to -6 mm

- for visible areas from 6 mm to -3 mm

- for wet surfaces from 4 mm to -3 mm.

The plus sign () refers to the increase of the thickness of the elements and the name with positive tolerance, and the minus sign (-) refers to the reduction of the thickness of the elements and is called the negative tolerance.

- deviation from the projected to 3 m / - 3 mm

- directions up to 6 m / - 6 mm

- deviations for all other projects me more / - 12 mm

With the maximum negative tolerance conditioned by the thickness of the elements which can not be less than 97% of the projected thickness.

4.4.11.3. Tolerance of unevenness

In accordance with the provisions of point 4.4.11.1. the height of the local unevenness for surfaces in the formwork must not exceed the following limit

- for buried surfaces / - 12 mm

- for visible surfaces / - 3 mm

- for surfaces in contact with water / - 2 mm

In case of exceeding the prescribed tolerances, both the dimension and the irregularity, the Supervisory Body is competent to decide in each specific case, depending on the type of overrun on other factors (fitting into equipment, aesthetic appearance, stress conditions in the construction, etc.), is this Overruns may tolerate or will be treated as non-quality concrete and ordered appropriate repairs in accordance with the provisions of point 4.4.13. of these Technical Terms.

4.4.12. Payment and scaffolds

4.4.12.1. General

In all places, where this is the shape and the position of the construction and the adopted working methods, the Contractor will build and mount the appropriate scaffolds and lay the appropriate formwork to form the concrete according to the dimensions given in the drawings and ensure that it is carried to the curing. The following types of formwork

- OR 1 Coarse flat formwork - according to the requirements of points 4.4.10.2/a and 4.4.11;

- OR 2 Ordinary flat formwork - according to the requirements of points 4.4.10.2 / b and 4.4.11;

- OR 3 Special flat formwork - according to the requirements of points 4.4.10.2/a and 4.4.11.

NOTE: The formwork between the working joints of concrete concrete blocks is not specifically specified. Its price is included in the price of concrete. The working joints include the surfaces of various niches and openings into which the coating is later deposited with secondary concrete.

4.4.12.2. Material of formwork and scaffolds

The material for the production of all formwork and scaffolding is not specifically specified and it is allowed to use various materials, such as wood, sheet metal, steel structures, hardwood and similar wood manufactures, prefabricated or made and tailored to the face of the site and the like that all materials applied meet the relevant standards and regulations regarding quality and carrying capacity, as evidenced by appropriate approvals. The use of steel formwork is mandatory for placing prefabricated elements.

4.4.12.3. The formwork and scaffolding

Contractor is obliged to submit to the Supervising Authority the approval of the formwork and scaffolding project that he intends to apply in the concrete project. The contractor is obliged to prove the stability and bearing capacity for all phases of the works. The stresses in all elements of the formwork and scaffold must be within the limits of the allowed voltages for the applied materials and temporary objects. All elements must be calculated for receiving the load from the weight of fresh concrete and horizontal pressing on the formwork. For all elements, calculations and deformations of the scaffolding and formwork must be calculated, and the project must anticipate the appropriate elevations.

Within the framework of the concession approval, the Supervisory Body will, after mounting and mounting and fixing the scaffolding and formwork, control all parts and dimensions. If the formwork and scaffold are relying on already constructed structures and structures, the load capacity of these loads must be demonstrated. The formwork and scaffolding project The contractor must ensure the load capacity of the whole load of own weight of concrete and useful cargo that can occur during this period. In the case of phase concreting, the Contractor may also be allowed to download the load on concrete of the next phase, by the construction of the previous phase, if such a sequence of execution is foreseen by the Main Project, or if the stability and the carrying capacity of the previous phase and the final state of the building are not jeopardized. If this order is approved upon the request or the Contractor's wish, and stability and load control demonstrate that additional reinforcement is necessary in relation to the main design, all this additional reinforcement will be at the expense of the Contractor's cost without the right to collect that additional reinforcement. Regardless the approval of the project and the execution of the layout and scaffolding is the sole responsibility of the Contractor for the correctness of the same, as well as their stability and security.

4.4.12.4. Making, installing and disassembling formwork and scaffolds

It is obligated to use on all objects and parts of buildings, where it is physically possible, the sliding system of assembly and disassembly panels with multiple use. Cutting of formwork on site will only be allowed in cases of cement mortar. The gap between the individual boards should not be greater than 1 to 2 cm. All the elements for forming the moldings that are bent as anchors in the previous phase or block must be sufficiently far from the edge of the concrete to avoid breaking and edging of the edges.

The design and assembly of the formwork and scaffolds must satisfy the theoretical forms given by the project and fulfill the requirements of item 4.4.10. and 4.4.11. These Technical Requirements. Cutting, assembly and disassembly of plates and scaffolds must be carried out only by qualified workers for this type of work. The contractor is obliged to take all safety measures for working on the altitude, for access to the plating and scaffolding, for cleaning, control and reception must be secured. The surface of the formwork must be cleaned, without the remains of cured concrete and other materials and, as a rule, coated with motor oil or the like easier separation of concrete. Prior to the concreting, all surfaces of the wooden formwork will be wetted with water to prevent the moisture from withdrawing from fresh concrete.

The lateral formwork will be removed only when the concrete reaches a compressive strength of at least 3 Mpa, which is determined from the previous testing of concrete. The horizontal, supporting formwork and scaffolding will be relaxed and removed only after the period determined by the scaffolding and formwork project, when the stability and carrying capacity of all phases of concrete has been demonstrated. The decision to relax and remove these forms and the appropriate scaffolds shall be made by the Supervisory Body.

4.4.12.5. Measurement for payment

The measure for payment of the formwork and scaffold will not be performed.

4.4.12.6. Payment

Payments will not be paid separately. All costs of formwork and scaffolding will be covered by unit prices of concrete.

4.4.13. Procedure with concrete that does not correspond to the proper quality

4.4.13.1. General

In all cases where the result of control tests of concrete shows that the built-in concrete does not have the required quality characteristics, the Contractor is obliged to perform repairs of concrete in the manner specified by the Supervisory Body, unless the Supervisory Authority orders the removal and replacement of such concrete. In addition, in all cases, even when the control tests of the concrete prove the required quality, but visual inspection of the performed concrete reveals poor and non-performing areas.

The Contractor is obliged to make corrections to these zones in the manner specified by the Supervisory Body. Under poor and inadequately developed zones, there is the existence of insufficiently homogeneous and unbounded and segregated nests, the appearance of cracks, humidification and leakage through concrete, as well as any other concrete damages that are evidently results of incomplete and ineffective work and failure to comply with the provisions of these Technical Terms or instructions received from the Supervisory Authority.

Concrete repairs will be done in principle by replacing low-quality concrete or by injecting it, with the Authority may also determine a combination of these two modes or a third method, which is not specified in these Technical Terms, when it can be clearly concluded that the specified repair method will not provide satisfactory the results, or when the Contractor proposes, and the Supervisory Body approves such a method of repairing concrete. It shall be for the Contractor to perform any repair work on concrete before the Supervisory Authority has ordered or approved the manner of performing the repair for each specific place.

All repair costs for concrete whose test results show that the built-in concrete does not have the required characteristics, the costs of additional work caused by it and the costs of repair described in item 4.4.11.3. These Technical Terms will be fully met by the Contractor.

4.4.13.2. Replacement of non-quality concrete

The replacement of low-quality concrete will be done by punching or by some other means removing the concrete area that is determined as being ineffective and re-concreting. Depending on the size of the removed concrete, and the accessibility and possibilities of placing the formwork, the Supervisory Authority will prescribe the maximum aggregate grain and fill method, which can be concreting or ticketing.

The connection with the 'old' concrete will in principle be achieved by the appropriate shape of the hammer, but the inspection body may require coating with materials that will accelerate the adhesion of the new concrete to the old for the open openings. The edges of the hammer must be sharp with a minimum depth at the margin of 1.5 maximal aggregates.

If the zone of poor quality concrete involves the place where the reinforcement is installed or any other building element, the Contractor is obliged to remove the poor quality concrete without damaging the reinforcement or the installation element and clean it in the proper way before the new concrete. In all places where the replacement of low quality concrete or if the replacement takes place on the vital supporting parts of the building, the Supervisory Authority may order the installation of additional reinforcement, anchor or steel grids to more firmly connect new and old concrete. All these works shall be performed by the Contractor at their own expense.

4.4.13.3. Injection of low-quality concrete

The injection of non-quality concrete will be done in principle in cases where the results of the control tests are unsatisfactory and the quality of the concrete must be improved in the interior of the block as well as in the cases of humidification and leakage through improper concrete. The number, direction and dimensions of the injection wells, the composition of the mixture, and all other injection parameters will be determined by the Supervising Authority for each specific case, depending on the position and dimensions of the non-quality zone, ie the block and the role in the constructive system and the performance possibilities. In cases of humidification and leakage through non-quality concrete, injecting with special mixtures and materials such as 'syprogel' and the like will be applied.

The working pressures for the injection of low quality concrete will be determined so that no further damage and spraying of concrete occurs. All works are to be performed by the Contractor at their own expense.4.4.13.4. Demolition of Concrete Structures If the Supervisory Body determines that the constructed concrete structure substantially deviates from the required quality and designed measures and thus endangers the general stability of the building, its function and endangers the life and health of people and that the above findings can not be remedied, the order will be issued to the Contractor for demolition of quality The works for the demolition of improperly executed works listed in the previous paragraph can also be issued during the technical reception of the building.

4.4.13.5.              Payement

The conditions for the quality of concrete that are carried out in aggressive environments are as follows:

• the concrete must be as thick as possible

• the granulometric composition of the aggregate and the amount of cement must be chosen so that the freezing, uniformly dense concrete can be obtained by the compression procedure,

• the aggregate is added to the concrete the air content is limited to max. 5%

• Make concrete bodies preferably without interruption, and if the working joints are necessary then predict them below or above the level of fluctuation of water levels.

When setting the concrete, carefully remove the concrete sludge layer on the connecting surfaces. Removing this layer is done after the finished bonding

• Contact with aggressive water and fresh concrete mass is not allowed. If the Authority exceptionally allows concreting in aggressive water, the cement content must not be less than 400 kg / m3 of ready-mixed concrete.

In very aggressive waters it is necessary to take additional measures of concrete insulation with coatings, pastes, asphalt mastic, artificial sheaths, clinker cladding, or clay isolation or insulating strips.

• The protective layer for the reinforcement must be min. 4 cm thick

• All reinforced concrete elements must be made of the same type of cement.

4.4.14.3. Measurement and Payment

The payment method will be made according to the dimensions of the concrete section, which is protected against the aggressiveness of water and soil. Payment will be made per m3 of specially built concrete resistant to the aggressiveness of water or soil.

The Contractor is obliged to add additional cost per unit of concrete measure at the indicated place of his offer, which is caused by adapting the quality of concrete to the aggressive effects of water and soil.

4.4.15. Measurement and payment of concrete

4.4.15.1. Measurement for payment

The measure for the payment of concrete works will be done according to the amount of embedded concrete within theoretical formwork lines and theoretical excavation lines, including the concrete built into the approved overpass. The volume of the embedded reinforcement and the upleted parts and holes, or the otter in concrete whose individual volume is less than 2000 cm3, or an individual surface of less than 20 cm3, regardless of length, will not be deducted from the amount of embedded concrete.

4.4.15.2. Payments

Payments will be made according to unit prices for cubic meter of embedded concrete. The unit price includes, among other things, all work, material, machinery and all other costs related to the purchase of concrete components, including washing, separating aggregates for concrete; procurement, transport and storage of concrete additives; water consumption, then the handling, transport, installation and maintenance of concrete; construction, assembly and dismantling of all scaffolding and formwork, including formwork of all holes and holes, galleries and shafts; all processing of concrete surfaces, preparation of all basic and working joints and other concrete surfaces, including all preliminary and control tests of concrete and its components, the production of all concrete projects with accompanying drawings and calculations, and lighting needs for work at night, all in harmony with the provisions of these Technical Terms and the relevant applicable Regulations, Regulations and Standards and the Agreement. The Contractor will not be granted any request for additional payment of any costs related to concrete, but the Contractor is obliged to include all the costs in the unit price of concrete.

5. ARMY WORKS

5.1. GENERAL PROCEDURE with reinforcement works, in terms of these Technical Terms, means the procurement, cutting, cutting, bending, folding, cleaning, setting and fixing of steel reinforcement.

5.2. TYPES AND QUALITY

The reinforcement works will be performed according to the shapes and dimensions given in the drawings or as ordered or approved by the Supervisory Authority. All reinforcement works will be used with high-grade naturally rigid steel RA400 / 500-2 and smooth steel GA 240/360 . The quality of steel and its characteristics must satisfy all the requirements and requirements laid down in the Ordinance on Technical Measures and Conditions for Concrete and Reinforced Concrete, as well as the Regulations on Technical Regulations for the Use of Concrete Steel for Reinforced Concrete and the Regulations on Technical Measures and Conditions for the Use of Reinforcement in reinforced concrete structures.

5.3. PURCHASE AND STORAGE OF ARMATURE

In each delivery of reinforcement, the Contractor is obliged to submit to the Supervising Authority the appropriate attestation on the quality of steel. It is not allowed to ship to the construction site and store any reinforcement without proper attestations, as well as reinforcement which according to the attestations does not meet the prescribed and required quality.

The reinforced reinforcement on the construction site must be sorted by diameter and stored. The armature can be stored in an open space. All fittings must be fitted to the appropriate holders so that storage is not allowed directly on the ground. If the Authority does not determine, the reinforcement must be separated by individual deliveries, so that the reinforcement of the same diameters and the different deliveries is not mixed. When it comes to deliveries from different manufacturers, that is, deliveries with different quality to attestations, this separation by delivery is mandatory.

The supervisory authority may allow the Contractor to deliver to the construction site pre-cut and reinforced reinforcement by position, whether cutting or tailoring by the manufacturer of the reinforcement in his workshop or a permanent Workshop of the Contractor or his co-producers outside the construction site. In this case, the Contractor is obliged to enable the Supervisory Body control of the work in all cases, the Contractor is obliged to place visible and transparent plates with markings of the diameter of the reinforcement, the manufacturer and the date of delivery, as well as the number of positions of the appropriate one in the armature warehouse. element.

The Contractor is obliged to provide on the construction site sufficient ammunition of all necessary diameters in order to be able to carry out all works according to the dynamics, and in case when the Supervisory Authority in accordance with the provisions of item 5.6. stop the use of individual deliveries. The Contractor is obliged to keep a record of the delivered, stored and installed armature through the journal of reinforcement works, so that at any time the Authority can determine the exact condition of the reinforcement at the site.

5.4. SETTING ARMATURES

Armature attachments must be executed in the manner and in places as shown in the detailed drawings, or as ordered or approved by the Supervisory Authority. In the event that the Contractor wishes to continue the reinforcement at the place where this is not provided for in detailed drawings, such sequences can only be carried out upon approval by the Supervisory Authority. The attachments will be carried out by appropriate welding or welding, provided that it has an attestation that the concrete steel is weldable. Fixing of the reinforcing rods The contractor will perform the automatic machine on the interface in such a way that the welded joint has at least the same tensile and tear strength as the base material. In exceptional cases, the Inspection Body may also permit welding or overhanging on the spot, but only the certified assemblies with suitable electrodes may be used, and according to the regulations for welded steel structures.

5.5. CONSTRUCTION, FITTING AND SETTING ARMATURES

The contractor is obliged to perform according to detailed drawings and specifications, as well as any amendments ordered by the Supervising Authority. The reinforcement of the reinforcement rods shall be performed by the Contractor in the cold state. The forward rods must be discarded and replaced with a new one. When fitting, each reinforcement bar must be cleaned of rust, oil, grease, earth or any other material which may cause a reduction in adhesion between steel and concrete.

The installed reinforcement must be firmly fixed and connected. Fixing of the reinforcement to the projected position can be done by means of steel or concrete pads, armature skeletons and stools, without the use of article pads with outer surfaces. Interconnecting and fixing of the reinforcement will be carried out with ignition wires and hedging - short guards. For the use of the fitting of the reinforcement into a unique grounding system, the Contractor will perform the appropriate valves on the fitting. The quality, thickness and arrangement of the welds will be given to the main project. All welding of the reinforcement, either for earthwork purposes or for fixing purposes or on-site construction, can be carried out exclusively by certified assemblies with appropriate equipment and electrodes.

Unless otherwise indicated in the drawings, the protective layer of concrete is 4 cm for all the dirty surfaces and surfaces in contact with the ground. Permissible tolerance in the thickness of the protective layer is - 0.5 cm. The distance between the two adjacent bars distributed by a meter can vary by ± 1/20 of the projected distance, but the distance between each quarter of the rod can not be higher than the projected one. At the beginning of the concreting, within the concession approval, the Authority will execute inspection and control of the reinforcement in accordance with the provisions of item 4.4.2. of these Technical Terms.

5.6. PRELIMINARY AND CONTROL EXAMINATIONS

Preliminary and control tests of the armature shall be considered all the manufacturer's attorneys to be submitted by the Contractor to the supervisory authority, before the commencement of delivery and for any new deliveries. The contractor is obliged, through an authorized organization, to carry out preliminary and control tests of the reinforcement of the reinforcement by welding. Prior to the commencement of works, as in all cases of technology change, that is, the welding machine, preliminary testing of the strength of the tensile and tearing of the interfering welding will be carried out on ten samples of all diameter of the reinforcement (which will continue to be welded) welded to the interface by the construction automatic station. The same scope of testing The contractor is obliged, through an authorized organization, to perform the control examination at least once every six months for the duration of reinforcement works. These control tests may omit the diameter of the reinforcement that will not be used on the construction site until the next control test. In exceptional cases when the Authority suspects the quality of the delivered and stored reinforcement (unevenness of the results given in the certificates, significant corrosion and the like), the Contractor will be obliged , at the request of the Supervisory Authority and through an authorized organization, carry out control tests of such reinforcement.

The type of control tests in this case will be prescribed by the Supervisory Authority, but it may be necessary to inspect the diameter, tensile strength and stretching limits, maximal elongation at break and flexural flexibility. Tensile strength control, stretching limits and maximum elongation will be carried out on ten samples, and other tests will be carried out on six samples.

In order to perform preliminary and control strength tests for tensile and tearing of welded joints, as well as possible inspection of the reinforcement, the Contractor may, if economically justified in the construction laboratory, install the appropriate equipment and instruments, and may carry out this testing in the laboratory of an appropriately authorized work organization, approved by the Authority. In this case, sampling, packaging and referral to the control examination of the samples will be carried out with the full control of the Supervisory Authority. The costs of the preliminary and control tests of the welded joints and fittings will not be paid in particular.

If any inspection of the welded joints on tightening and tearing does not give the required results, the Contractor will not be allowed to use the welded reinforcement further. The re-use may only be allowed after a further preliminary examination proves the validity of the welding technology technology. If the results of any inspection of the reinforcement made by the special request of the Supervisory Authority do not show the required results, the Supervisory Authority will decide on the further treatment of this reinforcement, including any order for removing it from the site.

5.7. MEASUREMENTS FOR PAYMENTS

The payment for reinforcement works will be based on theoretical weights, and according to the detailed specifications given to the contractor drawings, that is, approved or ordered by the Supervisory Authority. All fittings that the Contractor installs for their needs will not be measured and paid.

5.8. PAYMENTS

Payment will be made according to unit prices per kilogram of reinforcement. Unique prices are unique for individual objects or parts of objects, and according to specification in the area. The contractor is obliged to take into account the complexity of the design and mounting of the reinforcement on the basis of his experience and project backgrounds when forming unit prices in the offer.

The unit prices include all work, material, machinery and all other costs related to the procurement, transport, storage, continuation, cutting, bending of all shapes, cleaning and installation of reinforcement, including all welds for grounding purposes and all auxiliary skeletons, sticks and other fittings for the Contractor's needs and inspection, all in accordance with the provisions of these Technical Terms.

5.9. RULES, REGULATIONS AND STANDARDS (ARMING WORKS)

The reinforcing steel must comply with the following regulations and standards

• Regulation on Technical Measures and Conditions for Concrete and Reinforced Concrete (PBAB) No.11 / 87

• Technical regulations for the use of corrugated concrete steel for reinforced concrete from 13.8.1965

• Rulebook on technical measures and conditions for the use of mesh reinforcement in reinforced concrete structures

• General structural steels, technical conditions for the production and delivery of JUS CB021

• Concrete steel round (hot rolled) JUS C.6.020

• Tor - steel - technical regulations for making JUS C.K6.021;

• Concrete steel - shape and dimensions of JUS C.K6.120.

6. CARPENTRY WORKS

These must have the characteristics of the PTP (Solution MG FNRJ No.11850 / 49). All carpentry works must be carried out according to the project, details and instructions of the Supervisory Body. The wood material, used as permanent in the structures, must be sound, dry, in accordance with the applicable technical regulations, the dimensions of the project and the correct connections according to the calculation of connections. The quality of the material can be tested load. The costs of testing are borne by the Contractor if the result of the test is negative. The contractor is legally responsible for the stability of the scaffold and support.

The tenderer (sole tenderer or joint venture/consortium) must fulfil the following criteria:

- Has following Key personnel:

  - one architect-conservator, with appropriate licences of responsible work executor for construction, and appropriate diploma/certificate for conservation and minimum 5 years of post graduate professional experience,

  - one civil engineer with appropriate licences of responsible work executor for construction and minimum 5 years of post graduate professional experience,

  - one person with appropriate licences of responsible work executor for electro works (strong current) and minimum 5 years of post graduate professional experience,

  - one person with appropriate licences of responsible work executor for electro works (weak current) and minimum 5 years of post graduate professional experience,

  - one person with appropriate licences of responsible work executor for mechanical installations and minimum 5 years of post graduate professional experience,

  - one person with appropriate examine/certificate about safety and healthy at work and minimum 5 years of post graduate professional experience,

  - one graduated archaeologist with minimum 3 years of post graduate professional experience

The tenderer must submit in the tender the CV and copies of diploma/degrees/ employer`s certificates proving the qualifications and experience claimed in the CV of the staff above.