# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply and delivery of fire fighting vehicle	p 1 /…

Publication reference : RORS37/Mosna/TD6

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Fire fighting vehicle |  |  |  |
|  | Vehicle basis – chassis |  |  |  |
|  | Engine Engine power: minimum 185 kW Maximum torque: minimum 850 Nm No of cylinders: 6 cylinders in line Euro class engine: minimum EURO 5 Fuel: Euro-diesel Fuel filter heater Engine cooling system: water Operating capacity of the engine: maximum 6000 cm3 |  |  |  |
|  | Transmission Manual, 6 synchronized gears + 1 reverse |  |  |  |
|  | Driving formula 4х4 |  |  |  |
|  | Wheels Single - front and rear with two pads for the wheels, Tool for changing wheels and tires refilling hose |  |  |  |
|  | Pneumatics For road and off-road conditions (5 pieces) |  |  |  |
|  | Cab Short, daily, 1+2 seat Driver's seat with air suspension and safety belt and installed porter of breathing apparatus for respiratory protection at the seatbacks and headrests Passenger seats with safety belts and installed porters of breathing apparatus for respiratory protection at the seatbacks and headrests Air-conditioner Digital tachometer Electric glass lifters Tinted glass Trip computer Cruise control Tipper cab |  |  |  |
|  | Suspension: Front and rear parabolic springs, telescopic hydraulic shock absorbers |  |  |  |
|  | Braking system: ABS |  |  |  |
|  | Steering system: Servo, adjustable per height and gradient |  |  |  |
|  | Chassis dimensions: Length: maximum 6000 mm Width: maximum 2500 mm, without rearview mirror Height: maximum 3100 mm |  |  |  |
|  | The maximum permissible total weight: 15000 kg |  |  |  |
|  | Maximum length of vehicle (chassis with vehicle bodies): 6200 mm |  |  |  |
|  | Vehicle bodies/upgrades |  |  |  |
|  | Water tank: Capacity: min. 3000 liter Material: stainless steel Reliance: via elastic supports on the sub-frame Construction: with anti-wave divisions, with hatch for revision with a lid diameter of minimum 500 mm and an overflow system with a valve for pressure and vacuum; Equipment: electric water level sensor; connection for the hydrant chrging with stable and blind coupling and the ball-shape valve; ports for connection to the pump and the tank discharge |  |  |  |
|  | Foam tank: Capacity: min. 300 liter Material: stainless steel Equipment: filling hatch; valve for pressure and vacuum; electric encoder of foam level; ports for connecting to a system for mixing and emptying tank |  |  |  |
|  | Firefighting pump: Type: combined firefighting pump with the medium pressure level (10 bar) and the high pressure level (40 bar) Capacity: min. 2000 l/min at 10 bar and 400 l/min at 40 bar Standard: the pump must be in compliance with standards for firefighting pumps Materials: Case and rotor for medium pressure level - aluminum; Case for high pressure level – stainless steel; Rotor for high pressure – bronze; Pump shaft – stainless steel Construction: one rotor for medium pressure and one rotor for high pressure at the same shaft; the pump must provide work with medium pressure separately, with high pressure separately and simultaneously with both medium and high pressure; the pump must be equipped with a vacuum device to suck up water from external sources to a depth of 8 m; the pump must be equipped with a system to protect against overheating, which turns on automatically at 55oC and a tap to drain water to prevent freezing; Drive: via transmission cardan shaft Position: in the rear storage of the body/upgrade Mixing system: the pump must be equipped with a pre-mixer for dispensing the foam, from 0-6%; with possiblity to withdraw foam from the outer dispenser. |  |  |  |
|  | Control panel: Commands – minimum requirements: water from the tank into the pump; tank filling - circular; turning on high pressure capstan; turning on medium pressure capstan Instruments – minimum requirements: Mano-vacuum-meter for pressure at the pump inlet; Manometer for medium pressure; Manometer for high pressure; Electric water level indicator in the tank; Electric foam level indicator in the tank; Speedometer pump with counter of operating hours; Switch for adding and subtracting the gas (hand-held gas); STOP switch |  |  |  |
|  | Storage boxes: The supporting structure of upgrades must be made of stainless steel. The interior space is coated with anodized aluminum and stainless ribbed tin. The roof the upgrade is coated with anodized aluminum and stainless ribbed non-slipping tin, with the built-in supporters for the equipment that will be located on the roof of the vehicle and the low, protective fence. The sides and back box are closed by aluminum rolling doors with rubber gaskets, which prevent the penetration of water and dust. Aluminum rolling door are closed by external locking and are completely protected against atmospheric influences. The storage boxes must be equipped with the installed carriers, holders and compartments for all the required equipment. At the back of the upgrade, on the right, there should be built-in bending ladders ladder for climbing onto the roof of the vehicle. |  |  |  |
|  | Installation of water and foam: Material: The installation must be made of steel pipes protected by hot galvanizing. All small elements of the installation of water and foam (elbows, branches, connectors) must be made of stainless materials. Connectors: Intake from the tank to the throttle valve and flexible connection; connection for the withdrawal of water out of the vehicle, with coupling ø110, two discharge ports on the pressure valves and fittings ø75 on both sides of the vehicle; pressure pipeline for secondary capstans of medium and high pressure, pressure pipeline of the water or foam jetter. Vehicle auto-protection system: allows the maximum safe access to the fire and must consists of a nozzle for auto-protection of the wheels and windscreen and pipeline with navigation valves. |  |  |  |
|  | Monitor water/foam: Capacity: min. 1200 l/min at 8 bars Jet range: water - min. 50m, foam – min. 35 m Handling: manual Rotation: horizontal - 360°, vertical – from - 15 to +75° Jet form: full and dispersed Other: two brakes to ensure the horizontal and vertical position; manometer |  |  |  |
|  | Fast-intervention capstan: Medium pressure capstan: equipped with a minimum 25 m of incompressible rubber hose, inside diameter of Ø32 mm and the nozzle for water or foam, capacity minimum 200 l / min at 8 bars - with adjustable spray pattern (full and dispersed) High-pressure capstan: equipped with a minimum 60 m of incompressible rubber hose, inside diameter of Ø19 mm and the nozzle for water or foam, capacity minimum 200 l / min at 40 bars - with adjustable spray pattern (full and dispersed) and extension for foam that is placed on the nozzle enabling extinguishing by foam. Winding: winding of capstans should be by electrical drive with the possibility for manual winding. Other: capstans must be equipped with a security brake and the hose router. |  |  |  |
|  | Auto-pulling capstan: Pull power: min. 6800 kg on a single cable Drive: electric motor, 24V Command: wireless radio control Cable length: min. 28 m Cable diameter: min. 10 mm Brake: automatic, direct in drum, with limiter Gear: three-stage planetary |  |  |  |
|  | Light signaling: Voltage: 24V Navigation: from the cab Roof signaling: integrated device – console with LED blinkers flashing blue, protected from mechanical damaging. Blinkers: two LED blinkers flashing blue on the front side of the vehicle; four LED blinkers flashing blue on the back side of the upgrade; voltage 24V; navigation from the cab Outside lights: two LED spotlights on the outer sides of the upgrade roof – on the left and on the right to light the area around the vehicle; Inside lights: boxes for equipment and pump area must be lighted. |  |  |  |
|  | Sound signals: Howling siren and sound system (amplifier 100W) with a microphone in the cab. |  |  |  |
|  | Color: Vehicle: fire-red RAL3000 or equivalent, Chassis: black Fenders: white |  |  |  |
|  | Equipment: Pressure hoses Ø 52, min. 15 m, 10 pieces Pressure hoses Ø 75, min. 15 m, 4 pieces Universal keys, 3 pieces Key to underground hydrant, 1 piece Hydrant extension, 1 piece Intake hoses Ø 110, L=1,6m, 4 pieces Intake basket Ø 110, 1 piece Ropes for tying the intake port, 2 pieces Divider B/CBC, 1 piece Collector A/2B, 1 piece Backpack, 10 pieces Fire axes, 2 pieces Pick, 1 piece Shovel, 1 piece Extinguishing broom, 10 pieces Handsaw, 1 piece Chainsaw, 1 piece Universal nozzles Ø 52, 3 pieces Universal nozzles Ø 75, 2 pieces Nozzle for medium foam Ø 52, 1 piece Nozzle for heavy foam Ø 52, 1 piece Portable fire extinguisher S-9, 3 pieces Set of hydraulic demolishing tools, 1 piece |  |  |  |
|  | Specification of hydraulic demolishing tools |  |  |  |
|  | Driving aggregate: Type of engine: four-stroke engine Type of fuel: gasoline Engine power: minimum 2 KW No. of ports: 2 pieces Weight: maximum 25 kg |  |  |  |
|  | Scissors: Pressure: maximum 720 bars Cutting force: minimum 920 kN Opening of operating parts: minimum 180 mm Cutting round bars: minimum Ø 38 mm Weight: maximum 23 kg Type of scissors: pliers |  |  |  |
|  | Breaking tool: Operating pressure: maximum 720 bars Range of operating parts in the end position: minimum 680 mm Demolition force: minimum 300 kN Weight: maximum 22 kg |  |  |  |
|  | Piston: Operating pressure: maximum 720 bars Type telescopic piston: 2 or more degrees Body length of the piston: maximum 750 mm Tool length in the operating position: minimum 1200 Stretching force of 1st degree: minimum 250kN Stretching force of 2nd degree: minimum 80 kN Weight: maximum 22 kg Piston support –“P”: 1 piece |  |  |  |
|  | Obligatory equipment Installation with two connections (installation of operating and reverse pressure in one joint): 2 pieces Installation length: minimum 5 m |  |  |  |
| 2 | After-sale service Proposal for after-sale service with recommendation of the manufacturer |  |  |  |
| 3 | Training Training for drivers of the vehicle: duration minimum 2 hours; Training technical utilization of the firefighting equipment: duration minimum 6 hours; |  |  |  |