# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Purchasing of field fire fighting vehicle and fire fighting cistern	p 1 /…

Publication reference : RORS 40-9/2018

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | All requirements mentioned and requested in this document are minimal and mandatory |  |  |  |
| 1. | FIELD FIRE FIGHTING VEHICLE |  |  |  |
|  | Type of vehicle – Forest firefighting vehicle Driving formula: 4x4. Motor: Diesel 4-tact, turbo charger Fuel: Euro-diesel Engine power: minimum 120 kW Cooling: water based Characteristics of undercarriage Axial range: Minimum 3000 mm Length of undercarriage: Minimum 5200 mm Width of undercarriage: Minimum 1800 mm Useful bearing capacity of the vehicle: Minimum 870 kg Cabin: airconditioned Seats: 2+3 Crew seats equipped with safety belts Doors: 4 Installation for vehicular radio station with installed antenna Gear box , manual gearbox with clutch Breaks: with ABS system Steering mechanism: servo device Steering wheel: adjustable from the left side in direction of the vehicle motion Wheels: standard for manufacturer, for basic chassis and spare wheel of the same type and size as the basic wheels Color of the vehicle: painted parts of the vehicle, cabin, upgrade, construction are to be painted in red color RALL 3000 Light signalization: integral device with rotating blue lightings, protected at the upper part of the vehicle. Sound signalization with howling siren, surround and megaphone (siren 100W), command board with installed microphone situated in the cabin. Day light set Electrical installation voltage minimum 12 v Tools and equipment for the vehicle: Standard tools for replacement of pneumatics-wheels Medical first aid kit, tow hook, reflecting vest, triangle, spare bulbs Fog lights Self-pulling owing winch installed at the front side of the vehicle. Euro hook Upgrade Upgrade is manufactured of aluminum profiles connected by welding of screw connections, which guarantees adjustable hight of shelves in the formed boxes. Hight of the upgrade may exceed the hight of the cabin for maximum 10-15 cm. Depository of cargo space Closed type with possibility of fast assembly/disassembly, with minimum 2 side roll-doors and 1 door with vertical opening on the back side (to approach the pump). Water reservoir capacity 300 liters, stainless steel – inox, hatch at the top of the reservoir, outlet with connection for pump, outlet for loading the reservoir and outlet for discharge of water. Reservoir for foaming agent – bucket 20 liters Pump aggregate Fixed pump is installed in depository at the rear side of the vehicle with possibility of fast assembly/disassembly. High pressure of minimum 40/100. High pressure reel length minimum 60 meters with gun nozzle and extension for extinguishing fires by foam. Sub-mixer Automatic dosage in all scopes of pump operations 0% or 6% Possibility to draw foaming agent from external container Control dashboard – water-resistant with manual switch on/switch off of drive and operating functions (switches), positioned at the back part of the upgrade. Drive SUS motor of minimum 12 KS. Equipment All equipment must be in line with SRPS, DIN standard or EN norms, intended for usage by professional firefighting units, has to have its bearings and to be individually fixated with special girders. Installed equipment needs to be individually fixed with girders in their bearings. List of equipment Mobile accumulator lamp – hand reflector with charger, "Ex" protection - 1pcs Hand lamp with charger - 1 pcs Manual mobile fire extinguisher with dust, S-9 kg - 1 pcs Mobile device, CO2-5 kg - 1 pcs Backpacks, water capacity 25 liters- 3 pcs Brushes - 3 pcs |  |  |  |
| 2. | FIRE FIGHTING CISTERN |  |  |  |
|  | Type of the vehicle – Truck for extinguishing fire with water and foam Type of the vehicle – Truck for extinguishing fire with water and foam Undercarriage is delivered with installed original manufacturer`s parts, homologues Driving formula: 4x4. Continuous drive is in rear axis with engaging front axis as needed. Motor: Diesel, 4-tact, turbo charger Fuel: Euro-diesel Engine power: minimum 190 kW Cooling: Water-based Characteristics of undercarriage Wheel base from 3500-4500 Length of undercarriage minimum 7000 mm Width of undercarriage: minumum 2300 mm Hight of lowest vehicle point (clearance) nominally weighted- -Minimum 280 mm Access angle: Minimum 20o Outgoing angle: Minimum 20o Obstacle angle:: Minimum 20o Useful bearing capacity of the vehicle: Aligned with the weight of upgrade and equipment Cabin enables transportation of 2 firefighters-rescuers, airconditioned Number of doors: 2 Number of seats: 1 driver + 2 seats for crew members with safety belts Installation for vehicular radio station with installed antenna, vehicular radio station, VHS system and 2 manual radio stations VHS system Audio system – standard for manufacturer Gear box: manual gearbox with clutch Useful bearing capacity of the vehicle: Aligned with the weight of upgrade and equipment Total permissible weight: maximum 20.000 kg Steering mechanism: servo Steering wheel: adjustable from the left side in direction of the vehicle motion Wheels: standard for manufacturer, for basic chassis and spare wheel of the same type and size as the basic wheels Bottoming of front axis: steel Bottoming of rear axis: steel Fuel reservoir: capacity min. 300 km of travel and min. 4 working hours of installed equipment Color of the vehicle: painted in firefighting red color RAL 3000 Day light: set Electrical installation: voltage 24 V Vehicle tools and equipment: Standard tools for replacement of pneumatics-wheels Ser of accompanying tools, Standard set for basic chassis Medical first aid, tow hook, reflecting vest, triangle, spare bulbs Wheel chock for ensuring parking Reflecting foils installed at side and rear edge of the vehicle as per relevant regulations Self-pulling owing winch installed at the front side of the vehicle. Poll for hauling vehicles Front assembly for hauling vehicles Fog lights Tachograph GPS device with integrated map of Republic of Serbia Upgrade DESCRIPTION: Upgrade is manufactured of aluminum profiles connected by welding of screw connections, which guarantees adjustable hight of shelves in the formed boxes. Hight of the upgrade may exceed the hight of the cabin for maximum 10-15 cm. Installing water reservoir: for the water reservoir capacity of 6000 l, envision suitable elastic support which are installed at the undercarriage Material: Construction of aluminum profiles, covered by aluminum tin. Inner space: Covered by ribbed aluminum tin. Depositories: Minimum 4 side pieces and 1 on the rear side, with roll or classic doors with complete protection from external influences, and with outer locking system in accordance with EN 1846-2. Depository space: sufficient for envisioned equipment. For all equipment envision suitable holders with fixating stripes. Installed movable depositories – shelves need to be additionally secured by delimiters which prevent slipping out of the slider. Access to equipment: in accordance with EN 1846-2. Upper part of the upgrade: covered by protective material against sliding. Equipment installed at the upper part of the upgrade needs to be appropriately fixed and protected. Light signalization: protected, at the upper part of the vehicle, blinking blue lighting console with several modules. Modules are installed at the front and rear part of the vehicle within the upgraded size. Sound signalization: audio signalization with howling siren, surround and megaphone (siren 100W), command board with installed microphone situated in the cabin. Bearers at the upgrade: bearers for ladder, grapple and brushes Ladder for climbing the upper part of the upgrade: in line with EN 1846-2. External and internal lightning of the upgrade: external in accordance with EN 1846-2, internal in all boxes with manual switching. Reflector: reflector for illumination installed at the front, outer right side of the vehicle cabin, with operating commands from the vehicle cabin, with possibility to change the angle of the light beam on vertical and horizontal axis, minimum power 70W with installation of 24V. Water reservoir Capacity 6.000 liters, stainless steel - inox, Adjusted to manufacturing recommended support with breakwaters within the reservoir for prevention of sudden overspill. Reservoir hatch: at the top of reservoir, with the possibility to be opened from the upper part of the upgrade. Outlet with connection for the pump: F 110mm Outlet for loading the reservoir: F 75 mm on the side of vehicle Outlet for release of water: F 52 mm, at the lowest point of reservoir with extension toward the side of the vehicle with ball valve Venting pipe at reservoir Reservoir for foaming agent capacity 200 liters. Artificial material Reservoir hatch: at the top of the reservoir, with possibility to open from the upper part of the upgrade, with sub/pressure valve and possibility to measure the level Outlet connection sub-mixer of the pump F 25 mm Connection for release of foaming agent F 25 mm PUMPS: Fixed pump: installed in the depository on the rear side of the vehicle. Medial pressure minimum 10/2000 and high pressure minimum 40/400, and drive with cardan doors. Pump must have possibility to operate during the vehicle driving (at lower speed) Medial pressure: minimum 2 outlets with thrust irreversible valves f75 mm on the rear side of the vehicle (left and right from the pump) Medial pressure reel: 1 outlet F 32,5 mm High pressure reel: 1 outlet F 32,5 mm Vacuum device Sub-mixer: automatic dosage in all scopes of pump operation 3% or 6% Possibility to draw foaming agent from external container Control dashboard – water-resistant with manual switch on/switch off of drive and operating functions (switches). Positioned at the back part of the upgrade. Commands for medial pressure reel, high pressure reel, switching on the vacuum device, return line, water from reservoir, pre-mixer, setting the mano-vacuum-meter, manometer of medial pressure, manometer of high pressure, clock counter, measurer of water and foaming agent levels. Monitor Nozzle Installed at the roof of the vehicle or the upgrade. Nozzle of variable flow. Possibility of horizontal rotation to 360 degrees and vertical from +80 to -15 degrees. Minimal flow 1600 litres/minute WATER INSTALLATION Intake line: f 110 mm, for external draw of water Thrust line: minimum 2 outlets with connections f 75 mm, 1 outlet for medial pressure reel 1 outlet for high pressure reel Return line for unloading of thrust track Line for pre-mixer Live for circular work of the pump Sub-mixer: Automatic dosage in all scopes of pump operation 3% or 6% Control dashboard – water-resistant with manual switch on/switch off of drive and operating functions (switches). Positioned at the back part of the upgrade. Commands for medial pressure reel, high pressure reel, switching on the vacuum device, return line, water from reservoir, pre-mixer, setting the mano-vacuum-meter, manometer of medial pressure, manometer of high pressure, clock counter, measurer of water and foaming agent levels. Medial pressure reel: reinforced rubber hose according to standard EN 1947, length 25m f 32,5 mm with gun nozzle High pressure reel: reinforced multilayered rubber hose according to standard EN 1947, length 60m f 25 with gun nozzle at fast-binding connection FIREFIGHTING RESCUING EQUIPMENT All equipment must be in line with SRPS, DIN standard or EN norms, intended for usage by professional firefighting units, has to have its bearings and to be individually fixated with special girders. List of equipment Mobile accumulator lamp – hand reflector with charger, "Ex" protection-2pcs Hand lamp with charger-2pcs Devices for protection of breathing organs with composite tanks, for compressed air capacity approx. 6 – 7 lit 300 bar, with ''T'' bend-3pcs Hood for rescuing endangered person, mounted to device via “Y” or “T” bend connection-2pcs Spare composite tanks for compressed air capacity approx. cca 6 – 7 lit 300 bar, for device for protection of breathing organs-2pcs First aid kit in the bag, dimensions 400h300h150-1pcs Semi-static climbing rope, diameter up to Ø 10,5 mm length 30 m-2pcs Manual mobile fire extinguisher with dust, S-9 kg-2pcs Mobile device, CO2-5 kg-1pcs Transient connector A/B, wrought, “storc”, manufactured in line with standard DIN 14343 -2pcs Transient connector B/C, wrought, “storc”, manufactured in line with standard DIN 14342-2pcs Transient connector C/D, wrought, “storc”, manufactured in line with standard DIN 14341-2pcs Universal nozzle Ø 52 mm for water, with handle and arm with ball valve and turbine, minimum flow 100 - 400 l/min at 8 bar, with possibility of variety forms of water jet full to dispersed. Connection type “storc”, manufactured in line with standard DIN 14367 - 2pcs Bucket for foaming agent 25 liters, plastic 1pcs Thrust hose Ø 52 mm, length 15 m, with handles. Connection type wrought, “storc”, manufactured in line with standard DIN 14811 - 10pcs Thrust hose Ø 75 mm, length 15 m, with handles. Connection type wrought, “storc”, manufactured in line with standard DIN 14811 – 10pcs Thrust hose F 25, length minimum 20 meters. Connection type wrought, “storc”, manufactured in line with standard DIN 14811 - 5pcs Universal wrench ABC, for “storc” connections – 2pcs Universal wrench for surface hydrant 1pcs Shield – fan nozzle, Ø 52 mm – 1pcs Inter-mixer for foam with “storc” connections, 200 lit/min. – 2pcs Backpacks, water capacity 25 liters – 3pcs Rubber fishing boots, numbers: 43, 44, 45 -3pcs Dividing line C/2D with handles and ball valves, “storc” connections, manufactured in line with DIN 14365 2pcs Ladder, 4 connecting parts, aluminum, total length 6,4 m. 1psc Brushles 3pcs Grapples 2pcs Nozzle for medial foam, 200 lit/min – 1pcs Nozzle for heavy foam, 200 lit/min – 1pcs Nozzle for heavy foam, 400 lit/min. – 1pcs Pickaxe – 1pcs Bus bar A/2B. with “storc” connections, manufactured in line with DIN 14355. – 1pcs Dividing line B/CBC, with ball valves – 1pcs Plug (blind) joint, Ø 52 mm. – 1pcs Plug (blind) joint, Ø 75 mm – 1pcs Thrust hose, Ø 75 mm. length 5 m. – 1pcs Intake basket, Ø 110 mm. Connection type “storc”, manufactured in line with standard DIN 14362 - 1pcs Intake hose, Ø 110 mm, length 1,6 m. Connection type wrought “storc” manufactured in line with standard DIN 14557 4pcs Rope with the satchel for binding intake line, Ø 10 mm 2pcs Hydrant extension B/CC - 2pcs Wrench (T) for underground hydrant with za alternative square key – 1pcs Shovel, standard, length 1,5 m. – 1pcs Universal nozzle F 75, with ball valve and possibility of variety forms of water jet full to dispersed. Connection type “storc”, manufactured in line with standard DIN 14365 - 2pcs Snubber of jet reactions F 75 – 1pcs Universal nozzle F 25, with possibility of variety forms of water jet full to dispersed and possibility to stop water flow. Manufactured in line with standard EN 671. 2pcs |  |  |  |
|  | TRAINING |  |  |  |
|  | Training of the vehicle crew, for both vehicles and upgrades |  |  |  |