# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Purchasing of balloon, stage and info desk	p 1 /…

Publication reference : RORS92/City of Pozarevac /T4

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 1 Prefabricated stage - Balloon

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1.1. | Designing, procurement and set-up of a tensile sports hall with the gross dimensions of 24x50x10m (width / length / height) for the purposes of conducting cultural and sports events without the need for infrastructural equipment. -pcs 1 Contents of the inside of the hall: -the total available space for the exhibitions is 24x45m and within this space is an artificial turf covered sports court with the dimensions of 20x40m - changing rooms (of the width from the available space to the border of the hall, the length is up to the designer) – pcs 2 - toilets along with the changing rooms – pcs 2 The procurement is to be made according to the TURNKEY clause, which includes: - Procurement of the tensile sports hall with delivery and set-up at the site - Creation of project technical documentation with all of the necessary approvals and certificates - Set-up of the tensile sports hall according to the project documentation for which the Decision on the construction of temporary structures was issued by the competent authority of the Požarevac City Administration for the purposes of the set-up of the tensile sports hall. |  |  |  |
| 1.2. | Project technical documentation The connection of the facility to the communal and water supply infrastructure will be foreseen according to the conditions issued by the Public Utility Enterprise “Vodovod i kanalizacija.” Heating of the facility through the connection to the heating network will be foreseen according to the conditions issued by the Public Utility Enterprise “Toplifikacija.” The procuring entity is required to connect the heating installation/network to the site designated by the project designer. The contracting authority is required to request conditions from “Elektroprivreda Srbije”. To illuminate the interior of the hall, provide a sufficient number of adequately powerful LED flood lights, to provide illumination according to the standards applicable for this type of facility. Lighting fixtures must comply with the standard in their degree of moisture protection. Provide a solution for the drainage of atmospheric water along the outside of the facility. The facility – the tensile sports hall, must have all of the required fire protection documentation and equipment provided. |  |  |  |

LOT 2 Stage

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 2.1 | ROOF 10X8R pcs 1 The Trussing system with the following minimum technical specifications: Double pitch roof system dimension 10x8m with tower 4 pcs for height 7 m The Tower - Maximum loading capacity 1000kg, maximum height 7,5m, square truss segment 290x290mm build of main tube diameter 50mmx4mm and the cross braces diameter 20mm x 2mm from material Alloy EN-AW 6082 T6 (AIMgSil T6), included steel base and stabilisator with assist arm Roof top support - square truss segment 400x400mm build of main tube diameter 50mmx2mm and the cross braces diameter 25mm x 2mm from material Alloy EN-AW 6082 T6 (AIMgSil T6) Handwinch 4 pcs with load capacity 900kg included handwinch holder and steel rope Rigging accessories The Canopy set including PVC cover and relevant accessories included roof canopy, side canopy and back canopy |  |  |  |
| 2.2. | STAGE 10X8M pcs 1 The stage system with the following minimum technical specifications: The stage dimension 10x8m = 80m2 The dimensions of one platform is 200cm x 100cm, frame made of aluminum, a total of 40 pieces. The top layer of the platform is made of 12mm thick plywood, of waterproof and fireproof material, and with anti-slip surface. Designed for external use. The platform is certified for a load of 750kg / m2. Each frame of the platform is equipped with 4-piece for leg supports and an accessory for connecting between platform and legs. Coupling between the platforms, the insert between the platforms for leveling the stage on uneven terrain, the four-leg coupling between the aluminum legs of the bicycle segments, the coupling of the double between the aluminum legs of the bicycle segments. With the stage, the telescopic square legs are delivered 160 pieces The legs are made of aluminum and adjustable height from 60 to 100cm. The adjustable steps for height from 60 to 100cm with handrails, 3 pcs. |  |  |  |
|  | 48ch digital desk - 1 set The audio console with the following minimum technical specifications: 46 busses: 16 x flexi-busses Mono/Stereo (equivalent of 32 DSP busses), Stereo Master (2), Solo busses (2 stereo, 4 total), and 10 x 8 Matrix (8) ,10 x Control Groups, 1 x Compressor per channel and buss, 1 x Gate per channel and buss (switchable to ducker, or compressor with side chain access),16 x assignable 32 band Graphic EQs 8 x FX engines (reverbs, delays, w/modulation and enhancer) 4 x assignable DiGiTuBes, 4 x assignable Multiband Comps User definable Macros An extremely high power headphone amplifier with 1/4 inch and mini jack socket 96kHz as standard 24 mic line inputs,12 analogue outs , 2 AES I/O (mono) Word Clock I/O,1 GPI and 1 GPO DVI out (for an external monitor) 2 DMI Card slots (up to 64 I/O per slot) 2 Ethernet connections for Networking 2 x 24 segment master/solo meters Touch sensitive rotaries with integrated switch & HTL 2 x multi-touch screens 21 x touch sensitive moving faders |  |  |  |
|  | 2 x multi-touch screens 21 x touch sensitive moving faders 4 x layers of banks of 10 faders Customisable bank and channel layout Snapshots Integrated USB2 Audio I/O interface for recording and playback of up to 48 channels 40 x Input flex-channels Mono/Stereo (equivalent of 80 DSP channels) Audio Specification Sample Rate 48kHz, 96kHz Processing Delay 2ms Typical @ 48k(60 Stereo Channels, Stage Input Through L-R Buss to Stage Output) 1.1ms @ 96kHz Internal Processing Up to 40-bit, Floating Point A>D & D>A 24-bit Converter Bit Depth Frequency Response +/- 0.6dB (20Hz – 20kHz) THD <0.05% @ Unity Gain, 10dB Input @ 1kHz Channel Seperation Better Than 90dB (40Hz – 15kHz) Residual Output <90dBu Typical Noise (20Hz - 20kHz) Microphone Input Better Than -126dB Equivalent Noise Maximum Output Level +22dBu Maximum Input Level +22dBu |  |  |  |
| 2.3 | - ACTIVE TWO-WAY ARRAY pcs 4 Active Line source speaker system with the following minimum features: Minimum 3x10 "LF speaker +3" HF (winding circle) in an asymmetric horn with vertical radiation of +5 degrees, -25 degrees, horizontally 90 degrees with a maximum deviation of 10%. Maximum sound pressure at 1 meter distance can not be less than134dB. Frequency oscillation with a maximum deviation of +, - 3db must be from 45Hz to 20kHz or cheeses. The loudspeaker system must have 3 or more LF speakers in line with the tweeter. LF speakers can be 10"or larger. - Active 18" subwoofer 2 pcs Active 18" subwoofer of the following minimum features: The maximum sound output per 1 m distance must be 136db or greater. The frequency range must be from 30hz to 120hz or wide in the lower band with a maximum deviation of +/- 3db. The subwoofer must have a Delay function with setup mode so that it can be set in the "END Fire cardio" configuration. It must be able to reset the upper cutting frequency from 90hz to 120hz or wide. |  |  |  |
|  | - MECHANIC SET set 1 A set of brackets and mechanical accessories for linking the system Stagebox 24in/8out with multicore cable length 25m and audio connectros pcs 1 - Speaker stand, Pole mount M20, fi 35mm, adjustable height from 700mm to 1100mm pcs 2 - Microphone stand, with Boom arm length 805 mm, adjustable height from 900 to 1,600 mm, threaded connector 3/8’’ set 6 - Microphone dynamic, hypercardidal, for vocals and instruments, wide frequency range of 50 Hz - 18 kHz 2 sets - The wireless microphone system with hand-held microphone 4, The antenna combiner 1 pc, amplifiers 2 pcs and 2 pcs of the antenna, minimum 1,600 tunable frequency. Bandwidth 40MHz minimum. Automatic scanning band. The wired rack cabinet. With the offer is required to submit a certificate of conformity for wireless microphone system issued by the authority that meets the Rules on radio equipment and telecommunications equipment treminalnoj for regular import the required wireless microphones 1 set |  |  |  |
|  | - AUDIO SET set 1 A set of hybrid cables for connecting the system (signal cable 3-pin XLR + powercon connectors - CORE SET set 1 The lighting console with the following minimum technical specifications: 1.General requirements A digital light control system shall be supplied. The lighting control system shall be provided either as a console only or a networked control system, where all individual devices use a standardized Ethernet network for inter-communication. Operators shall be able to swap single devices in the case of a fault. Under no conditions shall a single faulty device affect the remaining system or parts of it. For purposes of reliability and standardization, analogue control signals and analogue control voltage in system links shall not be used. The lighting control system shall comply with state of the art technology and generally accepted codes of practice. It shall be manufactured so that it operates properly without risk (in regular or irregular use or in a malfunction), so the manufacturer shall provide: a) Protection against electric shock b) Protection against the impact of high temperatures c) Isolation of functional sections of the electrical systems through the use of galvanic isolation. 2. Lighting control system implementation A versatile lighting control system shall be provided, suitable for the operating requirements of theatres, television broadcasts, special events and live performances. The primary focus of the lighting control system shall be to control moving lights, video servers, LED and conventional luminaires (also called fixtures). It shall be intuitive to use and guide the user with additional information displayed at runtime. |  |  |  |
|  | 3.Hardware: Dimensions, In- and Outputs Dimensions shall not exceed 427 x 546 x 161mm / 16.8 x 21.5 x 6.4 inch. The power supply shall be designed for operation at 120/230V AC @ 50/60Hz and a maximum load of 100VA.The console shall support a variety of peripheral equipment. Therefore the ports listed below shall be integral by default: 1 x IEC 60320 C13/C14 (IEC cord) 1x etherCON (dot2-Net, sACN, Art-Net) 4 x DMX512-A Outputs (5 pin XLR female) 1 x DMX512-A Input (5 pin XLR male) 1 x MIDI-In (5 pin DIN female) to receive MIDI notes or MIDI timecode 1 x MIDI-Out (5 pin DIN female) 1 x Linear time code input (3 pin XLR female) to synchronize to an external LTC-SMPTE signal 1 x DC remote control port (SUB D 15 female) for remote triggering with a standard touch board 1 x Audio in (3 pin XLR female) for sound to light functionality 3 x USB 2.0 (type B) socket to connect a computer mouse, mouse-type devices (e.g. a touch screen) or a computer keyboard 1 x DVI-D connectors for external monitor (optional touchscreens) 1 x Desklamp (4 pin XLR female) 4.Controls and Displays Screens The console shall run a graphical user interface (GUI) that can be operated with a touchscreen. Therefore 2 touchscreens with a minimum diagonal viewable size of 7” shall be provided. One external monitor, with optional touchscreen functionality, shall be able to be connected to the console. These monitor shall be connected to the console using DVI-D port for video information and USB type B connector for the touchscreen input. (Consult with console manufacturer for monitor and/or touchscreen type.) Control All console keys shall be grouped in function blocks, like the numeric keypad, screen navigation, user command and control keys. All key buttons shall be silent. Additionally all keys shall be backlit for ease of data entry in dimmed environments. Additional keyboard functions A virtual keyboard shall be implemented for the touchscreens. An optional external keyboard shall be able to be connected via USB. Fader The console shall have 6 independent faders with minimum 60mm stroke length. Each fader strip shall be provided with 2 additional buttons. Fader and buttons shall be able to be assigned with multiple functions: |  |  |  |
|  | Control All console keys shall be grouped in function blocks, like the numeric keypad, screen navigation, user command and control keys. All key buttons shall be silent. Additionally all keys shall be backlit for ease of data entry in dimmed environments. Additional keyboard functions A virtual keyboard shall be implemented for the touchscreens. An optional external keyboard shall be able to be connected via USB. Fader The console shall have 6 independent faders with minimum 60mm stroke length. Each fader strip shall be provided with 2 additional buttons. Fader and buttons shall be able to be assigned with multiple functions: a) Playback Cuelist, where button functions can be mapped individually b) Groupmasters – limit intensity for groups |  |  |  |
|  | c) Speed/Rate-Fader – for adjusting the overall rate of all effects d) Programmer Time Fader – to individually adjust the programmer time Playbacks shall only be allowed per single page and therefore active playbacks need to remain active even during page-changes. Universal recall/executor buttons The console shall have 12 universal recall/executor buttons with individual assignments to playbacks, chaser and macros. The assignments are stored on pages, which can be changed. Main Playback The console shall have two main playback faders, with a stroke length of 100mm, with adjacent Go+, Go- and Pause-Buttons, for playing back the main cuelist. On the screen above these faders information in plain text shall be shown for the actual played back cue. Grandmaster The grandmaster shall adjust the master output level. Adjacent to the grandmaster, there shall be a blackout button. Encoder The console shall provide four push encoder for multiple controls. The encoders should be used case-sensitive according to the above screen. The encoder shall be push-sensitive to enable data entry. All encoders shall be magnetic rotary encoders to resist excessive use and avoid wearing. All four push encoders shall provide control to multifunctional devices and to access all attributes and functions which are displayed in the screen above the encoders.Level wheel |  |  |  |
|  | To control the intensity of certain parameters, including the intensity parameter of moving lights and multimedia devices, a dedicated vertical high-resolution intensity wheel shall be provided. This wheel shall operate on all selections of different luminaires and fixtures, single or in groups. External mouse It shall be possible to connect a regular 2-button scroll wheel mouse via USB. 5.Operating and software General operation The keypad shall give access to select dimmers, moving lights and multimedia devices by using logic operations. It shall be possible to use the numeric keypad, the command line, the encoders or the level wheel to assign different values to that selection. These values shall be able to be stored into a set amount of cues or presets, limited only by the system’s capacity. Patch The console shall provide a predefined library of well-known and popular fixtures. This shall include dimmer channels, moving lights and |  |  |  |
|  | multimedia devices by various manufacturers. The corresponding control channels shall have a resolution of 8, 16 and 24 bit. Every fixture library shall have written function sets for certain attributes (which describe their properties) to give direct access for selecting colours, gobos or any other function. This includes functions, such as striking and dowsing discharge lamps or resetting the logics and/or homing mechanical parts of a fixture. These function sets shall also be used in the guided user interface. Bitmap files shall be used to describe and display gobos in the graphical user interface for a pleasant and easy operation. The bitmap files shall also be used to show beams and shading in the 3D visualizer. Four encoders shall control the various functions of the fixtures. All controlled fixtures shall be assigned to a fixture number. A virtual dimmer function shall allow control of standard RGB fixtures and to simulate dimmer functions and control intensity levels independently from the colour attributes. For automated luminaires the possibility must be given, to invert pan and/or tilt values. To get an overview of all patched fixtures it shall be possible to call helping views containing information about DMX channel assignment or the fixture start addresses. Screen configurations For convenience and the ease of use the following views should be provided by the console: a) Fixture sheet to display and change the values of selected channels b) DMX sheet to display the raw data output of the console throughout all universesc) Cuelist sheet for playbacks, to give detailed cue related information and display and change timings |  |  |  |
|  | c) Cuelist sheet for playbacks, to give detailed cue related information and display and change timings d) Groups view to list, edit and recall groups e) Macro view to list and recall macros f) Page view to display and change fader and button assignments g) Preset pool views for all predefined preset-types to store, list and recall presets for all fixtures. h) Special Dialogue to graphically display and select all features and attributes of selected fixtures (automatically populated by the console to guide the user). i) Playback view to display assignments and information about playbacks on faders and buttons. j) Command line view to display all entered commands. General editing and programming functions It shall be a fundamental requirement that all items like presets, groups, playbacks and pages can be moved, copied and deleted. It shall also be a fundamental requirement that all items like fixtures, groups, effects and pages can be selected by logic functions like “+”, “-“ and “thru”. All attributes and functions of a multifunctional device shall be addressable by a single fixture number. |  |  |  |
|  | The use of intelligent selections shall be available to perform logic divisions on selections of fixtures (e.g. to select odd/even). By the use of hard key buttons it shall be possible to step through all subsets of the selection. It shall be possible to select fixtures: a) if they are already in use b) if they are unused and still available c) if they are stored in a certain cue or preset d) if they a currently in a certain preset Relative or absolute intensity changes for selected fixtures shall be made by the use of the level wheel, an encoder or the hard key “AT” (followed by a numeric entry). Just as the hard key “AT” there shall be “FULL” and “HIGHLIGHT” buttons. The hard key “FULL” shall set the selected fixtures to full intensity. The hard key “HIGHLIGHT” shall set the output of the selected fixtures temporarily to a defined intensity and attribute setting without affecting the programmer. It shall be possible to define the colour feature of any type of fixture by a dedicated colour picker in a graphic user interface that uses RGB, CMY and HSB values with a pointer on the screen. There shall be command line commands to directly change DMX channels and all features of moving lights and media devices. |  |  |  |
|  | It shall be possible to define the colour feature of any type of fixture by a dedicated colour picker in a graphic user interface that uses RGB, CMY and HSB values with a pointer on the screen. There shall be command line commands to directly change DMX channels and all features of moving lights and media devices. It shall be possible to copy intensities and attributes from one selection to another selection of fixtures. There shall be an option to park fixtures (in whole or just certain attributes) or DMX channels on specific values to keep them at a defined state without readdressing them in programmer or cues. Oops (Undo) shall be implemented to step back in the command history. It shall be possible to assign dedicated timing values to each presettype, using the command line or an encoder. The use of macros shall be implemented: Predefined macros should be provided and already implemented. To create and edit effects, a graphical user interface shall be implemented. It shall be possible to adjust several functions to a selection of fixtures. This includes but is not limited to Size, Speed, Phase, Width and Softness of an effect. It shall be possible to copy all programmed data from a selection of fixtures. That implies to copy all Preset-, Grouping-, Cue- and Effect-information from one selection to another selection of fixtures. It shall be possible to store a selection of fixtures as a group. It shall also be possible to store can be recalled for later cue editing. When working with presets in cues, they should be recorded as a reference. |  |  |  |
|  | a selection of fixtures as a group. It shall also be possible to store dedicated fixture attributes in presets, which can be recalled for later cue editing. When working with presets in cues, they should be recorded as a reference. For storing cues, it shall be possible to define whether the programmer shall store active values (only changes) or store a whole look (all fixtures with dimmer greater zero and all other fixtures with dimmer at zero). When changing existing cues, there shall be the choice between overwriting, merging or removing data in the corresponding cue when storing. If only one cue is existent there shall be an additional option of creating a second cue. When storing cues it shall be possible to assign dedicated values for fade and delay times via the command line. There shall be a graphical user interface for later display and editing the cue timings. All cues of a Cuelist shall be editable to specify the cue trigger: a user “GO”, auto following directly or after a given time, SMPTE timecode or the external sound input. All cues shall have the possibility to execute commands and keystrokes when played back. The commands have to be a batch of command line commands. |  |  |  |
|  | Playback features A playback can be a fader strip (fader plus 2 recall buttons) or a single recall-button. Playbacks can be turned into chasers, where all cues are auto triggered via a user-defined speed or an external sound input. The playbacks are Soft-LTP per fader and shall have an autostart and autostop function. Every single button playback shall be toggle functionality by default. Further functions like GO, GOBACK, FLASH, PAUSE and LEARN shall be assignable. Backup show-files The show shall contain at most: a) 999 groups b) 10 x 999 predefined presets c) 999 playback pages d) 22 playback fader plus Main Playback and 140 playback button with e) 9,999 cues each The total number of showfiles is only limited by the capacity of the integral solid-state-drives. All relevant data shall be saved in a show-file, including the fixtures’ 3D coordinates and corresponding 3D objects. The console shall also come with demo showfiles and tutorials that are read-only and cannot be modified by the user - by intention or accident. 6. Network Full tracking backup There shall be an option to set up a full tracking |  |  |  |
|  | backup system with other consoles or PC based online software editors. A backup system shall always be provided with the latest show data to be in sync and operational. If the master console fails a synchronized backup system can instantly take over and resume executing commands and playbacks to transmit control and without any noticeable data-loss. Remote control The console shall be remote controllable via MIDI, DMX, SMPTE linear time code (LTC) or MIDI time code (MTC). When using MIDI for remote controlling MIDI “Note On” commands will trigger buttons or change fader level. There shall be a system wide option to activate or de-activate MIDI remote control. It shall be possible to remotely control the console via MIDI time code or SMPTE linear time code. The switching between different time code formats, such as 24, 25 or 30 frames per second, shall be implemented (auto-detect). The console shall provide 12 DC remote controls (contact closures) that can be assigned to dedicated functions. 7.Manual and documentation The manufacturer, supplier or the vendor has to provide an installation manual and an operation manual. The Help-manual should be available online and inside the console in English, German, French, Spain and Portuguese. 8.Service, maintenance and repair At time of order placement, bidders shall nominate a company and/or a person for service, maintenance and repair for the offered light control system. This company and/or person must be familiar with the construction and design of the offered light control system and must work on behalf of the manufacturer. |  |  |  |
|  | Intimidator pcs 4 The moving fixture spot with the following minimum technical specifications: Light Source: 1 LED (white) 60 W (2.3 A), 50,000 hours life expectancy Beam Angle: 15° Prism: 3-facet round, static DMX Channels: 8 or 13, DMX Connectors: 3-pin XLR Pan and Tilt: 540°/270° Colors: 8 + white, split colors, continuous scroll at variable speeds Gobos: 7 + open,rotating, interchangeable, slot-n-lock, continuous scroll at variable speeds Strobe Rate: 0 to 20 Hz lluminance: 10,400 lux @ 2 m, 1,600 lux @ 5 m Power Linking: 10 units @ 230 V, Input Voltage: 100 to 240 VAC, 50/60 Hz (auto-ranging) Included hooks and safety cable Slimbank USB pcs 8 The wash static fixture with the following minimum technical specifications: Light Source: 18 LEDs (Tri-Color RGB) 3 W (1050 mA), 50,000 hours life expectancy Beam Angle: 17°, Field Angle: 32° lluminance: 4,120 lux @ 2 m DMX Channels: 3, 5, 9 or 13, DMX Connectors: 3-pin XLR |  |  |  |
|  | Strobe Rate: 0 to 22 HzPWM Frequency: 1.2 kHz Power Linking: 24 units @ 230 V Input Voltage: 100 to 240 VAC, 50/60 Hz (auto-ranging) Included barndoor, hook and safety cable IRC6 pcs 1 Infrared remote control (IRC) for IRC compatible lighting fixtures. It provides instant access to pre-programmed scenes, color mixing options and on-the-fly speed adjustments. This compact and lightweight remote has an approximate range of 9m POWER BOX pcs 1 Power box with power connetors and protected system, rubber POWER SET set 1 Set of power cables for connecting audio and lighting system SERVICE Delivery, install and setup of system |  |  |  |

LOT 3 Wooden info desks

| 1. Item Number | 2. Speciications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 3.1 | WOODEN INFO PULTS pcs 20 Purpose: For work and for products exposition BASIC DIMENSIONS : Lower part: ( W x L x H) 100 x 200 x 80 Roof-Canopy: at the height of (200 to 220 cm), two-sided Description Pedestal Metal frame of square profile 4 x 4 cm with four-points and with stem screws Upgrade A box in the form of a square with a vertical bulkhead and with two doors on the inside for disposal of goods from wooden panels 2 cm thick Roof-Canopy On two wooden pillars, which are removable on a wooden box on which rests a two-sided wooden roof. The cover on the roof is made of tegola or aluminum sheet in the color of tile imitation. |  |  |  |