# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

p 1 /…

Contract title : Purchasing equipment for processing, drying and packing fruit, vegetables and plants.

Publication reference :   RORS 10/ County prison in Pancevo/03

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

When brand names/specifications referring to or describing products of a given brand or origin are used in the technical specifications, they are used in descriptive purposes only or should be interpreted as equivalent.

- PURCHASING EQUIPMENT FOR PROCESSING, DRYING AND PACKING FRUIT, VEGETABLES AND PLANTS

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 Small machine for drying fruits, vegetables and plants Quantity: 1 | Must have following characteristics: - must be ”chamber type machine” with trays; - machine thermo-generators must work on electricall energy in combination with solar panels for the use of solar energy as an additional source of energy for drying; - external dimensions must be: height from 2300mm to 2600mm; width from 1500mm to 1800mm; length from 1300mm to 1500mm; - engaged power must be maximum at 8 kW - nominal consumption of energy must be maximum at 3 kw - range of temperature in the drying zone must be from 0 oC to 75 oC - range of humidity in the drying zone must be from 0% to 99% - with the machine must be delivered minimum 40 trays and minimum 2 special troleys. Trays must be made of stainless steel (inox, rostfraj, prochrom). The trays must be placed into special trolley that are compatible to the dimensions of the drying chamber. Special troley must also be made of inox. The drying chamber must be such that it can accept the trolley with the trays during the drying operation. Each tray must have an appropriate barrier to hold (fix) medicinal plants during the drying of medicinal plants; - capacity of the machine must be minimum 240 kg based on the plum as raw material; The drying chamber must have: - digital control unit for drying process; - moisture and temperature sensors for monitoring and controlling the drying process; - two way air flow, which ensures uniformity of drying parameters in all parts of the chamber; - turbine for moisture removal; - possibility for a variable work mode that allows optimal adjustment of the drying process for each product. |  |  |  |
| 2 Large machine for drying fruits, vegetables and plants Quantity: 1 | - must be ”chamber type machine” with trays; - machine thermo-generators must work on electricall energy in combination with solar panels for the use of solar energy as an additional source of energy for drying; - external dimensions must be: height from 2300mm to 2600mm; width from 1500mm to 1800mm; length from 1600mm to 1900mm; - engaged power must be maximum at 10 kW - nominal consumption of energy must be maximum at 4 kw - range of temperature in the drying zone must be from 0oC to 75o C - range of humidity in the drying zone must be from 0% to 99% - with the machine must be delivered minimum 80 trays and minimum 4 special troleys. Trays must be made of stainless steel (inox, rostfraj, prochrom). The trays must be placed into special trolley that are compatible to the dimensions of the drying chamber. Special troley must also be made of inox. The drying chamber must be such that it can accept the trolley with the trays during the drying operation. Each tray must have an appropriate barrier to hold (fix) medicinal plants during the drying of medicinal plants; - capacity of the machine must be minimum 480 kg based on the plum as raw material; The drying chamber must have: - digital control unit for drying process; - moisture and temperature sensors for monitoring and controlling the drying process; - two way air flow, which ensures uniformity of drying parameters in all parts of the chamber; - turbine for moisture removal; - possibility for a variable work mode that allows optimal adjustment of the drying process for each product. |  |  |  |
| 3 Machine for homogenizing powdered and grain matter Quantity: 1 | must have the following characteristics: - must be with minimum power of 0,50 kw; - must be with minimum volume of 210 liters; - must be made of stainless steel (inox, rostfraj, prochrom); - must have safety measures which insures the operator working on the machine can not be harmed; - electric power supply. |  |  |  |
| 4 Machine for packing finished grain and powder products with weldering machine Quantity: 1 | Must have the following characteristics: - must be able to pack in polyethylene and polyester bags; - packaging range must be between 10 grams and 1000 grams; - packing quantity per minute must be minimum 5 bags per minute; - must have option of electronic scale measurment; - electric power supply; - must have pneumatic drive (powered with compresed air) - machine for packing must be equiped with air compresor of minimum power between 1,5-2,5 kW, with air flow of minimum capacity between 240-350 liters/minutes, with air pressure between 8-10 bar and reservoir of minimum 50 l. |  |  |  |
| 5 Work table Quantity: 4 | must have the following characteristics: - must have an integrated waste throwing hole; - must have the following dimensions: height from 800mm to 850mm; width from 650mm to 700mm; length from 1300mm to 1400mm; - must be made of stainless steel (inox, rostfraj, prochrom). |  |  |  |
| 6 Machine for thermal processing of vegetables and fruit before drying (blanching) Quantity: 1 | Must have the following characteristics: must be with minimum power of 8 kw; - must be with minimum volume of 110 liters; - must be maximum dimensions 1000mm x 1500mm x 1000mm; - must be made of stainless steel (inox, rostfraj, prochrom); - electric power supply; - must have safety measures which insures the operator working on the machine can not be harmed. |  |  |  |
| 7 Machine for washing fruits and vegetables Quantity: 1 | Must have the following characteristics: - must have washing capacity based on small fruits and vegetables from 400 kg per hour to 600 kg per hour; - must be equiped with appropriate technological solution which allows separating dirt (i.a soil and small stones) from the raw material during the washing process; - must be equiped with appropriate technological solution which allows fine rinsing of the raw material after the process of dirt separation; - must be equiped with appropriate technological solution which allows transportation of washed raw material out of the machine; - electric power supply. |  |  |  |
| 8 Machine for chopping fruit and vegetables Quantity: 1 | must have the following characteristics: - must be made of stainless steel (inox, rostfraj, prochrom); - must have safety measures which insures the operator working on the machine can not be harmed; - must have the capacity of processing raw materials from approximately from 150kg per hour to 300 kg per hour; - along with the machine, it is necessary to deliver a set of minimum 10 interchangeable blades for cutting raw material into rings, cubes and slices; - electric power supply. |  |  |  |
| 9 Machine for milling dried fruits and vegetables Quantity: 1 | Must have the following characteristics: - granulation size of the milling product must be between 60 mikrons and 3 milimeters; - must have the capacity of producing milled products between 40 kg per hour to 150 kg per hour - parts of the machine which are in direct contact with the raw product and milled products must be appropriate for processing raw product for human consumption; - must be equiped with appropriate technological solution which allows the so-called "cold milling" which ensures the preservation of all usefull ingredients in the milled product; - must have safety measures which insures the operator working on the machine can not be harmed; - electric power supply. |  |  |  |
| 10 Storage containers for dried products Quantity: 5 | Must have the following characteristics: - the containers must be made of plastics (PVC); - the containers must have minimum dimensions of 1000 mm height and 400 mm width and minimum 200 liters of volume - must be round shape with appropriate cover of same shape and dimensions. |  |  |  |
| 11 Small Packaging bags Quantity: 3000 | Must have the following characteristics: - must be appropriate for packing grain and powdered dried products; - bags must be made of polyethylene and polyester, must have health certificate and must be airproof and waterproof; - expiration date of the bags must be minimum one year; - bags must have minimum dimensions 200mm x 300mm. |  |  |  |
| 12 Large Packaging bags Quantity: 2000 | Must have the following characteristics: - must be appropriate for packing grain and powdered dried products; - bags must be made of polyethylene and polyester, must have health certificate and must be airproof and waterproof; - expiration date of the bags must be minimum one year; - bags must have minimum dimensions 250mm x 350mm. |  |  |  |
| 13 Scale for measuring Quantity: 2 | Must have the following characteristics: - must be able to measure a weight of up to 40 kg; - must show the result of measuring on display; - must work on electric power supply or replacable bateries. |  |  |  |
| 14 Scale for measuring with printer for labels Quantity: 1 | Must have the following characteristics: - must be able to measure a weight up to 40 kg - must be equiped with enough supplies to print minimum 5000 labels. - must show the result of measuring on display; - must work on electric power supply or replacable bateries. |  |  |  |
| 15 Machine for washing with hot water under pressure Quantity: 1 | Must have the following characteristics: - must have engine power minimun 2,5 kW - must have working pressure of minimum 150 bar; - must have water flow of minimum 450 liters per hour; - must be able to heat water to minimum 70oC; - must have fuel consumtion of maximum 3 kg per hour; - must have integrated compression hose minimum 15 meters long; - must be equiped with extended hand spray gun, compressor gun, jet tube minimum 500mm long and with option to turn off the pressure; |  |  |  |
| 16 Crates Quantity: 50 | Must have the following characteristics: - minimum exterior dimensions must be 600mm x 400mm x 300mm; - must be able to be used on temperaturs from -40°C to +70°C; -must have minimum volume of 48dm3; - must have minimum weight of 2 kg |  |  |  |
| 17 Scale for humidity measurement Quantity: 1 | Must have the following characteristics: - weighing area from 0 grams to maximum 110 g - area of humidity measurement from 0% humidity to 100% absolute humidity and 0% of dry matter to 100% of dry matter; - maximum weight of the sample 110 g - resolution of scaling 0.01% (moisture/dry matter) or 0.005 grams (when used as a scale) - heating must be with minimum 2 halogen heaters of minimum 90 W - temperature range must be from +50 ° C to +160 ° C (adjustable in increments of 1 ° C) - drying time must be from minimum 2 minutes to 9 hours 59 minutes (adjustable in steps of 1second) - must have an option for monitoring the drying process on display - electric power supply |  |  |  |
| 18 Knives Quantity: 10 | Must have the following characteristics: - blade must be made of stainless steel (inox, rostfraj, prochrom); - must have maximum blade length 15 cm; |  |  |  |
| 19 Kitchen cutting boards Quantity: 10 | Must have the following characteristics: - must be made of termoresistant plastics; - must have minimum dimensions 30 cm x 20 cm |  |  |  |
| 20 Spatula for drainage Quantity: 2 | Must have the following characteristics: - must be made of termoresistant plastics; - must be minimum length 30 cm |  |  |  |
| 21 Kitchen spatula Quantity: 2 | Must have the following characteristics: - must be made of stainless steel (inox, rostfraj, prochrom); - must be minimum length 30 cm |  |  |  |
| 22 Kitchen bowl Quantity: 5 | Must have the following characteristics: - must be made of stainless steel (inox, rostfraj, prochrom); - must be minimum width 25 cm -must be minimum height 14 cm |  |  |  |
| 23 Plates Quantity: 10 | Must have the following characteristics: - must be made of stainless steel (inox, rostfraj, prochrom); - must be minimum width 25 cm |  |  |  |