# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Procurement of Equipment for hydrographic measuring for project BEGA	p 1 /7

Publication reference: VI-84/24

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Riverboat with installed equipment for hydrographic measuring – 1 set with next characteristics as minimal requirements: - Riverboat With next characteristics: - Cabin boat with passage through the cabin to the bow and stern part of the vessel - boat must be made of fiberglass - minimum 2 seats for 2 crew members must be placed to each other with a passage between (skipper & operator seats) - boat must be appropriate for installation of hydrographic measuring equipment within a hull of the boat. All cables, power supplies and other connection must be installed in the hull of the boat. - Boat must be able provide electricity with minimum 150W flexi solar panels on board the vessel, with regulator and accumulator of minimum 90Ah, so-called service battery, for users who are required to signal the vessel for a minimum of 12h and a crushing battery of minimum 90Ah. - Boat must have a chemical toilet, reservoir with minimum 40 l of technical water. - Boat must have fuel tank min. volume 100 l - Motor power – from 80 up to 100 HP - min. dimensions: 6,00 m x 2,00 m - draught (depth of the boat below the water level) up to 0,5 m - Multi beam Frequency – from 120 up to 500 kHz. Coverage - Up to 12 times depth Along track resolution - 0.50° Across track resolution - 0.04° Beam opening angle – min. 120° Depth resolution – max. 2 cm Min ping rate - 10 Hz Min depth – 1.0 m Max depth – 150 m Compact Deck Unit Control – Laptop and Ethernet interface must be included. Transducer head Software features - Complete acquisition and processing included including texture analysis software for side scan data normalisation and mosaicking. Calibrated backscatter. GSF output. Real time processing. Area based calibration. OS with characteristics similar to Windows 10 or equivalent OS |  |  |  |
|  | - Motion reference unit Performance Angle Accuracy static - 0.02º RMS Angle Accuracy Dynamic @ ±5º simultaneous roll and pitch - 0.03º RMS Resolution Angle –. 0.001º Resolution Heave - 0.01m Angle range Roll/Pitch - ± 30º Heave range - ±10m Heave Accuracy – min. 5cm or 5% Acceleration accuracy – max. 0.01 m/s2 RMS Communications Configuration Software - The IMU is shipped with configuration software allowing on site setup Output Signal Protocol - Multiple, user selectable Output Protocols ASCII NMEA and binary Communications Interface - Output RS422 and RS232. Analog with remote converter (optional) 2 x RS232 External inputs, (not available on all models) Velocity input formats RMC, RMA, VTG, VBV, VHW; Heading input formats HDT, HDG Physical Dimensions (W x H) – min. tube Ø89, mounting plate Ø134, flange Ø110) x 127 Weight – up to 3 kg Housing Material – Titanium Environmental Temperature (absolute max) - 0º to +55º Celsius (-10º to +65º); Storage Temperature - 40º to +65º Celsius Mounting Orientation - Vertical or Horizontal mounting Power requirements - 12 - 30 VDC; 2 W MTBF (computed) - 50 000 hours Depth rating - IP66 (standard); IP68 30 meter depth rated (optional) Standard - Complies with the IEC 60945 - GNNS Position system Performance Receiver Type - L1 C/A code, with carrier phase smoothing Signal Tracking - Dual L1 GPS receiver design, parallel tracking GPS Sensitivity - 142 dBm SBAS Tracking - 2-channel, parallel tracking Update Rate - 20 Hz standard Horizontal Accuracy - < 0.1 m 95% confidence (DGPS1) < 0.5 m 95% confidence Heading Accuracy - < 0.30˚ rms Pitch/Roll Accuracy - < 1˚ rms Heave Accuracy - 10 cm rms Power Input Voltage - 6 to 36 VDC Power Consumption - 3.3 W nominal ~ 350 mA @ 9 VDC ~ 265 mA @ 12 VDC ~ 200 mA @ 16 VDC - Sound velocity sensor Performance Sensor length - 25mm Range - 1375 - 1900m/s Resolution - 0.001m/s 100mm Random noise (point to point) - ±0.002m/s Max systematic calibration error - ±0.013m/s |  |  |  |
|  | Max systematic clock error - ±0.002m/s Total max theoretical error - ±0.020m/s Electrical Voltage - 8 - 30vDC Power - 0.25W (SV only) Connector - Subconn MCBH6F or equivalent Material - Titanium housing Power Consumption - 3.3 W nominal ~ 350 mA @ 9 VDC ~ 265 mA @ 12 VDC ~ 200 mA @ 16 VDC Mechanical Dimensions – min. 66.3 L x 20.9 W x 14.6 H (cm) Weight – up to 2,5 kg |  |  |  |
| All items | Installation performed by contractor or authorised service provider. All the equipment must include all necessary parts and standards for its installation. |  |  |  |
| All items | Testing of all basic functions of the instrument on a set of producers standard samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Training one-day for minimum 2 persons (upon delivery) on equipment handling (familiarisation during installation - working with the equipment in all basic functions of the equipment), equipment maintenance. Details of proposed training methodology, shall be presented in the tender proposal and shall refer to the related equipment |  |  |  |
| All items | Technical documentation for equipment (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | Warranty Tenderers must provide local reliable warranty service agent providing maintenance and the rapid supply of equipment spare parts and consumables for the Warranty duration of one year. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed maximum response time to submitted maintenance support request (fax or e-mail) of 1 (one) working day; - Guaranteed that any requests for services will be attended to within 24 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 72 hours; - Guarantee that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 1 year (after the end of 1 year standard warranty) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |