# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply and delivery of furniture and equipment for Emergency Situation Management Centre 	p 1 /…

Publication reference : RORS28/Mosna/TD10

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | Office cabinet – O1 The office cabinet width is 800mm, height 2000mm and depth 420mm (with wings). It is made from chipboard of thickness d = 18mm in the BEECH NATUR design. The sides of the cabinets are provided with full height, while the horizontal elements "fall" between the sides with which they are tied through the anchors. The cabinet has a 100 mm high clam, while under the lowest shelf are four adjustable, adjustable height adjustable feet that accommodate the bulk of the load and provide unstable structural stability. The inside of the cabinet contains 5 shelves adjustable in height. All edges of the plate elements are dangled with an ABS band with a thickness of d = 2mm. The back of the cabinet is made of HDF - board with a thickness of 3mm. The pages are flat on the back side, so the back is falling into the falcon. Each wing has a handle, the handle is made of metal, grey. Install the locking device into the wings. Quantity: 12 |  |  |  |
|  | Glass office cabinet – O2 The cabin width is 800mm, height 2000mm and depth 420mm (with wings). It is made from a chipboard of thickness d = 18mm in the design BEECH NATUR. The sides of the cabinets are provided with full height, while the horizontal elements "fall" between the sides with which they are tied through the anchors. The cabinet has a height of 100mm, while the four shelves are adjustable under the lowest shelf, adjustable in height, which take up the bulk of the load and provide an unstable stability to the construction. The cabinet is vertically divided into two parts, each of which has a separate pair of wings, and within each part two height adjustable shelves are placed. The front parts of the first part are made of chipboard, while the depth of the other part is 300mm and the front is made of glass. All edges of the plate elements are dangled with an ABS band with a thickness of d = 2mm. The back of the cabinet is made of HDF - board with a thickness of 3mm. The pages are flat on the back side, so the back is falling into the falcon. Each wing has a handle, the handle is made of metal, grey. Install the locking device into the wings. Quantity: 5 |  |  |  |
|  | Metal wardrobe Metal wardrobe is made of steel sheet thickness 0.7mm in dimensions with one cabin. Dimensions: Height: 1800-2000 mm Width: 300 mm Depth: 500mm Colour: grey Quantity: 10* *It can be offered 5 pieces of double metal wardrobe |  |  |  |
|  | Office desk – ST1 The office desk is made of chipboard thickness d = 25 mm thick in BEECH NATUR design. The workpiece is 1400h600mm in size, 590h725mm in size. Inside the table there is a vertical casing, horizontal plate height of 400mm. In the body of the table there is a fixed cube with 3 drawers, total height 400mm. The horizontal plate, the cube body and the forehead are made of chipboard d = 18 mm, and the drawer corners are made of d = 18mm thickness in a basic white colour. The metal handle was fitted to the forehead. In the side of the table on the floor, touch the adjustable, adjustable height adjustable feet. All edges of the plate elements are trimmed with the ABC band 2mm thick. Dimensions: 1400x600x750mm Quantity: 6 |  |  |  |
|  | Conference table – ST2 The conference table was made from the chipboard of thickness d = 25 mm in the design of the BEECH NATUR. The dashboard is 1200h800mm, a site measuring 790h725mm. Install vertical twisting between horizontal sides, horizontal plate height 400mm. In the side of the table on the floor, touch the adjustable, adjustable height adjustable feet. All edges of the plate elements are trimmed with the ABC band 2mm thick d = 2 mm. Dimensions: 1200x800x750mm Quantity: 4 |  |  |  |
|  | Office table – ST3 Table is made from a chipboard of thickness d = 25 mm in a design BEECH NATUR. The worktop is 1600h600mm, the size of the page is 590h725mm. Inside the table there is a vertical casing, horizontal plate height of 400mm. In the body of the table there is a fixed cube with 3 drawers, total height 400mm. The horizontal plate, the cube body and the forehead are made of chipboard d = 18 mm, and the drawer corners are made of d = 18mm thickness in a basic white colour. The metal handle was fitted to the forehead. In the side of the table on the floor, touch the adjustable, adjustable height adjustable feet. All edges of the plate elements are trimmed with the ABC band 2mm thick. Dimensions: 1600x600x750mm Quantity: 1 |  |  |  |
|  | Working kitchen table – ST4 Table is made of worktop and metal construction. The worktop is made from chipboard of thickness d = 25mm in the design of BEECH NATUR. Metal construction dimensions 990h590h725mm is made of cross-sectional cross-section profiles 40h40mm in grey. The metal construction follows the worktop on all edges. Below the worktop, in the middle of the table, an additional reinforcement is placed between the two long edges. The legs of the table are on the sides, in contact with the floor, connected by a horizontal element so that the entire shorter edge of the table rests on the backing. Set the adjustable height adjustable tabs between the base and the leg. Dimensions: 1000x6000x750mm Quantity: 2 |  |  |  |
|  | Dining table – ST5 Material for manufacturing the desk legs of the cross-section dimensions being 40x40mm and a profiled rail of the cross-section dimensions being 10x20mm made of beech massif and surface treatment provided by glazing and varnishing by using a polyurethane lacquer in the tone selected by the order maker. Desk slab is made of chipboard thickness: 25m thick in the BEECH NATUR design, trimmed with ABC band 2mm thick. Dimensions: 900x900x750mm Quantity: 1 |  |  |  |
|  | Club table – ST6 Desk made of chipboard thickness: 25mm, in BEECH NATUR design. The desk is consisted of the top and bottom slabs of the dimensions being 800x600mm and side dimensions 600x450 mm. Between two sides of the table is to be installed vertical slab to achieve rigidity 200 m high, made of chipboard 18, thick in the BEECH NATUR design. Adjustable pads with adjustable height re to be installed to the desk sides to make contact with the floor. All edges are to be trimmed with the ABC band 2mm thick. Dimensions 600x500x500mm Quantity: 6 |  |  |  |
|  | Working garage table – ST7 Table is made of worktop and metal construction. The worktop is made from a chipboard of thickness d = 25mm in the design of BEECH NATUR. The metal structure measuring 1790h690h825mm is made of cross-sectional cross-section profiles 40h40mm in gray. The metal construction follows the worktop on all edges. Below the worktop, in the middle of the table, an additional reinforcement is placed between the two long edges. The legs of the table are on the sides, in contact with the floor, connected by a horizontal element so that the entire shorter edge of the table rests on the backing. Set the adjustable height adjustable tabs between the base and the leg. Dimensions: 1800x700x750mm Quantity: 1 |  |  |  |
|  | Chairs for presentation room The upper part of chair must be made of pressed beech veneer, 10mm thick, with anatomic shape. Surfaces are ground with trimmed edges. The surface is varnished in beech colour. The chair must be upholstered with fabric in the material and colour which is acceptable for Contracting Authority. On the back of the chair there is an anatomical grip in the shape of an ellipse. Chair construction made of cold-rolled metal pipes Ø20mm, pipe wall thickness being minimum 1,5mm, plasticised in grey colour, with the legs placed in plastic pads. Pop rivets should be used to fasten the elements to the construction. Dimensions: Height: 840-870 mm Width: 460-480 mm Depth: 470 – 500 mm Quantity: 132 Standard: EN 1729 or similar |  |  |  |
|  | Office chairs 1 The upper part of chair must be made of pressed beech veneer, 10mm thick, with anatomic shape. Surfaces are ground with trimmed edges. The surface is varnished in beech colour. The chair must be upholstered with fabric in the material and colour which is acceptable for Contracting Authority. On the back of the chair there is an anatomical grip in the shape of an ellipse. Chair construction is provided with five wheels covered with rubber and a gas mechanism with adjustable height with the possibility of adjusting the chair back in two positions. Dimensions: Height: 840-870 mm Width: 460-480 mm Depth: 470 – 500 mm Quantity: 20 |  |  |  |
|  | Office chair 2 Chair construction is provided with five wheels covered with rubber and a gas mechanism with adjustable height with the possibility of adjusting the chair back in two positions. Chair construction is made of pressed beech veneer 0mm thick and with anatomic shape. A part of the construction on its lateral side is raised and serves the function of a handle. The seat and the backside of the chair or whole chair must be covered with anatomic sponge and must be padded with eco-leather by the order maker's choice. Surfaces are ground with trimmed edges. The surface is varnished with polyurethane lacquer. Quantity: 1 |  |  |  |
|  | Club chairs Club chair has a cube shape and the minimum dimension 500x700x700mm. The seat is at the height of 380 - 420mm. The bearing construction of the chair is made of beech massif while the surface elements are treated by a good quality. Cahir legs should beat the height of 80 - 100mm, made of beech massif and diagonally interconnected with the horizontal elements. The chair seat and backside are made of plywood of at least 14mm thickness. The visible chair structure elements have glazed surfaces in the colour selected by the order maker, followed by applying the base lacquer in two coats, with an interphase grinding and finishing matte lacquer of high density. The chair is handle-less and padded with sponge and upholstered. Quantity: 18 |  |  |  |
|  | Dining room chairs Chair must be made of beech massif and glazed on surface as well as varnished with polyurethane lacquer in the colour selected by the order maker. The seat and the chair backside are made of plywood 10 mm thick. The seat's dimensions being at minimum 400x400x10mm and the backside dimensions being minimum 180x400x10mm. Legs are made of beech massif minimum dimensions 35x35mm. Chair height to the seating element is 440-450mm and the height of the entire chair is 800 - 840mm. Legs are secured with rubber pads. Chair legs are fastened with a plywood piece. Quantity: 4 |  |  |  |
|  | Clothes hanger – C1 The base is made of chipboard of dimensions 2000x300mm, thickness d = 18mm in a design BEECH NATUR. All edges are dangled with ABS tape d = 2 mm. On the surface, 13 double-hooked hooks are installed in two rows. The first row consists of 7 hooks placed 100mm from the upper edge of the panel, while the second row has 6 hooks from the upper edge of the 230mm plate. The axial distance between the hooks is 250mm. The base is attached to the wall using screws and anchors. The total depth of hooks with hooks is 50mm. Dimensions: 2000x300x50mm Quantity: 3 |  |  |  |
|  | Clothes hanger – C2 The base is made of chipboard of dimensions 1600x300mm, thickness d = 18 mm in a design beech "NATUR". All edges are dangled with ABS tape d = 2 mm. On the surface, 11 double-hooked hooks are installed in two rows. The first row consists of 6 hooks placed 100mm from the top edge of the board, while the second row has 5 hooks from the upper edge of the 230mm plate. The axial distance between the hooks is 250mm. The base is attached to the wall using screws and anchors. The total depth of hooks with hooks is 50mm. Dimensions: 1600x300x50mm Quantity: 2 |  |  |  |
|  | Panel curtains Total surface is 54 m2. The dimensions and colour will be upon desire of Contracting Authority. |  |  |  |
|  | Wastepaper basket Material: metal Colour: grey Dimension: 300mm height minimum Quantity: 7 |  |  |  |
|  | Kitchen furniture – KUH1 The kitchen consists of lower elements and upper elements. Lower kitchen element is made in the "BEECH NATUR" design and the work plate thickness is 40mm in colour at the customer's request. In the lower part three fields are 600mm width, of which the third is designed to set the inox sink, then the element of 500mm width, in which are placed 4 drawers and a terminal of 600mm width where is set the electrical plate with 4 burners. The upper elements have dimensions 2900x600x300mm. To the left, they are subdivided into 3 parts, 600 mm long, each is closed with 2 fronts doors, then one open element of 500 mm width and at the end, the 600 mm width element is repeated. All fronts and visible parts of the cube are made of chipboard thickness of 18mm in the BEECH NATUR design, while the lower body corps is made of white chipboard base. All visible edges are trimmed with the ABC band 2mm thick. All wings are secured to the sides by the hinge hinges. Handles are made of metal in grey. On the left side of the kitchen elements should be placed refrigerator with minimum volume of 300 litres. |  |  |  |
|  | Kitchen furniture – KUH2 The kitchen consists of lower elements and upper elements. Lower kitchen element is in the BEECH NATUR design and the work plate thickness d = 40mm in colour at the customer's request. The lower element is divided into three parts. The first part has a width of 400mm and on the worktop there is an embedded electrical plate with two burners. The other two sections are 600mm wide, the middle one has INOX sink. The back of the tract pulls 50mm to the inside and makes them from the moulded plywood thickness d = 18mm. All edges are trimmed with ABS tape thickness d = 2mm. The upper element is divided into three parts. The first part is 400mm wide, has one shelf in the middle and the front is composed of one wing. The middle and third sections are closed type, width is 600mm and has one shelf in the middle. All fronts and visible parts of the cube are made of chipboard thickness of 18mm in the BEECH NATUR design, while the lower body corps is made of white chipboard base. All visible edges are trimmed with the ABC band 2mm thick. All wings are secured to the sides by the hinge hinges. Handles are made of metal in grey. On the left side of the kitchen elements should be placed refrigerator with minimum volume of 300 litres. |  |  |  |
|  | Wardrobe bench – SE6 The bench is consisting of a seating plate and metal construction. The seating panel is 2000x400mm and is made from a chipboard of thickness d = 25mm in a design BEECH NATUR. All edges are canted ABS tape thickness d = 2mm. The metal structure is in the dimensions of 1990h390h425mm and is made of boxed profiles 40h40mm, so that the profile tracks all four edges of the board and rests on the floor over four vertical elements at the corners. On the lateral sides, the supports are additionally stiffened by a horizontal element that is raised from the bottom of the floor by 50mm, while under the panel, between the longer edges of the construction, there are two more elements that reduce the range and increase the stability of the benches themselves. Below the foot of the bench, in contact with the floor, install the adjustable height adjustable feet. Dimensions: 2000х400х450mm Quantity: 3 |  |  |  |
|  | Wardrobe bench – SE7 The bench is consisting of a seating plate and metal construction. The seating panel is a dimension of 1600h400mm and is made of a chipboard d = 25mm thick in BEECH NATUR design. All edges are canted ABS tape thickness d = 2mm. The metal structure is 1590h390h425mm in size and made of 40x40mm box profiles, so that the profile tracks all the four edges of the seating plate and relies on the floor over four vertical elements at the corners. On the lateral sides, the supports are additionally stiffened by a horizontal element that is raised from the bottom of the floor by 50mm, while under the panel, between the longer edges of the construction, there are two more elements that reduce the range and increase the stability of the benches themselves. Below the foot of the bench, in contact with the floor, install the adjustable height adjustable feet. Dimensions: 1600x400x450mm Quantity: 2 |  |  |  |
|  | Office hanger Material: metal Dimension: 1600 – 2000 mm height Quantity: 8 |  |  |  |
|  | Holder for liquid soap in the toilets Material: inox Quantity: 8 |  |  |  |
|  | Electrical hand drier Material: inox Quantity: 8 |  |  |  |
|  | Toilet can Material: inox Quantity: 16 |  |  |  |
|  | Holder for toilet paper Material: inox Quantity: 8 |  |  |  |
|  | Toilet brush Material: inox Quantity: 8 |  |  |  |