CORRIGENDUM No: 1

to the

CONTRACT NOTICE

Publication Ref: RORS-40/2018

- Subject: Supply of 2 fire trucks and 1 multifunctional tool

Location: Romania, Recas, Timis

The contract notice is corrected or modified as follow:

Document:

Technical Specifications (Annex II) & Technical Offer (Annex III) (c4f_annexiitechspeciiitechoffer_en)

Item number 1 Fire truck 2 pieces

Instead of:

Engine:

- Diesel

- Power: minimum 300 HP

- Pollution norm: Euro 6

- Fuel tank: minimum 100 liters

- Fuel heating system

Read:

Engine:

- Diesel

- Power: minimum 300 HP

- Pollution norm: Euro 6 or equivalent

- Fuel tank: minimum 100 liters

- Fuel heating system

Instead of:

Transmission:

- Traction 4x4

- Manual gearbox

- Gearbox cooling system

Read:
Transmission:

- Traction 4x4 with part time rear axel

- Manual gearbox

- Cooling system

Instead of:
Brakes:

- ABS system

- Pneumatic brake control

- Parking brakes

- Drum brakes

Read:

Brakes

- ABS system

- Brake control system

- Parking brakes

- Drum brakes

Instead of:

Cabin:

- Air suspensions

- Stabilizer bar

- Towing device

- Illuminated steps

- Heated windscreen

- Doors with doubled windows

- Electric windows for driver and passenger

- Electrically adjustable and heated rear view mirrors

- Panoramic mirror for driver and passenger

- Close up mirror

- Front mirror

- Centralized lock

- Sunshield

- Cabin interior light

- Driver seat with air shock absorber

- Doors with storage pockets

- Air conditioning system

- Counter for engine operating hours

- Air pressure display

Read:

Cabin:

- Double cabin

- Laminated windscreen

- Doors with tinted windows

- Electric windows for driver and passenger

- Electrically adjustable and heated rear view mirrors

- Right and left heated rear view mirrors for low objects

- Right and left heated rear view mirrors for panoramic view

- Front mirror on passenger side

- Sunshield

- Cabin interior light

- Driver seat with air shock absorber

- Storage compartments

- Fog lights

- Air conditioning system with automated temperature control

- Counter for engine operating hours

- Air pressure display

Instead of:

Other features and accessories:

- Steering system with double circuit

- Day lights

- Height adjustable light beam

- Hose for tire inflation

- Jack

- Wheel chocks – 2 pcs

- First aid kit

- Fire extinguisher

- Reflective triangle – 2 pcs

- Tool box

Read:

Other features and accessories:

- Reverse alarm

- Hose for tire inflation

- Wheel chocks – 2 pcs

- First aid kit

- Fire extinguisher

- Reflective triangle – 2 pcs

- Superstructure: shall be made of made of aluminium profile and will be linked to a stable structure using aluminium plates. The welds shall be executed accordingly to ISO 9606-1:2013 and ISO 9606-2:2005 standards and the supplier must provide the certificates issued by independent organizations to prove that the personnel is trained for this type of welds.

Instead of:

Pumping aggregate:

- Centrifugal pump located at the rear of the vehicle

- The components that come in contact with water will be made from corrosion resistant materials

- The pump and all the hose systems will provide possibility for complete water evacuation

The pump control panel will contain at least the following:

- Manovacuumeter

- Low pressure manometer

- Water and foam level

- Indicator for engine temperature

- Indicator for number of operating hours

- The panel will be provided with lights for night operation

- Automatic priming unit

- Two pressure levels (a normal pressure level from 0 ÷ 16 bar and a second level with high pressure 35 ÷ 40 bar )

- Suction pipe (a pipe of 4” for natural water sources and a B type inlet for pressure water sources)

- Water and foam discharge pipes for low pressure (two type B, two type C)

- High pressure hoses on two reels with guns for high pressure delivery

Read:

Pumping aggregate:

- Centrifugal pump located at the rear of the vehicle

- The components that come in contact with water will be made from corrosion resistant materials

- The pump and all the hose systems will provide possibility for complete water evacuation

- Start and stop of the pump must be executed inside the cabin

- The pump capacity: minimum 3000 l/minute at 10 bar pression and minimum 400 l/minutes at 40 bar pression

The pump control panel will contain at least the following:

- Manovacuumeter

- Low pressure manometer

- Water and foam level

- Indicator for engine temperature

- Indicator for number of operating hours

- The panel will be provided with lights for night operation

- Suction pipe

- Two exit pipes

- 1 exit for drum hose

Instead of:

Water and foam tanks

- Water tank with a capacity of minimum 5000 litres, made of fiberglass reinforced plastic

- Foam tank integrated in the water tank, capacity minimum 5000 liters

- The tanks will be provided with longitudinal and transversal sections to avoid excessive balance

- Manhole with diameter minimum 500 mm

- External inlets for water and foam filling

- Level indicators on the pump control panel

- There will be provided a heater to protect the water inside the tank against freezing

Read:

Water and foam tanks:

- Water tank with a capacity of minimum 5000 litres

- The tanks will be provided with longitudinal and transversal sections to avoid excessive balance

- External inlets for water and foam filling

- Level indicators on the pump control panel

- The tenders must provide a certificate issued by the Minister of Health or any equivalent organization to prove that the water tank is suitable for drinking water.

- Foam tank integrated in the water tank, capacity minimum 1000 liters

Item number 2: Multifunctional tool

Instead of:

Accessories:

- Snow plow

- soil bucket

- mower with articulated arm

- water tank with spraying installation

- leaf bucket, rotary brush, collecting sweeper

- equipment for high pressure cleaning

Read:

Accessories:

- Snow plow

- soil bucket

- mower with articulated arm

- water tank with spraying installation

The unit has to provide the contracting authority the possibility for further equip of at least the following work attachments: snow cutter, snow bucket, pallet fork, leaf bucket, rotary brush, collecting sweeper, equipment for spreading anti-skid material and platform for working at heights

Instead of:

Engine:

- turbo diesel engine

- 4 cylinders 3300 cm3

- minimum 100 kW

- pollution norm: minim Euro 4

Read:

Engine:

- turbo diesel engine

- 4 cylinders 3300 cm3

- minimum 100 kW

- pollution norm: minim EU stage 4

Instead of:

Transmission:

- hydrostatic transmission with variable displacement pump

- minimum 2 range of speeds:

- 1st step, maximum speed: minimum 20 km/h

- 2nd step, maximum speed: minimum 50 km/h

Read:
Transmission:

- hydrostatic transmission with variable displacement pump

- minimum 2 range of speeds:

- 1st step, maximum speed: minimum 20 km/h

- 2nd step, maximum speed: minimum 50 km/h

- Function for very slow movement (inching) with actuation possibility from pedal

- Traction force:

- In 1st speed step: minimum 4200 daN

- In 2nd speed step : minimum 2100 daN

Instead of:

Features:

- Hydraulically actuated rear lift

- Central lubrication system

- Filling pump for central lubrication system

- Hydraulic system with pilot operated valves

- Automatic differential lock on front and rear axles

- Heated rear view mirrors

- Engine heater

- Automatic driving lights

- LED work lights at front and rear

- Foldable beacon

- Adjustable steering wheel

- Heated seat

- Air conditioning system

- Safety belt

- Filling pump for hydraulic oil

- FOPS safety system – protection against falling objects

Read:

Features:

- Hydraulically actuated rear lift

- Central lubrication system

- Filling pump for central lubrication system

- Hydraulic system with pilot operated valves

- Automatic differential lock on front and rear axles

- Heated rear view mirrors

- Engine heater

- Automatic driving lights

- LED work lights at front and rear

- Foldable beacon

- Adjustable steering wheel

- Heated seat

- Air conditioning system

- Safety belt

- Filling pump for hydraulic oil

- FOPS safety system – protection against falling objects

- Articulated hydrostatic steering system

- Hydraulic installation

- ROPS ISO 3471 safety system – rollover protection

All the technical specifications will be considered as minimal. Where technical specifications indicating a particular origin, source, production, special process, factory or trade mark appear, a patent of intent, a manufacturing license will be read or equivalent

All other terms and conditions of the contract notice remain unchanged. The above alterations and /or corrections to the contract notice are integral part of the contract notice.