# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Fireman equipment and vehicles	p 1 /…

Publication reference : RORS38/Municipality of Golubac/72041/TD8

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Impotant Note:

- Tenderer should fill data only in tables for lots he wants to apply and other tables should be deleted.

- If applicable/possible for the purpose of evaluation tenderers should provide Manufacturer Catalogue of the each device with marked minimum technical features.

LOT 1 Fireman Vehicles

| 1. Lot 1 Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| L1.1 | Fireman Vehicle VP7000/700: 1 pieces Description and purpose of vehicle: firefighting vehicle for fire extinguishing actions of flammable liquids and solid flammable substances. The vehicle should be capable for drive on roads, and offroad. The vehicle should be formed from the chasiss and superstructure and must have all required firefighting equipment in it. The vehicle should be supplied with a fire pump with installations, water tanks (7000 lit.) and foam, (700 lit.), as well as firefighting equipment for firefighting and protection of rescue firefighters. The supplier must have a service on the territory of the Republic of Serbia for all installed parts and equipment. Required Firefighting equipment must be constructed according to SRPS EN, EN, DIN or equivalent standards. Substructure (Vehicle): - Engine power: min 410 HP - Max engine torque: min 2000Nm - Engine type: diesel, EURO V or better - Engine displacement: min 12500ccm - Engine breaks power: min 370kW - Cold start system - Fuel tank: min 300 liters, made from aluminium or stainless steel. - fuel filter heater included - Automatic or semiautomatic transmission with 4x4 drive - Ground clearance: min 300mm - Batteries min 2 with min 170Ah - AC device included - Digital tachograph included - Cruise control system included - Left side steering wheel with servo included - Fire fighting vehicle blinker lights mounted and emergency vehicle siren. - Installation for mounting of radio communication equipment - Protection of headlights made from steel - LED daylights included - Heavy duty front bumper made from steel or similar - Crankcase made from steel - Alarm for reverse movement included Superstructure - Description: Made from aluminium or other similar non corrosive material with similar strength. Space reserved for equipment, pump and installations. - Internal storages for equipment and tools with Aluminium rolling shutters for protection against external influences and external locking. - Upper/top side of superstructure must be covered with anti slip material and superstructure need to have suitable ladder for access to top side - Water reservoir type and capacity: made from stainless steel (min 7000 liters) for water with internal seawalls. - On top of water reservoir reservoir there should be access shaft min 500mm in diameter - Water reservoir needs to have overspill exhaust, electric water level measurement, Ø75mm (type B) connector for hydrant, and connector for refill and drain with pump - Water reservoir attached to substructure via elastic support to allow off-road driving of vehicle - Foam reservoir made from non corrosive material, with capacity 700 liters. Needs to have connection for refill, vent for overpressure and vacuum, electric measure of level and connection for system for mixing of water and foam and draining foam. - Water pump description : centrifugal driven vehicle by driveshaft capable with regulation of water pressure - Water pump pressure: min 8 bars at 1600liters/minute - Water pump material: made from metal resistant to corrosion - Control panel board - with manual switching on / off of drive and control functions (switches) Positioned in the rear of the superstructure - command of the speed control of the engine motor (pumps) - water and foam level gauges -pressure indicator -STOP switch -other if relevant - Foam mixer: automatic dosage in all pump work modes 1-6% - Fast intervention fire fighting winch on side of vehicle must contain 25m incompressible hose of internal diameter min 25mm, with capacity at least 200l/min and pressure adequate for pump supplied, exit "C" connection Ø52mm, nozzle capable for capacity 0-200 liters per minute with possibility of facilitation firefighting with foam, possibility of full or scattered jet - Installation of the internal pipelines: Pipes must be made of steel and hot galvanized or stainless or similar. The entire installation must be resistant to corrosion. - Water cannon: the barrel must be placed in the upper part of the superstructure, Max capacity at least 1600 liters per minute (with valve for adjustment) max range when working with water: min 60m max range when working with foam: min 50m 360 ° rotation horizontally. Rotation vertically from -15 ° to + 80 ° Possibility of full or scattered jet - Blinker lights on the upper part of the vehicle cabin, and rear of vehicle in line with vehicle purpose. - Appropriate sound signal siren min 100 W - External and internal superstructure lighting (storage boxes and equipement). Aux Equipment: - Standard fire hoses Ø 75 mm, length 15 m. with couplings (in according with DIN 14811) or similar x 2pcs. - Standard fire hoses Ø 52 mm, length 15 m. with couplings (in according with DIN 14811 or similar) x4pcs - Reduction coupling 75/52mm (in according with DIN 14342) x2pcs - Universal key ABC, for coupling shafts (in according with DIN 14822) x2 pcs - Suction hose ø 110 clamp, 1.6 meters with all necessary couplings 2sets - Hand-held portable fire extinguisher, S-9 x 1pcs - Hand-held portable fire extinguisher, CO2-5 x1pcs - Universal nozzle with adjustable flow and variation of level of spray up to 400l / min at 6 bar with maximal range of minimum 32m ( in according with DIN 15182 or similar) x 2pcs - Hydrant connection with required couplings x1pcs - T tool for hydrant connection mounting 1pcs Color: in according with Serbian legislative for this type of vehicles. RAL 3000 Markings on vehicle should be in line with IPA Romania- Serbia crossborder programme VIM. http://www.romania-serbia.net/?page_id=212&lang=en_GB |  |  |  |
| L1.2 | Fireman Vehicle V300: 1 pieces Description and purpose of vehicle: Multi-purpose emergency vehicle with superstructure for fast response in fire fighting and other purposes suitable for quick deploying in remote areas and with off road capability. Substructure (Vehicle): - Engine power: max 120KW - Max engine torque: max 360Nm - Engine displacement: max. 2000 ccm - Engine type: diesel, EURO VI or better, no AdBlue - Transmission: 4WD with transfer case - Ground clearance: min 220mm - Vehicle approach angle forward: min 30o - Vehicle approach angle backward: min 20o - Cabin type: 4 doors - Vehicle category: N1 - Carrying capacity: min 1000kg - Trailer towing force min. 3500 kg - AC device - Minimal safety equipment: 6x airbag, ABS, ESP, TCS, EBD - 16 inch steel wheels Superstructure - Description: Made from aluminium or other similar non corrosive material with similar strength. Space reserved for equipment, pump and installations. Side/back openings can be closed by rolling panel/door with lock for security reasons and protection from dust and water. - Reservoir type and capacity: made from polyester, stainless steel or similar material - min 300 litres for water with internal seawalls and connection for hydrant refill - min 25 litres for foam tank with connection for refill and mixing with water equipment and discharge - Water pump description : high-pressure pump aggregate. Compact design includes a drive motor gasoline/diesel type with electric start and manual start, fuel tank, winch with a high pressure hose of min 60 m length. On the end of the hose jet nozzle should be mounted with manual adjustment from full flow to water mist. - Water pump capacity: min 20 liters/minute at pressure 250 bars - Foam mixer: manually adjusted Electrical equipment - All lights required by traffic legislation in Serbia - Lights inside cabin and superstructure - Fire fighting vehicle blinker lights mounted and 100 W emergency vehicle siren. - Installation for mounting of radio communication equipment Aux Equipment: - at least two pull out boxes for equipement - All necessary tools/parts for vehicle operation. - Two-legged firefighting ladder min 2m in length - Equipment obligatory by Serbian traffic legislation. Color: in according with Serbian legislative for this type of vehicles. RAL 3000 - Markings on vehicle should be in line with IPA Romania- Serbia crossborder programme VIM. http://www.romania-serbia.net/?page_id=212&lang=en_GB |  |  |  |

LOT 2 Fireman Equipement

| 1. Lot 1 Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| L1.3.1 | Fire extinguishing backpack used in forest firefighting – 12 pcs, with the following minimum technical requirements: - Volume: 18-20 litres, - Water tank equipped with filter and 3600 coupling for water outlet. - With shoulder straps and a handle for carrying as well additional straps for carrying of tools. - With nozzle for simple jet stream and diffused stream/mist, - Max range of simple jet stream– ≥ 10 meters, Note: Fashion model sizes to be delivered will be determined by Contracting Authority depending on size standard used by producer after contract is signed. |  |  |  |
| L1.3.2 | Firefighting waist belt - 12 pcs - Certified as to DIN 14927 or equivalent. |  |  |  |
| L1.3.3 | Fire fighting gloves (Extreme protective gloves). – 12 pcs, - According to EN 659 or equivalent. - The gloves are water and windproof and breathable. |  |  |  |
| L1.3.4 | Protective boots (Extreme protective boots) – 12 pcs, - According to EN 15090 or equivalent. - The skin should be fire resistant and the device should be of waterproof Note: Fashion model sizes to be delivered will be determined by Contracting Authority depending on size standard used by producer after contract is signed. |  |  |  |
| L1.3.5 | Fire fighting helmet – 12pcs, - In line with EN 16471 or equivalent - Helmet should be low weight, comfortable with adjustable headband - each one to be delivered with suitable firefighter goggles |  |  |  |
| L1.3.6 | Protective jackets-12 pcs, - According to EN 15614 or equivalent - Equipment should be comfortable and easy to carry. Note: Fashion model sizes to be delivered will be determined by Contracting Authority depending on size standard used by producer after contract is signed. |  |  |  |
| L1.3.7 | Waist trousers (Protective trousers) - 12 pcs - According to EN 15614 or equivalent - Equipment should be comfortable and easy to carry - resistance to friction and wear inherent to a forest setting Note: Fashion model sizes to be delivered will be determined by Contracting Authority depending on size standard used by producer after contract is signed. |  |  |  |
| L1.3.8 | Firefighter mask –12 pcs - The equipment will be used by firefighters, in case of forest fire extinguishing. It should protect the face and neck from radiation burns and to help reducing smoke and ash particles inhalation. It should incorporate filter made from activated carbon or equivalent/better - Protective half mask according to EN 140 or EN 149 or equivalent, with one filter for the protection of the respiratory organs - Along with the mask, deliver a spare filter. |  |  |  |
| L1.3.9 | Fire swatter - 12pcs - Strudy design suitable and designed for fighting forest fires - Total length 1.8-2.2m - Capable for dismatling for easy transportation |  |  |  |

LOT 2 SUV Vehicle 4x4

| 1. Lot 2 Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| L2.1 | SUV Vehicle 4x4 - 1pcs Description and purpose of vehicle: passenger SUV 4x4 vehicle for transport of firefighters with high off-road capabilities. - Engine power: min 60KW - Engine type: EURO VI or better - Transmission: 4WD - Ground clearance: min 0.2m - Fog lights included - ABS brakes included - Power steering included |  |  |  |