# ANNEX II + III:	 LOT 2 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply and delivery of firemen equipment 	p 1 /…

Publication reference: RORS28/Mosna/TD7

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Radio station with 5 mobile units – 1 set |  |  |  |
|  | - VHF digital-analogue base radio station with frequency range 136-174 MHz, minimum 16 channels, power supply adapter, base vertical antenna with frequency range 160-174 MHz and minimu 20 m coaxial cable for connection between radio station and antena, for instalation in Emergency situation management centre – 1 piece; - VHF digital-analogue mobile radio station with frequency range 136-174 MHz, minimum 16 channels, mobile vertical antenna with frequency range 160-174 MHz, minimum 3 m coaxial cable with for connection between radio station and antena, for instalation in the fire-fighting vehicle – 1 piece; - VHF digital-analogue hand radio station with frequency range 136-174 MHz, minimum 16 channels, aditional microfon with speaker for connection with radiostation, for firemen – 4 pieces; |  |  |  |
| 2 | Loud siren for the public warning – 1 set |  |  |  |
|  | Power supply: batery Range: 1 km Activation: hand activation and WiFi connection activation |  |  |  |