# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Purchasing of Truck for winter maintenance of the roads										p 1 /…

Publication reference : RORS 40-15/2018

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | All requirements mentioned and requested in this document are minimal and mandatory |  |  |  |
| 1. | TRUCK FOR WINTER MAINTENANCE OF THE ROADS |  |  |  |
|  | Type of vehicle – Truck with dumper for winter maintenance of the roads Driving formula: 4x2 Motor: Diesel 4-tact, turbo charger Fuel: Euro-diesel Engine power: minimum 160 kW Cooling: water based Characteristics of undercarriage Length of undercarriage: Minimum 5.800 mm Width of undercarriage: Minimum 2.000 mm maximum mass of the vehicle: 16.000 kg Cabin: airconditioned, digital tachograph Seats: 1+1 minimum Crew seats equipped with safety belts Doors: 2 minimum Gear box , manual gearbox with clutch Breaks: with ABS system Steering mechanism: servo device Steering wheel: adjustable from the left side in direction of the vehicle motion Wheels: standard for manufacturer, for basic chassis and spare wheel of the same type and size as the basic wheels Color of the vehicle: choice of provider Light signalization: device with rotating yellow light above the cabin Day light set Electrical installation voltage minimum 24 v Tools and equipment for the vehicle: Standard tools for replacement of pneumatics-wheels Medical first aid kit, tow hook, reflecting vest, triangle, spare bulbs Fog lights Euro hook Upgrade 3-side dumper upgrade of min. approx. 4,5m3 capacity Metal cargo space of the dumper upgrade with 3-side dumping (left, right, rear) Dimensions of the dumper box aligned with the allowed load of the chassis minimum: 4.100mm length 2.000 mm width 500mm height of side and rear boards Frontal board of the box is elevated ensuring protection of the space between cabin and upgrade Plastic fenders for driving axle, Carrier of the spare tire placed in chassis. Tarpaulin for covering of the load included in delivery Installed side protection from sliding Steering – dumping of the load from cabin Steel parts of the upgrade – dumper sand-blasted and double-coated in color of the vehicle chassis. Load box in the color of the vehicle cabin. Ensured signalization of the vehicle and the load box in line with the Law on traffic safety Snow coulter Designed to remove the snow deposits at the road Composed of the blade and coulter body, hydraulic system and control box. The blade and coulter body manufactured from steel plate with hardness of min. 400 Hardox Coulter width: min. 2400 mm Coulter height: min. 800 mm Hydraulics for raising and rotation. Rotation angle min. 25 degrees Springs for amortization of the hits Armored rubber blade TOW SPREADER OF SALT Tow spreader of salt is intended primarily for spreading the local roads with grit and salt and mixture of these spreading materials. Powered by its own wheels. Loading from the load box of the truck. Volume of minimum 1 m3 Towing elements: universal tow bar with adjustable height Dimensions adjustable to vehicle chassis and load box. Completely sand-blasted, protected and painted in RAL color 2011. Installed reflecting barriers at the rear part of the spreader LED light for night working conditions Ensured tarpaulin for covering the spreader. Ensured light installation of signalization at the rear end of the spreader and yellow rotating light at the rear end of the spreader, as per the Law on traffic safety. Auxiliary wheel for parking maneuvers Protective rubber curtain on the front side of the spreader Dimensions adjustable to vehicle chassis and load box. |  |  |  |
|  | TRAINING |  |  |  |
|  | Training of the vehicle crew, for both vehicles and upgrades |  |  |  |