# VOLUME 3

# TECHNICAL SPECIFICATIONS

TECHNICAL DESCRIPTION

The following project aims to increase the efficiency of the public lighting system from Municipality of Recaş by replacing the existing lighting bulbs and lighting poles. After the implementation of the new public lighting system the national and international standards will be reached.

After analysing the present situation, a list of public lighting parts with the basic data on the number and type of poles and luminaires, as well as installed capacity was elaborated. The results showed that the luminaires, lighting bulbs and the poles are in a poor condition and the total bills for the electricity consumed for public lighting indicate, that in the following period, it is necessary to upgrade the public and decorative lighting system

Taking into account that the European road E70 is the main street of City of Recas, pedestrian sidewalk will be equipped with lighting sources with 4000 K neutral white colour, while the rest of the luminaires will be 3000 K warm white colour. Due to this lighting sources a higher level of illumination well be available and the pedestrians shall visible form a greater distance and the drivers attention shall be higher. By using this kind of lighting sources, it is possible to reduce the risk of accidents in these areas. Through this project in the road intersection area is created an additional level of illumination over the rest the streets.

For energy efficiency and cost optimization a remote-control system will be implemented. The remote-control system will have the ability to manage and monitor the entire lighting infrastructure (lighting fixtures, their power supply systems, points of

ignition, energy measurement, etc.) and the hardware components will be installed or attached to each luminaire. The system will be controlled by a server and the access to the server will be made by using an encrypted communication, from anywhere, with a PC or portable devices connected to the internet. No additional wiring for data transmission will be used. The information sent in the system will be bidirectional and the luminaires will have the ability to receive and interpret the commands transmitted by the server. The system will allow an unlimited number of queries with each luminaire and data traffic will be unlimited for these queries

In addition, the remote-control system will have the following functions: a single technology for data transmission and control should be used as much as possible and the system will allow the management of a restricted area as well as an extended area to the level of thousands of luminaires on the same platform. At the same time the system will allow the introduction of new luminaires connected to the platform. Power consumption will be available for any query, and can be generated comparative graphical representations of energy consumption over time intervals. Also, the system will allow automatic and manual control commands remotely (via GPRS) and the preprogrammed scenarios will be stored. In the absence of communication, the system will operate autonomously according to the stored scenarios. For higher energy efficiency the system will allow dimming steps, in addition to 100% and 0%, and these steps can be programmed remotely. The system /user will be able to reduce light and consumption on each of the luminaires using preprogrammed scenarios. A user will have the possibility to checked the energy consumed at each point and the access can be done by any user registered in the system by the administrator on any type of terminal connected to the internet. 	All collected data is stored and kept in a SQL database, making the information available for a longer period of time, primarily for the analysis of electricity consumption, prediction lifetime of the light source, detection of problems. Using the SQL a user will be able to run queries to set alarms and filter them according to severity, alarm type, alarm status (closed / open), alarm time. If the lighting fixtures are malfunctioning or defective, the system will be able to detect and report the errors. A registered user can use the system to generate reports in both HTML and Excel format.

The remote system will have an intuitive user interface and allow the creation of visible groups for easy identification of the lighting devices. The groups can be sorted by street, intersection, neighborhood, etc. and will provide a communication function between users. The remote management system will integrate a map with all the components of the lighting system (luminaires ,lighting bulbs, lighting fixtures, etc.). The map will be plotted as visible as possible, and will contain the status of the luminaires and the GPS coordinates. The serial number and hardware components of each selected luminaire, the name/number assigned for each cabinet, the name of each luminaire, will be displayed by the user interface. The system will also include smartphone / tablet applications that can be used by the police force, firefighters, or intervention teams, and in case of incidents or accidents, they will be able to turn off / on the lighting in a certain area, or cancel the hourly reduction (dimming) so that lighting can work at 100% intensity. The interface will be opened through the API for communication with other types of interfaces used in SMART solutions (video surveillance, traffic control, emergency systems, etc.).