# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Procurement of Aquatic Weed Harvester for project "The repairing of the navigation infrastructure on Bega Canal"

Publication reference: VI-85/3-1

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Aquatic Weed Harvester Quantity: 1 set |  |  |  |
|  | The hull is suited with separate compartments and topped with a fully closable deck. The carrier is a compact tool carrier on the nose. Mowing boat is provided with hydraulic engine side shift for counter balance. Standard load-sensing hydraulic system and gear pumps. All major hydraulic functions provided with pressure compensated and low-pressure servo assisted armrest integrated controls. Executed with two anti-coiling auger. On the right side, the boat is fitted with a hull-integrated embankment cutter with top or bottom driven. Aquatic Weed Harvester set must include set of next elements: General and technical requirement: Dimensions - min. 4.60 x min. 1,70 x min. 0,90 m hull min. 4,00 x 1,75 x 0,70 m material 3 mm St. 52 or equivalent Weight - from 1.800 kg to 2.350kg draft from 0,35 – 0,45 m Engine: Engine 4-cylinder from 30 to 40 kW Diesel hydraulic side-shift emission class EU stage 11IA Fuel consumption 4 - 6,5 l/hr Cooling system Keel cooling, Electrical installation 12V or equivalent Propulsion - Two 400-600 mm hydraulic anti coiling auger Steering Servo - assisted with steering angle of 2 x 50° Hydraulic system, load sensing pump with variable displacement for auger(s) and implements gear pump for hydraulic cylinders, 1x LS pump and 1x gear pump with electric proportional valves Fuel tank must be from min. 75 L Hydraulic tank up to 120 L with suction - and return filter. Integrated oil cooling Carter drain pump or equivalent Crew 1 operator, speed 6 - 7 km/hr Height adjustable auger(s), incl. hydraulic function Lifting frame with quick coupling release system Mechanical preparation for embankment cutter and trailing knife system, Electric proportional controls for cylinders and implement functions, Heavy duty auger with electric proportional foot control, hydraulically tiltable, height and angle adjustable steering column, Adjustable comfort-seat with armrests and heating, Engine control panel with hour counter, rev. counter, fuel/temp gauge and 2 x 12 Volt connector, acoustic and optical signals for oil pressure and coolant temperature Anti-slip treatment of deck area, Anti-vandalism cover over operator compartment, 4 lifting eyes for lifting with a 4 leg chain sling, 1 towing eye, wearing strips on hull corners |  |  |  |
| 2 | Arms and Implements Quantity: 1 set |  |  |  |
|  | Arms and Implements must include set of next elements: A) Front implements • Tool carrier with quick coupling • Push rake with quick coupling release It is used to collect mowed vegetation and floating debris. After collecting, the load can be easily unloaded onto the embankment. The push rake is mounted on the nose of the boat (tool carrier). The rake is equipped with fixed min. 0.45 long rigid teeth. The lifting frame are hydraulically operated. Working width min 1.75 m. • Push rake attachments - duck weed bucket working width min. 1.65 m - push rake mower B) Right side implements • Embankment cutter Mowing boat is fitted on the right side with an embankment cutter with double-acting blades from 1.20 m to 1.4 m long, top drive version. A horizontal rotation angle of 180° can be achieved. C) Rear implements • Trailing knives The trailing knife system is situated in the back of the boat. It is operated on the arm-rest console and is hydraulically driven. Two knife sets in a V-shape are towed behind the boat. The trailing knives drag through the soil to cut the vegetation close to the core. Minimum working width must be 4.5 m. • Propulsion two hydraulic anti-coiling auger The anti-coiling auger provides that no vegetation or objects (rope) can get caught in the auger. Double auger is height-adjustable and hydraulically foldable. Hydraulic hoses are included. |  |  |  |

| All items | Supply, delivery and installation performed by contractor or authorised service provider. All the equipment must include all necessary parts and standards for its installation. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Testing of all basic functions of the instrument on a set of producers standard samples commonly used for the corresponding instrument. |  |  |  |
| All items | Training at customer site for minimum 3 persons (upon delivery) on equipment handling (familiarisation during installation - working with the equipment in all basic functions of the equipment), equipment maintenance. Details of proposed training methodology, shall be presented in the tender proposal and shall refer to the related equipment |  |  |  |
| All items | Technical documentation for equipment (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. Technical documentation with drawings for Aquatic weed harvester, Arms and Implements must be delivered for the purpose of aquatic weed harvester registration as technical floating object. |  |  |  |
| All items | - Warranty - Tenderers must provide local reliable warranty service agent providing maintenance and the rapid supply of equipment spare parts and consumables for the Warranty duration of one year. - Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Help Desk (phone) support, which must be available during working hours, 09.00 – 17.00 CET; - Guaranteed maximum response time to submitted maintenance support request (fax or e-mail) of 2 (two) working day; - Guaranteed that any requests for services will be attended to within 72 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 72 hours; - Guarantee that genuine spare parts and consumables will be available for a period of minimum 10 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty Min. 1 year from the date of delivery or min. 1000 hours of operation in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |