# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of CNC lathe	p 1 /…

Publication reference : 394/SCNT/TD5

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | CNC lathe – 1 set which include: CNC lathe - 1pc - Type of machine:Compact PC controled 2 axis table-top CNC turning machine - Clockwise/counter clockwise spindle rotation supported - Max spindle speed : min 4000rev/minute - Max spindle torque: min 10Nm - Spindle motor power: min 1kW - Number of tool turret stations: min 8 - Work area max turning diameter: min 60mm - Work area max part length: min 200mm - Safety glass window in door - Machine should be in line with ISO 23125:2015 or equivalent related to safety of turning machines - Emergency stop push button installed - Feed drive axis feed max. force: min 1000N - Feed drive axis position error: - max 10-5m Control unit with software 1pc which inlude: - Control software (all software required for normal operation without obligatory licence subscription) - CNC machine keyboard with LCD Screen for 2D graphic simulation with autozoom, capable to be used with PC computer. - 3D Simulation software for offline testing of required operation programme. The system should check the programmed travel of the tool to prevent collision with clamping devices and blanks. - Suported languages must include Serbian and/or English ACCESSORIES - Leveling Element - Manual Tailstock - LED Light strip CLAMPING DEVICES FOR WORPIECE - 3-jaw lathe chuck diameter 74 mm - Set of 14 collets TAILSTOCK ACCESSORIES - Live center ACCESSORIES FOR TOOL TURRET - Set of 8 reduction collets for dia.2-9 mm TOOLS - Copying tool right - Copying tool left - Copying tool neutral - Thread-cutting tool holder left hand - Threading die holder f. external threads - Boring bar holder 10x60 mm - Cut-off holder - (10 pcs) carbide indexable inserts - Indexable inserts for alu (10pcs.) - 5 pcs. carbide indexable inserts 2 mm - 5 pcs indexable inserts for OD thread - 5 pieces carbide tips indexable inserts - Centre drill A8 DIN 333 - Set 9 twist drills 2-10 mm - Twist drill 12x10 mm SETTING TOOLS - Optical tool presetting device Important notice: system set should contain software and equipment required for regular operation and user maintenance, including but not limited to, cables, specialized tools and software. Important notice: System specific operation and user maintenance training, for 2-5 operators, will be organized by Contractor before Provisional acceptance is signed by Contracting Authority. Operators will receive training to safely utilize CNC lathe. No training in programming of CNC will be required. |  |  |  |