# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of “Water rescue equipment”	p 1 /33

- Publication reference : RORS 27- TD 07

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required |  |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - All requirements mentioned and requested in this document shall be considered as the minimum acceptable criteria. - All requirements mentioned and requested in this document are accompanied by the mention "or equivalent”. |  |  |  |  |  |  |
|  | - The tenderer is obliged to provide supporting documentation (catalogues, guides, brochures, manuals, etc.) confirming that the offered specification meets the required conditions. |  |  |  |  |  |  |
|  | - Supplier is obliged to provide the possibility of servicing the delivered equipment in the Republic of Serbia within the warranty period and beyond the warranty period. |  |  |  |  |  |  |
|  | - The Contractor is obliged to submit all the necessary documentation that is needed for custom procedure, registration and technical inspection of Trailers in accordance with the legislation of Republic of Serbia. |  |  |  |  |  |  |
| Lot no 1:Rafting and motor boats for water rescue |  |  |  |  |  |  |  |
| - Rafting boats for water rescue |  |  |  |  |  |  |  |
| ## 1.1.1. | Rafts (big) | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Rafting boat suitable for rescue operations in swift waters. |  |  |  |  |  |  |
|  | Boat has bow on both sides with multiple chambers as well as two chambers in the central part of the boat. |  |  |  |  |  |  |
|  | Boat is manufactured of material very resistant on sharp object impact reinforced with additional layer of material. |  |  |  |  |  |  |
|  | Floor of the boat is manufactured in “roll-up” system with self-buoyant floor and drainage holes. |  |  |  |  |  |  |
|  | Material must have non-slip surface on tubes, resistant to acids, algae, fungi, organic fuels and oils and additional resistance on mechanical damages, wear and tear, UV radiation and temperature differences from -30° to +80° C |  |  |  |  |  |  |
|  | Boat is equipped with foot straps and 6handles, 3 on each side for easy caring. |  |  |  |  |  |  |
|  | Boat is equipped with 2 circles on each side with ropes for tying. |  |  |  |  |  |  |
|  | Boat can be used for transport from 8 – 10 persons. |  |  |  |  |  |  |
|  | Fabric – CSM Hypalon neoprene and rubber canvas or equivalent |  |  |  |  |  |  |
|  | Outer length – min 4m |  |  |  |  |  |  |
|  | Inner length – min 3m |  |  |  |  |  |  |
|  | Outer width – min 2m |  |  |  |  |  |  |
|  | Inner width – min 1m |  |  |  |  |  |  |
|  | No of chambers – min 7 (5+2) |  |  |  |  |  |  |
|  | Diameter of tube – min 50 cm |  |  |  |  |  |  |
|  | Branding of rafting boat |  |  |  |  |  |  |
|  | Stickers with emblem of SEM with dimensions 30 x 30 cm |  |  |  |  |  |  |
|  | Beneath the emblem emergency number 193 |  |  |  |  |  |  |
|  | On the side of boat Cyrillic inscription“ВАТРОГАСЦИ-СПАСИОЦИ” (size and colour of letters will be defined later) |  |  |  |  |  |  |
| ## 1.1.2. | Rafts (small) | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
| . | Technical characteristics |  |  |  |  |  |  |
|  | Rafting boat suitable for rescue operations in swift waters. |  |  |  |  |  |  |
|  | Boat has bow on both sides with multiple chambers as well as two chambers in the central part of the boat. |  |  |  |  |  |  |
|  | Boat is manufactured of material very resistant on sharp object impact reinforced with additional layer of material. |  |  |  |  |  |  |
|  | Floor of the boat is manufactured in “roll-up” system with self-buoyant floor and drainage holes. |  |  |  |  |  |  |
|  | Material must have non-slip surface on tubes, resistant to acids, algae, fungi, organic fuels and oils and additional resistance on mechanical damages, wear and tear, UV radiation and temperature differences from -30° to +80° C |  |  |  |  |  |  |
|  | Boat is equipped with foot straps and handles, on each side for easy caring. |  |  |  |  |  |  |
|  | Boat is equipped with 2 circles on each side with ropes for tying. |  |  |  |  |  |  |
|  | Boat can be used for transport from 4 – 6 persons. |  |  |  |  |  |  |
|  | Fabric – CSM Hypalon neoprene and rubber canvas or equivalent |  |  |  |  |  |  |
|  | Outer length – min 3m |  |  |  |  |  |  |
|  | Outer width – min 1,2m |  |  |  |  |  |  |
|  | No of chambers – min 4 |  |  |  |  |  |  |
|  | Diameter of tube – min 30 cm |  |  |  |  |  |  |
|  | The dimensions of the boat must ensure that spinal boar or stretcher can be placed in the boat |  |  |  |  |  |  |
|  | Branding of rafting boat |  |  |  |  |  |  |
|  | Stickers with emblem of SEM with dimensions 30 x 30 cm |  |  |  |  |  |  |
|  | Beneath the emblem emergency number 193 |  |  |  |  |  |  |
|  | On the side of boat Cyrillic inscription“ВАТРОГАСЦИ-СПАСИОЦИ” (size and colour of letters will be defined later) |  |  |  |  |  |  |
| ## 1.1.3. | Paddles with "T" ending | Quantity: 84 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Paddle handle is made of non-corrosive and very rigid material, aluminium alloys or polyurethane fibres or similar. |  |  |  |  |  |  |
|  | Handle ends in “T” shape with curved edges so they form a hook shape. |  |  |  |  |  |  |
|  | Paddle area is minimum 20 x 55 cm |  |  |  |  |  |  |
|  | Paddle height 155 - 165 cm |  |  |  |  |  |  |
|  | Handle diameter 2,5 – 3 cm |  |  |  |  |  |  |
|  | REMARK: Additional strength is required in connecting the handle with “T” ending since rescuers can use the “T” ending as a pulling hook |  |  |  |  |  |  |
| ## 1.1.4. | Accessories for the boat, mending kit, foot pump | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Additional equipment and repair kits for the rafting boats. |  |  |  |  |  |  |
|  | Pump for boat inflating - 2 pieces |  |  |  |  |  |  |
|  | Transport bag for boat when deflated |  |  |  |  |  |  |
|  | Spare valve – 2 pieces |  |  |  |  |  |  |
|  | Wrench for mounting/dismounting of valves |  |  |  |  |  |  |
|  | Repair and maintenance kit |  |  |  |  |  |  |
|  | User and maintenance manual in Serbian |  |  |  |  |  |  |
| ## 1.1.5. | Axle trailer boat for transportation |  | Quantity: 6 pcs |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Trailer for transport of rescue boats to emergency |  |  |  |  |  |  |
|  | No of axles - max 1 |  |  |  |  |  |  |
|  | Useful payload – min 500 kg |  |  |  |  |  |  |
|  | GTW – max 800 kg |  |  |  |  |  |  |
|  | Length of the trailer – max 6000 mm |  |  |  |  |  |  |
|  | Allowed towing hook pressure – max 75kg |  |  |  |  |  |  |
|  | Construction – steel (pipes or profiles or similar) protected from corrosion |  |  |  |  |  |  |
|  | Rubber suspension unbrake axle |  |  |  |  |  |  |
|  | Boat winch – min 400 kg |  |  |  |  |  |  |
|  | Support wheel – 1 |  |  |  |  |  |  |
|  | Trailer is delivered with connector for electric installation and installation for lights hook connector |  |  |  |  |  |  |
|  | Trailer is equipped with clearance lights |  |  |  |  |  |  |
|  | Trailer is equipped with sleds with rubber support for easier pulling of boat on the trailer or equivalent system |  |  |  |  |  |  |
|  | Spare wheel |  |  |  |  |  |  |
|  | Spanner – 2 pieces Total length: 5m (short section 0.5m) + (long part 4.5m) short section: strong stable robust pull-out tension on one end of the spanner double hook long part: a very strong tear-resistant tape with double hook on one end. Width – min 75cm Material – polyester Pull-out tension mast have safety lock Tensile force – min 5000 daN / 10000daN |  |  |  |  |  |  |
|  | Trailer is attested and with 12 months’ warranty |  |  |  |  |  |  |
| ## Motor boats for water rescue |  |  |  |  |  |  |  |
| ## 1.2.1. | Small rubber boats | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Rubber suitable for transport max 10 people (rescuers and victims). |  |  |  |  |  |  |
|  | Boat |  |  |  |  |  |  |
|  | Tube Boat is made of Hypalon-Neoprene CSM |  |  |  |  |  |  |
|  | Flooring material – aluminium |  |  |  |  |  |  |
|  | Length – max 4,8 m |  |  |  |  |  |  |
|  | Inner length – max 3.9 m |  |  |  |  |  |  |
|  | Width – max 2,1 m |  |  |  |  |  |  |
|  | Inner width – max 1,1 m |  |  |  |  |  |  |
|  | Tube diameter – max 50 cm |  |  |  |  |  |  |
|  | Weight – max 150 kg |  |  |  |  |  |  |
|  | Number of chambers – min 6 |  |  |  |  |  |  |
|  | Maximum load – 1300 kg |  |  |  |  |  |  |
|  | The boat must be able to install an outboard engine |  |  |  |  |  |  |
|  | Material must have non-slip surface on tubes, resistant to acids, algae, fungi, organic fuels and oils and additional resistance on mechanical damages, wear and tear, UV radiation and temperature differences from -30° to +80° C |  |  |  |  |  |  |
|  | Outboard engine |  |  |  |  |  |  |
|  | Power – min 40 HP |  |  |  |  |  |  |
|  | Bore and stroke – min 60 x 60 mm |  |  |  |  |  |  |
|  | Gearbox – Forward, Neutral and Reverse |  |  |  |  |  |  |
|  | Fuel tank – portable min 20l |  |  |  |  |  |  |
|  | Engine start – electronic and manual |  |  |  |  |  |  |
|  | Cooling system – water cooling with thermostat. |  |  |  |  |  |  |
|  | Engine should be equipped with propeller protection. |  |  |  |  |  |  |
|  | Branding of boat |  |  |  |  |  |  |
|  | Stickers with emblem of SEM with dimensions 30 x 30 cm |  |  |  |  |  |  |
|  | Beneath the emblem emergency number 193 |  |  |  |  |  |  |
|  | On the side of boat Cyrillic inscription “ВАТРОГАСЦИ-СПАСИОЦИ” (size and colour of letters will be defined later) |  |  |  |  |  |  |
| ## 1.2.2. | Aluminum boats with outboard motor and trailer boat for transportation | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Aluminium rescue boats with outboard engine suitable for transport of 10 – 12 people (rescuers and victims). |  |  |  |  |  |  |
|  | Boat |  |  |  |  |  |  |
|  | Boat is made of durable marine grade aluminium suitable for work in very shallow water. |  |  |  |  |  |  |
|  | Boat is constructed in a manner that provides stability in any situation regardless the weight distribution of load. |  |  |  |  |  |  |
|  | Boat floor is made of non-slip material. |  |  |  |  |  |  |
|  | Boat is manufactured with double hulls and sides in order to increase safety. |  |  |  |  |  |  |
|  | Boat is equipped with small wheels which can be deployed very fast without any tools so it can be dragged on the ground. Wheels can be completely detached for fast operations. |  |  |  |  |  |  |
|  | Boat is equipped with bar on the bow for pulling boots while on wheels. |  |  |  |  |  |  |
|  | Length – max 6m |  |  |  |  |  |  |
|  | Width – max 1,7m |  |  |  |  |  |  |
|  | Height – max 0,7m |  |  |  |  |  |  |
|  | Weight – max 300kg |  |  |  |  |  |  |
|  | Outboard engine |  |  |  |  |  |  |
|  | Power – min 30 HP |  |  |  |  |  |  |
|  | Bore and stroke – min 60 x 60 mm |  |  |  |  |  |  |
|  | Gearbox – Forward, Neutral and Reverse |  |  |  |  |  |  |
|  | Fuel tank – portable min 20l |  |  |  |  |  |  |
|  | Engine start – electronic and manual |  |  |  |  |  |  |
|  | Cooling system – water cooling with thermostat. |  |  |  |  |  |  |
|  | Engine should be equipped with propeller protection. |  |  |  |  |  |  |
|  | Branding of boat |  |  |  |  |  |  |
|  | Stickers with emblem of SEM with dimensions 30 x 30 cm |  |  |  |  |  |  |
|  | Beneath the emblem emergency number 193 |  |  |  |  |  |  |
|  | On the side of boat Cyrillic inscription “ВАТРОГАСЦИ-СПАСИОЦИ” (size and colour of letters will be defined later) |  |  |  |  |  |  |
|  | Trailer |  |  |  |  |  |  |
|  | Trailer for transport of rescue boats to emergency |  |  |  |  |  |  |
|  | No of axles - max 1 |  |  |  |  |  |  |
|  | Useful payload – min 500 kg |  |  |  |  |  |  |
|  | GTW – max 800 kg |  |  |  |  |  |  |
|  | Length of the trailer – max 6000 mm |  |  |  |  |  |  |
|  | Allowed towing hook pressure – max 75kg |  |  |  |  |  |  |
|  | Construction – steel (pipes or profiles or similar) protected from corrosion |  |  |  |  |  |  |
|  | Rubber suspension unbrake axle |  |  |  |  |  |  |
|  | Boat winch – min 400 kg |  |  |  |  |  |  |
|  | Support wheel – 1 |  |  |  |  |  |  |
|  | Trailer is delivered with connector for electric installation and installation for lights hook connector |  |  |  |  |  |  |
|  | Trailer is equipped with clearance lights |  |  |  |  |  |  |
|  | Trailer is equipped with sleds with rubber support for easier pulling of boat on the trailer or equivalent system |  |  |  |  |  |  |
|  | Spare wheel |  |  |  |  |  |  |
|  | Spanner – 2 pieces Total length: 5m (short section 0.5m) + (long part 4.5m) short section: strong stable robust pull-out tension on one end of the spanner double hook long part: a very strong tear-resistant tape with double hook on one end. Width – min 75cm Material – polyester Pull-out tension mast have safety lock Tensile force – min 5000 daN / 10000daN |  |  |  |  |  |  |
|  | Trailer is attested and with 12 months’ warranty |  |  |  |  |  |  |
| Lot no 2:Equipment for water rescue and individual safety equipment |  |  |  |  |  |  |  |
| 2.1. Equipment for water rescue |  |  |  |  |  |  |  |
| ## 2.1.1. | Floating bags with a rope of 30m | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Floating rope is packed in flexible bag |  |  |  |  |  |  |
|  | Rope is very strong, resistant to UV radiation and wear and tear and it is coloured in highly visible colour |  |  |  |  |  |  |
|  | Bag is made of very strong fabric, in highly visible colour equipped with net for rope ventilation and additionally strengthen with belts made of stabilized PES (or equivalent) which in the same time provides possibility of connecting of bag to the vest |  |  |  |  |  |  |
|  | Bag must be buoyant |  |  |  |  |  |  |
|  | Rope length: min. 30 m |  |  |  |  |  |  |
|  | Rope diameter: min. 8 mm |  |  |  |  |  |  |
|  | Rope load: min. 400 kg |  |  |  |  |  |  |
| ## 2.1.2. | Set of hand tools | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Set box wrench 6 – 22 |  |  |  |  |  |  |
|  | Set wrench 6 – 22 |  |  |  |  |  |  |
|  | Set inbus |  |  |  |  |  |  |
|  | Set torex |  |  |  |  |  |  |
|  | side cutter, 200 mm, insulated combination pliers, isolated, 200 mm long nose pliers, insulated, 160 mm adjustable wrench, 20 mm, insulated hammer, 300 g pipe wrench, isolated, 240 mm screw driver, cross recess, PH1, insulated screw driver, cross recess, PH2, insulated screw driver, cross recess, PH2, insulated screw driver, longitudinal slot, 0,6 x 2,8 mm screw driver, longitudinal slot, 0,6 x 4 mm screw driver, longitudinal slot, 1 x 6,5 mm |  |  |  |  |  |  |
|  | tool box, solid plastic |  |  |  |  |  |  |
| ## 2.1.3. | Floating ropes | Quantity: 10 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Floating rope, static suitable for height rescue and swift water rescue |  |  |  |  |  |  |
|  | Floating rope must be in accordance with EN 1891 |  |  |  |  |  |  |
|  | Made of material PA/PPV |  |  |  |  |  |  |
|  | Length - min 200m |  |  |  |  |  |  |
|  | Diameter - 10 – 11 mm |  |  |  |  |  |  |
|  | Tenacity - min 23 KN |  |  |  |  |  |  |
|  | Number of falls - min 20 |  |  |  |  |  |  |
|  | Weight – max 70 g/m |  |  |  |  |  |  |
|  | Percentage of sheath: max 50 % |  |  |  |  |  |  |
|  | Colour: highly visible (yellow, red, orange or similar) |  |  |  |  |  |  |
| ## 2.1.4. | Alpine ropes | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Apline rope type A semi-static or static suitable for height rescue |  |  |  |  |  |  |
|  | Alpine rope must be in accordance with EN 1891 |  |  |  |  |  |  |
|  | Made of Polyamide |  |  |  |  |  |  |
|  | Length: min 200 m |  |  |  |  |  |  |
|  | Diameter: 10.5 – 11.5 mm |  |  |  |  |  |  |
|  | Breaking strength min 27 kN |  |  |  |  |  |  |
|  | Strength tied with figure-eight knot min 18 kN |  |  |  |  |  |  |
|  | Number of factor 1 falls: min 15 |  |  |  |  |  |  |
|  | Weight per meter: max 75 g |  |  |  |  |  |  |
|  | Percentage of sheath: max 40 % |  |  |  |  |  |  |
|  | Static elongation: max 4 % |  |  |  |  |  |  |
|  | Shrinking in the water max 5% |  |  |  |  |  |  |
| ## 2.1.5. | Megaphones | Quantity: 6 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Power output - min 25 W |  |  |  |  |  |  |
|  | Power supply – alkaline batteries |  |  |  |  |  |  |
|  | Distance - min 1000 m |  |  |  |  |  |  |
|  | Wight without batteries max 1,7 kg |  |  |  |  |  |  |
|  | Dimension – max Ø 240 mm, L max 400 mm |  |  |  |  |  |  |
|  | Operation min 4 hours |  |  |  |  |  |  |
| 2.2.Individual safety equipment for the intervention in case of flood |  |  |  |  |  |  |  |
| ## 2.2.1. | Rafting Helmets | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Helmet is consisting of hard shell, tightening straps and shock absorber made of foam and fabric |  |  |  |  |  |  |
|  | Straps are tightened under the chin |  |  |  |  |  |  |
|  | Outer shell can be made of plastic (PVC), carbon or other light but sturdy material |  |  |  |  |  |  |
|  | Helmet must provide ear protection (from injures) |  |  |  |  |  |  |
|  | Helmet must be buoyant |  |  |  |  |  |  |
|  | Helmet must have drainage and ventilation holes |  |  |  |  |  |  |
|  | Helmet colour: Highly visible (red, yellow or orange) |  |  |  |  |  |  |
|  | Helmet must provide head protection level suitable for extreme water sports |  |  |  |  |  |  |
|  | Helmet must be produced in accordance with EN 1385 and labelled with “CE” mark |  |  |  |  |  |  |
| ## 2.2.2. | Two piece thermal suits | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Two-piece thermal insulating (wet) diving suit is made of multilayer laminar neoprene |  |  |  |  |  |  |
|  | Wet suit thickness: min. 5 mm |  |  |  |  |  |  |
|  | Inner part of the suit is made of plush or equivalent fabric. Suit is protecting rescuers skin from direct water splash with possibility that during the swim suits contain very thin layer of water which very quickly reaches body temperature. |  |  |  |  |  |  |
|  | Suit is consisting of upper part (jacket) with long sleeves, integrated hood and zipper and lower part (long johns) with suspenders |  |  |  |  |  |  |
|  | Equipment must be marked with “CE” sign or with Serbian mark for harmonization with EN norms. |  |  |  |  |  |  |
| ## 2.2.3. | Shoes with soles drainage | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Boots are made of thermal insulating fabric – neoprene or equivalent |  |  |  |  |  |  |
|  | Boots are made in a manner which allow rescuers to quickly putt on the boots, they are comfortable to wear and they are providing thermal protection |  |  |  |  |  |  |
|  | Boot are tightened with laces or with tightening strap |  |  |  |  |  |  |
|  | Boots should be reinforced in finger area and they should have hard rubber sole. They have to allow movement on rough, slippery terrain, but also in the water and under water. |  |  |  |  |  |  |
|  | Boots should be equipped with a drainage holes, and they can be used with offered models wet diving suit. |  |  |  |  |  |  |
|  | Boots have to complies with the EC directive for Personal Protective Equipment (Directive 89/686/EEC) and with EN ISO 20345. |  |  |  |  |  |  |
| ## 2.2.4. | Thermal protection gloves | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Thermal diving gloves must be compatible with wet diving suit |  |  |  |  |  |  |
|  | Gloves are made of multilayer laminar neoprene |  |  |  |  |  |  |
|  | Gloves thickness: min 2 mm |  |  |  |  |  |  |
| ## 2.2.5. | Waterproof flashlights | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Headlamp is easily mounted on the helmet by the elastic band |  |  |  |  |  |  |
|  | Lamp is waterproof and it has a mechanism for adjustment of light beam angle |  |  |  |  |  |  |
|  | Headlamp has LED lamp and it is powered with alkaline batteries |  |  |  |  |  |  |
|  | Headlamp is water resistant to minimum IPX 7 and it can be submerged to the depth of 1 m for 30 min |  |  |  |  |  |  |
|  | Angle of light beam: min. 3 positions |  |  |  |  |  |  |
|  | Light beam range: min. 70 m |  |  |  |  |  |  |
|  | Quantity of emitted light: min. 60 lumens |  |  |  |  |  |  |
|  | Working autonomy under maximal load: 10 h |  |  |  |  |  |  |
|  | Equipment must be manufactured in accordance to EN norms and labelled with “CE” mark |  |  |  |  |  |  |
| ## 2.2.6. | Overalls | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Overall suit consists of basic fabric additionally strengthen in shoulder area, elbows, knees and in rear. |  |  |  |  |  |  |
|  | Overall is made of 50% (±3%) and polyester 50% (±3%), weight 260gr (±5%) |  |  |  |  |  |  |
|  | In the chest area, there are two pockets with double partition and with zipper. Above the right pocket there is a Velcro tape for name tag, and above the left pocket Velcro tape for rank. |  |  |  |  |  |  |
|  | There are 2 pockets on the hips and on side of the legs there are 3 dimensional pockets with zipper |  |  |  |  |  |  |
|  | On the front side of the overall there are a full length 2 zipper systems with 1 zipper slider each. Collar is done/undone with Velcro tape and in collar there is an inner hood with lace |  |  |  |  |  |  |
|  | Legs and sleeves ends with cuffs with Velcro tape for adjustment |  |  |  |  |  |  |
|  | Strengthen area on elbows and knees are open on the upper side and in case of need rescuers can insert rubber protectors for knees and elbows. |  |  |  |  |  |  |
|  | Overall is equipped with elastic tape in waist for straining horizontally and vertically |  |  |  |  |  |  |
|  | Back side is manufactured with a pleat in order to provide comfort during work |  |  |  |  |  |  |
|  | On the back, there is a Velcro bas for team insignia. Insignia will be defined latter. |  |  |  |  |  |  |
|  | At the upper part of the right sleeve there is a Velcro base for emblem and on the left sleeve there is Velcro base for flag 9,5 x 6,5 cm |  |  |  |  |  |  |
|  | On the shoulder area of the sleeves there are reflective stripes in grey colour 2cm wide. |  |  |  |  |  |  |
|  | Reflective stripes are positioned on the legs too, and they are placed 15 cm above lower edge of the trouser |  |  |  |  |  |  |
|  | Overall is in the waist area equipped with 6 belt loops 2 cm wide and 4,5 – 5 cm in height |  |  |  |  |  |  |
|  | REMARK: For every overall supplier is delivering: 1 working belt made from ballistic canvas wide 4 cm, tested for load of 160 kg with appropriate case made from ballistic canvas suitable for handheld radio, 2 cases (in size suitable for pistol CZ 99 clip), 1 belt loop with metal ring and 1 belt loop with carabineer. 1 Thermal insole with stitched insole (100 % polyester 60 gr ±10 %), filled with micro polyester fibres melted 100 % (tinsulate) 100 gr/m2 |  |  |  |  |  |  |
| ## 2.2.7. | Rescue vests | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Vest must be produced in accordance with SRPS EN 393, EN ISO 12402-5/12402-6 |  |  |  |  |  |  |
|  | Buoyancy: min. 50 N |  |  |  |  |  |  |
|  | Vest is equipped with reflecting stripes on front and rear side as well as on the shoulder area |  |  |  |  |  |  |
|  | Vest must have at least two pockets on the front side (smaller pockets for multi tool knife, carabineer, whistle etc.) |  |  |  |  |  |  |
|  | Vest must be equipped with at least 7 adjusting points from which 2 on each side, 2 on shoulders and one with “fast releasing” buckle positioned on waist |  |  |  |  |  |  |
|  | Vest is manufactured of polyester fabric, Cordura 220D or equivalent |  |  |  |  |  |  |
|  | Vest color is Red or Yellow |  |  |  |  |  |  |
|  | Vest must be equipped with rescuers pealess whistle |  |  |  |  |  |  |
|  | OPTIONAL: pocket on the back side of the vest, suitable size for the bag with the floating rope |  |  |  |  |  |  |
| ## 2.2.8. | Fishing rubber suits | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Fishing rubber suit should allow to the rescuers to move through water |  |  |  |  |  |  |
|  | Height of the suit (for the foot size 42) minimum 1420 -/+ 5mm |  |  |  |  |  |  |
|  | Outer surface of the suit is made of natural or synthetic caoutchouc with adjustable belts and pocket on the front of the suit |  |  |  |  |  |  |
|  | Inner layer is made of cotton and with pressed sole |  |  |  |  |  |  |
| ## 2.2.9. | Raincoats | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Two-piece rain suit is protecting rescuers from rain and wind during emergencies |  |  |  |  |  |  |
|  | Rain suit consists of jacket and trousers |  |  |  |  |  |  |
|  | Jacket is equipped with hood, zipper and snap buttons, 2 large pockets with overlaps and possibility of cuff adjustment |  |  |  |  |  |  |
|  | Trousers with adjustable cuffs and waist size |  |  |  |  |  |  |
|  | Suit is made of 100% polyester with layer of polyurethane or PVC. Seams on jacket and trousers must be welded. |  |  |  |  |  |  |
|  | Suit is made in black or dark blue colour with reflective stripes |  |  |  |  |  |  |
|  | Suit must be manufactured in accordance with SRPS EN 343 |  |  |  |  |  |  |
| ## 2.2.10. | Sleeping bags | Quantity: 120 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |  |
|  | Product type, model |  |  |  |  |  |  |
|  | Technical characteristics |  |  |  |  |  |  |
|  | Transport bag (dry bag) is used by rescuers for transport of water rescue equipment |  |  |  |  |  |  |
|  | Bag is made of reinforced PVC or equivalent fabric, resistant to tear and wear and UV radiation |  |  |  |  |  |  |
|  | Bag volume: min. 80 l |  |  |  |  |  |  |
|  | Belts (straps) are made of stabilized PES or equivalent fabric and they are allowing carrying of the bag in hand and on the back |  |  |  |  |  |  |
|  | Transport bag is 100% waterproof |  |  |  |  |  |  |