# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of firefighting vehicles and equipment	p 1 /…

Publication reference : 283/City of Vrsac/TD8

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Fireman Vehicle VP3000/300: 1 pieces Description and purpose of vehicle: firefighting vehicle for fire extinguishing actions of flammable liquids and solid flammable substances. The vehicle should be formed from the chasiss and superstructure and must have all required firefighting equipment in it. The vehicle should be supplied with a fire pump with installations, water tanks (3000 lit.) and foam, (300 lit.), as well as firefighting equipment for firefighting. The supplier must have a service on the territory of the Republic of Serbia for all installed parts and equipment. Required Firefighting equipment must be constructed according to SRPS EN, EN, DIN or equivalent standards. Substructure (Vehicle): - Engine power: min 240 HP - Max engine torque: min 900Nm - Engine type: diesel, EURO V or better - Engine breaks power: min 130kW - Fuel tank: min 300 liters. - Batteries min 2 with min 170Ah maintenace free - AC device included - Tachograph included - Left side steering wheel with servo included - Installation for mounting of radio communication equipment - Vehicle approved/ homlogized to 14 tonnes - Wheelbase min 3800mm - EBS, ABS systems - Power windows on both sides - Alarm for reverse movement included - LED daylights - Fog lights - Cabin capacity: min 2+1 firemen Superstructure - Description: Made from aluminium or stainless steel. Space reserved for equipment, pump and installations. - Internal storages for equipment and tools with Aluminium rolling shutters for protection against external influences and external locking. - Upper/top side of superstructure must be covered with anti slip material and superstructure need to have suitable ladder for access to top side - Water reservoir type and capacity: made from stainless steel (min 3000 liters) for water with internal seawalls. - On top of water reservoir reservoir there should be access shaft min 500mm in diameter - Water reservoir needs to have overspill exhaust, electric water level measurement, Ø75mm (type B) connector for hydrant, and connector for refill and drain with pump - Water reservoir attached to substructure via elastic support to prevent torsional twisting. - Foam reservoir made from stainless steel, with capacity 300 liters. Needs to have connection for refill, vent for overpressure and vacuum, electric measure of level and connection for system for mixing of water and foam and draining foam. - Water pump description : centrifugal driven vehicle by driveshaft and overheat protection - Water pump pressure: min 10 bars at 1800liters/minute - Vehicle need to have pump for refill water from external source with min H=7m - Water pump material: made from metal resistant to corrosion - Two delivery outputs “Storz” 75mm on back of the vehicle - Pump should have option of manual discharge of water for freezing protection. - Control panel board - with manual switching on / off of drive and control functions (switches) Positioned in the rear of the superstructure - command of the speed control of the engine motor (pumps) - water and foam level gauges -pressure indicator -STOP switch -other if relevant - Foam mixer: manual dosage in all pump work modes 1-6% - Fast intervention fire fighting winch on side of vehicle must contain 30m incompressible hose of internal diameter min 25mm, with capacity at least 200l/min and pressure adequate for pump supplied, nozzle capable for capacity 0-200 liters per minute with possibility of facilitation firefighting with foam, possibility of full or scattered jet - Installation of the internal pipelines: Pipes must be made of steel and hot galvanized or stainless or similar. The entire installation must be resistant to corrosion. - Blinker lights on the upper part of the vehicle cabin, and rear of vehicle in line with vehicle purpose. - Appropriate sound signal siren min 100 W - External and internal superstructure lighting (storage boxes and equipement). Aux Equipment: - Fire hoses type “B”, length 15 m. x 2pcs. - Fire hoses type “C”, length 15 m. x 6pcs - Universal key ABC, for coupling shafts (in according with DIN 14822) x4 pcs - Suction hose with all necessary couplings 2sets - Hand-held portable fire extinguisher, S-9 x 1pcs - Hand-held portable fire extinguisher, CO2-5 x1pcs - Universal nozzle with adjustable flow and variation of level of spray up to 200l / min x 2pcs - Extension for foam x2pcs - Coupling B/C 3pcs - Coupling A/B 1pcs - Hydrant coupling B/2C 1pc - Distributor with valves B/CBC 1pc - T tool for hydrant connection mounting 2pcs Color and markings: in according with Serbian legislative for this type of vehicles. RAL 3000 Markings on vehicle should be in line with IPA Romania- Serbia crossborder programme VIM. http://www.romania-serbia.net/?page_id=212&lang=en_GB |  |  |  |
| 2 | Fireman Vehicle V300: 1 pieces Description and purpose of vehicle: Multi-purpose emergency vehicle with superstructure for fast response in fire fighting and other purposes suitable for quick deploying in remote areas and with off road capability Substructure (Vehicle): - Engine power: max 120KW - Max engine torque: max 360Nm - Engine displacement: max. 2000 ccm - Engine type: diesel, EURO VI or better, no AdBlue - Transmission: 4WD with transfer case - Ground clearance: min 220mm - Vehicle approach angle forward: min 29o - Vehicle approach angle backward: min 20o - Cabin type: 4 doors - Vehicle category: N1 - Carrying capacity: min 1000kg - Trailer towing force min. 3500 kg - AC device - Minimal safety equipment: 6x airbag, ABS, ESP, TCS, EBD - 16 inch steel wheels Superstructure - Description: Made from aluminium or stainless steel. Space reserved for equipment, pump and installations. Side/back openings can be closed by rolling panel/door with lock for security reasons and protection from dust and water. - Reservoir type and capacity: made from polyester, stainless steel or similar material - min 280 litres for water with internal seawalls and connection for hydrant refill - min 20 litres for foam tank with connection for refill and mixing with water equipment and discharge - Water pump description : high-pressure pump aggregate. Compact design includes a drive motor gasoline/diesel type with electric start and manual start, fuel tank, winch with a high pressure hose of min 60 m length. On the end of the hose jet nozzle should be mounted with manual adjustment from full flow to water mist and foam extender. - Water pump capacity: min 42 liters/minute at pressure 100 bars - Foam mixer: manually adjusted Electrical equipment - All lights required by traffic legislation in Serbia - Lights inside cabin and superstructure - Fire fighting vehicle blinker lights mounted and 100 W emergency vehicle siren and microphone with loudspeaker. - Blinkers on front and rear of the vehicle - Installation for mounting of radio communication equipment Aux Equipment: - at least two pull out boxes for equipement - All necessary tools/parts for vehicle operation. - Equipment obligatory by Serbian traffic legislation. Color and markings: in according with Serbian legislative for this type of vehicles. RAL 3000 Markings on vehicle should be in line with IPA Romania- Serbia crossborder programme VIM. http://www.romania-serbia.net/?page_id=212&lang=en_GB |  |  |  |
| 3 | Bodycamera: 2pcs Mount: Clip mount rotatable, capable for attaching to uniform or similar Fuctionalities: recording of audio and video Max resolution of video: min full HD at 30fps Codec: H264, MP4 or other modern hi compression codec Night vision: Yes, with automatic engagement in dark environments IP protection rating: min 65 Wi-Fi access: supported to authorized users GPS support: Yes Storage: capable to record min 8 hours of video at 720p Battery type and autonomy: rechargeable with device autonomy of more than 300 minutes Camera weight: max 130 grams Aux equipment to be included: transport hard case, charger, management software and hardware if necessary |  |  |  |
| 4 | Handheld fire fighting thermal image camera: 1 pieces Type of grip: pistol type grip with neck strap included IR resolution: 240x180 or better Display size diagonal: min 100mm Max temperature of object to be recorded: min 500oC Operating temperature range: -20 oC to 80 oC or wider IP protection rating: min 67 Capable to withstand drop on concrete floor from attitude of at least: 2 meters Capable to withstand temperature of 250 oC for at least: 5min Battery type and autonomy: rechargeable with device autonomy of more than 200 minutes Camera weight: max 1150 grams Aux equipment to be included: transport hard case, charger, vehicle charger |  |  |  |