# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Acquisition of meteorological stations

Publication reference: RORS 380 / Politehnica University Timisoara  / 209

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| - 1. Meteorological stations, 10 pieces |  |  |  |  |  |
| - Motherboard |  |  |  |  |  |
|  | Digital inputs | 5 digital inputs, automatic sensor recognition, supporting sensor chains (max. 600 sensors) |  |  |  |
|  | Extension connector | Radio access point or Sentek Drill & Drop or ultrasonic wind sensor or two extra chain connectors, cable nodes |  |  |  |
|  | Memory | Min 8 MB flash memory |  |  |  |
|  | Internet connectivity | GSM - GPRS, EDGE, HDSPA, CDMA, UMTS, Satellite |  |  |  |
|  | Alerts | SMS, user configurable via website |  |  |  |
|  | Dimensions without sensors | 41 cm L x 13 cm W x 7 cm H |  |  |  |
|  | Weight without sensors | rougly 2.2 kg |  |  |  |
|  | Measuring interval | 5 minutes (by default) |  |  |  |
|  | Logging interval | 10-120 min (user selectable) |  |  |  |
|  | Transmission frequency | User selectable |  |  |  |
|  | Power supply | Battery 6V, 4.5AH, Operating range: -35 °C to 80 °C |  |  |  |
|  | Solar panel dimensions: | 13.5 x 13.5 cm, min.2 Watt solar panel |  |  |  |
|  | Sensors required | Air temperature and relative humidity sensor |  |  |  |
|  |  | Wind speed sensor |  |  |  |
|  |  | Sensor for measuring soil temperature and humidity |  |  |  |
|  |  | Global solar radiation sensor |  |  |  |
|  |  | Leaf wetness sensor |  |  |  |
|  |  | Rain gauge sensor |  |  |  |
| - Air temperature and relative humidity sensor |  |  |  |  |  |
|  | Temperature sensor | PT1000 1/3 Class B or equivalent |  |  |  |
|  | Humidity sensor | ROTRONIC Hygromer® IN-1 or equivalent |  |  |  |
|  | Accuracy with standard adjustment profile | at 23°C and 10, 35, 80 % rh ± 0.8%rh / ± 0.1 °C |  |  |  |
|  | Resolution, AirChip 3000 | Typically 0.02 %rh, 0.01 °C |  |  |  |
|  | Long-term stability | < 1 %rh, 0.1°C / year |  |  |  |
|  | Humidity response time t 63 | 3 seconds |  |  |  |
|  | Measurement range | 0…100 %rh, -100…200 °C |  |  |  |
|  | Electronics operating range | -50-100 °C and 0-100 %rh |  |  |  |
|  | Output signals | Serial port RS485 |  |  |  |
|  | Audit trail & electronic records | FDA 21CFR Part 11 and GAMP compliant |  |  |  |
|  | Power supply & consumption | 3.2 V / 4 mA |  |  |  |
|  | Housing/probe material | Polycarbonate |  |  |  |
|  | Filter | Polyethylene insert, polycarbonate cage |  |  |  |
|  | Standards | CE-compliant 2007/108/EG |  |  |  |
| - Wind speed sensor |  |  |  |  |  |
|  | Range | 0 to 50 m/s, gust survival 60 m/s |  |  |  |
|  | Sensor | 12 cm diameter cup wheel assembly, 40 mm diameter, hemispherical cups |  |  |  |
|  | Turning factor | 75 cm |  |  |  |
|  | Distance constant (63 % recovery) | 2.3 m |  |  |  |
|  | Threshold | 1.1 m/s |  |  |  |
|  | Transducer | Stationary Coil |  |  |  |
|  | Transducer output | AC sine wave signal induced by rotating magnet on cup wheel shaft. 100 mVpp at 60 rpm. 6 Vpp at 3600 rpm |  |  |  |
|  | Output frequency | 1 cycle per cup wheel revolution. 0.75 m/s per Hz |  |  |  |
| - Sensor for measuring soil temperature and humidity |  |  |  |  |  |
|  | Temperature accuracy | ±2 °C at 25 °C |  |  |  |
|  | Operating Temperature | -20°C – +60°C |  |  |  |
|  | Method | Capacitance based technology |  |  |  |
|  | Outer Probe Diameter (Top-Bottom) | 27-29.5 mm |  |  |  |
|  | Probe length | 60 cm |  |  |  |
|  | Moisture precision | ±0.03 % vol. |  |  |  |
|  | Resolution | Moisture 1:10000, Temperature ±0.3 °C |  |  |  |
| - Global solar radiation sensor |  |  |  |  |  |
|  | Sensor | Photodiode |  |  |  |
|  | Calibration | Calibration against Kipp and Zone CMP3 under daylight. Absolute error max. 5%, typically 3% |  |  |  |
|  | Stability | 2% drift on 2 years use |  |  |  |
|  | Time to measure | 10 µs |  |  |  |
|  | Temperature dependency | 0.15% per °C |  |  |  |
|  | Cosines correction | Sensor corrects up to 80° degrees |  |  |  |
|  | Azimuth | 1% error over 360 degree at 45 degree elevation |  |  |  |
|  | Working temperature | -20°C to 65°C |  |  |  |
|  | Relative humidity | 0 to 100% |  |  |  |
|  | Housing | Weatherproof PAS case with acrylic diffuser, stainless steel hardware |  |  |  |
|  | Size | 35 mm diameter, 45 mm height |  |  |  |
|  | Weight | Max. 115 g |  |  |  |
|  | Evaluation | Pulse Wide Modulation 0-80% = 0-2000 W/m² |  |  |  |
|  | Spectral range | 300-1100 nm |  |  |  |
| - Leaf wetness sensor |  |  |  |  |  |
|  | Supply voltage | 4.75 V – 5.25 V |  |  |  |
|  | Supply current | max. 1500 μA |  |  |  |
|  | Short circuit protection | Infinite (within supply voltage range) |  |  |  |
|  | Dry / Wet threshold | 220 – 390 kOhm |  |  |  |
|  | Output | Dry: max. 0.4 VDC, Wet: min. VCC-0.4VDC |  |  |  |
|  | Electronic | Totally plastic encapsulated – SMD |  |  |  |
|  | Evaluation | Digital – Filter paper |  |  |  |
|  | Dimensions | 127 mm x 254 mm x 508 mm |  |  |  |
|  | Cable length | 5 m |  |  |  |
|  | - Rain gauge sensor |  |  |  |  |
|  | Sensor Type | Double tipping bucket rain gauge |  |  |  |
|  | Output | Switch signal |  |  |  |
|  | Switch | Reed contact, solid state |  |  |  |
|  | Sensitivity | 1 tip per 0.2 mm or 1 tip per 0.5 mm |  |  |  |
|  | Collector Surface | 200 cm² |  |  |  |
|  | Evaluation | Digital |  |  |  |
|  | Maximum Rain | 12 mm/minute |  |  |  |
|  | Dimensions | 185 mm diameter x 250 mm H |  |  |  |
| - Equipment cage |  |  |  |  |  |
|  | Dimensions | 1000x800x2800 mm |  |  |  |
|  | Structure | Metallic frame (profile 40x40x3 mm) surrounded by steel welded wire mash net (min. Ø 6 mm). One vertical sided door with hinges and safety lock. Internal central pillar for station equipment. |  |  |  |
| - Installing and commissioning |  |  |  |  |  |
|  | Meteorological stations will be put into operation at 10 locations indicated by contracting authority according to the project map (Timis conty). |  |  |  |  |

Unless otherwise specified, the requirements in this Technical Specifications are presented as minimum standard, which the offered goods must meet. The offers that will not satisfy completely and correctly the minimum requirements mentioned above, will be disqualified.
	When brands names/specifications referring to or describing products of a given brand or origin are used in this technical specifications, they are used in descriptive purposes only and should be interpreted as equivalent.