Works on arrangement of left bank of the Navigable Begej canal from km 26+500 to km 26+720 in Zrenjanin and installation of floating dock

ARRANGEMENT OF THE LEFT BANK OF THE NAVIGABLE BEGEJ CANAL FROM 26+500 TO KM 26+720

1. INTRODUCTION

The capacity of DTD hydrosystem is recognised for its potential to promote navigation and cooperation between Serbia and Romania, for the needs of the building of the port for recreational boats, aimed at the development of tourism in the field of navigation, regional connecting and enabling cooperation between Serbia and Romania.

A special need was expressed by the local self-government as regards the development of the city and the fact that there is no regulated port for recreational boats in Zrenjanin despite the long-lasting needs. It is, also, reasonable to expect that the quality of the canal water will be significantly improved (since the upstream polluters are, so to speak, neutralised with the construction of a purification plant), thanks to the development of environmental impacts and the growing impact on the awareness of citizens (along with the development and harmonisation of national legislation with EU regulations).

With the thus formed overall perception of requirements and needs for the construction of the port for recreational needs and regional cooperation, there is a realistic need for the construction of a port in the City of Zrenjanin.

2. DESIGN BASES

Conducted for the needs of design was the geodetic surveying of cross sections of the canal bed and banks on every 25 m, on the Navigable Begej canal, at chainage from km 26+500 to km 26+720.

The geodetic survey is submitted within the graphical part of the project.

Designer had various contacts with pontoon producers within the procedure of selection of the construction type and the kind of material of the floating facility – dock. Following several different analyses, in consultation with the Investor, the designer opted for the concrete pontoons as a traditionally recognised ecological, material with sufficient strength and compactness, with time-tested resistance to UV and other impacts.

3. LOCATION OF FLOATING FACILITY - DOCK

The design brief defines that the installation of the floating facility be carried out on the location of the Begej Canal, on km 26+612km, at the site of boat turning platform. The zone belonging to the turning platform occupies the area from 26+500 to 26+720 km of the Begej canal.

During the project development stage, following a detailed geodetic survey, field visit and meetings with the representatives of the Investor and the Municipality of Zrenjanin, it was agreed that the location of the floating facility be at the starting point of the turning platform, on km 26+525 because of the closeness to existing roads and pedestrian bridge over Begej canal.

4. GEOMECHANICAL PARAMETERS

Field exploration works were conducted for design needs, based on which the Geomechanical elaborate was made by A.D. Vojvodinaprojekt, Novi Sad, (ref.no. E-G/59-18), December 2018.

Submitted with the Geomechanical elaborate as documentation material was the Report on examination of sediments, made at the Laboratory of the Chemistry, Biochemistry and Environmental Protection Department of the Novi Sad Faculty of Sciences in August 2018.

The conclusion of the Geomechanical elaborate was that the surface part of the exploration area builds the embankment representing the humus layer mixed with construction and communal debris under which there is a sandy-clay dust of yellow colour to the exploration depth. No groundwater was found to the exploration depth during exploratory works (early October 2018). Due to heterogenous composition (presence of communal waste and humus in the filled material) and low compactness, it is necessary to remove the filled material before conducting any works on the rehabilitation of the existent slope.

Based on the examination of the surface and in-depth samples of sediments from Begej Canal it was concluded, regarding the concentration of pollutants, that they were within classes 0, 1 or 2, in accordance with the “Regulation on limit values of pollutants in surface and ground Waters and deadlines for their reaching” (Official Gazette of the Republic of Serbia no. 50/2012). Such sediments are considered to be insignificantly polluted and their disposal without special protection measures is allowed within an area of up to 20 m width in the vicinity of the watercourse. Due to a greater organic substance content, established by ignition (mainly >7%) this material cannot be used for embankment building.

5. TECHNICAL SOLUTION

5.1. PONTOON MODULES AND ACCOMPANYING EQUIPMENT

The technical solution for the installation of pontoons belongs to the group of standard solutions which include the implementation of finished pontoon modules, whereby the final, desired pontoon shape is formed by their connecting.

Used as the accompanying equipment in addition to the basic pontoon module are the access bridge to the pontoon, pontoon anchors, the equipment holding the pontoon at a distance from the coast and, in particular – purpose-built bitts for securing the pontoon on water.

The pontoon modules and accompanying equipment of pontoons are further elaborated in the Project of the floating facility, also made by A.D. Vojvodinaprojekt Novi Sad, December 2018 (Project number E-H/1231, 77/18-3.2.).

5.2. WORKS REQUIRED FOR PONTOON INSTALLATION

The works preceding the pontoon installation include the geodetic works, preparatory works and bank arrangement works and, as such belong to the group of regular hydro-construction works.

The geodetic works include the preparation, i.e. the making and inspection of the “0” survey in the canal bed and on the dry parts of the bank, their marking and staking out, monitoring, i.e. maintenance during construction, as well as the final survey of the as built state.

Based on the geodetic survey, preparatory works are carried out, i.e. works on removal of cane and mud.

The works include:

1. Removal of can and mud on one half of size of a part of turning platform where pontoon is planned to be placed (from km 26+478.54 to km 26+625), to the elevation of 72.30 m.a.s.l.,

2. Additional excavation to the elevation of 71.50 m.a.s.l. (final bottom elevation, defined in design brief) only in the zone envisaged for the building of the bank revetment (from km 26+500 to km 26+575)

3. Removal of cane and mud on the second half of the turning platform surface (from km 26+625 to km 26+775) to the elevation of 72.70 m.a.s.l.

As regards the cleaned part of the turning platform, where the pontoon is planned to be placed, it is required to conduct the bank arrangement, that is form full profile bank slope – for the final profile form that consists of:

stone feet of - “carpet” made of 6-15 cm crushed stone, with at least 30cm layer thickness.

Prism of broken stone, size 15-45 cm, with free declining slopes 1:1.5, thickness 1.2 m (to elevation 73.00 m.a.s.l.) shall be built on the crushed stone foot.

“Rolled” stone (mechanically installed stone), size 15-45 cm is built in from the elevation of the stone prism in underwater part, to the elevation of 74.75 m.a.s.l. (berm elevation).

Above this elevation, up to the slope top, i.e. to terrain level, stones of 15-45 cm size are laid manually and poured with fine-grained concrete.

The manually laid stones are placed on a base of compacted 16-64 mm gravel, under which geotextile is laid (300 gr/m2), as a measure of anti-filtration protection.

All bank revetment edges are reinforced with reinforced concrete beams and the upper final edge of the lining is reinforced also with broken stone beam. The thus made lining represents the final lining.

Reinforced 2.00 m wide concrete stairway is envisaged to be constructed as a descent to the access bridge.

It is important to stress that, while conducting works on the bottom, a natural slope 1:10 of underwater material should be formed, the role of which should be to bridge the height difference of 40 cm between surfaces with a deeper bottom (on elevation 72.30) and shallower bottom (on elevation 72.70).

6. STABILITY OF THE BANK REVETMENT SLOPE

The calculation of the bank revetment slope stability was made, aimed at enabling bank rehabilitation. GEO-SLOPE program was used for that purpose, as one of programs of the GeoStudio package. GEO-SLOPE is a widely accepted program in the world for this type of calculation for the reason that it proved to be very precise, simple to use and very quick in solving the equations on which it is based. The program enables the forming of the plane model and functions by finding several potential sliding planes to establish the one for which the lowest security coefficient is obtained. The software is very flexible because it enables the input of elements of various geometries, different material types, change of water level and different load types making an impact on the soil.

In accordance with the results of the calculation check (and bearing in mind that the bank revetment lining in the direction of the gradient of the slope additionally relies on the stone foot of the bank revetment) the bank revetment lining is stable when it comes to sliding at slope level.

The bank revetment slope stability calculation is an integral part of this project and is submitted under no. 3.6.1.

7. TECHNOLOGY OF EXECUTION OF DEMUDDING WORKS

The technology of works represents a combination of operation of land and waterway machinery. The demudding works on the bank revetment location would be conducted with amphibian excavators and floating facilities for transport to depositing location, where all works are conducted with land machinery.

7.1. DESCRIPTION OF WORKS

The demudding works can be divided into preparatory works at demudding location, preparatory works at depositing location and works on the very demudding of the location envisaged for port.

Conducted first within the preparatory works at the demudding location should be the cleaning of banks from bushy vegetation and stumps with adequate machinery and manually (if necessary) on both sides of the canal. After the removal of the vegetation, it is required to build a big descend ramp that shall serve for the movement of the required vegetation during works between the level of the surrounding area and the level of the canal bed and, after completion of all works, the ramp shall be used by citizens for letting their vessels in the canal. The exact location of the descend ramp shall be determined by the representatives of the investor and professional supervision with the contractor, immediately before the start of works.

An access road should be first formed on depositing location for unhindered movement of all required machinery, after which, two smaller ramps and two bigger ramps should be built. Smaller ramps shall serve to connect the access road and the landfill for disposal of all earth materials, while the bigger ramps shall serve for connecting the reloading plateau with the access road. After the ramps are built, the disposal location and the location of the reloading plateau should be cleaned from bushy vegetation, trees and stumps, after which they should be levelled and planned. It is required to conduct the demudding of the bank area on the location of reloading plateau to enable easier access of the floating facility to the plateau, while the demudded material should be transported and disposed on depositing location.

The demudding works shall be conducted by excavation to the designed elevations with hydraulic excavators on amphibian tracks with direct loading into the load space of a suitable vessel that transports all demudded materials to a maximum distance of 2 km from the reloading plateau. At the reloading plateau the demudded material is reloaded from the vessel to dump trucks with hydraulic excavators with 15 meter reach that transport the load via descend ramps and access road to disposal site, where earth material is unloaded. After the earth materials is drained and dried it is required to disperse it within the landfill, thus completing all demudding works and the preparation of location for the construction of bank revetments and port.

8. BANK BERTHING EQUIPMENT FOR FLOATING FACILITY

The equipment of the operating bank is determined according to the selected location of the floating facility, in accordance with the requirements for stable anchoring at the port.

Required berth elements

As regards the location of the floating facility, elements of additional equipment on the bank are defined, which are required for floating facility anchoring. In accordance with that, it is required to place, i.e. make two bitts for berthing floating facility, two recesses with support for bumper connection, as well as the bank support for the access bridge of the floating facility.

The bitt is made of a steel pipe DN 323.9 mm, with 5.6 mm wall thickness, filled with MB 20 concrete, with MB 20 concrete foundation. The bitt calculation is made and submitted in this project under no. 3.6.2. Taken for bitt dimensioning was the force of 90 kN, increased by security coefficient of 1.5.

As regards the placing of the floating facility, it is accepted that all elements of the operating bank part should be identical as the usual ones.

The support for bumper connection is designed in the form of a concrete block with dimensions BxL/H=150x100/100 cm, made of MB 20 concrete. Envisaged for the lateral side of the block, facing the bank, is the placing of an anchor plate for the connection of the bank end of the abutment. The connection is achieved through horizontal, cylindric joint that is an integral part of the floating facility (pontoon) equipment.

The bank support of the access bridge of the floating facility is designed in the form of a concrete block, dimensions BxL/H=100x150/100 cm, made of MB 20 concrete. This support is placed at the level of the lower path of the bank revetment, on the elevation of 74.75 m.a.s.l. The connection is achieved through steel support that is an integral part of the floating facility (pontoon) equipment.

Manner of marking of the floating facility - dock

The manner of marking of the floating facility should be in line with the conditions, i.e. in accordance with the Law on Navigation and Inland Ports.

9. PRICED BILL OF QUANTITIES

The priced bill of quantities for bank rehabilitation works was made for two stages of works, with an overall summary. The priced bill of quantities includes: geodetic works, preparatory works (cane cutting and mud excavation) and bank arrangement works, namely building of bank revetment.

10. CONCLUSION

By arrangement the bank and placing the floating facility - dock for the needs of recreational vessels, primarily the beginning of use of tourist potentials of DTD hydrosystem is achieved, as well as a long-term wish and intention of the citizens of Zrenjanin to turn to canal and continue the long-term regional cooperation with Romania.

The classical technical solution, within the available funding, enables the preparation of space for placing the floating facility – dock, as well as its installation and further usage.

The technology of works and the technical solutions applied are the usual ones and mainly belong to regular maintenance works.

11. PHOTOS OF THE LOCATION PLANNED FOR PLACING THE FLOATING DOCK

Navigable Begej canal – view from the pedestrian bridge - upstream

Old ship turning platform on the left bank of the Navigable Begej canal – view from the pedestrian bridge

Left bank of the Navigable Begej canal at the location of old turning platform

Left bank of the Navigable Begej canal at the location of old turning platform

PLACING OF FLOATING BERTH FACILITY

1. INTRODUCTION

DTD Hydrosystem with its potentials represents a significant opportunity for development of nautical tourism of the Autonomous Province of Vojvodina. Given the domestic and international demand in the field of nautical tourism, it is required, as a precondition, to provide the space and organisation preconditions regarding the basic infrastructure.

The development of nautical tourism requires the provision of nautical facilities. In this sense, the arrangement of the left bank of the Begej in Zrenjanin is planned, from km 26+500 to km 26+720, as well as the placing of the floating facility – dock for berthing recreational vessels. This location is situated in the Begej sector, outside the waterway, under controlled water regime, in the urban part of Zrenjanin, on the location of the old ship turning platform, which is derelict and covered by vegetation, mainly cane.

Before the floating dock is placed, it is required to clear the whole canal bed and bank. Envisaged in that regard is the removal of cane and deposits from canal bed bottom, after which, the arrangement of the bank is supposed to take place, with the forming of the bank revetment and placing of bitts for berthing floating facility. The mentioned works are elaborated in the Bank Revetment Project, that was also prepared by A.D. Vojvodinaprojekt Novi Sad, December 2018 (Project number E-H/1230, 77/18-3.1.).

2. LOCATION OF FLOATING FACILITY - DOCK

The design brief defines that the installation of the floating facility be carried out on the location of the Begej Canal, on km 26+612km, at the site of boat turning platform. The zone belonging to the turning platform occupies the area from 26+500 to 26+720 km of the Begej canal.

During the project development stage, following a detailed geodetic survey, field visit and meetings with the representatives of the Investor and the Municipality of Zrenjanin, it was agreed that the location of the floating facility be at the starting point of the turning platform, on km 26+525 because of the closeness to existing roads and pedestrian bridge over Begej canal.

3. FLOATING FACILITY DESCRIPTION

According to the requirements of Design Brief, the floating facility – dock has to be a floating, ecological facility of sufficient strength and compactness. The dimensions should be such to enable safe release of at least two vessels with up to 15 m length, meaning the dock length should be around 30 m.

For the stated reasons, the Designer opted for placing standard, finished HD floating concrete pontoons, of 2.40 m width, 0.95 m height (0.50 m above water; 0.45 m draft). The dock length of 30 m is achieved by connecting 3 pontoon elements, each 10.00 m long, which is the most optimal variant from the aspect of pontoon transport from the place of production to the place of installation. The pontoons are connected with special cords which are pulled longitudinally through both sides of all pontoons in line, from the beginning to the end and, after tightening on ends, locked with special “wrenches”. Rubber inserts, so called buffer’s are located on pontoon connection points.

HD (Heavy Duty) pontoons ae made of reinforced concrete, with special additives, reinforced by plastic fibres. The pontoon interior is filled with expanded polystyrene with 15-17 kg/m3 density. Steel reinforcement is zinced by hot-dip galvanization. The pontoon walking surface is non-slippery. Rubber bumpers are placed along pontoon edges.

HD pontoons are robust and can withstand the surface load up to 500 kg/m2. They are very stable, thanks to their large weight. They are fully resistant to water and do not require any anti-corrosion protection or overhaul. They are also able to withstand huge temperature differences (from -50ºC to +50ºC) and ice of thickness exceeding 700 mm. The pontoons are mounted quickly and easily, with the option of subsequent adding of new elements or moving to another location. The environment protection standards have been observed in full, while maintenance expenses are insignificant. They are highly aesthetic and, in the case of any damages occurring during use, repairs on spot are quick and simple. HD pontoon lifetime is over 50 years. In technical and economic sense the concrete pontoons belong to the group of materials most suitable for making floating pontoons.

4. FLOATING FACILITY EQUIPMENT

The procurement of the floating facility made of typical HD floating pontoons also includes the procurement of the following accompanying equipment:

- stainless steel fence, minimum height 0.90 m, around the whole floating facility (with chains at passage points)

- access bridge (BxL=1.00x8.00 m),

- 2 spacers (length up to 8.00 m),

- 2 steel berth ropes Ø18, each 30 m long, for berthing to bank

- stainless steel bitts (24 items),

- rubber bumpers (24 items),

- lifebelts or lifebuoys,

- light signalisation on the most prominent part of the floating facility.

NOTE: The floating facility must possess a valid certificate of the manufacturer (Type Approval Certificate), issued or recognised by the Authority for Determination of the Seaworthiness of the Republic of Serbia.

The equipment of the operating bank is determined according to the selected location of the floating facility, in accordance with the requirements for stable anchoring at the port.

The required berthing facilities for the location of the floating facility are defined by the elements od additional equipment on the bank, required for floating facility anchoring. In accordance with that, it is required to place, i.e. make two bitts for berthing floating facility, two recesses with support for bumper connection, as well as the bank support for the access bridge of the floating facility.

The elements of the equipment for berthing on the bank were elaborated in the Bank Revetment Project, that was also prepared by A.D. Vojvodinaprojekt Novi Sad, December 2018 (Project number: E-H/1230, 77/18-3.1.) and are herein provided for informational purpose:

The bitt is made of a steel pipe DN 323.9 mm, with 5.6 mm wall thickness, filled with MB 20 concrete, with MB 20 concrete foundation. The bitt calculation is made and submitted in this project under no. 3.6.2. Taken for bitt dimensioning was the force of 90 kN, increased by security coefficient of 1.5. As regards the placing of the floating facility, it is accepted that all elements of the operating bank part should be identical as the usual ones.

The support for bumper connection is designed in the form of a concrete block with dimensions BxL/H=150x100/100 cm, made of MB 20 concrete. Envisaged for the lateral side of the block, facing the bank, is the placing of an anchor plate for the connection of the bank end of the abutment. The connection is achieved through horizontal, cylindric joint that is an integral part of the floating facility (pontoon) equipment.

The bank support of the access bridge of the floating facility is designed in the form of a concrete block, with dimensions BxL/H=100x150/100 cm, made of MB 20 concrete. This support is placed at the level of the lower path of the bank revetment, on the elevation of 74.75 m.a.s.l. The connection is achieved through steel support that is an integral part of the floating facility (pontoon) equipment.

5. MANNER OF MARKING OF THE FLOATING FACILITY - DOCK

The manner of marking of the floating facility should be in line with the conditions, i.e. in accordance with the Law on Navigation and Inland Ports. The marking of the floating facility is not a subject of this project.

6. PRICED BILL OF QUANTITIES

All works referred to in the priced bill of quantities for the installation of the floating facility are covered by one position of works which includes procurement, transport and installation of the floating facility - dock, which consists of 3 items of HD concrete pontoons connected into one entity with all stated accompanying equipment.

7. CONCLUSION

By arrangement the bank and placing the floating facility - dock for the needs of recreational vessels, the conditions for exploitation of tourist potentials of DTD hydrosystem will be fulfilled, as well as a long-term wish and intention of the citizens of Zrenjanin to turn to canal and continue the long-term regional cooperation with Romania.

The classical technical solution, within the available funding, enables the preparation of space for placing the floating facility – dock, as well as its installation and further usage.

8. PHOTO OF THE LOCATION AT WHICH THE FLOATING DOCK IS SUPPOSED TO BE PLACED

The location for placing the floating facility on the left bank of the Navigable Begej canal – view from the pedestrian bridge