# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Thermal imaging camera with software and drone carrier	p 1 /…

Publication reference : RORS300/CityofZrenjanin/TD5

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Thermal imaging UAV system - 1set consisting of: Unmanned Aerial Vehicle - 1pcs - Type of aerial vehicle: multirotor - Type of propulsion system: electric or gasoline or alcohol or similar - Payload: min 1.3kg - Max Service Ceiling Above Sea Level: min 2000m - Max speed without wind: min 20 m/s - Max wind speed allowed for operation: min 10m/s - Flight autonomy: min 20 minutes with max.allowed takeoff weight - IP protection: min IP43 - Capable to use GPS and GLONASS systems - Horizontal positioning accuracy, near ground, using GNSS or other system: max 0.3m error - Vertical positioning accuracy, near ground, using GNSS or other system: max 0.3m error - Capable of return to home and safe landing on autopilot. - Capable of detecting and preventing obstacles collision at least in forward and downward direction. - Capable to simultaneously stabilize and utilize both supplied cameras and to control additional instruments - Remote Controller included with LCD/LED, 7” QXGA resolution daylight viewable screen or better, with min. 5km range in unobstructed interference-free conditions. - Operating temperature range: -20oC to 45oC or better Camera Optical - 1pc - Type: stabilized optical camera providing both realtime/live HD video and photogrammetric still imaging of structures - Detachable mount with gimbal stabilization in all 3 axis (roll, tilt, pan) and remotely controlled in tilt and pan axis - Camera resolution: min 20Mpix capable to record pictures and video - Lens with variable aperture, Field of View: min 80o - Equipped with Mechanical Shutter capable for 1/2000s speed or faster - Photo resolution: min 5400x3600px in PC compatible RAW format - Video resolution: min 4096x2160/50fps compressed in PC compatible codec - ISO range: at least 100-6400 - Capable for live view in HD resolution on supplied UAV Remote Controller screen or when using mobile phones with IOS and Android operative system - Storage system: SD Card or similar included - Weight of camera with lens and gimbal stabilization: less than 300grams Camera Thermal - 1pc - Type: stabilized infrared and optical camera providing simultaneously both thermal and visible light images/videos - Detachable mount with gimbal stabilization in all 3 axis (roll, tilt, pan) and remotely controlled in tilt and pan axis - Thermal camera Field of View: 30deg. horizontally or more - Thermal camera sensor type: uncooled sensor (no need for cryogenic gasses) - Thermal camera resolution: min 640×512, capable to record radiometric pictures and radiometric video at less than 9fps, capable for at least two gain settings - Thermal camera temperature recording range (high gain): -25 to 120oC or better - Thermal camera temperature recording range (low gain): -40 to 500oC or better - Max Sensitivity thermal (NEdT): less than 60mK - Visual camera: 12Mpix or better, capable for recording of 4k video and pictures, - Capable for live viewon supplied UAV Remote Controller screen or when using mobile phones with IOS and Android operative system - Storage system: SD Card or similar included - Weight of camera with lenses and gimbal stabilization: less than 600grams Software for photogrammetric modelling - 1 PC licence - Stand-alone PC software product for photogrammetric processing of digital images, to generate textured 3D models - Capable to generate, edit, classify and export dense point clouds, and to generate and export georeferenced orthomosaic maps and DEM and textured 3D models - Capable to perform custom calculations and scripts in high-level programming language - Capable for local network distributed processing - Capable to perform processing based on recordings from drone and terrestrial cameras Important notice: The UAV system set should contain software and equipment required for regular operation and user maintenance, including but not limited to battery chargers, cables, specialized tools and user serviceable parts. Important notice: The UAV system specific operation and user maintenance training, for 2 operators, will be organized by Contractor before Provisional acceptance is signed by Contracting Authority. Operators will receive training to safely utilize contracted UAV system for recording of buildings in thermal efficiency inspection process. Important notice: If the UAV is subject to certification or registration by Civil Aviation Directorate of the Republic of Serbia, Contractor will provide Contracting Authority support in the process. |  |  |  |