# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Emergency situation equipment procurement	p 1 /19

Publication reference : RORS/232/P3

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | LOT 1 Fire fighting truck and fire fighting equipment |  |  |  |
|  | Fire fighting vehicle 1 piece Ø Wheelbase: 3750mm Ø Engine: diesel, min EURO6 Ø Engine power: minimum 132kW/179HP Ø Gearbox: minimum Manual 6+1DRIVE: Ø Double axle chassis with 4x2 drive Ø Double wheels on the rear axle Standard vehiclecooling BRAKES: Ø Anti-lock brake system (ABS), Ø parking brake on rear wheels CABIN: Ø Double cabin Ø Doors: 4 Ø Number of seats min 1+4 Ø Tachometer Ø Taster ( power switch) for switching on and of the pump Ø The floor of teh cabin made of non-skid material ELECTRICAL SYSTEM: Ø Electrosystem min 12V Ø 2 acumulators according EN 1846 Ø Alternator FUEL TANK: Ø Tank cap with chain and locking WINCH FOR SELF-EXTRACTION Ø The winch for self-extraction has been built-in in the back side of the vehicle Ø The capacity of extractionmin 300l Ø The cord lenght min 30m Ø Remote controlled TECHNICAL FIREFIGHTING SUPERSTRUCTURE CHARACTERISTICS: SUPERSTRUCTURE: GENERAL : •System must have ability to throw micronized mixture of water and foam rapidly. System has to be powered by gasoline engine and built up steel chassis withprotection covers.System can use water from from tank or any external water source during operation. •The superstructure and the depository are made of stainless Al-profiles, interconnected by welding and/or screw connections, and the outer laying of the depository is made with aluminum flat sheets. The interior space is coated with aluminum anodised ribbed sheet metal. •The roof of the superstructure is coated with aluminium anodised ribbed sheet metal- , non-skid, and with low sideways protection, for safe moving on roof. •Sideways as well as the back box are closed with aluminium roller door which prevent the water and dust breakthrough in the depositories in accordance with EN 1846-2. •The aluminium roller door are closing with outside locking and are being locked with the same key ( one key locks all roller doors). •Two (2) doors are installed on the each side of the superstructure , paying attention on functionality, symmetry and aesthetics. One (1) door installed at the back side of the superstructure •Carriers and bulkheads for predicted equipment are set In dispositories. •Folding ladder for climbing on roof according EN 1826-2 are installed on the back side of the vehicle , •The light signaling consists of integral device on the vehicle cabin, with integrated LED blue rotating lights, and two blue LED flashing lights on the back part of the vehicle. •The sound signalization consists of three-way electronic siren and amplifiers of strenght 100 W. The device possesses also an announcement system with incorporated mycrophone. The managing is being performed through an comand desk in vehicles cabin. • The boxes for equipment storage and the space for pump are illuminated, and on the outside sideways edges of the roof superstructure, on the left and right side, the reflectors for space illumination around the vehicle are installed in accordance with EN 1846-2. WATER TANK: • The water tank relies on the metal carying frame, over the elastic supporters which prevent the torsion curls, and it is built of quality stainless steel (inox). •WATER tank capacity is V=300 lt built up from stainless steel material with enoughbuffles inside the tank which is equal to standard system of 3000 l. • Filling cover on the top to be used for filling from outer sources. • A level indicator near the tank • A drainage tap underneath the tank. • FOAM tank (V=30 lt), made of stainless steel with filling covers on the tanks and drainage tap underneath the tankwhich is equal to standard system of 300 l. • Level indicators available on the foam tank HOSE REELS: •min 2 hose reels. • Hose reels made fromstainless steel material. • min 40 mt x Ѕ” high pressure hose • Easy couplings for use in the system. • The hose must be easily mounted on the system, easy access and operation FIREGUN • min 2 fireguns • The fireguns consists of two lances. • These lances can be used with various nozzles depending of the type fire. • Trigger mechanism to start and to stop water flow . • Double function directional valve for water/foam throw. Possibility for change choice from water to foam throw by means of double directional valveimmediately. Easy couplings for firegun with hoses and water outlets • Throw distance: 12-15 meters • Usage of all functions from firegun without going back to system: - Far distance foam throw throughbelow foam nozzle - Close distance foam throw with wide angle through below foamnozzle - Micronized water fogging to far distance with upper nozzle. - water throw to close distance with wide angle through below nozzle - water throw to far distance through below nozzle - low-pressurized water&foam mixture for petrolium fire through blownozzle HIGH PRESSURE PUMP: • Pressure: min 100 bar. • Waterflow: • Type: ceramic plunger pump CONTROL PANEL • Possibility to operate-control all the system with this panel • The control panel consists of: - On/off switch for engine - A monometer showing the water pressure ENGINE : • Gasoline engine min.13 hp 420 cc with battery and electrical and manual starter accoupled to high pressure pump directlythrough gear box. OPERATION & FOAMDOSAGE SYSTEM • With the engine and pump,you have the chance to extinguesh the firethrough firegun. • Water flow and pressure can be adjusted through pressure regulator as peruser’s desire. • Foam flow can be adjust as 3 - 6% through foam dosage mixing valve . • System can mix and dosage AFFF or any type of Fire xtinguishing foam. • All the piping and connections are made from anti-rust material TELESCOPIC LIGHT MAST •Pneumatic actuated •With Cable Remote Control •4 x LED Lamps •3 Stages GENERATOR •1 unit of Diesel Generator –min. 2 kVa • min 50 m Electric Cable with Drum HAND TOOLS & ACCESSORIES 1 pcs shovel 1 pcs wood saw 2 pcs hammer 1 pcs crowbar 1 pcs of First Aid Kit 2 pcs of Axe 2 pcs x 20 lt of Metal Can for Spare Gasoline 4 pcs of Traffic Cone 2 pcs of Wheel Wedge 2 pcs DRAGER Breathing Apparatus Infra red camera 10 pieces • The TIC shall be capable of sensing, imaging, and displaying infrared radiation (“IR”) at 7 to 14 micron spectral wavelengths and shall have been designed specifically for use as adjunct firefighting equipment. • The TIC shall be hand held and portable allowing it to be passed from one firefighter to another. The TIC utilizes single green button operation. • The TIC shall meet International test Standard CEI IEC 529 (IP-67) that includesshort-term immersion in water to a depth of up to 3 feet, 6ft repeated concrete drop test and other requirements within the standard • The TIC shall use a single gloved hand accessible push type control to energize and de-energize the TIC. The TIC shall render a usable thermal and 30 seconds or less from on/off mode and less than 10 seconds from sleep mode. • The TIC shall include an LCD display that is no less than 3” diagonally. • The TIC shall have a minimum of a 160 × 120 Vanadium Oxide (VOx) Microbolometer. NO Exception Fire fighting helmets 10 pieces Exterior Body Composite material with high temperature resistance Interior Body Impact absorbing polyurethane Adjusting Mechanism Adjusting mechanism that can be adjustable to head Visor Made of material that is resistant to high temperatures and removable form inside Chinstrap Adjustable nonflammable materials Colour Ivory Product Certification EN 443: 2008 EN 14458:2004 Product Proporties Thanks to its photo luminascent feature, it provides high visibility Fire fighting gloves 10 pieces Palm Outershell Double-Face knit of Para-aramid with SEIZ® Silicon Carbon Coating Backhand Outershell Nomex® Fabric with reflective dots Moisture Barrier Waterproof and breathable GORE-TEX® X-TRAFIT TM membran Heat Barrier Flame, contact heat, convective heat, radiant heat resistant 100 % Para-aramid Properties material SEIZ® Senso Touch lining system. A Technology to guarantee a strong and permanent link between Para-aramid lining, GORETEX® membrane and the outer Shell Clasp that blocks the heat on the outer surface of the hand Elastic shirring at wrist 3M Scotchlite™ reflective tape at cuff end Cuff Hook and ring carabiner at cuffs Sewing Thread 100% Aramid Size According to the EN 659 and EN 420 Colour Black / NavyBlue Product Certification and Test EN659:2003 + A1:2008 Fire fighting clothing 10 pieces Outershell Fire Resist 75% Meta-Aramid 23% Para-Aramid 2% Antitistatic Fiber Mouisture Barrier Heat Resistant nonwoven laminated to a PTFE Membrane Heat Barrier Heat resistant nonwoven quited to Aramid/Viscose FR inner lining Sewing Thread 100% Aramid Velcros Heat Resistant Zipper Metal zipper, Panic Zipper Reflective Tape Certified according to EN 20471 and EN 469, yellow-silver-yellow, 5cm Colour Navy Blue Size S,M,L,XL,XXL according to TS EN ISO 13688 Product Certification and Test EN 469:2005 Level 2 (Xf2, Xr2, Y2, Z2) Reports EN 1149-3 & EN 1149-5 Logo The logo will be added on request. Fire fighting boots 10 pieces Outershell Material Outer surface is made of flame resistant and waterproof rubber material Mechanical Strenght Boots are produced with steel soles and steel toes to increase resistance against mechanical Chemical Strength Resistant against acid, alkaline, oil and petrol derivatives Sole Sole of boots are produced of special material that resists high temperatures exposed Reinforcements Boots have antiskid soles and soles are made of materials that have high abrasion Inner Lining Comfort enhancing canvas material Color Black, Yellow supported Product Certification and Test EN 15090:2006 F21S HI3 P T CI Reports EN ISO 20345:2004 + A1:2007 SRA SB P CI E Product Properties Non-flammable handles for easy wearing Self inflating flood barrier 2 pieces Type: Flood 1 • Overal size: 500 mm • Freeboard : 250 mm • Draft: 250 mm • Weight: 2,5 kg/m • Boom Fabric: PVC Coated Polyesther • Fabric Weight: 1.000 g/m2 • Boom Color: International Orange • Chambers: 3 • Tensile Strength: 3200 N/5 cm • Tear Strength: 400 N • Ballast: Water • Temparature: -35°C + 70°C • Storage: Boom Bag Water pump 1piece Stainless Steel Casing Waste Water Submersible Pump with Cutting blade • Capacity 6-15 m3/h • Engine: 1,5 HP • Outlet: 2” • Solid Component Size: 5 mm • Sealing: Mechanical Sealing • Speed: 2900 rpm • Temperature of Liquid: 1°C- 40°C • Density of Liquid: 1,2 gr/cm3 |  |  |  |
|  | LOT 2 Off road vehicle |  |  |  |
|  | Off road vehicle 1 piece Engine, transmission 1, 7 lt(volume) 8v(valves), 5MT(gear shift), 83 hp(engine power) Body Wheel arrangement/traction wheels permanent 4 WD (AWD) Engine location Front longitudinal Body type/number of doors sw I 3 seats 4 Length / width / height, mm 3640 / 1680 / 1640 Wheelbase, mm 2200 Front/rear track, mm 1440 / 1420 Road clearance, mm 205 Rear trunk capacity in passenger/commercial vehicle, 1 265 / 585 Engine Engine type gasoline F.eed system Port electronically controlled fuel injection Cylinder number and arrangement straight-4 Engine displacement; max 1700cm3 Maximum brake power, hp / kW / r.p.m. 61 (83) I 5000 Peak torque, Nm / r.p.m. 129 / 4000 Gas Premium gasoline Dynamic characteristics Maximum speed, km/h 142 Acceleration time 0-100 km/h, sec 17,0 Fuel consumption Urban cycle, 1/l00km 12,1 Country cycle, 1/100km 8,3 Combined cycle, 1/100km 9,9 |  |  |  |
|  | Weight Unladen weight, kg 1285 Gross weight, kg 1610 Fuel tank capacity, 4 2 Transmission No of gears: 5MT Final drive gear ratio 3,9 Suspension Front Independent on wishbone, spring-actuated, with oil-actuated telescopic shock absorbers and anti-roll bar Back Conventional, lever-type, spring-actuated, with double-acting shock absorbers Trailer for Off road vehicle 1 piece tray dimensions min 3060 x 1460mm -chassis , hot zinc-reinforced for +5mm -paw KF 7.5 750kg of load capacity - side-by-side added four places for binding to spaniers on both sides - The back sides with a marine plywood - manual hydraulic berth -on the front part the delimiter of the Quad from square tubes -winch 450kg on the carrier -The spare wheel holder, as well as the wheel, under the trailer -torsion axis VG 7L of load capacity up to 750 kg KNOTT non-braking -tires 155/80 R13” - wheel TK 48H –load capacity up to 100 kg -el.installation according to Eu standard - wheels are on the side of the car body - the height from the surface to the zero adjustable |  |  |  |
|  | position of the paw is 485 mm -servis 24h daily as well as the spare parts - gross official capacity of the trailer with an attestation 750 kg -net official capacity 500 kg - |  |  |  |
|  | LOT 3 Drone |  |  |  |
|  | Drone with equipment 1 piece Range 7km Batery LiPo 4S :30 minutes capacity Professional drone with a high-end 4K camera with 1 "20MP sensor and mechanical aperture that eliminates rolling shutter distortion, titanium and magnesium alloy body, Flight Automony dual rear vision system and infrared obstacle avoidance sensors, easy steering, take-off and a touch-back and an impressive 30 minutes of single-charge flight. Drone record H.264 4K videos in 60fps, H.265 4K videos in 30fps, both with 100Mbps bitrate, with built-in gyroscope for perfectly stabilized and smooth shots. |  |  |  |
|  | -The remote comes with a 5.5 "1080p screen where you can track the flight, which has a range of up to 7Km, as well as the Draw option of drawing a path on the screen during the flight, which allows focus on the camera for better and more complex shots. Climbing speed -6 m / s (Sport mode) Decay speed-4 m / s (Sport mode) |  |  |  |