# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Procurement of Multipurpose dredger for project "The repairing of the navigation infrastructure on Bega Canal"

Publication reference: VI-85/3

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Multipurpose dredger Quantity: 1 set |  |  |  |
|  | Multipurpose Dredger set must include set of next elements: - Four hydraulic stabilizers, excavator arm, power unit module, propulsion system and cabin + manuals and other documentation – 1 set - Side pontoons – 1 pair - Central pressure greasing system – 1 pc Mobility: - Dredger must be transportable as one complete unit with normal trailer; - Dredger must be able to load and unload itself on and from the trailer move independently in and out of the water without crane assisting in suction dredging mode; - Dredger must have own propulsion system to be able to cruise independently in water; - Main engine of the dredger must have water-air cooling system to be able to work also on dry ground; - In order to ensure steady and safe operation the dredger must be equipped with 4 stabilizers controlled hydraulically from cabin. Versatile: - Dredger shall be multipurpose in order to replace many single purpose machines. Dredger must be able to do: - suction dredging - backhoe dredging - raking - piling - Dredger must be equipped with submersible cutter pump to be able to do suction dredging on shallow water (0,5m) and from dry land side; - Dredger must be equipped with automatic central pressure greasing system. General and technical requirement: - Overall length of hull: minimal range between 8,00 – 11,00 m - Transport width of hull: minimal range between 2,9 - 3,4 m - Transport weight: from 15 to 20 tons; -Dredger can walk in and walk out of the water in both: a) suction dredging mode b) backhoe dredging mode - Dredger must have own propulsion system to be able to drive independently in water; - Propeller: The dredger must have one propeller, which can be hydraulically turned sideways for steering and lifted up from the water when walking from water to ground; - Drive speed with propulsion: minimum 3,4 knots; - Dredger must be able to operate and walk on dry ground; - Dredging depth: minimum reach at least 5,2 m from water level; - Dredging reach below water level in suction dredging: minimum reach at least 6,15 m; - Dredger must be equipped with 4 hydraulic stabilizers reaching 5,00 – 7,00 m below water level; - Pressure in the hydraulic system shall in no circumstances exceed standard level of 350 bar; - Dredger must be able to pump and excavate at least 100 m2 area in min. 0,9 meter depth without moving the machine itself; Engine: - Diesel engine, turbo charged; - Engine power: Minimum 200 kW; Cabin: - Ventilation: Air Condition; - Sun protection in all windows; - Minimum 10 working lights for operating during night time; - Safety: Cabin must be certified according safety certificate ISO 8083:2006 or equivalent. Certification issued by authorized accreditation service. Essential when pile driving or when working on construction sites, under bridges or in harbours. |  |  |  |
| 2 | Additional work tools Quantity: 1 set |  |  |  |
|  | Additional work tools must include set of next elements: - Suction dredging pump diameter must be 10”/ 250 mm, dredging capacity for water must be min. 800 m3/h – 1 pc - Backhoe bucket min. 600 l with teeth or equivalent for excavating – 1 pc - Rake, width min. 2700 mm for removing vegetation and trash – 1 pc - min. 600 l Clamshell bucket or equivalent for excavating – 1 pc - min. 1000 l Backhoe bucket without teeth for soft soils or equivalent for excavating – 1 pc - Piling bucket for wood piles or equivalent for driving piles – 1 pc |  |  |  |
| 3 | Pipeline components Quantity: 1 set |  |  |  |
|  | Pipeline components must include set of next elements: - Spray pipe – 1 pc - HDPE-pipeline with flanges (Ø 280 mm, pipe length 6m) – 2 x 100 meters - Floaters for 2 x 100 meters pipeline with attachment straps (Ø 225 mm) - Dredge hose (length 2m, outside Ø 285 mm) – 6 pieces |  |  |  |
| 4 | Accessories: Quantity: 1 set |  |  |  |
|  | Accessories must include set of next elements: - Service crane – 1 pc - Digital Measurement and indication of dredging depth – 1 pc - Hydraulically operated high-pressure washer – 1 pc - Spare and wear parts package (set) for min. 3000 operating hours maintenance – 1 set |  |  |  |

| All items | Supply, delivery and installation performed by contractor or authorised service provider. All the equipment must include all necessary parts and standards for its installation. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Testing of all basic functions of the instrument on a set of producers standard samples commonly used for the corresponding instrument. |  |  |  |
| All items | Training at customer site for minimum 3 persons (upon delivery) on equipment handling (familiarisation during installation - working with the equipment in all basic functions of the equipment), equipment maintenance. Details of proposed training methodology, shall be presented in the tender proposal and shall refer to the related equipment |  |  |  |
| All items | Technical documentation for equipment (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | - Warranty - Tenderers must provide local reliable warranty service agent providing maintenance and the rapid supply of equipment spare parts and consumables for the Warranty duration of one year. - Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Help Desk (phone) support, which must be available during working hours, 09.00 – 17.00 CET; - Guaranteed maximum response time to submitted maintenance support request (fax or e-mail) of 2 (two) working day; - Guaranteed that any requests for services will be attended to within 72 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 72 hours; - Guarantee that genuine spare parts and consumables will be available for a period of minimum 10 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 1 year from the date of delivery or 1000 hours of operation in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |