# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of laboratory equipment`s	p 1 / 13

Publication reference: RORS462/Technical Faculty Bor/106354/TD6

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Lot.1. Laboratory equipments |  |  |  |  |
| 1 | Portable conductivity, pH, temp meter Quantity: 1 pc. |  |  |  |
|  | Temperature range: min. 0.00 to 50.00 °C; pH range: 0.00 to 14.00 pH; Calibration : automatic Conductivity range: min. 0 to 200 μS/cm TDS (Total Dissolved Solids) range: min. 0 to 100 ppm |  |  |  |
|  | Warranty: minimum 1 year |  |  |  |
| 2 | Waterproof Portable Dissolved Oxygen and BOD meter Quantity: 1 pc. |  |  |  |
|  | Dissolved Oxygen range: min. 0.00 to 19.99 mg/l Dissolved Oxygen resolution: max. 0.01mg/l Barometric pressure range: 0-1100 mbar Barometric pressure resolution: max. 1mbar Temperature range: min. 0 to 60° C Temperature resolution: max. 0.1° C Oxygen saturation range: min. 0.00 to 199.99% Oxygen saturation resolution: max. 0.1 % Salinity range: 0 - 50 ppt PC connectivity: USB Weight: max 300g |  |  |  |
|  | Warranty: minimum 1 year |  |  |  |
| 3 | Reagents and gas for analysis Quantity: 1 set. |  |  |  |
| 3.1. | Technical calibration buffer with +/ 0.01 pH accuracy 4.01pH: Package,bottle, Size :500ml. Quantity: 1pc |  |  |  |
| 3.2. | Technical calibration buffer with +/ 0.01 pH accuracy 7.01pH, Package,bottles, Size :500ml. Quantity: 1pc |  |  |  |
| 3.3. | Technical calibration buffer with +/ 0.01 pH accuracy 10.01pH, Package,bottle, Size :500ml. Quantity: 1pc |  |  |  |
| 3.4. | Conductivity Standard 84 µS/cm on 25°C, ISO17025 certified, NIST traceable, Package,bottle , 500ml. Quantity: 1pc |  |  |  |
| 3.5. | Conductivity Standard 147 µS/cm, ISO17025 certified, NIST traceable, Package: bottle, Volume Capacity 500ml. Quantity: 1 pc |  |  |  |
| 3.6. | Conductivity Standard 1413 µS/cm on 25°C, ISO17025 certified, NIST traceable, 500ml . Quantity: 1pc; |  |  |  |
| 3.7. | pH Electrode Storage Solution, Package: bottle, Volume Capacity 500ml. Quantity: 1pc; |  |  |  |
| 3.8. | pH Electrode Cleaning Solution, Package: bottle, Volume Capacity 500ml. Quantity: 1pc |  |  |  |
| 3.9. | Cleaning and Disinfection Solution for Algae, Fungi and Bact. Package: bottle, Volume Capacity 500ml. Quantity: 1pc; |  |  |  |
| 3.10. | Quick Calibration Solution, Volume Capacity 500 mL bottle, Quantity: 1pc |  |  |  |
| 3.11. | ORP test solution 240 mV (20°C), Package: bottle, Volume Capacity 500ml. Quantity: 1pc; |  |  |  |
| 3.12. | ORP test solution 470 mV (20°C), Package: bottle, Volume Capacity 500ml. Quantity: 1pc; |  |  |  |
| 3.13. | Glass cuvettes, Volume: 10 ml Quantity: 16pc; |  |  |  |
| 3.14. | New cap for cuvettes , Material: plastic Quantity: 16pc; |  |  |  |
| 3.15. | Cuvette cleaning cloth, Material: cotton Quantity: 4pc; |  |  |  |
| 3.16. | Instrument Calibration Standard 5% HNO3 / tr Tartaric Acid/ tr HF; 100 µg/mL: Ag, Al, As, Ba, Be, Ca, Cd, Co, Cr, Cu, Fe, K, Mg, Mn, Mo, Na, Ni, Pb, Sb, Se, Sn, Sr, Ti, Tl, V, Zn; Package: bottle, Volume Capacity : 125 mL Quantity: 1pc; |  |  |  |
| 3.17. | Mercury (Hg) Pure Standard 1,000 µg/mL Hg 10% HNO3 Package: bottle Volume Capacity : 125 ml Quantity: 1pc; |  |  |  |
| 3.18. | Instrument Calibration Standard 5% HNO3 / tr H; 21,215 µg/mL: Cl 3,000 µg/mL: Ca 2,500 µg/mL: Package: bottle Volume Capacity : 125 ml Quantity: 1pc; |  |  |  |
| 3.19. | Fe, Na 2,000 µg/mL: C 1,000 µg/mL: Al, K, Mg, P, S 20 µg/mL: Mo, T Package: bottle Volume Capacity : 125 ml Quantity: 1pc; |  |  |  |
| 3.20. | Graduated Conical Bottom SuperClear Tubes with Teal Flat Caps Included Volume Capacity : 15 mL Material: glass or plastic Quantity: 500pcs; |  |  |  |
| 3.21. | Sample Autosampler Capacity: 60 pcs, Sample Vessel Volume Capacity: 15mL or16mL; Quantity: 2pcs; |  |  |  |
| 3.22. | Tubing Material:Non-Flared Solva MP2 Two-Stop Peristaltic Pump Tubing : Ø 0.76 mm Package: Pack with12 pcs; Quantity: 1 pack; |  |  |  |
| 3.23. | Tubing Material: Non-Flared Solva MP2 Two-Stop Peristaltic Pump Tubing : Ø 1.14 mm Package: Pack with 12 pcs; Quantity: 1 pack; |  |  |  |
| 3.24. | LLG-Microlitre pipette, Variable volume : 1000-10000 L; Quantity: 1 pc |  |  |  |
| 3.25. | Argon, Gas for analysis, Package: : bottle Weight: 40 kg, Quantity: 8 pcs; |  |  |  |
| 3.26. | Nitrogen, Gas for analysis, Package: : bottle Weight: 40 kg, Quantity: 1pc; |  |  |  |
| 3.27. | Graduated cylinders, Material: PP, Volume capacity: 100mL, Quantity: 5pcs |  |  |  |
| 3.28. | Membrane filter 25 mm cellulose acetate, 0.45 um, white, non sterile. Package : pack with 100 pcs Quantity: 1 pcs; |  |  |  |
| 3.29 | Narrow-necked bottles Material: PE Volume capacity :100 ml. Quantity: 30pcs |  |  |  |
| 3.30. | Narrow-necked bottles Material: PE Volume capacity: 500 ml, Quantity, 50 pcs |  |  |  |
| 3.31. | Narrow-necked bottles Material: PE Volume capacity :1000 ml, Quantity:20pcs; |  |  |  |
| 3.32. | Wide-necked square bottle : Material: HDPE or PP or PE Volume capacity 250 ml, Quantity: 30pcs; |  |  |  |
| 3.33. | Nitrile protective gloves Material: plastic Package : pack with 100 pcs |  |  |  |
| 4. | Additional Equipments for samples Quantity: 1 set |  |  |  |
| 4.1. | Electronic precision top loading and platform balances - Capacity max. 3100 g; Readability 0,01 g; Pan dimensions max. Ø 160 mm |  |  |  |
| 4.2. | Sample splitters (riffle boxes) Material Steel : Stainless; Slot width 1-2" – 12,7 mm; Max Size Sample mm 10; Slot number min 12 |  |  |  |
| 4.3. | Test sieve - diameter min. 8 " - aperture 0,075 mm, high 50 mm Quantity: 2pcs |  |  |  |
| 4.4. | Brush Double ended soft/hard nylon Quantity: 2pcs |  |  |  |
| 4.5. | Telescopic aluminum rod Adjustable rod length: 165 - 450cm; Angular beaker, Material : polypropylene ,Capacitate: max. 1000ml; |  |  |  |
| 4.6. | Multi sampler Rod operated, Material: transparent acrylic, Dimension : max. Ø 50 mm, length 1 m, Contents: min.1 l |  |  |  |
| 4.7. | Bayonet inclusive 1 sharp cutting head 1 cutting head with ball valve seating and ball |  |  |  |
| 4.8. | Cap Material : polyethylene, Inside dimension 40 mm, Quantity : 20 pcs |  |  |  |
| 4.9. | Ropes Stretch-proof, with noose, Length min.5 m. Material: Polyester covering, strong stretch-proof core Quantity : 2 pc |  |  |  |
| 4.10. | Material for accurate operation of pistons in core samplers. Complete with one carabine hook |  |  |  |
| 4.11. | Handle Dimension : normal, Length: max. 60 cm, Material: synthetic, Detachable grip (inclusive coupling sleeve), bayonet; Quantity : 1 pc |  |  |  |
| 4.12. | Extension rod Length: min. 100 cm (inclusive coupling sleeve) bayonet; Quantity : 1 pc |  |  |  |
| 4.13. | Carrying bag for field equipment, Number of shoulder straps : 2 pcs Inside dimension: min. Ø 17 x 150 cm; Quantity : 1 pc |  |  |  |
| 4.14. | Handle Dimension : normal, Length: max. 60 cm, bayonet (inclusive coupling sleeve); Quantity : 1 pc |  |  |  |
| 4.15. | Edelman auger bottom part, combination type, bayonet, diameter: min. Ø 7 cm; Extension rod. Length : min. 100 cm (inclusive coupling sleeve) bayonet. Quantity : 1 pc |  |  |  |
| 4.16. | Air Compressor with air pressure regulator max pressure 10 bar reservoir capacity minimum 50 liters air delivery minimum 250 l/min Weight: max. 50 Power supply: 230V, 1.5 KW; Quantity : 1 pc |  |  |  |
|  | Warranty: minimum 1 year |  |  |  |
| 5. | Particle Sizing Analyzer Quantity: 1 set. |  |  |  |
|  | Measuring range: wet dispersion: 0.01 - 2100 µm dry dispersion: 0.1 - 2100 µm |  |  |  |
|  | Analysis method : laser light scattering |  |  |  |
|  | Type of analysis : wet and dry measurement of the particle size of solids and suspensions |  |  |  |
|  | Measurement value : particle size |  |  |  |
|  | Standard: ISO 13320 |  |  |  |
|  | Number of particle size classes: max. 110 |  |  |  |
|  | Laser: 1 x green (λ = 532 nm, 7 mW) 1 x IR (λ = 850 nm, 15 mW) Linear polarization 10000 hours average service time |  |  |  |
|  | Laser beam alignment: automatic |  |  |  |
|  | Fourier lenses: 260 mm and 560 mm focal length (green or IR) 10 mm diameter of the laser beam in the Fourier lens |  |  |  |
|  | Cell spacings: 20 mm for green 540 mm for IR |  |  |  |
|  | Angle range: 0.014° - 165° |  |  |  |
|  | Time for capturing detector channels: max 41 µs (24 kHz) |  |  |  |
|  | Detectors: min 2 segments 1 x for vertical and 1 x for horizontal direction of the laser light polarisation57 elements |  |  |  |
|  | Typical measuring time: min.5s, max. 10s |  |  |  |
|  | Voltage: 240 V/1A~ |  |  |  |
|  | Frequency: 50 - 60 hz |  |  |  |
|  | Input power: min. 50 Watt |  |  |  |
|  | Wet dispersion unit: Dispersible particle size: 0.01 - 2100 µm Volume of sample circuit: 300 - 500 ml Ultrasonic power and frequency: 100 W / 36 kHz Liquid pump rate: min. 6.8 l/min Particle size which can be dispersed: max.2100 µm Quantity: 1 pc.. |  |  |  |
|  | Filter set for water supply including pressure regulator: Back flush filter 100 µm with pressure regulator and manometer, Two filter housing 10“, Filter 5 µm with AC, Filter 1 µm Quantity: 1 set. |  |  |  |
|  | Analysis according : Fraunhofer or Mie theory |  |  |  |
|  | Data export to ExcelTM and/or XML format |  |  |  |
|  | PC Controller (desktop PC or laptop) with min. 15“ monitor-display Quantity: 1 pc. |  |  |  |
|  | Warranty: minimum 1 year |  |  |  |
| 6. | X-ray fluorescence spectrometry analyzer Quantity: 1 set. |  |  |  |
|  | Excitation Source: X ray tube min. 4W Rh 6-50kV, 5-200 μA |  |  |  |
|  | Detector: Silicon Drift Detector (SDD) detector type, minimum resolution <145eV |  |  |  |
|  | Detector protection shield: The detector protection device should be constructed as a metal mesh placed in front of the beryllium window of detector. The openings on the metal mesh must be less than max. 0.5 mm in order to prevent the penetration of objects to detector. Metal mesh must transmit more than 60% of the sample signal across the entire range of the unit. |  |  |  |
|  | Camera: a built-in Color CMOS Camera for precise positioning of the measurement spot with adjustable lighting and ability to store up to 5 photos per assay |  |  |  |
|  | Filter: minimum 5 position automatic filter changer |  |  |  |
|  | Sample temperature: up to 150°C, with the possibility for operation at 350C with appropriate window |  |  |  |
|  | Collimator Spot size: Choice of 3,5 or 8 mm |  |  |  |
|  | Dimensions (length x width x height): max. 25 cm x 28 cm x 9 cm |  |  |  |
|  | Weight: max. 1.5 kg with battery |  |  |  |
|  | Display: Touchscreen, integrated color LCD, minimum 9.4 cm (3.7 inch) |  |  |  |
|  | Data storage: minimum 500 MB internal memory, external USB data storage |  |  |  |
|  | Data transfer: Bluetooth™, USB and Wi-Fi connectivity |  |  |  |
|  | Languages: Multilanguage menu including menu on Serbian/Croatian |  |  |  |
| 6.1. | Desktop Stand for measurement on desk Quantity: 1 pc. |  |  |  |
| 6.2 | GPS Receiver Position updates: at least once per second Maximum speed: 1150 mph Altitude: min. 18,000 m Bluetooth range: min. 10m Quantity: 1 pc. |  |  |  |
| 6.3. | XRF Sample Cups 32mm with lid (100) 1.24" x 1.1.5 (31.5 mm) OD X 1.36" (34.5mm max OD 25.9 mm with vented cup; double open ended with internal overflow 100 cups per bag Quantity: 1 pc. |  |  |  |
| 6.4. | XRF Sample Cup Foil PROLENE4 Roll 91m Material: roll of Prolene (TM) Max. 0.16 mils 4 microns 3" x 300' (7.6 cmx91.4 m) Quantity: 1 pc. |  |  |  |
| 6.5. | Alloy calibration package with light element (LE) capability. Includes type specific calibrations for stainless steels, tool steels, low alloy steels, nickel alloys, cobalt alloys, copper alloys, zinc alloys, light (Al, Mg and Ti) alloys and general calibration for all other alloy types. Including Alloy check sample. Quantity: 1 pc. |  |  |  |
| 6.6. | Heavy metals and Nutrients in soil Dual phase calibration for environmental compliance and elemental nutrients in soil. With check sample. Quantity: 1 pc. |  |  |  |
| 6.7. | GeoExploration application package Multiphase application package including GeoExploration. Calibration for Geochemical exploration and GeoMining for mining and ore processing. Automated and user selectable calibrations for oxide and sulfide matrices. With check sample. Quantity: 1 pc. |  |  |  |
| 6.8. | Restricted Materials calibration Restricted materials, consumer products calibration package with light element capability (Al-Bi). For all ROHSII, CPSIA, PROP65, WHEE compliance applications. With user selectable calibrations for plastics, mid-density materials and metals. With Plastic and Solder check sample. Possibility for Cl and Br measurement and that can be used also for Halogen free testing. Quantity: 1 pc. |  |  |  |
|  | Warranty: minimum 1 year |  |  |  |

Unless otherwise specified, the requirements in this Technical Specifications are presented as minimum standard, which the offered goods must meet. The offers that will not satisfy completely and correctly the minimum requirements mentioned above, will be disqualified.

When brands names/specifications referring to or describing products of a given brand or origin are used in this technical specifications, they are used in descriptive purposes only and should be interpreted as equivalent.