# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Acquisition of equipments, accessories, consumables and reagents                                                                  p 1 /  11

Publication reference : RORS 365/ University of Agricultural Sciences and Veterinary Medicine of Banat "King Mihai I of Romania" from Timisoara/ 02

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Unless otherwise specified, the requirements in this Technical Specifications are presented as minimum standard which the offered goods must meet. The offers that will not satisfy completely and correctly the minimum requirements mentioned bellow will be disqualified.

When brands names or EN-ISO standards/specifications referring to or describing products of a given brand or origin are used in this technical specifications, they are used in descriptive purposes only and should be interpreted as equivalent.

All supplies under this contract may originate from any country.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | FRX elemental Analyser - 1 piece |  |  |  |
|  | Min. 50 kV Rh-target X-ray tube Large area silicon drift detector (SDD) Factory-calibrated and ready to use Lightweight, max 1.5 kg Tested to MIL-STD 810G IP54 compliant Automatic 6-position filter changer Accessories included: light radiation shield to measure Al alloysand plastics, light stand and safety shield for small samples analysis, and background plate for plastics analysis Integrated LCD colour touch screen display min. 4.2” Integrated computer Integrated camera Magnification: x6. Image size: 640 pixels x 480 pixels. Image area on sample: approx. 7.5 mm x 6 mm. Camera technology: CMOS. Min. 16 GB internal memory -10°C to +50°C operating temperatures Store formin. 100,000 results Bluetooth, WiFi and USB data transfer Password protection. Infrared proximity sensor. Count rate sensor. Fail-safe warning lights Fundamental parameters calibrations for mining applications where metals are present in high concentrations (>1%). Standard element set: Mg, Al, Si, P, S, K, Ca, Ti, V, Cr, Mn, Fe, Co, Ni, Cu, Zn, As, Se, Rb, Sr, Zr, Nb, Mo, Ag, Pd, Cd, Sn, Sb, Ba, Ta, W, Pt, Au, Hg, Tl, Pb, Bi, Th, U; Standard element set: K, Ca, Sc, Ti, V, Cr, Mn, Fe, Co, Ni, Cu, Zn, As, Se, Rb, Sr, Mo, Ag, Cd, Sn, Sb, Ba, Ta, W, Au, Hg, Tl, Pb, Bi, Th, U; Inludes a soil CRM check sample Fundamental parameters calibration for rare earths mining application. Standard set includes: S, Cl, K, Ca, Sc, Ti, V, Cr, Mn, Fe, Co, Ni, Cu, Zn, Ga, As, Rb, Sr, Y, Zr, Nb, Mo, Ag, Pd, Cd, Sn, Sb, Ba, La, Ce, Pr, Nd, Hf, Ta, W, Pb, Bi, Th, U Fundamental parameters calibration for alloy analysis and grade identification. Analyses also special alloys such as Au, Ag, Pt, Sn and W Standard element set: Ag, Al, Au, As, Bi, Cr, Co, Cu, Fe, Hf, In, Ir, Mg, Mo, Mn, Nb, Ni, P, Pd, Pt, Pb, S, Se, Si, Sn, Sb, Ta, Ti, V, W, Y, Zn, Zr. Set of verification standards. Min. 25 pcs with usual grades in Fe, Cu, Al, Ni bases Control system for equipment, data storage and processing: processor minimum i5 (or equivalent), minimum 3,20GHz, memory RAM minimum 4 GB, hard disk minimum 480 GB, DDR5, DVD-RW, monitor LED minimum 18", mouse, keyboard CNCAN authorisation Warranty: min. 12 months Installation and training included for project team members |  |  |  |
| 2 | Equipment, accessories and specific consumables for chemical analisys – 5 set |  |  |  |
|  | 1. Calibration standard for XRF and reactives: 1.1 NIST 2711a, prepared in a sample cup, to verify Soil mode accuracy and stability – 1 pc 1.2 Fe ore sinter (BCS CRM377/6), prepared in a sample cup, to verify Mining mode accuracy and stability – 1 pc 1.3 Ni ore (GBM911-15), prepared in a sample cup, to verify Mining mode accuracy and stability – 1 pc 1.4 Cu ore (OREAS 904), prepared in a sample cup, to verify Miniing mode accuracy and stability – 1 pc 1.5. Chromium VI – Soil certified reference material, min 30 g – 1 pc 1.6. TROLOX – 5g Concentration: ≥97% CAS Number: 53188-07-1 Formula: C14H18O4 MW: 250,29 g/mol Melting Pt: 187°C - 194°C 1.7. Neocuproine - 25 g Concentration : ≥98% Formula: C14H12N2 MW: 208,26 g/mol Melting Pt: 161 - 163 °C CAS No.484-11-7 1.8.Copper (II) chloride – 1 kg Formula: CuCl2·2H2O MW: 170,48 g/mol Melting Pt: 100 °C Density: 2,54 g/cm³ (20 °C) Storage Temperature: ambient CAS Number: 10125-13-0 2.Solid phase chelation 2.1.Solid phase chelating disks Solid phase extraction disk 47 mm – 200 pcs Anion, C18 Sorbent Particles, C8 Sorbent Particles, Cation Exachange, Chelating, Filter + C18 Sorbent Particles, SDB-XC Sorbent Particles Used for purification and concentration of analytes from aqueous samples Filter unit for 47 mm disc filter made of borosilicate glass, contains flask, fritted glass holder base, funnel and aluminium clamps, complete set with 1000 ml flask Vacuum pump for filter unit 2.2.Polyvinylpyrrolidone – 2kg Concentration: ≥99% CAS Number: 9003-39-8 Heavy Metals <0.001% pH (5%, Water) @25 °C 3 – 7 2.3. Methylcellulose – 2.5 kg emulsifying and suspending agent viscosity 15 cPs soluble in cold water 3. Pellet preparation: 3.1 Mini-Pellet Press for XRF Min. 15 Ton Press Lower piston stroke: min. 25 mm Distance between pressing faces (max): min. 150 mm Distance between pressing faces (min): min. 35 mm Polycarbonate safety guards Adjustable upper bolster Pressure release valve Adjustable pressure control valve Vacuum ports Gauges for low pressure applications Pellet Die with Pellet Ring Holder 3.2 XRF Grinder Consumable – 1 set 4. Transfer pipettes 4.1.Variable volume micropipette – 1 pc Fully autoclavable at 121ºC without disassembling Volume: 100 - 1000 μl Precision: ±0,7% - 0,2% Increment: 5,0 μl Individually calibrated according to EN-ISO 8655 standard Set of min. 500 tips included 4.2Variable volume micropipette – 1 pc Fully autoclavable at 121ºC without disassembling Volume: 10 - 100 μl Precision: ±1,5% - 0,2% Increment: 1,0 μl Individually calibrated according to EN-ISO 8655 standard Set of min. 1000 tips included 4.3. Pipette filler – 10 pcs Made of natural rubber With 3 valves For pipette up to 100 ml 5. Laboratory glassware and other laboratory consumables 5.1. Erlenmeyer flask with GL thread and screw cap – 30 pcs Made of 3.3 borosilicate glass Volume: 500 ml With GL thread and screw cap with a PTFE joint According to DIN 12380, ISO 1773 5.2. Erlenmeyer flask with GL thread and screw cap – 60 pcs Made of 3.3 borosilicate glass Volume: 500 ml With GL thread and screw cap with a PTFE joint According to DIN 12380, ISO 1773 5.3. Volumetric flask class A with plastic stopper – 12 pcs Volume: 25 ml Made of 3.3 borosilicate glass Blue graduation Accuracy: ±0.040 ml Batch certificate included Calibrated to contain 5.4. Volumetric flask class A with plastic stopper – 20 pcs Volume: 50 ml Made of 3.3 borosilicate glass Blue graduation Accuracy: ±0.060 ml Batch certificate included Calibrated to contain 5.5. Volumetric flask class A with plastic stopper – 10 pcs Volume: 100 ml Made of 3.3 borosilicate glass Blue graduation Accuracy: ±0.100 ml Batch certificate included Calibrated to contain 5.6. Volumetric flask class A with plastic stopper – 4 pcs Volume: 500 ml Made of 3.3 borosilicate glass Blue graduation Accuracy: ±0.250 ml Batch certificate included Calibrated to contain 5.7. Volumetric flask class A with plastic stopper – 4 pcs Volume: 1000 ml Made of 3.3 borosilicate glass Blue graduation Accuracy: ±0.400 ml Batch certificate included Calibrated to contain 5.8. Melting crucible high form, with lid 30 pcs Capacity: 30 ml Glazed inside and outside H min. 45 mm 5.9. Melting crucible high form, with lid 10 pcs Capacity: 40 ml Glazed inside and outside H min. 51 mm 5.10. Test tube – 200 pcs Made of borosilicate glass Ø: 16 mm Capacity: min 12 ml L: 100 mm 5.11. Test tube rack – 4 PCS Made of stanless steel For min. 20 tube with Ø: <18 mm 5.12. Capsules with lid – 30 pcs Made of borosilicate glass Diameter: min. 50 mm Heigh: min. 30 mm 5.13 Syringe filter 5.13.1 Syringe Filter 0.22 µm – set of min. 100 pcs Diameter: min. 25 mm Housing: PP Conectors: FLL/MSL Membrane made of PTFE High thermal resistance 5.13.2 Syringe Filter 0.45 µm – set of min. 100 pcs Diameter: min. 25 mm Housing: PP Conectors: FLL/MSL Membrane made of PTFE High thermal resistance |  |  |  |
| 3 | Mince mixer - 1 piece |  |  |  |
|  | Discs Grinder Type Motor Power min. 0.5 HP Motor Speed min. 1400 rpm Grinding Discs min. 64mm Grinding Capacity 10 ... 14 Kg/h Power Supply 220 - 250 volts Accessories included: fully equipped ultrasonic homogenizer for 50 ml volumes |  |  |  |
| 4 | Specific consumables for microbiology – 5 set |  |  |  |
|  | 1 Variable volume micropipette – 1 pcs Fully autoclavable at 121ºC without disassembling Volume: 100 - 1000 μl Accuracy: ±0,7% - 0,2% Increment: 5,0 μl Individually calibrated according to EN-ISO 8655 standard 2 Accesories 2.1 Round stand for 6 micropipettes 2.2 Kolle handle, 200 mm lenght with 2 Ni-Cr inoculating loops – 6 pcs 2.3. PE transfer Pasteur pipettes, sterile, ind. wrapped, graduated each 0,25 ml up to 1 ml, overall vol. – 2000 pcs 3. Consumables 3.1 Petri dishes, Ø100 mm, Polystyrene, sterile, with 3 vents, 500 pcs 3.2 Inoculation loops, calibrated, 1 µl white color, sterile, 1 pc/peel pack, 500 pcs/pack – 3 sets 3.3. Glass bottle narrow neck, amber, DIN-18, 20 ml, screw cap included 300 pcs 3.4. Hydrophilic wadding 200gr – 20 pcs 3.5.Sterilization bag with fold 200 x 400 x 50 mm (100 pcs / pack) – 15 sets 4. Reactives 4.1 TSI agar 7,5 ml ISO 6579; 20 x 5 pcs 4.2 Simmons citrate agar 6 ml ISO 10273, ISO 22964; 20 x 5 pcs 4.3 Kovac's indole reagent for microbiology – 100 ml – 2 pcs 4.4 di-Potassium hydrogen phosphate gen-apex for molecular biology – 500 g – 2 pcs 4.5 Phenol red acs grade – 25 g – 4 pcs 4.6 Ammonium sulphate ≥99%, GPR 1 kg 4.7 Sanitary alcohol 500ml – 20 pcs 5. Reference strains 5.1. E coli ATCC10536 (Cod CBD02330); U.M.: 3 vials x 10 discs; 5.2. Clostridium perfringens ATCC 13124/NCTC8237 3 vials x 10 discs; |  |  |  |
| 5 | Metal cabinets for reagents and samples - 8 pieces |  |  |  |
|  | 1. Metal cabinets for reagents and samples -7 pcs dimensions: min. 500x500x2000 mnm (LxWxH) welded clothing made of quality board I two overlapping cabinets two doors fitted with yolk the upper closet has a pole, so 2 compartments the upper cabinet has a collection tray at the bottom the bottom closet has a shelf so two storage compartments electrostatic paint in the color requested by the beneficiary 2. Metal cabinets for reagents and samples -1 pcs aspirated and filteredsafety cabinet for the storage of 70 litres of chemical, acid and base products, in compliance with EN 61010-1, EN 16121, EN 16122 NORMS - CE MARK - TÜV CERTIFICATION Completely produced in pickled and electrogalvanized steel, 10/10 mm thickness, painted with epoxy acid-proof powders. External monolithic construction, internal body resistant to corrosive product Activated charcoal filter with electronic device Doors opening up to 180°. Internal Dimensions:min 500x400x1500 mm Key lock |  |  |  |
| 6 | Bactericidal Lamp UV - 1 piece |  |  |  |
|  | For bacteria, viruses, mold spores, yeasts, algae, protozoa UV-C light generating tube (wavelength 253.7 nm); Mounting type: wall bracket Equipped with an orientation device Efficient operating time of bactericidal tubes: min. 8500 hours Power: min. 30 W |  |  |  |
| 7 | McFarland densitometer - 1 piece |  |  |  |
|  | Factory calibrated Density accuracy Déviation maximale: 3,0 ±0,1 McF Supplied with D16 adapter for Ø 16 mm tubes Sample size 2 ml Light source LED |  |  |  |
| 8 | pH micrometer - 1 piece |  |  |  |
|  | pH range 0.00 to 14.00 pH pH resolution 0.01 pH pH accuracy ±0.2 pH calibration automatic, one or two points with three standard buffers included (ph 4.01, 7.01 and 10.01) battery type/life, 1.5V CR2032 (1) / approximately 1000 hours of continuous use electrode/probe: pH electrode with screw type connector (included) environment 0 to 50°C (32 to 122°F); RH max 95% |  |  |  |
| 9 | Climate equipment, for EEP Lab - 1 piece |  |  |  |
|  | General, cooling and heating capacity each min. 12000 BTU Surface: min. 70 mp Filter included: Antibacterian, Antiallergic, Self-cleaning Compressor: Digital Inverter 8-POLE Installation kit included Inverter included A ++ cooling energy efficiency Number of fan steps: min. 5 |  |  |  |