Lot 3 - Container for logistics support

CAMPING EQUIPMENT - MOBILE ENCAMPMENT IN CONTAINER SYSTEM

ANNEX II + III : TECHNICAL SPECIFICATIONS + TECHNICAL OFFER Contract title : Procurement equipments (Supply) p 1 /… Publication reference : RORS 283/8 eMS Columns 1-2 should be completed by the Contracting Authority Columns 3-4 should be completed by the tenderer Column 5 is reserved for the evaluation committee Annex III - the Contractor's technical offer The tenderers are requested to complete the template on the next pages: Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer), Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient) Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee. The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications. Any mentions regarding a trade mark, brand, model is only for a proper and issyier identification of product requirements, and should be understand as followed by phrase “or equivalent”.

| Item number | Specification required | Specifications offered | Notes, remarks, ref to documentation | Evaluation Committie’s notes |
| --- | --- | --- | --- | --- |
| 0 | The containerized system will provide logistical support for the transport of specific equipment and materials necessary for the optimal organization of camps for intervention in long-term emergency situations to combat or limit the effects of disasters. |  |  |  |
| 1 | Container - 1 piece The transportable container shall be constructed in accordance with DIN 30722 (which defines the height of the hook and the distances to the anchoring elements of the sub-frames on the transport platform). It shall be loaded on vehicles equipped with a hook system which allow loading / unloading of the following dimensions: 1. Container inner length: maximum 7000 mm; 2. Container outer width: maximum 2550 mm 3. Container height including load system: minimum 2200, maximum 2400 mm. 4. The length of the container to which the rail and the handling ring are attached to DIN 30722: maximum 7400 mm; 5. Handle height: 1570 ⁵⁵ mm. 6. Thickness of handling ring: Ø 50 mm. 7. The mass of the container assembly (including lifting and loading hook system) loaded with all materials / equipment in the complete inventory of the product shall not exceed 11,000 kg. The container will be provided on both sides (lengthwise, left and right) with openings protected with double-wall metal blinds for access to equipment and materials. When opening blinds, the operator must have access to any of the boxes / carts / drawers without moving or detaching another crate unless the boxes contain the same materials or equipment. Inside the container, the chassis / trolleys shall be fitted on sliding rack-type shelves with locking system for open / closed positions. Access to crates must be easy, regardless of the position of the chassis. The container will have a fixed ceiling access staircase located at the front of the container next to the loading system The container shall be provided with a foldable staircase fitted to each closure with the blinds so that access to the equipment can be made without lowering the container on the chassis of the transporter vehicle. It will have the following features: 1. Minimum width 800 mm 2. Stage number: minimum 6 3. Made of galvanized steel and anti-skid steps with a depth of 220 mm. The container will be provided with internal and external LED lighting. In each compartment, adequate lighting will be provided to identify the equipment. Perimeter lighting will be incorporated into the container structure. The power supply of the indoor and outdoor lighting system will be done with a battery of batteries, which can be charged from the vehicle's installation or from an external power source (external electric generator). It will be fitted with gauges on both sides. |  |  |  |
| 2 | Tent for shower - tent with accessories with 4 showers - 2 pieces A. TENT COMPONENT - External tent with inflatable structure and floor (Tarpauline insulation in solidarity with the tent) - Inner tent (with 2 modules) - Rug (tarpaulin) under the tent (protection against stinging under the tent) - Air pump and inflating hose set - repair kit - Anchoring kit - interior lighting system - preinstalled 4 shower system - evacuation system for used water 1. Tent Dimensions: - Width: minimum 5.00 m - Length: minimum 5.00 m - Shelf height: minimum 2.20 m - Height at the tail: max. 2.80 m - Floor surface: minimum 25 sqm 2. Standard Features - at least 1 entrance with swing/rolling doors and anchoring system at the top - inflatable resistance structure - Folding system with removable aluminum reinforcements for interior strengthening. - electric pump for inflation and deflation of inflatable resistance structure - inner tent for thermal insulation and weatherproofing with at least two chambers: dressing room and shower area. - windows with insect screen and shutters located on each side: min 2 pieces/side - windows with insect screen and shutters located on each side: min 2 pieces/side - each chamber have 2 windows and interior lighting system - interior lighting system: at least 2 lamps with minimum 36W each, individual switch, minim IP 65, the plug outside the tent (the cables are masked – manufacturer solution), - installation with 4 showers - protection tarpaulin under the tent (protection against punctures under the tent) - PVC insulation floor solidary welded with the tent - In order to avoid personnel slipping during or after the shower, the floor inside the tent in the shower area is provided with solid, removable, non-slip tiles - controlled drainage system for waste water - Anchor system (anchor ropes, anchor pins, hammer ...) - Access for air conditioning tubes and electric cables 3. Construction details: To achieve the proposed purpose, the tent is divided into two areas: the dressing room and the shower area. The area for dressing room located at the entrance to the tent is used for storing the equipment during the shower or for drying and maintaining the equipment and is provided with an inside tent made of light white fabric. In the shower area access can be made from the dressing area or through the second entrance of the tent located on the rear wall. In this area there are 4 mobile showers for 4 people, and the floor surface should prevent the staff from slipping during the shower. The technical solution of the tent must allow the collection of the waste water, the collection and the controlled discharge of it. Both areas have lighting system and access holes through which to heat the indoor air in cold weather. 4. Minimum requirements for materials used in the manufacture of the tent: 4.1. Inflatable Structure of resistance Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Total thickness; (DIN EN ISO 2286-2); 0.70±0.02; mm. Weave Style; (DIN ISO 9354); L1/1. Total Weight; (DIN EN ISO 2286-2); min. 800; g/m². Tensile Strength; (DIN EN ISO 1421); warp; min. 2800; N/5cm. weft; min. 2500. Tear Resistance; (DIN 53363); warp; min. 250; N. weft; min. 200. Adhesion; (ISO 6133:2015); min. 100; N/5cm. (ISO 2411). Remarks; Color grey. 4.2. External tent Properties; Test Standards; Values; Units. (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); min 1100; dtex. Weave Style; (DIN ISO 9354); L1/1. Yarn Count; (DIN EN 1049); warp; 8; yarn/cm. weft; 8; yarn/cm. Base Coat; PVC both sides, fungicide. Top Coat; acrylic lacquer. (DIN EN ISO 2286-2); min 630; g/m². Tensile Strength; (DIN EN ISO 1421); warp; min. 2800; N/5cm. weft; min. 2500; N/5cm. Tear Resistance; (DIN 53363); warp; min. 300; N. weft; min. 250; N. Remarks; color white. 4.3. Inner tent Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); 280; dtex. Weave Style; (DIN ISO 9354); L1/1 Oxford. Yarn Count; (DIN EN 1049); warp; 20; Fd/cm. weft; 17; Fd/cm. Base Coat; PU. Total Mass per Unit Area; (DIN EN ISO 2286-2); 150; g/m². Width; (DIN EN ISO 2286-1); 155; cm. Tensile Strength; ISO 13934-2; warp; 940; N/5cm. C.R.E Grab method; weft; 750. Tear Resistance; (ISO 4674-2); warp; 21; N. Pendulum method; weft; 17. Water Pressure Resistance; EN 20811:1992; >1000; mm WS. Remarks; Color white. Fluor – Carbon treated. 4.4. PVC insulation floor Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); min. 1100; dtex. Weave Style; (DIN ISO 9354); L1/1. Yarn Count; (DIN EN 1049); warp; 8; yarn/cm. weft; 8. Base Coat; PVC with embossing on one side. Total Mass per Unit Area; (DIN EN ISO 2286-2); min.850; g/m². Width; (DIN EN ISO 2286-1); min.205; cm. Tensile Strength; (DIN EN ISO 1421); warp; min.2900; N/5cm. weft; min.2600; N/5cm. Tear Resistance; (DIN 53363); warp; min.300; N. weft; min.250; N. Adhesion; (EN ISO 2411); warp; min. 80; N/5cm. weft; min. 80; N/5cm. Temperatures extremes; -30; °C. +70. Impermeability; (DIN EN 20811); > 1000; mm. Remarks; Color gray. anti-slipping. 4.5. PVC Protection tarpaulin under the tent Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); min. 1100; dtex. Weave Style; (DIN ISO 9354); Plain weave 1/1. Base Coat; PVC. Top Coat; Acrylic lacquer. Total Mass per Unit Area; (DIN EN ISO 2286-2); min. 610; g/m². Tensile Strength; (DIN EN ISO 1421); warp; min. 3000; N/5cm. weft; min. 3000. Tear Resistance; (DIN 53363); warp; min. 300; N. weft; min. 300. Adhesion; (EN ISO 2411); min. 100; N/5cm. Resistance to damage by flexing; (DIN 53359 A); 100.000 x. Temperatures extremes; -30; °C. +70. Color Fastness; (DIN EN ISO 877); >7. Remarks; color grey matt. fungicide finish. 4.6. Floor tiles for the shower area - the floor must cover the entire surface of the shower area - the surface must have anti-slip properties - must be easy to relocate and resume - mass: max 4 kg/ m² - load capacity: min. 1.000 kg/ m² - temperature range: at least - 20ºC, + 70° C B. ACCESSORIES FOR EACH TENT: 1. Lighting lamp - 4 pcs a) The lighting fixture is made with: • Unbreakable transparent polycarbonate tube, closed at the ends with cap and handle in rubbery thermoplastic material, with bumper edges; • Voltage disconnection system in case of removal of the handle or cap; • Switch for switching on and off inside the handle; • Two hooking systems to allow suspension and fixing to the tent structure. b) The lamp: • Fluorescent type – min. 36 W, white light • powered by an electronic anti-interference converter, operating at 230V / 50Hz. • Operation possible even with variations between -5% and + 15% of the rated voltage • operational in a temperature range, at least between - 15 ° C and + 40 ° C • possibility to connect one after the other by means of an entry / exit predisposition • plug and socket protection minimum IP67 • 220/230 V power supply 2. Electric pump - 1 pc - 220/230 V power supply - Working pressure: max 400 mbar, - Weight - maximum 5 kg - Electrical cable - The hose must assure simultaneous swelling / deflation of the inflatable arcs. 3. Water circuit for showers The supplier’s offer must contain a technical solution for the shower system which must provide the following: - Hot water flow for 1 shower: 0.10 liter/sec at a nominal pressure of 0.3 bar (according to MDRAP order no. 818/2015 annex 2 and https://ec.europa.eu/environment/gpp/pdf/criteria/sanitary/RO.pdf) - The 4 showers must operate simultaneously - The water used for bathing is taken from a tank, with or without pressure, located near the tent, outside; - After use, the used water should be taken from the shower area and drained outwards in a container (tank) with a capacity of at least 1 mc - the entire shower circuit must be thought and delivered as a complete system for each tent, from the cold water supply tank to the waste water tank and it will contain at least the following accessories: - hose connection from the water supply tank - instant water heater (the description and the requirements are listed below, in this table) - the connection hose for transporting the hot water from the instant heater to the showers) - shower – 4 pieces - evacuation system (absorption and transport) of the used water, from the shower tent to the waste water tank - waste water tank – at least 1 mc volume, made of plastic with metallic reinforcements or made of PVC material, pliable – for gaining space in the container |  |  |  |
| 3 | Instant water heater - 2 pcs - Hot water flow: minimum 4 l / min - Hot water at the outlet: minimum 30 ° C – maximum 70 ° C - Temperature indicator for the output of hot water - Protection against scalding - Fuel: diesel - Burning motor power: minimum 90 kW power - Diesel tank capacity: minimum 20 liters - Weight: if the weight is above 50 kg, the device must be provided with wheels |  |  |  |
| 4 | Tent for Communication - 1 piece Dimensions: - Width: minimum 5.50 m - Length: minimum 7.50 m - Shelf height: minimum 2.20 m - Height at the tail: max. 2.80 m - Floor area: minimum 40 sqm - 2 identical inputs (minimum / maximum) width: 140/160 cm; height 180/200 cm 2. Standard Features / Features - 2 entrances with swing doors at the top, one on the front and rear - possibility to connect to the two entrances with other identical tents - inflatable resistance structure - Folding system with aluminum struts for interior stiffening. - windows with insect screen and shutters located on each side: 6 pieces - Indoor tent for thermal insulation and weatherproofing - PVC protection floor tarpaulin under the tent, - PVC insulation floor , solid welded with the tent - Access air conditioning tubes, two on each side and two electric cable access tubes on the front and rear. - Electric pump for inflation and deflation of inflatable resistance structure - Anchor system (anchor ropes, anchor pins, hammer ...) 4. Minimum requirements for materials used in the manufacture of the tent: 4.1. Structure of resistance Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Total thickness; (DIN EN ISO 2286-2); 0.70±0.02; mm. Weave Style; (DIN ISO 9354); L1/1. Total Weight; (DIN EN ISO 2286-2); min. 800; g/m². Tensile Strength; (DIN EN ISO 1421); warp; min. 2800; N/5cm. weft; min. 2500. Tear Resistance; (DIN 53363); warp; min. 250; N. weft; min. 200. Adhesion; (ISO 6133:2015); min. 100; N/5cm. (ISO 2411). Remarks; Color grey, airtight. 4.2. External tent Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); 1100; dtex. Weave Style; (DIN ISO 9354); L1/1. Yarn Count; (DIN EN 1049); warp; 8; yarn/cm. weft; 8; yarn/cm. Base Coat; PVC both sides, fungicide. Top Coat; acrylic lacquer. Total Mass per Unit Area; (DIN EN ISO 2286-2); 630; g/m². Tensile Strength; (DIN EN ISO 1421); warp; 2800; N/5cm. weft; 2500. Tear Resistance; (DIN 53363); warp; 300; N. weft; 250. Remarks; B1/ M2 acc. to DIN 4102 color white; Flame retardant. 4.3. Inner tent Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); 280; dtex. Weave Style; (DIN ISO 9354); L1/1 Oxford. Yarn Count; (DIN EN 1049); warp; 20; Fd/cm. weft; 17; Fd/cm. Base Coat; PU. Total Mass per Unit Area; (DIN EN ISO 2286-2); 150; g/m². Width; (DIN EN ISO 2286-1); 155; cm. Tensile Strength; ISO 13934-2; warp; 940; N/5cm. C.R.E Grab method; weft; 750. Tear Resistance; (ISO 4674-2); warp; 21; N. Pendulum method; weft; 17. Water Pressure Resistance; EN 20811:1992; >1000; mm WS. Remarks; Color white. Fluor – Carbon treated. Flame retardant. 4.4. PVC insulation floor Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); 1100; dtex. Weave Style; (DIN ISO 9354); L1/1. Yarn Count; (DIN EN 1049); warp; 8; yarn/cm. weft; 8. Base Coat; PVC with embossing on one side. Total Mass per Unit Area; (DIN EN ISO 2286-2); 850; g/m². Width; (DIN EN ISO 2286-1); 205; cm. Tensile Strength; (DIN EN ISO 1421); warp; 2900; N/5cm. weft; 2600. Tear Resistance; (DIN 53363); warp; 300; N. weft; 250. Adhesion; (DEN ISO 2411); warp; N/5cm. weft. Temperatures extremes; -30; °C. +70. Impermeability; (DIN EN 20811); > 1000; mm. Remarks; Color gray. anti-slipping. Flame retardant. 4.5. PVC Protection tarpaulin under the tent Properties; Test Standards; Values; Units. Fiber Type of the Base Fabric; (DIN ISO 2076); PES. Titer of Yarn; (DIN EN ISO 2060); 1100; dtex. Weave Style; (DIN ISO 9354); Plain weave 1/1. Base Coat; PVC. Top Coat; Acrylic lacquer. Total Mass per Unit Area; (DIN EN ISO 2286-2); 610; g/m². Tensile Strength; (DIN EN ISO 1421); warp; 3000; N/5cm. weft; 3000. Tear Resistance; (DIN 53363); warp; 300; N. weft; 300. Adhesion; (EN ISO 2411); 100; N/5cm. Resistance to damage by flexing; (DIN 53359 A); 100.000 x. Temperatures extremes; -30; °C. +70. Color Fastness; (DIN EN ISO 877); >7. Remarks; Standard: grey. matt, fungicide finish, free of cadmium. B. ACCESSORIES: 1. Lighting lamp - 6 pcs The lighting fixture is made with: • Unbreakable transparent polycarbonate tube, closed at the ends with cap and handle in rubbery thermoplastic material, with bumper edges; • Voltage disconnection system in case of removal of the handle or cap; • Switch for switching on and off inside the handle; • Two hooking systems to allow suspension and fixing to the tent structure. The lamp that equips the lighting fixture is of the 36W fluorescent type, with white light; this is powered by an electronic anti-interference converter, operating at 230V / 50Hz. Operation is also guaranteed with variations between -5% and + 15% of the rated voltage and in a temperature range between - 15 ° C and + 40 ° C. The lamp is powered at high frequency (not less than 10 kHz) in order to eliminate stroboscopic effects. The lamp can be connected one after the other by means of an entry / exit predisposition realized with 1.5 m of H07RN-F type flexible electric cable section. 2 x 2.5 mm2 both in input and output, equipped with a flying plug input and a straight socket outlet. The plug and socket are of the standard CEE P17 230V 2P + T with a protection rating of IP67. The electric cable is fixed to the lighting fixture by means of a cable clamp and anti-tear / anti-torsion device, incorporated in the handle and in the cap with IP67 protection degree. 2. Electric pump - 1 pc - 220/230 V power supply - Working pressure: max 400 mbar, - Weight - maximum 5 kg - Electrical cable with a length of at least 2 m - Hose for inflation/deflation of the tent |  |  |  |
| 5 | Technical Tent - 1 piece Military Modular tent The tent is designed for acomodation, sheltering,material storage, dining, medical. Minimal requerements: - modular capacity - can be erected and used with or without walls. Minimum configuration Components; UNIT (pcs). Basic module (external tent); 1. Acces door; 1. Windows; 6. Acces module for the entrance; 1. Thermal protection inner tent; 1. Aluminium resistence structure; 1. PVC isolation floor; 1. Tubular climating passages; 4. Anchoring system; 1. Transport bag external tent; 1. Transport bag internal tent; 1. Transport bag PVC isolation floor; 1. Transport bag aluminum resistance structure; 2. Transport bag anchoring system and accesories (repair kit + hammer); 1. Technical construction features The tent wil have the following constructive dimensions: - Lenght without acces module minim 5,00 m - Lenght with module acces minim 6,00 m - Width minim 5,00 m - Minimal utile surface minim 25 m˛ - Wall hight minim 1,90 m - Top hight maxim 3,3 m - Entrance dimensions minim 1 x 2 m - Module acces dimensions (minimum values): entrance hight 1,75 m, exit hight 2 m, width 1 m, depth 1 m - Basic modul requierements - made by mechanically and water resistant fabric 50%/50% Cotton/Polyester - the inner tent is a 100% cotton - the floor is idependentlly designed, allowing the lateral walls, which are vertical, to be rolled up - vestibule entrance ont he front - the walls must terminate down part with PVC ground skirt. - Windows: minimum 60 x 40 cm with external blind, PVC tranparent foil External tent fabric properties; Test standards; Values; Units. Fiber type on the basic fabric; SR ISO 1833+A1*; Warp Weft; 50%/50% BBC/PES 50%/50% BBC/PES. Titer of yarn; SR EN ISO 2060:1997; Warp Weft; 50 +/-5% 50 +/-5%; Tex x2 Tex x2. Weave style; SR 6431; L 1/1. Weight; SR EN 12127:2003; 430 +/-5%; g/m2. Tensile strenght; SR EN 13934-1; Warp Weft; Min. 2200 Min. 1700; N/5cm. Tear resistance; SR EN 13934-1; Warp Weft; 75 +/-5% 60 +/-5%; N. Water column resistance; SR EN 20811:1996; > 500; mm H2O. Spreadind resistance (Spray test); SR EN 24920:1996; Note; Min. 80. Dimensional modifications; DIN 53892/2; Warp Weft; Max. ± 1,5; %. - Inner thermal protection tent requerements - the inner tent is done by a 100 % cotton fabric having the following features - the inner tent windows are provided with mosquito net Inner tent fabric properties; Test standards; Values; Units. Weight; SR EN 12127:2003; Warp Weft; 210 ± 5%; g/m˛. Fiber type on the basic fabric; SR ISO 1833+A1*; Warp Weft; 100 % Cotton. Titer of yarn; SR EN ISO 2060:1997; Warp Weft; 36 ± 5% 42 ± 5%; Yarn/cm Yarn/cm. Weave style; SR 6431; L 1/1. Tensile strenght; SR EN 13934-1; Warp Weft; Min. 600 Min. 600; N/5 cm. Tear resistance; SR EN 13934-1; Warp Weft; Min. 12 Min. 10; N. Water column resistance; SR EN 20811:1996; > 250; mm H2O. Dimensional modifications; DIN 53892/2; Warp Weft; Max. ± 5 Max. ± 3; %. - PVC isolation floor and the lateral protection of the external tent features PVC insulated fabric for floor and lateral protection; Test standards; Value; Units. Weight; SR EN ISO 2286-2:2000; 610 ± 5%; g/m˛. Width; SR EN ISO 2286-1:2000; Min. 230; cm. Fiber type on the basic fabric; SR EN ISO 2076:2014; 100 % PES. Main coating; PVC. Upper finish; Acrylic laqued. Tensile strenght; SR EN ISO 1421:2002; Warp Weft; Min. 2700 Min. 2700; N N. Tear resistance; DIN 53363; Warp Weft; Min. 280 Min. 280; N N. Adhesion; SR EN ISO 2411:2002; Min.100; N/5 cm. - Aluminium resistance structure - aluminium structure - done by longitudinal welded pipe material. - each pipe must be under 2m in order to allow the transport into the container. - Min. 8 pcs. pipe AL φ 42 x 2780 mm (two bars jointed of 1338 mm) for the roof; - Min. 8 pcs. pipe AL φ 40 x 1905 mm with fixing ground sole for the pillars; - Min. 15 pcs. pipe AL φ 40 x 1660 mmfor the horisontal joint of the roof. - Min. 2 pcs conectors AL with 3 arms; - Min. 2 pcs conectors AL with 4 arms; - Min. 4 pcs steel conectors with 3 arms galvanised. - Min. 4 pcs steel conectors with 4 arms galvanised - Min. 4 pcs double conector. - Min. 4 pcs simple conector; - Min. 8 pcs ground sole for vertical pillars: Entrance vestibul resistance structure - Telescopic pasivised steel pillar with top and round base – 2 pcs - Telescopic pasivised steel bar – 1 pcs - Packing requirements The tent is packed as follows: -External tent transport bag - 1 pcs. Weight - maxim 65 kg - Inner tent transport bag - 1 pcs Weight - maxim 30 kg -PVC isolation floor transport bag - 1 pcs Weigt - maxim 25 kg -Aluminium structure transport bag - 2 pcs Weight / bag: maxim 50 kg -Anchoring sytem and accesories bag - 1 pcs Weight: maxim 30 kg |  |  |  |
| 6 | Portable air conditioning - 1 pcs BTU min 14.000 - max. 18,000 - Cooling / heating capacity min. 4000 W - max. 5000 W - Power absorbed Cooling / Heating min 1400 W - max. 3000 W - Absorbed current (inside 27 ° C, 65%, outside 32 ° C) max 16 A - Cooling air volume in three stages min 450 m³ / h - Heating air volume min 600 m³ / h - Noise level max 70 dB (A) - Weight: max 50 Kg |  |  |  |
| 7 | Diesel air heater - 3 pcs - Fuel: diesel - Heating capacity: minimum 20 kW - Heating air volume min: minimum 1450 m³/h - Power supply: minimum 220-230 V, 50 Hz - Tank volume: minimum 30 liters - Equipped with evacuation kit |  |  |  |
| 8 | Set of table with 6 chairs -2 sets Mass dimensions: 180 x 75 x 75 ± 5 cm Folding table dimensions: 90 x 75 x 8 ± 5 cm Seat dimensions: 45 x 50 x 88 ± 5 cm Package: 1 table and 6 chairs Package weight: 40 kg ± 5% Descriptors: Plastic upper, anticorrosive coated steel frame with powder coating, plastic foot protector. Static mass loading capacity: minimum 200 kg Static static load capacity: minimum 150 kg |  |  |  |
| 9 | Tripod set with two projectors - 4 sets - TRIPOD CONSTRUCTIVE CHARACTERISTICS Height adjustable stand: close to 700 mm In extended position: minimum 1700 mm Weight: maximum 4 kg (without projectors) Material type: metallic, corrosion protected Usage direction: inside and outside. Other Features: Possibility to hold at least two LED projectors - PROJECT CHARACTERISTICS type: LED lights Power (W): minim 2 pcs X 50 W each Power supply 220V - 240V AC Frequency 50 Hz Luminous flux (Lm): minim 3500 Lm Cable length minimum 5m Degree of protection IP65 Lifetime: minim 20000 hours Angle pitch 160˚ Power factor> 0.97 External use destination Optional manual switch |  |  |  |
| 10 | Cable drum - 5 pcs Socket number 2-4 pcs, self-covering Power supply 220V - 240V AC Cable section (mm) 3 x 1.5 mm² - 2 pcs Cable section (mm) 3 x 2.5 mm² - 2 pcs Cable section (mm) 3 x 2.5 mm², 400 V - 1 pcs Minimum cable length of 30 m Power (W) min. 2000 W/ 3000 W / 4000 W Amperage (A) min. 10 A Protection class minimum IP 44 External use destination |  |  |  |
| 11 | Boat - 1 pcs The boat is designed for the fast and safe movement of intervention personnel to remove victims (people or animals) fallen into frozen waters. TECHNICAL DATA 1. Dimensions: - Width: minimum 1.00 m - maximum 1.40 m - Length: minimum 4.00 m - maximum 5.00 m - Minimum floor length of 2.20 m - Minimum load (buoyancy) (kg): minimum 900 kg - Maximum weight of 30 kg 2. Accessories - 2 paddles/divisible - filling system kit (filling hose, adapters, pressure reducer) – compatible with compressed air cylinder - repair kit CONSTRUCTION INFORMATION The buoyancy of the boat is created by the tube, inflated around the perimeter of the flat bottom. For more safety there must be several independent air chambers. The openings, created between the inflatable tubes and the bottom, on the front and back of the boat, allow the rescuer to easily remove a person from the water directly on the bottom of the boat. The boat has handles or a strong rope placed around the entire perimeter of the boat for easy transport. The inflatable tube is provided with reinforcements on the sliding side for increased resistance to ice travel. |  |  |  |
| 12 | Inflatable walkway - 2 sets The inflatable walkway is intended to save people or animals caught or left captive in swampy, sludge, alluvial areas where access to the lifeboat is not possible or is carried out with difficulty. It must allow people to stand up without bending or sinking into mud or water. 1. Dimensions: A complete consists of 3 segments, each having the following dimensions: - Width: minimum 1.30 m - maximum 1.50 m - Length: minimum 3.00 m - maximum 6.00 m - The cumulative length of the track in 3 segments at least 15 m - Minimum load (buoyancy) (kg): minimum 250 kg / segment - Maximum weight of 25 kg / segment 2. Standard features - Structure of corrosion-resistant strength and climatic factors - waterproof material - The segments are provided with elements that allow them to be linked together on either side |  |  |  |
| 13 | Rescue accessories 1. Lifeline - 4 pcs 1.1. length (m): minimum 20 m 1.2. diameter (mm): minimum 10 mm 1.3. material: polyamide or other materials with similar properties 1.4. weight: maximum 69 g / m 1.5. static resistance: minimum 28 KN 1.6. maximum elongation 3.1% 2. safety carabiner - 4 pcs 2.1. made of aluminum alloy; 2.2. maximum weight 80 g; 2.3. Automatic closing system; 2.4. high axis resistance: minimum 25 kN; 2.5. resistance with open flap: minimum 8kN; 2.6. low axis resistance: minimum 8 kN; 2.7. opening flap: minimum 23 mm; 2.8. must comply with the requirements of SR EN 12275: 2013 - Climbing and climbing equipment. Connecting parts. Safety requirements and test methods and SR EN 362: 2005 - Personal protective equipment against falls from a height. Connecting parts 3. Backpack for string - type pretension - 4 pcs |  |  |  |
| 14 | Power generator - 3 pcs As the technical specifications in the annexes are considered as minimum requirements, the sizing of the generator / power generators shall be designed in such a way as to ensure the simultaneous operation of the electric consumers provided for the operation of the base (air conditioners, interior tents lighting and perimeter lighting, instant for shower water heating if the case etc). Minimum requirements for each Power generator: Voltage: 230V / 400V Frecvency: 50 Hz Electric start Sockets: at least 2 sockets x 230V 16A |  |  |  |
|  | For visibility actions, please refer to art 9 point 9.9 from Special Conditions. All products will be delivered at Contracting Authority headquarter in Resita, will be put in place and a proper function test will be performed, inluding showers, heating water, sewage. A main focus will be on electric generators, so all electrical equipments must be functional in the same time. |  |  |  |