# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Motor vehicles	p 1 /…

Publication reference : RORS 283/9 eMS

LOT 1-Truck 4x4

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Any mentions regarding a trade mark, brand, model is only for a proper and issyier identification of product requirements, and should be understand as followed by phrase “or equivalent”.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Truck 4X4 - WITH HYDRAULIC SHUTTER (HYDRAULIC LIFT) #### GENERAL CONDITIONS - DESTINATION 1.1. The truck will be used to transport specific equipment and materials, urban and interurban roads on and off roads, outside of them (all terrain), for the crossing of doves with a depth of minimum 0,6 m, in specific weather conditions of all seasons. - APPROVAL - The vehicle must be approved by R.A.R. - R.A. for public road traffic. - All the equipment fitted to the vehicle must be approved in accordance with the European standards in force at the date of delivery, and have all the necessary authorizations to put them into service. |  |  |  |
|  | - Vehicle category: N2G or N3G. - Body type: BA truck with tarpaulin. - Vehicle must hold EC certificate of conformity according to EC Directive 46/2007 (minimum for truck). - ENVIRONMENTAL IMPACT - Pollution standard: according to the regulations in force in the EU at the time of delivery. #### TECHNICAL CONDITIONS - CONSTRUCTIVE CHARACTERISTICS 1. DIMENSIONAL CHARACTERISTICS OF MASS AND VOLUME 1.1. Length: min. 7000 mm, max. 8500 mm. - Height (unloaded, tarpaulin): max 4000 mm. 1.3. Wheelbase: min. 3500 mm, max. 4300 mm. 1.4. Ground clearance: minimum 300 mm. 1.5. Payload: minimum 5000 kg. 1.6. The height of the interior of the cargo compartment (from floor to crag) at least 2000 mm. |  |  |  |
|  | 2. MOTOR AND APPENDIX INSTALLATIONS 2.1. Diesel fuel type. 2.2. Max. Power (DIN): minimum 200 kW. 2.3. Fuel tank of at least 200 liters. 2.4. Ad Blue Tank - min. 30 liters. - Cold start system (which ensures intake air heating). 2.6. Fuel filter (with resistance). 2.7. Intake air intake at height (minimum at the steering wheel). 3. TRANSMISSION 3.1. Traction formula: 4x4 permanent or with the following modes of operation: 1.3.1.1. Rear traction mode. 1.3.1.2. Full drive mode by coupling the front axle. 3.2. Gearbox: automatic or manual, min. 6 + 1 (walk back) steps. 3.3. Transfer box with two-tiered gear ratio: Normal range 1/1 and reduced range for rough terrain access, longitudinally differential lockable with command at the driving position. |  |  |  |
|  | 3.4. Differential rear axle locked with command at the driving position. 3.5. Differential front deck differential with command at the driving position 3.6. Rear axle with single wheels. 4. SUSPENSION 4.1. Front axle suspension: mechanical with parabolic springs and stabilizer bar. 4.2. Rear axle suspension: mechanical with parabolic springs and stabilizer bar. 5. WHEEL 5.1. Number of wheels - 5 pieces, of which a spare. 5.2. Minimum wheel diameter 1000 mm. 5.3. Tires M + S, mounted on steel wheels (including spare wheel). 5.4. Reserve wheel of the same size type with those mounted on the vehicle. 5.5. A spare wheel mounted without reducing the passing capacity, with a device to help raise the spare wheel to the fastening position. |  |  |  |
|  | 6. DIRECTION SYSTEM 6.1. Servo. 7. BRAKE SYSTEM 7.1. Servo. 7.2. The wheel brake, for both bridges, will be drum and shuttle. 7.3. Anti-lock braking system of wheels (ABS or equivalent). 7.4. Additional deceleration braking system (retarder or similar). 8. ELECTRICAL INSTALLATION. 8.1. Electrical system voltage 24 V. 8.2. Two non-maintenance batteries. 8.3. A general switch that interrupts all vehicle users. 9. LIGHTING-SIGNALING INSTALLATION 9.1. Daytime running lights. 9.2. Fog lights with halogen bulbs. Fog light bulbs with halogen bulbs. |  |  |  |
|  | 10. CABIN 10.1. Metal type, closed, 1 + 2 seats, with suspension and anti-corrosion protection. 10.2. Suspension and adjustable driver's seat in at least two directions, fitted with a three-point seat belt and head restraint. 10.3. Passenger seats with three-point seat belt and head restraint. 10.4. Interior sun visor. 10.5. Rear view mirror system with deflection and electric drive. 10.6. Cabin air conditioning (with heating and air conditioning). 10.7Audio system provided with radio with CD / USB + MP3 10.8. Cab color: red (no sticker allowed) - the hue will be set at the end of the contract. |  |  |  |
|  | 11. CAR-BODY (SUPERSTRUCTURE) 11.1. Type: fixed platform with shutters and tarpaulin. 11.2 Non-slip, scratch-resistant metal floor with grooves and tie downs points for anchoring goods. 11.3. Metal shutters, folding on the sides with a height of at least 900 mm. 11.4. Heavy-duty waterproof tarpaulin that closes over the rear bell, with two windows (skylights) on each side. 11.5. Metalic shed, removable type, for self-suporting of tarpaulin. 11.6. The color of the body: RED. The hue will be established at the signing of the contract. If the shutters are made of aluminum, the color of the metal will remain. |  |  |  |
|  | 12. SPECIAL SUPPLEMENTARY SUPPLY 12.1. Under-run bar with the possibility of being folded into rough terrain. 12.2. Towing hook/ Towing coupling corresponding to the maximum towing capacity of the truck. 12.3. Two 13-pin universal socket (and two 7-pin cable adapter) for connecting the trailer electrical system for both 24V and 12V. 12.4. Coupling devices for the pneumatic braking system of the trailer. 12.5. Optic signaling device (stroboscopic sign) with the following features: 12.5.1. A led light bar, red colour, placed on the cabin roof 12.5.2. The calotte of the device will be provided with protection (cage type) against mechanical action. 12.6. One set (minimum 2 pieces) of flashers led strobe lights, red colour, mounted in the front grille |  |  |  |
|  | 12.7. Sound signaling system with the following features: 12.7.1. A loudspeaker with minimum 100 W. In case it will be placed outside the roof, it will be provided with cage type protection against mechanical actions . 12.7.2. Operable from the passenger compartment. 12.7.3. Minimum 3 siren tones. 12.7.4. Provided with a public address system and a "push to talk" microphone that cancels all other siren functions. 12.8. Hydraulic winch - 1 piece (mounted at the front of the truck) with the following features: 12.8.1. driven by a socket power coupled to the transmission system of vehicle. 12.8.2. traction force of at least 10,000 daN. 12.8.3. Minimum cable length of 20 m. |  |  |  |
|  | 12.9. Requirements of firefighters specific inscriptions and logo of the financing programme: 12.9.1. The "POMPIERII" inscriptions will be done with the "Arial Narrow" font at the maximum sizes offered by the layout space. The inscription "POMPIERII" placed on the front of the vehicle will be written in the mirror; 12.9.2. Unique emergency number "112" 12.9.3. The logo of Caraș-Severin County Inspectorate of Emergency Situations, the type of fonts, their size and other marking elements of the financing programme will be set at the conclusion of the contract; 12.9.4. In order to comply the Visual Identity Manual requirements, an electronic template of the inscriptions will be submmit to Contracting Authority at least 20 days before the product is delivered; |  |  |  |
|  | 12.10. Work light lamp with the following characteristics: 12.10.1. Powered by the electrical installation of the truck; 12.10.2. Power: minimum 100 watts if a filament lamp is used or equivalent for other types of light sources; 12.10.3. Built in the front right side of the cab, detachable, with 2-way: up-down or left-right beam orientation; 12.10.4. Coupling to the truck's electrical system will be done with a moisture-proof plug in the cab near the projector via the projector's cable. 12.10.5. At the rear of the superstructure, a bracket will be provided to position the projector so that it illuminates the rear loading / unloading process. 12.11. Communication Module The module of communications for special vehicles is intended to provide the communication channels of these means for the management of specific missions and is composed of: |  |  |  |
|  | ###### TETRA mobile terminal with accessories: 1. Standardization and interoperability a. The offered TETRA terminal must comply with the applicable TETRA standards, of which we do not exhaustively list: a. 300 392 TETRA V+D air interface; b. 300 394 TETRA V+D conformance testing; c. 300 395 TETRA speech codec; d. 300 396 TETRA OMO. b. The terminal offered must be certified by TCCA / ISCOM for the Motorola TETRA infrastructure (DIMETRA IP 6.1 or later). The tenderer must provide the relevant pages of the interoperability certificate. c. The terminal offered must be certified by TCCA / ISCOM for the TETRA Airbus infrastructure (Airbus NTS ver 5.5 or later). The tenderer must provide the relevant pages of the interoperability certificate. d. The offered terminal must be certified according to the procedure published on the S.T.S. website, URL: www.stsnet.ro/pdf/Procedura_TETRA_ro_v3.pdf. For the offered terminal model, the certificate issued by S.T.S. |  |  |  |
|  | 2. Environment The offered TETRA terminal must have the following environmental characteristics: - Operating temperature: -20…..+55 Celsius degrees; - Storage temperature: -30…..+60 Celsius degrees; - Dust and water protection: minim IP54. 3. RF (Radio Frequency) The offered TETRA mobile terminals must have the following RF characteristics: - Banda RF (TMO&DMO): 380-400 MHz; - Duplex gap: 10 MHz; - Adjacent channel ecart: 25kHz; - Static sensitivity: 112 dBm; - Dynamic sensitivity: 103dBM; - Receiver Class A - Power Tx : min 10W. 4. TETRA services and functionalities The offered TETRA terminal must support the following TETRA services and functionalities 4.1. Voice services a. Group call (GC) in TMO & DMO (minimum 500 pre-programmed TMO groups, minimum 10 pre-programmed DMO groups);b. Emergency call; c. Individual call (IC); d. Full duplex telephone call (PABX / PSTN) |  |  |  |
|  | 4.2. Other services / functionalities a. TPI; b. CLIP; c. DGNA; d. Late entry; e. Scanning groups; f. Tx inhibit; g. The offered TETRA terminal must be able to operate in conjunction with a repeater-gateway equipment; thus, the terminal must have the capacity as at the reception of the transmitted signal of an equipment with the TMO-DMO repeater / gateway function to register in the working mode repeater (or TMO-DMO gateway), in which the group communications are carried out only through the terminal with repeater / gateway function and only within the radius of radio coverage provided of this. If the use of the terminal as a repeater / gateway requires a license, it must be an integral part of the offer (it is included in the price of the terminal). h. The offered TETRA terminal must be able to function as a repeater-gateway. |  |  |  |
|  | In case the activation of the repeater / gateway functionality requires a license, it must be an integral part of the offer (it is included in the terminal price). i. Integrated WAP browser. Note: if the activation of the WAP browser, repeater / gateway functionality requires a license, it must be an integral part of the offer (it is included in the terminal price). 4.3. Data services a.SDS / SDS-TL; b.Message Status; c.IP PDS, both single slot and multislot packet data (licenses included in the terminal price). 4.4. Mobility a.The TETRA terminal offered must initially support cell selection, cell reselection: undeclared cell reselection, unannounced cell reselection, announced cell reselection type 3 or higher); b.The TETRA terminal offered must support multiple networks (MNCs) based on a list programmed in the terminal. |  |  |  |
|  | 4.5. Security a. Authentication. The authentication keys must be fumigated in a format according to TCCA recommendations. The authentication keys must be provided in compliance with the security rules established by the TCCA SFPG recommendations. b. Encryption on the radio interface (supported algorithms: TEA1 and TEA2). Terminals must be fumigated with the TEA2 algorithm. c. Security class: class 1, class 2 and class 3. d. Remote disable / enable. 5. User Interface (MMI) 5.1. display a. Resolution: minimum 240x320 pixels; b. Minimum 65k colors; c. Diagonal: minimum 2.8 inches; d. Indicators displayed on the display for working modes (for example: TMO, OMO, inhibited Tx) |  |  |  |
|  | 5.2. Keyboard a. alphanumeric keypad; b. Menu navigation keys; c. Emergency call button / key; d. button / key for group selection; e. volume button / key. 5.3. Phone book capacity: minimum 400 contacts. 6. Interface a. Interface for connecting audio accessories; b. Multifunctional interface for programming / data transmission / AT commands. 7. The offered TETRA terminal must be equipped with an integrated GPS receiver. The GPS receiver has the following specifications: a. satellites received simultaneously: minimum 10; b. sensitivity: minimum -152 dBm; c. GPS antenna: active, integrated in TETRA antenna. |  |  |  |
|  | 8. License for programming the terminal If the license for programming the terminal is required or the programming applications are licensed at the number of terminals, for each terminal delivered the license required for programming or, as the case may be, the license to increase the number of terminals available in the programming applications. The license must be an integral part of the offer and is included in the price of the terminal. 9. Constructive model For easy installation, the terminal must be in "split" configuration (control unit + radio unit connected by cable). 10. UHF omnidirectional antenna for the car with integrated GPS active antenna installed on the chassis. 10.1 Technical specifications for the UHF antenna a. band: 380 ... 400 MHz b. directivity feature: omnidirectional antenna c. VSWR: <1.5 in the band of interest d. impedance: 50 ohms e. provided with installation kit: fixing, RF cable, connectors, etc. (all the necessary elements for installation will be delivered). |  |  |  |
|  | 10.2 Technical specifications for GPS antenna a. band: 1.5 ... 1.6GHz b. VSWR: <2, 5 in the interest band c. impedance: 50 ohms d. provided with installation kit: fixing, RF cable, connectors, etc. (all the necessary elements for installation will be delivered). e. GPS antenna to be integrated in UHF 11. Considerations related to the installation of mobile terminals The contractor must perform the installation of the delivered TETRA mobile terminals - Before starting the installation, the Contractor must elaborate a generic installation project and perform a representative installation ('sample installation') for each sub-category, agreed between the • Beneficiary and the Contractor, and the solutions used for this installation will be replicated. (with any necessary adaptations) for the rest of the vehicles in the respective sub-category; - The contractor must certify / approve the "sample installation" at the authorized representative of the vehicle manufacturer, so that the Beneficiary will continue to have the warranty of the vehicle. The costs of the certification services will be incorporated into the costs of the installation services. |  |  |  |
|  | - All mobile terminal installations must allow easy access to the radio unit of the TETRA terminal, so that the beneficiary can program the terminal without dismantling it from the vehicle. In this respect, various technical solutions proposed by the Contractor are accepted (for example, the proper positioning of the radio unit or the connection of an extension cable, pinout J to 1, at the PEI port of the terminal, to facilitate access to the radio unit). - All cables, connectors and accessories necessary for complete installation in the vehicle will be provided (including screws for all components that are provided with clamping systems and 24Vcc / 12Vcc adapters for powering the terminal if necessary). 12. Accessories and installation kit for installing TETRA mobile terminals a. Standard microphone with PTT and mounting bracket; b. speaker with connecting cable and mounting bracket; c. Power supply cable provided, relay for disconnecting the TETRA terminal from removing the key from the contact (or equivalent) and corresponding connectors (min. 5m, adapted to practical situations). The supply voltage of the TETRA terminal is 12Vdc; |  |  |  |
|  | d. vehicle mounting bracket for the radio unit; e. mounting bracket in the vehicle for the control unit; f. Interconnecting cable radio unit - cornea unit; g. UHF omnidirectional antenna containing active GPS antenna, At delivery, each TETRA terminal must be accompanied by the user manual in Romanian. The technical documentation provided by the Bidder must include the specifications / brochures of the main elements (antennas, microphone & speaker, etc.) in the set. 13. If licenses are required to meet the requirements of these Technical Specifications, these licenses must form an integral part of the offer and be included in the terminal price. 14. The contractor must deliver equipment belonging to the last generation from the manufacturer's portfolio; the fumigated equipment must be manufactured no more than six months before delivery date. |  |  |  |
|  | 15.The programming of the terminals will be carried out by the Beneficiary. At the request of the Beneficiary, the Contractor must provide technical assistance, free of charge, whenever it is needed, during the entire programming period of the terminals, including in the phases of introducing the authentication / encryption keys, respectively of registering the terminals in the TETRA infrastructure. 16. Terminal programming Parameter programming (DMO / TMO groups, OMO channels, network parameters, contacts, terminal IDs, etc.) in the TETRA terminals must be possible by the Purchaser through a graphical application running on a COTS PC / laptop, provided with a USB port and operating system Windows 7, Windows 8 or Windows 10, rarely need to connect to the Internet. Two (2) programming sets must be fumigated for the entire quantity of TETRA terminals. It must contain all the elements necessary for programming the TETRA terminals delivered. The set must contain at least the following: a. Two USB programming cables for mobile terminals; b. CD with the programming application; |  |  |  |
|  | c. The hardware license / key for the programming application (if applicable). d. Data cable. e. Any other hardware / software element required for programming / operation, except PC / Laptop (this is provided by the Purchaser). 17. Warranty minimum 12 months, preferable 24 months ###### VHF Mobile radio station with accessories Standard ETSI Working frequency range 136-174 MHz Gap between channels 12,5/25 kHz Number of channels Min. 100 programmable by the beneficiary Working mode Simplex and duplex, analog and digital Modulation type Frequency modulation (analog) Digital 4 FSK(voice, data, voice and data) Encryption Scan Based on at least one predefined list (analog, respectively digital) Individual call (digital mode) Group call (digital mode) Display GPS |  |  |  |
|  | Antenna connector impedance 50 ohm Operating temperature range -25 ℃…+55℃ Frequency stability ≤0,6 p.p.m. Supported technologies Conventional(analog) IP Site Connect (Single and Multi Site) Capacity Plus (Single and Multi Site) TRANSMITTER RF Output power Min.25W-programmable Maximum transmitter deviation ±5 kHz (analog) Ratio signal/noise ≥40dB Digital vocoder type AMBE+2™ RECEIVER Sensitivity ≤0,3µV –SINAD 12 dB Selectivity ≥60dB Intermodulation ≥70dB Audio distortions <5% Ratio signal/noise ≥40dB Audio power ≥3 W |  |  |  |
|  | ACCESSORIES INCLUDED Wired microphone and PTT Microphone mounting bracket Unit E/R mounting bracket Control panel mounting bracket Unit E/R interconnect cable with control panel Power cord equipped with protective fuses, connectors and slippers corresponding External speaker with connection cable and mounting bracket Other accessories required for installation Radio antenna 1 auto-magnetic antenna, omnidirectional, GPS/RF VHF combined Interface (with required accessories) and programming software Ensures the connection on the USB port of a PC with windows 7,8 and 10 operating system. It will be delivered one set per 10 equipment but not less then one per contract Operating manual in Romanian language |  |  |  |
|  | 12.12. Hydraulic rear bumper (hydraulic lift) with the following features: 12.12.1. Aluminum integral construction 12.12.2. - 4 cylinder hydraulic equipment: 2 platforms for lifting the platform and 2 for turning it 12.12.3. Platform with automatic planeity 12.12.4 The control box prepared for heavy climatic conditions is mounted on the side 12.12.5 Foot controls to drive the elevator 12.12.6 General switch in the cab for stopping / lowering the lift 12.12.7 Warning lights during lift handling 12.12.8 Castors on the ground 12.12.9 Under-run bar 12.12.10 The platform is equipped with 2 warning flags |  |  |  |
|  | 13. FEATURES 13.1. medical kit approved by R.A.R. 13.2. Powder fire extinguisher, self-supporting type, minimum 2 pieces of minimum 6 kg each; 13.3. Set of bulbs and spare fuses. 13.4. Rubber mats for interior cabin. 13.5. Reflective vest - 1 pc. 13.6. Reflective triangles - 2 pcs. 13.7. Wheel wrench. 13.8. Hydraulic jack to allow a wheel to be replaced with the fully loaded truck 13.9. Air hose and hose with manometer for inflation of vehicle tires. 13.10. Wheel chock set (2 pcs/set) for parking |  |  |  |
|  | DELIVERY CONDITIONS - The maximum accepted delivery time is 6 months from contract signing - At delivery, the equipment must be accompanied by the following: - Operating and maintenance manual - A list of the spare parts and consumables recommended by the manufacturer - CE declaration of conformity - Warranty and quality certificate There will be provided a training proposal for the personnel regarding the equipment exploitation and maintenance WARRANTY Minimum 12 months, preferable 24 months - The warranty period shall commence on the date of provisional acceptance and may recommence in accordance with Article 32.3 of the General Conditions, Manuals of instructions The provider will deliver a full set of instruction manuals for vehicle and special equipments in romanian language for: - Instruction manual for vehicle - Instruction manual for Hydraulic winch pct. 1.12.9. - Instruction manual for Hydraulic rear bumper (hydraulic lift) point. 1.12.14 |  |  |  |
|  | Transport costs The supplier will deliver the product to the headquarters of the CA or will bear the transport costs (fuel, additive, road charges, provisional traffic charges etc.) of the vehicle up to the headquarters of the CA, including travel and accommodation costs for 2 persons representing the CA in order to take the vehicle from the supplier's headquarters. Special system mounting requirements All the specific equipment (including control systems and transmission and reception antennas) will be mounted masked (manufacturer's solution) |  |  |  |

Acronyms

ARL-Automatic Resource Location

ATV – All Terrain Vehicle

AVL-Automatic Vehicle Location

CLIP - Calling Line Identification Presentation

CT- Cerinte Tehnice

DCE- Data Communication Equipment

DMO- Direct Mode Operation

DCE- Data Communication Equipment

DMO- Direct Mode Operation

DGNA-Dynamic Group Number Assignment

DTE-Data Terminal Equipment

EU - European Union

GC-Group Call

GPS -Global Positioning System

IC - Individual Call

IEEE - Institute of Electrical and Electronics Engineers (

IP - Internet Protocol

ISCOM- Istituto Superiore delle Comunicazioni e delleTecnologiedel l'Informazione

LCD TFT - Liquid Crystal Display Thin Film Transistor

LIP- Location Information Protocol

MNC-Mobile Network Code

MCC- Mobile Country Code

MoU - Memorandum of Understanding

PABX - Private Automatic Branche Xchange

PC - Personal Computer

PDS - Packet Data Services

PEI- Peripheral Equipment interface

PTT- Push To Talk

PSTN - Public Switched Telephone Network

RF - Radio Frequency

Rx - Receive)

SFPG – Security and Fraud Prevention Group

SD - Secure Digital

SOS -Short Data Services

SwMI- Switching and Management Infrastructure

TEA-TETRA Encryption Algorithm

TETRA-Terrestrial Trunked Radio

TL- Transport Layer

TMO -Trunk Mode Operation

TPI - Talking Party Identification

Tx -Transmit

URL- Universal Resource Locator

V+D- Voice+ Data

VOX- Voice Operated .

VSWR- Voltage Standing Wave Ratio

WAP - Wireless Application Protocol .