ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: RESEARCH EQUIPMENT AND SUPPLIES

# Publication reference: RORS-337 / WUT-07

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the Tenderer

Column 5 is reserved for the Evaluation Committee

# Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

IMPORTANT NOTICE:

Unless otherwise specified, the requirements in this Technical Specifications are presented as minimum standard, which the offered goods must meet. The offers that will not satisfy completely and correctly the minimum requirements mentioned below will be disqualified. In the technical tender of the tenderer, tenderers may indicate more details about the deliveries, including the brand name, if applicable, with reference to the requirements below. The requirements imposed will be considered as minimal. In this respect, any tender presented, which deviates from the provisions mentioned below, will be considered, but only to the extent that the TECHNICAL PROPOSAL CONTAINS THE CLEAR DESCRIPTION AND NAME OF THE PRODUCTS and implies the assurance of a qualitative level higher than the minimum requirements mentioned below, otherwise, the offer will be declared NON-COMPLIANT. The offer of products with characteristics inferior to the ones provided below attracts the disqualification of the bidder.

When brands names/specifications referring to or describing products of a given brand or origin are used in this technical specifications (even in column 2), they are used in descriptive purposes only and should be interpreted as equivalent. Technical specifications that indicate a particular origin, source, production, special process, trademark, patent, manufacturing license, are mentioned only for easy identification of product type and are not intended to favor or eliminate certain economic operators or certain products. These specifications will be considered as having "or equivalent".

When appropriate, the warrantee for the research equipment must be 3 years, starting with the installation date.

The reception of the services and products, at the address indicated by the purchaser (Advanced Environmental Research Laboratories, Oituz Street 4, Timișoara, 300086), is done by checking the correspondence between the specifications in the invoice and the specifications in the tables below, by the representative of the purchaser in the presence of the supplier's representative. Services (installation, trainings and other services as appropriate) and products that do not meet the requirements of this specification will be redone, respectively replaced with appropriate products at the supplier's expense.

# Category 1. Multimode microplate reader (with accessories, installation, training)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | The equipment must fulfil all of the following technical requirements: |  |  |  |
|  | - The instrument must allow the incorporation of a hybrid optical system, consisting of a monochromator fluorescence optical system, as well as a filter-based fluorescence optical system - which will ensure best optical performance regardless of the application. |  |  |  |
|  | - The monochromator optics and the optical system with filters must have each their own independent light source, detectors and completely separated light paths, to prevent any compromise in their optical performance. |  |  |  |
|  | - The instrument must be equipped with a monochromator for UV-Vis absorbance and full light luminescence |  |  |  |
|  | - The instrument must have monochromator for fluorescence optical system, for both excitation and emission light |  |  |  |
|  | - The equipment must be equipped with top and bottom monochromator fluorescence, monochromator UV-Vis absorbance and full light luminescence. |  |  |  |
|  | - The instrument equipped with the monochromator-based optical system must be upgradable later at any time with the filter-based optics option. |  |  |  |
|  | - The equipment must allow the distribution of reagents (by injection) in any of the detection modes, by adding a dual-injection module |  |  |  |
|  | - The equipment must include a microvolume accessory – which allows 1-2 μL low volume readings, with 16 or 48 sample micro-spots, useful for direct quantification of nucleic acids, as well as for fluorescence and luminescence measurements. |  |  |  |
|  | - The equipment must allow the choice of gas control option in the environment where the samples are read, either for CO2 / O2 or only for CO2 and has to be equipped with a controlling unit of these gases |  |  |  |
|  | - The equipment must have a temperature control up to +45°Celsius, through a precise temperature control system in at least 4-zones, ensuring a uniformity of temperature across the microplate of ± 0.20 Celsius (at 37°C), to limit the "edge effects" in the assays performed with temperature incubation |  |  |  |
|  | - The equipment must have an automatic adjustment of the reading height "z", to adapt automatically to microplates having different heights as well as to different fluid levels in the wells |  |  |  |
|  | - The equipment must have an automatic individual well-by-well absorbance path length correction to automatically calculate 1 cm path length corrected results and compensate for pipetting errors. |  |  |  |
|  | - The equipment must allow the connection to an external microplate stacker. |  |  |  |
|  | - A powerful PC must be included with the unit, consisting of a pre-configured high-performance computer - optimized for use with the imaging system, a 24'' monitor, keyboard and mouse, with the following minimal configuration: |  |  |  |
|  | - Preferably Mini PC, or similar |  |  |  |
|  | - Operating System: MS Windows 10 Pro, 64-Bit, or similar |  |  |  |
|  | - Processor: Intel i9 – or higher |  |  |  |
|  | - Memory: 16 GB DDR, or higher |  |  |  |
|  | - Storage: 512 GB SSD, or higher + 2 TB hard disk drive |  |  |  |
|  | - Input Device: USB keyboard and mouse |  |  |  |
|  | - Power: External power adapter |  |  |  |
|  | - Ports & Connectors: at least (4) USB 3.1, (2) USB 2.0; |  |  |  |
|  | - Serial Port, HMDI, Network |  |  |  |
|  | - MONITOR Minimal Configuration: |  |  |  |
|  | - Display size (diagonal): minimum 24” |  |  |  |
|  | - Native resolution: 1920 x 1080 pixels (minimum) |  |  |  |
|  | - The reader must be equipped with a performant data analysis software which must provide direct control of the equipment, as well as processing and analysis of the data; |  |  |  |
|  | - The software must be able to control the microplate reader equipment and the automated microscope equipment as well, both instruments being connected to the same computer system (described at point 2) |  |  |  |
|  | Accessories that must be also included in the delivery: |  |  |  |
|  | - Analyzer for measurement of dimensions distribution (Size), Zeta Potential and Molecular weight of particles and nanoparticles: |  |  |  |
|  | - Particle Size Distribution range at least: 0.3 nm – 10 μm; |  |  |  |
|  | - Theories "Dynamic Light Scattering"; |  |  |  |
|  | - Proposed / preferred Light source: Green Laser (510 – 560 nm); |  |  |  |
|  | - Proposed / preferred detector type: Photomultipliers Tube Detectors (PMT); |  |  |  |
|  | - Minimal number of detectors: Three (two for size one for Zeta Potential); |  |  |  |
|  | - Temperature range, at least: 0-80⁰C; |  |  |  |
|  | - Size Specifications: |  |  |  |
|  | - Principle: Photon correlation spectroscopy method; |  |  |  |
|  | - Proposed optical configuration: |  |  |  |
|  | - 2 angles of detection: Backscattering, 173° (for low medium or high concentrated samples) and 90° (for low concentrated samples); |  |  |  |
|  | - 2 focalization points for backscattering angle (173°): On the wall or on the center of the cell to cover larger concentration rang; |  |  |  |
|  | - Manual and Automated adjustment of the best angle and focalization point; |  |  |  |
|  | - Accuracy: at least or better 2 ± on NIST traceable 100nm latex; |  |  |  |
|  | - Measurement of the Zeta potential of particles and Nanoparticle (proposed characteristics): |  |  |  |
|  | - Principle of measure: Laser Doppler electrophoresis; |  |  |  |
|  | - Measurement Voltage range at least: -500 mV to 500 mV; |  |  |  |
|  | - Sizes of sample measurement at least: 3.6 nm at 100 μm; |  |  |  |
|  | - Proposed: Green Laser light source (510 – 560 nm); |  |  |  |
|  | - Proposed detector: Photomultipliers Tube Detector (PMT); |  |  |  |
|  | - Angle of measure 13 ⁰; |  |  |  |
|  | - Capability to work with low distance between electrodes, less that 10 mm, to reduce voltage applied and risk of sample damage; |  |  |  |
|  | - Molecular weight proposed specifications: |  |  |  |
|  | - Principle of measurement Molecular weight: Debye plot method (static light scattering) |  |  |  |
|  | - Measured range 1000 Da to 2 x 107 Da; |  |  |  |
|  | - Angle of Measure 90 ⁰. |  |  |  |
|  | - Analyser control Software and interpretation of results: |  |  |  |
|  | - Easy software, Measurement in 3 “clicks” - selection of type of measurement (size, Zeta or molecular weight) - Definition of working conditions parameters (SOP) - Start measurement |  |  |  |
|  | - Ensures that the operating conditions are saved for later use; |  |  |  |
|  | - Include the list of solvent parameters, refractive index, viscosity, dielectric strength constant vs. temperature, etc.; |  |  |  |
|  | - At least two algorithms for size (Cumulant method, for monodispersed sample, and Inverted histogram method for polydisperse sample); These are only examples, any other algorithms that have the same or better functions will be accepted |  |  |  |
|  | - Display the auto-correction function and the distribution of dimensions; |  |  |  |
|  | - Display of frequency shift graph and mobility graph; |  |  |  |
|  | - Database data storage with customized display parameters, that can be saved as default settings |  |  |  |
|  | - Data can be recomputed with different models (Volume, Number based using MIE theory), viscosity values, data treatments (monomodal multimodal, …) |  |  |  |
|  | - Will be delivered with: |  |  |  |
|  | - Cells with lid for measuring dimensions: at least 500 pcs. |  |  |  |
|  | - Cells for Zeta Potential with lid for potential Zeta measurement with electrode coated with carbon: at least 50 pcs. |  |  |  |
|  | - A powerful PC to control the accessory instrument must be included with the unit, consisting of a pre-configured high-performance computer - optimized for use with the imaging system, a 24'' monitor, keyboard and mouse, with the following minimal configuration (given here only as example, not to be limitative, as the tenderer may supply any other configuration with the same or better characteristics): |  |  |  |
|  | - Preferably Mini PC, or similar |  |  |  |
|  | - Operating System: MS Windows 10 Pro, 64-Bit, or similar |  |  |  |
|  | - Processor: Intel i9 – or higher |  |  |  |
|  | - Memory: 16 GB DDR, or higher |  |  |  |
|  | - Storage: 512 GB SSD, or higher + 2 TB hard disk drive |  |  |  |
|  | - Input Device: USB keyboard and mouse |  |  |  |
|  | - Power: External power adapter |  |  |  |
|  | - Ports & Connectors: at least (4) USB 3.1, (2) USB 2.0; |  |  |  |
|  | - Serial Port, HMDI, Network |  |  |  |
|  | - MONITOR Minimal Configuration: |  |  |  |
|  | - Display size (diagonal): minimum 24” |  |  |  |
|  | - Native resolution: 1920 x 1080 pixels (minimum) |  |  |  |
|  | - The offer must include all the necessary power cables adapted for Romania, communication cables, connectors, adaptors, network board, etc. that will assure the connectivity between the instruments and computers (network with cable or Wi-Fi). |  |  |  |
|  | - The offer must include the installation of all the instruments at the location of the end-user: Advanced Environmental Research Laboratories (Laboratoarele de Cercetari Avansate de Mediu), Str. Oituz 4, 300086, Timisoara, ROMÂNIA |  |  |  |
|  | - Technical documentation: The equipment must be accompanied by warranty certificates, model approvals and initial verification bulletins (if applicable), operating and service manuals in the following languages: Romanian and English. |  |  |  |
|  | - The supplier must ensure a professional two periods training sessions for the end-user team, at the final location, for the instruments, accessories and for the software. |  |  |  |
|  | - In the first session the designated persons of the end-user team will be instructed to use the instruments and to perform the required operations, methods, techniques required to obtain professional results after analyzing the biological samples. |  |  |  |
|  | - After 2 weeks, the second session of instruction will take place and the end-user team will be evaluated and certified to use the instruments. |  |  |  |
|  | - For both these session of training, the consumables, reagents, chemicals, materials, cultures, etc. has to be included in the offer. |  |  |  |
|  | - Instruments must be covered by a minimal 3 (three) years warranty, after the installation. |  |  |  |
|  | - Service during the warranty period must be provided by authorized and trained personnel. Service intervention must be done within 48 working hours at the beneficiary's premises. If the appliance can not be repaired at the customer's premises, it will be done at the supplier's premises. The maximum repair time must not exceed 30 days from the time the device was removed from the recipient. |  |  |  |
|  | - Post-guarantee conditions: post-warranty service must be provided on the basis of a post-guarantee service contract that will provide, besides troubleshooting services, transportation, assistance and other services, as appropriate, that will be negotiated by the supplier with the beneficiaries. Upon the beneficiary's firm order, the supplier must undertake to deliver the required consumables both during the warranty period and afterwards. |  |  |  |
|  | - Spare parts and consumables: The supplier must provide spare parts and consumables for a period of between 5 and 10 years. |  |  |  |

# Category 2. Automated complex brightfield inverted microscope (with accessories, installation, training)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | The automatic equipment for visualizing and analyzing living cells, motorized table (x, y, z) must meet all the following requirements: |  |  |  |
|  | - The instrument must perform image acquisition in (1) fluorescence, (2) brightfield, (3) color brightfield as well as (4) phase contrast modes. |  |  |  |
|  | - The microscope must include a turret for the objectives, with 4 to 6 positions. |  |  |  |
|  | - The unit must accept objectives with magnification powers between 1.25x to 60x without oil immersion, and for 60x to 100x with oil immersion |  |  |  |
|  | - The offer for the microscope must include the following Plan Fluorite objectives for fluorescence imaging and bright field imaging: |  |  |  |
|  | - 10x Objective (preferred Olympus Plan Fluorite phase, or better) |  |  |  |
|  | - 20x Objective (preferred Olympus Plan Fluorite phase, or better) |  |  |  |
|  | - 40x Objective (preferred Olympus Plan Fluorite phase, or better) |  |  |  |
|  | - 100x Objective (preferred Olympus Plan Fluorite, or better) |  |  |  |
|  | - The microscope must include phase contrast accessory / kit, that must include a centering telescope (pre-centered slider with rings) and a Zernike phase condenser lens. |  |  |  |
|  | - More than 20 imaging filters cubes must be available, with different colors |  |  |  |
|  | - It must allow simultaneous loading of at least 4 filter cubes for fluorescence imaging. |  |  |  |
|  | - The instrument must include, together with the corresponding LED cubes, for each filter, at least the following filters: |  |  |  |
|  | - filter cube for DAPI |  |  |  |
|  | - filter cube for GFP |  |  |  |
|  | - filter cube for Propidium Iodide (PI) |  |  |  |
|  | - The microscope must accept at least the following additional filter cubes for: GFP, Texas Red, YFP, CY7, CFP, Acridine Orange, CFP-YFP FRET, Chlorophyll A, PE, CY5.5, TagBFP, GFP Ex-CY5 Em; RFP Ex-CY5 Em; CFP FRET V2; CFP/YFP FRET V2; 377/647; Oxidized roGFP2; 400/647; and all their corresponding LED image cubes (for each mentioned filter); |  |  |  |
|  | - The instrument must have a high-resolution lead screw driven stage, with a resolution of 0.1 microns and open access; |  |  |  |
|  | - The microscope must include an image based and laser based autofocus, with a special laser autofocus cube included; |  |  |  |
|  | - The microscope must include at least one microplate/stage insert and a 2-position slide adapter and holder |  |  |  |
|  | - The instrument must include a special environmental control cover, for temperature control and incubation up to 40° Celsius, CO2/O2 control module support and “darkroom” fluorescence, allowing image acquisition under strictly controlled conditions of light. It is accepted also the solution when the microscope is included in external thermostated chamber (hood, incubator) adapted to host the microscope, but in this case, the incubator must be included in the offer. |  |  |  |
|  | - The unit must include a humidity chamber which maintains the humidity parameters for long term kinetics. The humidity chamber must have a re-charge port for rapid CO2/O2 recovery, according to the needs of each assay. Any other configuration / solution that conduct to the same result / environment will be accepted. |  |  |  |
|  | - The microscope must allow the connection to an optional gas control module for CO2/O2 concentration in the environment; |  |  |  |
|  | - The equipment must allow the distribution of reagents (by connection with an optional dual-reagent injection module with two syringes, having an accuracy of +/- 1 µL or 2%) |  |  |  |
|  | - The offer for the instrument must include a kit to run wound healings assays: the Auto-Scratcher (for special wound healing assessment tests, in cell monolayers, in 96- and 24-well microplates) and the software module dedicated to wound healing assays must be included in the offer. |  |  |  |
|  | - The offer must include the spare tips or pins, for at least 100 experiments (if these accessories cannot be cleaned and reused). |  |  |  |
|  | - The offer for the instrument must include a multi-vessel adapter (an adjustable holder for use with a variety of labware, including slides, Petri/cell culture dishes, chamber slides and more). The unit must work with microplates having 6 to 1536 wells. |  |  |  |
|  | - It will be accepted the possibility to have several adapters for different type of slides, petri dishes, microplates, etc., as long as these are included in the offer and are compatible with the motorized microscope table of the microscope: |  |  |  |
|  | - Adapter for one or two microscope slides |  |  |  |
|  | - Adapter for Petri dishes (if not adapted to any dimension, at least for 100 mm). |  |  |  |
|  | - Adapter for T25, T75 cell culture flask (adapter, stage insert) |  |  |  |
|  | - Multi-vessel adapter (an adjustable holder for use with a variety of labware, including slides, Petri/cell culture dishes, chamber slides and more). |  |  |  |
|  | - The microscope must allow the connection with a special joystick, which controls the automated stage movements and focusing. This accessory has to be included in the offer (or a similar solution has to be provided). |  |  |  |
|  | - The microscope must include a label-free high contrast brightfield cell counting kit |  |  |  |
|  | - The instrument must be a compact system, occupying a small surface area (for example 50x50x50 cm; Width x Depth x Height) and must be as light as possible (less than 30 Kg)(these characteristics are not limitative) |  |  |  |
|  | - A powerful PC Imaging System Controller must be included with the unit, consisting of a pre-configured high-performance computer - optimized for use with the imaging system, a 24'' monitor, keyboard and mouse, with the following minimal configuration (or better): |  |  |  |
|  | - Preferably Mini PC, or similar |  |  |  |
|  | - Operating System: MS Windows 10 Pro, 64-Bits |  |  |  |
|  | - Processor: Intel i9 – or higher |  |  |  |
|  | - Memory: 16 GB DDR, or higher |  |  |  |
|  | - Storage: 512 GB SSD, or higher + 2 TB hard disk drive |  |  |  |
|  | - Input Device: USB keyboard and mouse |  |  |  |
|  | - Power: External power adapter |  |  |  |
|  | - Ports & Connectors: at least (4) USB 3.1, (2) USB 2.0; |  |  |  |
|  | - Serial Port, HMDI, Network |  |  |  |
|  | - MONITOR Minimal Configuration: |  |  |  |
|  | - Display size (diagonal): minimum 24” |  |  |  |
|  | - Native resolution: 1920 x 1080 pixels (minimum) |  |  |  |
|  | - The system must also include a powerful image processing software for full control of the microscope and cell image analysis, but also for advanced image analysis. The software capabilities must include the following options (at least): |  |  |  |
|  | - Scatterplots/histograms of cell level data |  |  |  |
|  | - Image stitching and digital phase contrast |  |  |  |
|  | - Advanced features for secondary mask for measuring cytoplasm or whole cell |  |  |  |
|  | - Spot Counting for cellular analysis |  |  |  |
|  | - Object level measurements in cellular analysis |  |  |  |
|  | - Subpopulation analysis using custom object-level measurements |  |  |  |
|  | - Boolean operators for flexibility in cellular analysis subpopulation criteria |  |  |  |
|  | - Dynamic thresholding (iso-data) as option for cellular analysis masks |  |  |  |
|  | - Deconvolution image processing |  |  |  |
|  | - Intelligent z-projection method: focus stacking |  |  |  |
|  | - Easy image batch processing tools: fully automatic image pre-processing and analysis on large image collections |  |  |  |
|  | - The software must include licenses for minimum 2 installations |  |  |  |
|  | - The software must be able to control the automated microscope equipment and also the microplate reader equipment, both connected to the same computer system (as described above) |  |  |  |
|  | - High precision electronic pipettor robot, with capability to accurately dispense 1uL into 384 well-plates, capable to fast fill a 96-well plate in 20 seconds. |  |  |  |
|  | - The instrument must be user-friendly: graphical interface to design protocols and with an open configuration compatible to work with various type of tips and labware (microplates) |  |  |  |
|  | - The instrument must have automatic pipettes, with 1 and with 8 channels, that can be easily changed in order to cover the range of dispersed volumes between 1 μL until 1000 μL. For example, the instrument must have the following automatic / electronic pipettes: |  |  |  |
|  | - 1- 20 μL (8 channels) |  |  |  |
|  | - 5 – 50 μL (8 channels) |  |  |  |
|  | - 30 – 300 μL (8 channels) |  |  |  |
|  | - 1- 20 μL (1 channel) |  |  |  |
|  | - 20 – 300 μL (1 channel) |  |  |  |
|  | - 100 – 1000 μL (1 channel) |  |  |  |
|  | - The offer must include a fully automated thermocycler module (compatible with the pipettor robot), with lid that opens and closes automatically, with a program lid temperatures up to 110°C and a program block temperatures from 4-99°C and with the possibility to work in volume range: 10-100 μL (that can accommodate vials for PCR 25-100 μL). |  |  |  |
|  | - The offer must include a magnetic module (compatible with the pipettor robot) that can engage and disengage strong ferrous magnets with various type of labware (including microtiter plates). This module can be used for magnetic bead based protocols. The magnetic beads has to be included in the offer. |  |  |  |
|  | - The offer must include a temperature module (compatible with the pipettor robot) comprising a hot and cold plate module that holds reagents at temperatures between 4°C and 95°C. This module has to accommodate 96-well plates and other labware directly on deck and the has to contain the aluminum blocks required to hold 1.5 mL and 2 mL tubes, as well as PCR strips. |  |  |  |
|  | - The offer must include at least a set of the following labware (or similar, but compatible with the pipettor robot): |  |  |  |
|  | - NEST 12-Well Reservoirs, 15 mL |  |  |  |
|  | - NEST 1-Well Reservoirs, 195 mL |  |  |  |
|  | - 4-in-1 Tube Rack Set |  |  |  |
|  | - Aluminum Block Set |  |  |  |
|  | - Opentrons 10µL/20µL Tips |  |  |  |
|  | - Opentrons 50uL/300uL Tips |  |  |  |
|  | - Opentrons 1000uL Tips |  |  |  |
|  | - A powerful PC must be included in order to control the pipettor robot, consisting of a pre-configured high-performance computer - optimized for use with the imaging system, a 24'' monitor, keyboard and mouse, with the following minimal configuration (or better): |  |  |  |
|  | - Preferably Mini PC, or similar |  |  |  |
|  | - Operating System: MS Windows 10 Pro, 64-Bit, or similar |  |  |  |
|  | - Processor: Intel i9 – or higher |  |  |  |
|  | - Memory: 16 GB DDR, or higher |  |  |  |
|  | - Storage: 512 GB SSD, or higher + 2 TB hard disk drive |  |  |  |
|  | - Input Device: USB keyboard and mouse |  |  |  |
|  | - Power: External power adapter |  |  |  |
|  | - Ports & Connectors: at least (4) USB 3.1, (2) USB 2.0; |  |  |  |
|  | - Serial Port, HMDI, Network |  |  |  |
|  | - MONITOR Minimal Configuration: |  |  |  |
|  | - Display size (diagonal): minimum 24” |  |  |  |
|  | - Native resolution: 1920 x 1080 pixels (minimum) |  |  |  |
|  | - The offer must include all the necessary power cables adapted for Romania, communication cables, connectors, adaptors, network board, etc. that will assure the connectivity between the instruments and computers (network with cable or Wi-Fi). |  |  |  |
|  | - The offer must include the installation of all the instruments at the location of the end-user: Advanced Environmental Research Laboratories (Laboratoarele de Cercetari Avansate de Mediu), Str. Oituz 4, 300086, Timisoara, ROMÂNIA |  |  |  |
|  | - Technical documentation: The equipment must be accompanied by warranty certificates, model approvals and initial verification bulletins (if applicable), operating and service manuals in the following languages: Romanian and English. |  |  |  |
|  | - The supplier must ensure a professional two periods training sessions for the end-user team, at the final location, for the imager (microscope), accessories and for the software. |  |  |  |
|  | - In the first session the designated persons of the end-user team will be instructed to use the instruments and to perform the required operations, methods, techniques required to obtain professional images of biological samples |  |  |  |
|  | - After 2 weeks, the second session of instruction will take place and the end-user team will be evaluated and certified to use the instruments |  |  |  |
|  | - For both these session of training, the consumables, reagents, chemicals, materials, cultures, etc. has to be included in the offer. |  |  |  |
|  | - Instruments must be covered by a minimal three years warranty, after the installation. |  |  |  |
|  | - Service during the warranty period must be provided by authorized and trained personnel. Service intervention must be done within 48 working hours at the beneficiary's premises. If the appliance can not be repaired at the customer's premises, it will be done at the supplier's premises. The maximum repair time must not exceed 30 days from the time the device was removed from the recipient. |  |  |  |
|  | - Post-guarantee conditions: post-warranty service must be provided on the basis of a post-guarantee service contract that will provide, besides troubleshooting services, transportation, assistance and other services, as appropriate, that will be negotiated by the supplier with the beneficiaries. Upon the beneficiary's firm order, the supplier must undertake to deliver the required consumables both during the warranty period and afterwards. |  |  |  |
|  | - Spare parts and consumables: The supplier must provide spare parts and consumables for a period of between 5 and 10 years. |  |  |  |

# Category 3. Laboratory equipment and instruments (with accessories, installation, training, as appropriate)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 1) Product name: Benchtop pH/ion meter 2) Characteristics: Preferably with a large colour display (4.3 ″) Automatic 1-/2-/3-/4-/5-point calibration with predefined buffer groups or 1 user-defined group Automatic and manual temperature compensation Automatic, manual or timed endpoint detection Internal measured value memory: Saving by pressing key or timed (intermittent) saving (interval is variable from 1 s to 40 min) USB ports for exporting data via USB stick (datasets are exported as a text (.txt) file) or for connecting a USB keyboard The instrument must measure ion concentrations using additional ion-selective electrodes (ISE) –all the electrodes described below should be compatible (belong to) this pH/ion-meter. The instrument must have a pH single-rod measuring cell with integrated temperature gauge. pH range: 0 to 14. Operating temperature: 0 to +100 °C. Ceramic diaphragms, liquid-electrolyte, plastic shaft. Shaft length/Ø: 120/12 mm. The instrument must include pH single-rod measuring cell, buffer solutions (pH 4.01, 7.00, 9.21 and 10.00, 2 x 20 ml of each), electrode holder and protective case. Other technical characteristics: Conductivity measuring range from – -2 to +20 pH resolution – 0,001 pH accuracy – ±0.002 Conductivity measuring range from – -2000 to +2000 V Voltage resolution – 0,1 V Voltage accuracy – ±0,2 mV Temperature measuring range from – -5 to +130°C Temperature resolution – 0.1 °C Temperature accuracy – ±0.1 °C Temperature sensor – NTC 30 kΩ, Pt 1000 (or better) Calibration points – 1/2/3/4/5 (pH) Calibration memory – Current calibration (incl. date and time) Data memory – 2000 datasets (or more) Port – RS 232, USB-A (printer) USB-B (PC) Connection – BNC (pH) , Cinch (°C) Power supply – Mains adapter Power supply – 100 – 240 V, 50/60 Hz 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ion-Selective Epoxy Combination Electrode, for determination of copper ions, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: 120 mm long and 12 mm in diameter Application – For determination of copper ions Filling Solution Type – Ion Electrolyte D Length (Metric) Cable – 1.2 m Temperature (Metric) Operating – 0° to 80°C pH Range – 2 to 12 Membrane – Solid state Connector Types – BNC Material – Epoxy Shaft Interferences – Hg2+, Ag+ Electrode – Solid state 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ion-Selective Epoxy Combination Electrodes, for determination of Nitrate ions, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: 120 mm long and 12 mm in diameter Application – For determination of nitrate ions Filling Solution Type – Ion Electrolyte F Length (Metric) Cable – 1.2 m Temperature (Metric) Operating – 0° to 40°C pH Range – 2.5 to 11 Membrane – Polymer matrix Connector Types – BNC Material – Epoxy Shaft Interferences – NO3- 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ion-Selective Epoxy Combination Electrodes, for determination of Pb+2 ions, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: Click & Clear™ junction allows for an optimal contact of the electrolyte solution and the sample Application – For determination of Pb+2 ions Filling Solution Type – ISA Temperature (Metric) Operating – 0° to 80°C Electrode – Combined ISE Electrode Connector Types – BNC Operating range – 0.2 to 20700 mg/L 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ion-Selective Epoxy Combination Electrodes, for determination of Potassium ions, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: 120 mm long and 12 mm in diameter Application – For determination of Potassium ions Filling Solution Type – Ion Electrolyte E Length (Metric) Cable – 1.2 m Temperature (Metric) Operating – 0° to 40°C pH Range – 2.5 to 11 Membrane – Polymer matrix Connector Types – BNC Length (Metric) – 120 mm Material – Epoxy Shaft Interferences – K+ 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ion-Selective Epoxy Combination Electrodes, for determination of Silver/Sulphide, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: 120 mm long and 12 mm in diameter Application – For determination of Silver/Sulphide ions Filling Solution Type – Ion Electrolyte B Length (Metric) Cable – 1.2 m Temperature (Metric) Operating – 0° to 80°C pH Range – 2 to 12 Membrane – Solid state Connector Types – BNC Material – Epoxy Shaft Interferences – 10 mol/L NaOH Electrode – Solid state 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: pH micro Combination Electrode, compatible with the pH-Meter described in Category 3, Item No. 1 2) Characteristics: Ideal for measuring in very small vessels such as vials or cuvettes Combination design avoids use of separate reference and indicating electrodes. Different cables can be connected to the S7 screw head allowing use with a wide range of meters. • 3 mm diameter shaft • Combination design • Detachable cable Material – Glass shaft, Glass, U membrane Description – Reference electrolyte 3 mol/L KCl – Reference system ARGENTHALTM with Ag+ -trap – Without cable Feature – S7 connector Temperature measuring range – 0-80°C Length x diameter – 60 mm x 3 mm pH range – 0-14 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Washing machine with drier 2) Characteristics: Centrifugation speed – 1400 rpm Washing capacity – 8 kg Drying capacity – 4.5 kg Electric efficiency – Class A (from a range from A to G) Digital inverter motor Loading type – frontal Noise level for centrifugation – 73 dB Noise level for washing – 54 dB Window opening – left Command panel – digital Color – white Dimensions – Depth: 60 cm – Width: 60 cm – Hight: 85 cm – Weight: 71 kg Safety functions – Error signaling – Interface access blocking – Program finalization signaling Centrifugation speed steps – 5 Temperature steps – 5 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Precision balance 2) Characteristics: Top-loading Balance The balance communicates directly to a PC with Windows® applications (PC not included in the offer). Unit conversion – Weight value is presented in 22 units and modes, including percentage, carat, specific gravity, lb, oz, and so on. Users can choose any combination of units depending on their needs Capacity – 800 – 1200 g Minimum display – 0.001 g Calibration capability in the range of the measurement Repeatability (σ) - ≤0.001 g Linearity – ±0.003 g Response time – 1.5-2.5 s Ambient temperature and humidity – 5–40°C 30–85% (No Condensation) Temperature Coefficient Sensitivity (10–30°C) – ±3 ppm/°C Display – LCD with backlight Power requirements – compatible with Romanian power supply 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: UV-Vis spectrophotometer 2) Characteristics: Easy-to-use user interface design Ergonomic touch-screen display The instrument must be equipped with an ultra-high-speed scan, which can acquire a spectrum in a few seconds, with the lowest level of stray light and noise in its class The stages of the entire measurement process and the current status must always be shown on the display Stray light must be at 0.5 % max. (198 nm), making accurate measurements possible up to the vicinity of 2 Abs even in the ultraviolet region Spectra must be able to be acquired in the range 25,000 – 30,000 nm/min The photometric repeatability accuracy must be 0.0002 Abs max. (0.5 Abs and 1.0 Abs) Data must be able to be transferred to a PC (not included in the offer) The instrument shouldn’t require a time to warm up The following measurement modes must be available: photometric, spectrum, quantitation, kinetics, time course and biomethod measurements A standard software must be included for PC for both instrument control and data analysis 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Viscometer (standard set including four standard spindles, protective bracket, temperature sensor, quick connector, hook connector and stand) for viscosity measurement range of 1 - 6000000 mPa s with adapter for 2.1 mL samples that includes the necessary spindle and chamber for 2.1 mL samples 2) Characteristics: Viscometer: - viscosity measuring range - 1 - 6,000,000 mPa s - at least 4.3″ digital display - measurement accuracy level – for both Newtonian and non-Newtonian fluids must be ± 1 % of the measuring range - reproducibility –s ± 0.2 % - must have stepless speed adjustment Adapter: - must be compatible with the viscometer - for measuring the viscosity of especially valuable and expensive samples - double jacket with temperature sensor to quickly heat up or cool down the sample - coaxial cylinder system for the analysis of the sample at a specific shear rate must be included Spindle for 2.1 mL: - must be compatible with the viscometer and its adapter - for an extended viscosity range - especially suitable for sample volumes of 2.1 ml Chamber for 2.1 mL - - must be compatible with the viscometer, its adapter and the 2.1 mL sample spindle - especially suitable for expensive and valuable samples - temperature sensor must be already integrated in the chamber and thus allows for temperature measurement during viscosity determination 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Analytical balance 2) Characteristics: The balance communicates directly to a PC with Windows® applications (PC not included in the offer). Unit conversion – Weight value is presented in 22 units and modes, including percentage, carat, specific gravity, lb, oz, and so on. Users can choose any combination of units depending on their needs Capacity – 100 - 200 g Minimum display – 0.01 mg (0.00001 g) Auto-Calibration capability Repeatability (σ) - ≤0.01 mg Linearity – ±0.03 mg Response time – 1.5-3 s Ambient temperature and humidity – 5–40°C 30–85% (No Condensation) Temperature Coefficient Sensitivity (10–30°C) – ±3 ppm/°C Display – LCD with backlight Power requirements – compatible with Romanian power supply 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |

# Category 4. Chemicals and reagents (according with the detailed list)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 1) Product name: 1,10-Phenanthroline hydrochloride monohydrate (o-Phenanthroline hydrochloride monohydrate) 2) Characteristics: Purity 99%, analytical reagent for colorimetric determination of iron, Formula C12H8N2xHClxH2O, Formula Weight 234.68 g/mol 3) Measurement unit: 25g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 4-Methylumbelliferyl β-D-N,N′,N′′-triacetylchitotrioside 2) Characteristics: Molecular Formula C34H47N3O18, Formula Weight 785.8 g/mol, Purity ≥98%, Stability ≥ 2 years 3) Measurement unit: 5 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 4-Nitroaniline, ≥98,5 %, for synthesis 2) Characteristics: Empirical formula C6H6N2O2, Molar mass (M) 138.12 g/mol, Density (D) 0.951 g/cm³, Boiling point (bp) >800°C, Flash point (flp) 100.8°C, Melting point (mp) 158 °C 3) Measurement unit: 1 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 4-Nitrophenol, ≥99 %, for synthesis 2) Characteristics: Empirical formula C6H5NO3, Molar mass (M) 139.11 g/mol, Density (D) 1.48 g/cm³, Boiling point (bp) 279°C, Flash point (flp) 169°C, Melting point (mp) 115°C 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 4-Nitrophenyl β-D-N,N′,N′′-triacetylchitotriose 2) Characteristics: chitinase and lysozyme substrate, Empirical Formula (Hill Notation) C30H44N4O18, Molecular Weight 748.69 g/mol 3) Measurement unit: 1 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 5-(4-Dimethylaminobenzylidene) rhodamine, p.a. 2) Characteristics: Analytical reagent for silver. Empirical formula C12H12N2OS2, Molar mass (M) 264.37 g/mol, Melting point (mp) 280°C 3) Measurement unit: 10 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Acid Yellow 17 2) Characteristics: Dye content 60%, Empirical Formula (Hill Notation) C16H10Cl2N4Na2O7S2, Molecular Weight 551.29 g/mol 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Acridine orange (C.I. 46005), for microscopy 2) Characteristics: Empirical formula C17H20Cl3N3Zn, Molar mass (M) 438.09 g/mol, Melting point (mp) 165°C, Fluorescence indicator. Fluorescent dye with DNA intercalating properties. 3) Measurement unit: 10 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Acrylamide 2) Characteristics: 99.5%, Molecular Biology Grade 3) Measurement unit: 500 g 4) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Adenine ≥98.0%, high purity 2) Characteristics: Formula: C₅H₅N₅, MW: 135.13 g/mol, Density: 0.99741 g/cm³ (25 °C) 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Adenine hemisulfate salt 2) Characteristics: powder, BioReagent, suitable for cell culture, Linear Formula C5H5N5 x 1/2H2SO4, Molecular Weight 184.17 g/mol 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Agar, BioScience Grade, powdered 2) Characteristics: Agar of high purity for all culture media used in molecular- and microbiology. Recommended for plating in genetic engineering (GMOs). 3) Measurement unit: 2.5 kg 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Agarose, for DNA/RNA electrophoresis 2) Characteristics: Melting point (mp) 95°C, General agarose for routine gels and standard analyses (1 kb-20 kb). 3) Measurement unit: 1 kg 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Ammonium iron(III) citrate, 2) Characteristics: ~21 % Fe, brown 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ammonium iron(III) sulphate dodecahydrate, ≥98,5 %, p.a., ACS, ISO 2) Characteristics: Empirical formula (NH4)Fe(SO4)2x12H2O, Molar mass (M) 482.19 g/mol, Density (D) 1.71 g/cm³, Melting point (mp) 41°C 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ammonium thiocyanate, ≥99 %, p.a., ACS, ISO 2) Characteristics: Empirical formula NH4SCN, Molar mass (M) 76.12 g/mol, Density (D) 1.3 g/cm³, Melting point (mp) 150°C 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Anti-His-Tag antibody, Rabbit monoclonal 2) Characteristics: recombinant, expressed in HEK 293 cells, clone RM146, purified immunoglobulin. This antibody reacts to recombinant proteins containing the 6xHis-Tag or 10xHis-Tag fused to either the amino or carboxy terminus. No cross reactivity with other endogenous protein in mammalian or bacteria cells. 3) Measurement unit: 100 µg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Bentonite, pure 2) Characteristics: Carrier in chromatography, Density (D) 2.6 g/cm³, Melting point (mp) >450°C 3) Measurement unit: 1 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Benzamidine hydrochloride hydrate, ≥98%, for biochemistry 2) Characteristics: Inhibitor of serine proteases. Competitive inhibitor of trypsin and trypsin-like enzymes, acrosin, thrombin and plasmin. Empirical formula C7H8N2xHClxH2O, Molar mass (M) 174.63 g/mol, Melting point (mp) 87°C 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Calcium nitrate hydrate, 99.997% trace metals basis 2) Characteristics: Linear Formula Ca(NO3)2·xH2O, Molecular Weight 164.09 g/mol (anhydrous basis) 3) Measurement unit: 10 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Carbon disulfide, ACS reagent, ≥99.9% 2) Characteristics: Empirical Formula (Hill Notation) CS2, Molecular Weight 76.14 g/mol 3) Measurement unit: 1 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Carbon tetrachloride, anhydrous, ≥99.5% 2) Characteristics: Empirical Formula (Hill Notation) CCl4, Molecular weight 153.82 g/mol 3) Measurement unit: 100 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Carmine (C.I. 75470) MIC 2) Characteristics: Molecular Formula C44H37AlCaO27·3H2O, MW: 492.3 g/mol 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Casein hydrolysate, standard, acid-hydrolysed, for molecular biology 2) Characteristics: Ingredient of molecular and microbiological nutrient media, vitamin-free. 3) Measurement unit: 500 g 4) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Catalase reagent 2) Characteristics: The catalase test allows to differentiate or identify microorganisms by verifying their ability to produce catalase, an enzyme that protects microorganisms from toxic oxygen metabolites. Generally only aerobic microorganisms produce catalase. 3) Measurement unit: 30 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitobiose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C12H24N2O9·2HCl, Molecular weight: 413.25 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitoheptaose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C42H79N7O29·7HCl, Molecular weight: 1401.33 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitohexaose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C36H68N6O25·6HCl, Molecular Weight: 1203.72 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitopentaose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C30H57N5O21·5HCl, Molecular Weight: 1006.10 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitotetraose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C24H46N4O17·4HCl, Molecular weight: 808.48 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chitotriose Hydrochloride 2) Characteristics: Purity: not less than 95% determined by HPLC, Molecular Formula: C18H35N3O13·3HCl, Molecular weight: 610.87 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chromotrope FB (C.I. 14720), for microscopy 2) Characteristics: Empirical formula C20H12N2Na2O7S2, Molar mass (M) 502,43 g/mol, Density (D) 0.629 g/cm³, Boiling point (bp) 907.5°C, Flash point (flp) >225°C, Melting point (mp) >300 °C 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chymostatin, microbial 2) Characteristics: Solubility: DMSO: 10 mM (Stock solutions stable for months at −20°C.) Inhibits the lysosomal proteinase cathepsin B, and the soluble Ca2+- activated proteinase. 3) Measurement unit: 25 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ciprofloxacin, ≥98.0% (HPLC), powder 2) Characteristics: Empirical Formula (Hill Notation) C17H18FN3O3, Molecular Weight 331.34 g/mol, Mode of action: DNA synthesis ¦ interferes, enzyme ¦ inhibits Antibiotic activity spectrum: Gram-negative bacteria, Gram-positive bacteria 3) Measurement unit: 5g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cobalt(II) acetate tetrahydrate, ≥98 %, p.a., ACS 2) Characteristics: Empirical formula Co(CH3COO)2x4H2O, Molar mass (M) 249.08 g/mol, Density (D) 1.7 g/cm³, Melting point (mp) 140°C 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: cOmplete™, EDTA-free Protease Inhibitor Cocktail 2) Characteristics: cOmplete, EDTA-free Protease Inhibitor Tablets, inhibit a broad spectrum of serine and cysteine proteases 3) Measurement unit: vial of 20 tablets 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: D(+)-2-Phosphoglyceric acid sodium salt hydrate, ≥75% (calc. on dry substance, enzymatic) 2) Characteristics: Empirical Formula (Hill Notation) C3H7O7P xNa+ yH2O, Molecular weight 186.06 g/mol (anhydrous free acid basis) 3) Measurement unit: 50 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: DAPI, ≥98 %, for biochemistry 2) Characteristics: Empirical formula C16H17Cl2N5, Molar mass (M) 350.3 g/mol, Melting point (mp) 330°C 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Diethanolamine, ≥99 %, p.a., ACS 2) Characteristics: For biochemistry. Empirical formula C4H11NO2, Molar mass (M) 105.14 g/mol, Density (D) 1.09 g/cm³, Boiling point (bp) 271°C, Flash point (flp) 176°C, Melting point (mp) 28°C 3) Measurement unit: 500 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Diisopropyl ether, ≥99 %, p.a., stabilised 2) Characteristics: Empirical formula C6H14O Molar mass (M) 102.18 g/mol, Density (D) 0.72 g/cm³, Boiling point (bp) 70°C, Flash point (flp) -28°C, Melting point (mp) -86°C 3) Measurement unit: 1 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: di-Sodium hydrogen phosphate, ≥99 %, p.a., ACS, anhydrous 2) Characteristics: Molecular biology; Reagent for the detection of Mg, Li, Pb and uric acid. Empirical formula Na2HPO4, Molar mass (M) 141.96 g/mol, Density (D) 1.53 g/cm³, Melting point (mp) 250°C 3) Measurement unit: 2.5 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 1,4-Dithiothreitol (DTT), ≥99 %, p.a. 2) Characteristics: For biochemistry and molecular biology, Empirical formula C4H10O2S2, Molar mass (M) 154.2 g/mol, Density (D) 1 g/cm³, Flash point (flp) >109 °C, Melting point (mp) 43°C 3) Measurement unit: 10 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Dithizone, ≥85 %, p.a., ACS 2) Characteristics: Metal indicator, Empirical formula C13H12N4S, Molar mass (M) 256.32 g/mol, Melting point (mp) 168°C 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: D-Luciferin, ≥99 %, for biochemistry 2) Characteristics: Substrate of Luciferase, Empirical formula C11H8N2O3S2, Melting point (mp) 204°C 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Endo Hf (Endoglycosidase H) 2) Characteristics: Purity > 95% as determined by reducing SDS-PAGE, Molecular weight, 30.23 kDa, Fusion-Tag: C-6 His tag 3) Measurement unit: 5 µg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Eosin Y (C.I. 45380), for microscopy 2) Characteristics: Empirical formula C20H6Br4Na2O5, Molar mass (M) 691.86 g/mol, Melting point (mp) 296°C 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: (S)-(+)-Epichlorohydrin, 98% 2) Characteristics: Empirical Formula (Hill Notation) C3H5ClO, Molecular weight 92.52 g/mol 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Ethanol, ≥99,5 %, Ph.Eur., extra pure 2) Characteristics: Empirical formula C2H6O Molar mass (M) 46.07 g/mol, Density (D) 0.79 g/cm³, Boiling point (bp) 78°C, Flash point (flp) 12°C, Melting point (mp) -114°C 3) Measurement unit: 1 L 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Ferroin indicator solution, for waste water analysis 2) Characteristics: pH:4-7 (20°C in H2O) Density:1.01 g/cm3 at 20°C 3) Measurement unit: 500 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Gelatin Agar (DEV), for microbiology 2) Characteristics: For enumeration of the total microbial population and detection of gelatin-liquefying microorganisms in water samples. 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glutaraldehyde 25%, for electron microscopy 2) Characteristics: 25% solution in water, Empirical formula C5H8O2, Molar mass (M) 100.12 g/mol, Density (D) 1.06 g/cm³, Boiling point (bp) >100°C, Melting point (mp) -6°C 3) Measurement unit: 1L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glycerol, ≥99,5 %, p.a., anhydrous 2) Characteristics: This anhydrous glycerol is used for the storage and concentration of enzymes and other samples at low temperatures. It is also used for the purification of bacteriophages and other biological molecules by a glycerol step gradient. Due to its high density it is often a component of electrophoresis loading buffers. 3) Measurement unit: 5 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Goat Anti-Mouse IgG Antibody, (H+L) Alkaline Phosphatase conjugate 2) Characteristics: concentration: 1 mg/mL, biological source: goat, antibody product type: secondary antibodies, clone: polyclonal, species reactivity: mouse, application(s): ELISA, western blot 3) Measurement unit: 100 µL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Guaiacol, ≥98 %, extra pure 2) Characteristics: Empirical formula C7H8O2, Molar mass (M) 124.14 g/mol Density (D) 1.129 g/cm³, Boiling point (bp) 205°C, Flash point (flp) 82°C, Melting point (mp) 29°C. 3) Measurement unit: 100 mL 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Hematoxylin (C.I.75290), for microscopy 2) Characteristics: For staining of cell nuclei, Empirical formula C16H14O6, Molar mass (M) 302.29 g/mol, Flash point (flp) 304.5°C, Melting point (mp) >145°C 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Hydrogen peroxide 35% pure, stabilised 2) Characteristics: Empirical formula H2O2 Molar mass (M) 34.02 g/mol, Density (D) 1.132 g/cm³, Boiling point (bp) 108°C, Melting point (mp) -33°C 3) Measurement unit: 500 mL 4) Requested quantity: 6 |  |  |  |
|  | 1) Product name: Levofloxacin, ≥98.0% (HPLC), 2) Characteristics: Empirical Formula (Hill Notation) C18H20FN3O4, Molecular weight 361.37 g/mol 3) Measurement unit: 1g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Luciferase from Photinus pyralis (firefly) 2) Characteristics: recombinant, expressed in E. coli, lyophilized powder, ≥1×1011 units/mg protein 3) Measurement unit: 2 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Luminol, ≥95 %, for synthesis 2) Characteristics: Fluorescence indicator, peroxidase reagent, Empirical formula C8H7N3O2, Molar mass (M) 177.16 g/mol Melting point (mp) 330°C 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: L-α-Hydroxyglutaric acid disodium salt, ≥98.0% (GC) 2) Characteristics: Formula C5H6O5Na2, Molecular weight 192.08 g/mol 3) Measurement unit: 50 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Manganese (II) nitrate tetrahydrate, ≥98 %, p.a. 2) Characteristics: Empirical formula Mn(NO3)2 x 4H2O, Molar mass (M) 251.01 g/mol, Density (D) 2.13 g/cm³, Boiling point (bp) 129°C, Melting point (mp) 37°C 3) Measurement unit: 250 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Meat extract, powdered, for culture media 2) Characteristics: High-grade nutrient media additive also ideal for supplementation of media for fermentation studies. 3) Measurement unit: 500g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Menadione, ≥98.0% (HPLC), crystalline 2) Characteristics: Empirical Formula (Hill Notation) C11H8O2, Molecular Weight 172.18 g/mol 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Mono sodium citrate, ≥99 %, anhydrous, extra pure 2) Characteristics: Empirical formula C6H7NaO7, Molar mass (M) 214.11 g/mol Density (D) 0.89 g/cm³, Melting point (mp) 210°C 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Monoclonal Anti-c-Myc antibody produced in mouse, clone 9E10, ascites fluid 2) Characteristics: primary antibodies, unconjugated. Recognizes both N- terminal and C-terminal c-Myc tagged proteins. 3) Measurement unit: 100 uL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: NBT-BCIP® solution 2) Characteristics: BioReagent, suitable as substrate for alkaline phosphatase in dot blots 3) Measurement unit: 10 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: N-Laurylsarcosine sodium salt 2) Characteristics: Synonym: N-Dodecanoyl-N-methylglycine sodium salt, Sarkosyl NL Molecular Weight 293.38 Linear Formula CH3(CH2)10CON(CH3)CH2COONa Purity required (minimum) ≥94% form powder solubility H2O: 100 mg/mL, clear to slightly hazy, colorless to faintly yellow 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Nzamine, for molecular biology 2) Characteristics: Highly nutritive peptone, produced by pancreatic digest of Casein under distinct conditions. Best source of nitrogen and amino acids in nutrient media for molecular biological assays, particularly broth acc. to Luria/Bertani (LB). 3) Measurement unit: 250 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Oxidase reagent according to Gordon-McLeod, for microbiology 2) Characteristics: Composition: N,N-dimethyl-p-phenylenediamine hydrochloride, 0.15 g 3) Measurement unit: 100 mL 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Oxidase test strips, Bio Analysis 2) Characteristics: Rapid test system for detection of cytochrome oxidase in microorganisms. Rapid test system for detection of oxidase-positive microorganisms. 3) Measurement unit: 1 pack (100 pieces) 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Paclitaxel, from Taxus brevifolia, ≥95% (HPLC), powder 2) Characteristics: Empirical Formula (Hill Notation) C47H51NO14, Molecular weight 853.91 g/mol, Solubility DMSO: 50 mg/mL (can be stored frozen for several months), methanol: soluble 50 mg/mL, clear, colorless (undergoes transesterification), ethanol: soluble, Mode of action: DNA synthesis ¦ interferes, antibiotic activity spectrum: viruses. 3) Measurement unit: 1 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: PAR (4-(2'-Pyridylazo)-resorcinol), 96% 2) Characteristics: Empirical Formula (Hill Notation) C11H9N3O2, Molecular Weight 215.21 g/mol 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pepsin, ≥2 000 FIP-U/g, for biochemistry 2) Characteristics: Molar mass (M) 35000 g/mol Storage temp. +4°C 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Phenylmethyl sulphonyl fluoride (PMSF), ≥99 %, for biochemistry 2) Characteristics: Empirical formula C7H7FO2S, Molar mass (M) 174.2 g/mol, Density (D) 0.79 g/cm³,Melting point (mp) 94°C 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Phosphoramidon disodium salt, ≥97% (HPLC) 2) Characteristics: Empirical Formula (Hill Notation) C23H32N3Na2O10P, Molecular Weight 587.47 g/mol, solubility H2O: 50 mg/mL 3) Measurement unit: 5 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Polygalacturonic acid, ≥85 % 2) Characteristics: Empirical formula (C6H8O6)n, Molar mass (M) ~25000 g/mol, Melting point (mp) 60°C 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Polyvinylpolypyrrolidone 2) Characteristics: Autoignition temp.: 440°C Potency: >2000 mg/kg LD50, oral (Rat) Mp: 220°C (decomposes) Density: 1.2 g/cm3 at 20°C, bulk density 330 kg/m3, storage conditions: Store at +5°C to +30°C. 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Potassium cyanide, BioUltra, ≥98.0% (AT) 2) Characteristics: Formula KCN, Molecular Weight 65.12 g/mol 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Potassium dihydrogen phosphate, ≥99 %, p.a., ACS 2) Characteristics: Empirical formula KH2PO4, Molar mass (M) 136.09 g/mol, Density (D) 2.34 g/cm³, Melting point (mp) 255°C 3) Measurement unit: 1 kg 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Powdered milk, Blotting Grade, powdered, low in fat 2) Characteristics: Low-fat powdered milk, especially suited for use as blocking reagent in immunological assays like Western-Blot analyses or ELISA. Fine powdered for easy application. 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Premium Western blotting membranes, nitrocellulose 2) Characteristics: 0.2 µm pore size, roll W×L 300 mm × 4 m 3) Measurement unit: 4 m 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pyrogallol, ≥99 %, p.a., ACS 2) Characteristics: Empirical formula C6H6O3, Molar mass (M) 126.11 g/mol, Density (D) 1.45 g/cm³, Boiling point (bp) 309°C, Flash point (flp) 117.9°C, Melting point (mp) 135°C 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Rhodamine B (C.I. 45170), for microscopy 2) Characteristics: Empirical formula C28H31ClN2O3, Molar mass (M) 479.02 g/mol, Density (D) 1.31 g/cm³, Melting point (mp) 201°C 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Rose Bengal (C.I. 45440), for microscopy 2) Characteristics: Empirical formula C20H2Cl4I4Na2O5, Molar mass (M):1017.65 g/mol 3) Measurement unit: 50 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Safranin O (C.I.50240), for microscopy 2) Characteristics: Basic azin dye, soluble in water and ethanol, well adapted to visualise chromosomes. Safranin is used for bacteria staining, e.g. during Gram-staining or spore staining acc. to Wirtz. Empirical formula: C20H19ClN4, Molar mass (M) 350,85 g/mol 3) Measurement unit: 10 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Salicylic acid, ≥99 %, p.a., ACS 2) Characteristics: Empirical formula C7H6O3, Molar mass (M) 138.12 g/mol, Density (D) 1.44 g/cm³, Boiling point (bp) 256°C, Flash point (flp) 157°C, Melting point (mp) 160°C 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Silver sulphate, ≥99 %, p.a. 2) Characteristics: Empirical formula Ag2SO4, Molar mass (M) 311.80 g/mol, Density (D) 5,45 g/cm³, Melting point (mp) 655°C. 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Simmons Citrate Agar 2) Characteristics: ISO 10273, for microbiology. For differentiation of microorganisms on the basis of citrate utilization. 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Sodium citrate dehydrate, ≥99%, FG 2) Characteristics: Molecular Weight 294.10 g/mol 3) Measurement unit: 1 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Sodium dihydrogen phosphate dehydrate, ≥98 %, Ph.Eur., USP 2) Characteristics: Empirical formula NaH2PO4 x 2H2O, Molar mass (M) 156.01 g/mol, Density (D) 1.92 g/cm³ ,Melting point (mp) 60 °C 3) Measurement unit: 2.5 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Tetrachloroauric(III) acid trihydrate, ≥99,9 %, p.a. Ultra Quality 2) Characteristics: Empirical formula HAuCl4 x 3H2O, Molar mass (M) 393,83 g/mol, Density (D) 3,9 g/cm³, Melting point (mp) 30°C 3) Measurement unit: 1 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: N,N′,N′′-Triacetylchitotriose, ≥93% (HPLC) 2) Characteristics: Empirical Formula (Hill Notation) C24H41N3O16, Molecular Weight:627.59 g/mol 3) Measurement unit: 10 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: tri-Ammonium citrate, ≥97%, pure 2) Characteristics: Empirical formula C6H17N3O7, Molar mass (M) 243,22 g/mol 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Trichloroacetic acid 2) Characteristics: For biochemistry, ≥99%, Ph.Eur., extra pure, Empirical formula: CCl3COOH, Molar mass (M): 163,40 g/mol, Density (D) 1,62 g/cm³, Boiling point (bp) 197°C, Flash point (flp) >110°C, Melting point (mp) 56°C. 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Triple Sugar Iron Agar, Ph.Eur., for microbiology 2) Characteristics: For identification and differentiation of enterobacteria and for determination of Salmonellae. 3) Measurement unit: 500 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Triton® X 100, extra pure 2) Characteristics: Non-ionic detergent. For biochemistry and molecular biology. Empirical formula: C33H60O10, Molar mass (M): 624 g/mol, Density (D): 1,1 g/cm³, Boiling point (bp) >200°C, Flash point (flp) 251°C. 3) Measurement unit: 1 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Trizma® base 2) Characteristics: Primary Standard and Buffer, ≥99.9% (titration), crystalline, aminopeptidase substrate, 3) Measurement unit: 500 g 4) Requested quantity:1 |  |  |  |
|  | 1) Product name: Tryptone/Peptone ex casein 2) Characteristics: Rich in short-chained peptides and amino acids, including trytophane. Essential ingredient of many nutrient media like LB, 2YT. Produced by pancreatic digestion of casein. It is used as an additive to nutrient media in the production of toxins, vaccins, enzymes and further biological products. 3) Measurement unit: 1 kg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: TWEEN® 20, viscous liquid 2) Characteristics: non-ionic, assay ≥40.0% (GC), form: viscous liquid, mol wt ~1228, composition: lauric acid, ≥40% (balance primarily myristic, palmitic, and stearic acids), application(s): DNA extraction, RNA extraction, cell culture¦mammalian, protein purification, Density: 1.095 g/mL at 25 °C (lit.), HLB 16.7 3) Measurement unit: 1 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Yeast-Nitrogen-Base (without Amino Acids) 2) Characteristics: Yeast Nitrogen Base (without amino acids) contains ammonium sulphate, all vitamins and all organic salts needed to cultivate yeasts, except for the amino acids and carbohydrate sources. 3) Measurement unit: 500 g 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Zinc chloride anhydrous EPR 2) Characteristics: Specifications: Assay ≥ 98% Oxychlorides (ZnO) ≤ 2.4%, Sodium (Na) + Potassium (K) + Calcium (Ca) ≤ 1%, Iron (Fe) ≤ 0.002%, Copper (Cu) ≤ 0.002%, Lead (Pb) ≤ 0.002%, Cadmium (Cd) ≤ 0.001% 3) Measurement unit: 1 k 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Zinc nitrate hexahydrate, ≥98 %, p.a. 2) Characteristics: Empirical formula: Zn(NO3)2x6H2O Molar mass (M): 297.47 g/mol, Density (D): 2.065 g/cm³, Melting point (mp): 36°C 3) Measurement unit: 100 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Z-Phe-OH, 99% 2) Characteristics: Optical activity: [α]20/D +5°, c = 5 in acetic acid, mp: 85-87°C (lit.) 3) Measurement unit: 25 g 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Zymolyase® 20T ≥20 U/mg, for biochemistry and molecular biology 2) Characteristics: Zymolyase®, produced by a submerged culture of Arthrobacter luteus. Stability: Maintain 70% of the lytic activity at 30°C, more than 3 months. Optimum temperature and pH: at pH 7.5 - 35°C for lysis of viable yeast cells, at pH 6.5 - 45°C for hydrolysis of yeast glucan. Specificity (lytic spectrum): Ashbya, Candida, Debaryomyces, Eremothecium, Endomyces, Hansenula, Hanseniaspora, Kloeckera, Kluyveromyces, Lipomyces, Metschnikowia, Pichia, Pullularia, Torulopsis, Saccharomyces, Saccharomycopsis, Saccharomycodes, Schwanniomyces, etc. 3) Measurement unit: 500 mg 4) Requested quantity: 1 |  |  |  |

# Category 5. Laboratory materials (according with the detailed list)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 1) Product name: Centrifugal Filter Unit, 10 kDa NMWCO 2) Characteristics: Regenerated cellulose membrane 2 mL sample volume Polypropylene tube (for collection) Styrene-butadiene housing Non-sterile Centrifugation suitable (protein concentration and desalting) Protein extraction suitable Protein purification suitable Length – 119.7 mm Diameter – 15.9 mm Filtration area – 1 cm2 Pore size – 10 kDa NMWCO 3) Measurement unit: 24 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Centrifugal Filter Unit, 3 kDa NMWCO 2) Characteristics: Regenerated cellulose membrane 2 mL sample volume Polypropylene tube (for collection) Styrene-butadiene housing Non-sterile Centrifugation suitable (protein concentration and desalting) Protein extraction suitable Protein purification suitable Length – 119.7 mm Diameter – 15.9 mm Filtration area – 1 cm2 Pore size – 3 kDa NMWCO 3) Measurement unit: 24 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Centrifugal Filter Unit, 30 kDa NMWCO 2) Characteristics: Regenerated cellulose membrane 2 mL sample volume Polypropylene tube (for collection) Styrene-butadiene housing Non-sterile Centrifugation suitable (protein concentration and desalting) Protein extraction suitable Protein purification suitable Length – 119.7 mm Diameter – 15.9 mm Filtration area – 1 cm2 Pore size – 30 kDa NMWCO 3) Measurement unit: 24 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Centrifugal Filter Unit, 50 kDa NMWCO 2) Characteristics: Regenerated cellulose membrane 2 mL sample volume Polypropylene tube (for collection) Styrene-butadiene housing Non-sterile Centrifugation suitable (protein concentration and desalting) Protein extraction suitable Protein purification suitable Length – 119.7 mm Diameter – 15.9 mm Filtration area – 1 cm2 Pore size – 50 kDa NMWCO 3) Measurement unit: 24 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Bacterial cell spreaders, L-shaped, 90°, stainless steel 2) Characteristics: Must be autoclavable at 121°C Made of stainless steel Total length – 195 mm Total width – 40 mm Diameter – 4 mm 3) Measurement unit: 1 pcs 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Class A measuring cylinders, blue markings (10 mL) 2) Characteristics: Hexagonal base Class A Made of - top quality 3.3 borosilicate glass In compliance with ISO 4788, USP and Eichordnung (the German Federal Weights and Measures Regulations) Volume – 10 mL Blue markings 3) Measurement unit: 1 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Measuring cylinders, blue markings (25 mL) 2) Characteristics: Hexagonal base Class A Made of - top quality 3.3 borosilicate glass In compliance with ISO 4788, USP and Eichordnung (the German Federal Weights and Measures Regulations) Volume – 25 mL Blue markings 3) Measurement unit: 1 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Measuring cylinders, blue markings (5 mL) 2) Characteristics: Made of Borosilicate glass 3.3 Must be autoclavable at 121°C Tall type With hexagonal base Calibrated to IN. Acc. to DIN ISO 4788 Volume – 5 mL Graduation 10 ¦ Tolerance - ± 5.00 mL Diameter – 65 mm Height – 465 mm Blue graduations 3) Measurement unit: 2 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Measuring cylinders, blue markings (50 mL) 2) Characteristics: Hexagonal base Class A Made of - top quality 3.3 borosilicate glass In compliance with ISO 4788, USP and Eichordnung (the German Federal Weights and Measures Regulations) Volume – 50 mL Blue markings 3) Measurement unit: 1 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Single channel 0.5-10 µL pipette 2) Characteristics: Volume range – 0.5-10 µL Volume locking mechanism Dual volume adjustment Stainless steel tip ejector Adjustable ejector height, “click-to-fix” system Universal shafts Color coding for volume & user identification Easy maintenance & recalibration Fully autoclavable 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Single channel 100-1000 µL pipette 2) Characteristics: Volume range – 100-1000 µL Volume locking mechanism Dual volume adjustment Stainless steel tip ejector Adjustable ejector height, “click-to-fix” system Universal shafts Color coding for volume & user identification Easy maintenance & recalibration Fully autoclavable 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Single channel 20-200 µL pipette 2) Characteristics: Volume range – 20-200 µL Volume locking mechanism Dual volume adjustment Stainless steel tip ejector Adjustable ejector height, “click-to-fix” system Universal shafts Color coding for volume & user identification Easy maintenance & recalibration Fully autoclavable 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Single channel 500-5000 µL pipette 2) Characteristics: Volume range – 500-5000 µL Volume locking mechanism Dual volume adjustment Stainless steel tip ejector Adjustable ejector height, “click-to-fix” system Universal shafts Color coding for volume & user identification Easy maintenance & recalibration Fully autoclavable 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Dissection scissors, straight pointed, 145 mm 2) Characteristics: Stainless steel Type – dissection straight, pointed/pointed Length – 145 mm 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Freezing container for steady 1°C/min freezing 2) Characteristics: Polycarbonate container Blue high-density polyethylene closure White high-density polyethylene vial holder Foam insert Provides critical, repeatable, 1°C/min cooling rate required for successful cryopreservation of cells Easy to use in any mechanical freezer Container should be imprinted with graphic instructions Requires only isopropyl alcohol for use Rigid vial holder keeps vials from contacting alcohol, preventing contamination by wicking Vial holder floats to allow thawing in a water bath Numbers molded for identification All components must withstand repeated freeze/thaw cycles Must be stackable Dimensions – H 86 mm x diam. 117 mm Holds 18 x 1.2 and 2.0 mL cryovials 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass beaker, high form (1000 mL) 2) Characteristics: Tall (high) form Graduated, with a spout Capacity – 1000 mL Made of glass Must be autoclavable at 121°C 3) Measurement unit: 1 pcs/pack 4) Requested quantity: 6 |  |  |  |
|  | 1) Product name: Glass Petri dish 100 mm Ø 2) Characteristics: Made of soda-lime glass Base diameter – 100 mm Height – 20 mm 3) Measurement unit: 36 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass Petri dish 150 mm Ø 2) Characteristics: Made of soda-lime glass Base diameter – 150 mm Height – 25 mm 3) Measurement unit: 36 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass Petri dish 80 mm Ø 2) Characteristics: Made of soda-lime glass Base diameter – 80 mm Height – 15 mm 3) Measurement unit: 24 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass syringes, with metal cone and Luer-Lock-fitting, 10 mL 2) Characteristics: Made of Borosilicate glass Precisely adapted piston Printed on brown graduation Autoclavable up to 134°C Volume – 10 mL Glass/metal cone Luer-Lock fitting 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass syringes, with metal cone and Luer-Lock-fitting, 20 mL 2) Characteristics: Made of Borosilicate glass Precisely adapted piston Printed on brown graduation Autoclavable up to 134°C Volume – 20 mL Glass/metal cone Luer-Lock fitting 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Graduated pipettes for tissue culture 10 mL 2) Characteristics: Made of glass With main-point ring graduation Short shape for use in sterile benches Brown graduated Bottom zero point Calibrated to Ex Volume – 10 mL Graduation – 0.1 mL 3) Measurement unit: 12 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Autoclavable inoculation loop holder 2) Characteristics: Must be autoclavable at 121°C Made of Stainless steel For inoculation loops with wire Ø 0.6–1.0 mm Loops are fixed with union nut Set of assorted colors: blue, yellow, red, black Length – 245 mm 3) Measurement unit: 4 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Autoclavable micropestle for microtubes 2) Characteristics: Made of polypropylene Autoclavable at 121°C One end is for breaking up solids in 1.5 mL microtubes, and the other end to be used in 0.5 mL microtubes Length – 73 mm 3) Measurement unit: 1 pcs 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Autoclavable microtube floating rack for 16 1.5 mL tubes 2) Characteristics: Made of polypropylene To heat or cool microtubes in a waterbath and avoid them to sink Stackable and with an alphanumerical identification of each position Autoclavable at 121°C 3) Measurement unit: 1 pcs 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Microtube rack, stainless steel for 1.5 mL Eppendorf microtubes 2) Characteristics: For Eppendorf 1.5 mL microtubes Stainless steel For 10 x 5 microtubes (1.5 mL) 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Reusable bottle-top filter holders, autoclavable, 45 mm thread 2) Characteristics: Made of PSF For screwing on vacuum-tight bottles with thread size 45 mm Suitable for vacuum filtration with 47 mm or 50 mm membranes With graduation Tube connection for tubes with inner-Ø 6-8 mm Funnel volume – 500 mL 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pipette brushes, total length 160 mm, brush length 70 mm, brush diameter 10 mm 2) Characteristics: Total length – 160 mm Brush length – 70 mm Brush diameter – 10 mm With undyed natural bristles Hard-wearing, highly resistant to chemicals, heat resistant to approx. 150 °C 3) Measurement unit: 5 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pipette brushes, total length 400 mm, brush length 100 mm, brush diameter 5/10/5 mm 2) Characteristics: Total length – 400 mm Brush length – 100 mm Brush diameter – 5/10/5 mm With undyed natural bristles Hard-wearing, highly resistant to chemicals, heat resistant to approx. 150 °C 3) Measurement unit: 5 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pipette brushes, total length 450 mm, brush length 180 mm, brush diameter 12/25 mm 2) Characteristics: Total length – 450 mm Brush length – 180 mm Brush diameter – 12/25 mm With undyed natural bristles Hard-wearing, highly resistant to chemicals, heat resistant to approx. 150 °C 3) Measurement unit: 5 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Pipette brushes, total length 490 mm, brush length 190 mm, brush diameter 4/20/4 mm 2) Characteristics: Total length – 490 mm Brush length – 190 mm Brush diameter – 4/20/4 mm With undyed natural bristles Hard-wearing, highly resistant to chemicals, heat resistant to approx. 150 °C 3) Measurement unit: 5 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: PES Concentrator 30K MWCO 2) Characteristics: 30K MWCO Disposable Ready-to-use The crossflow cassette filters and concentrates samples from 100 mL to 3 mL Thin channel flip-flow recirculation path No need for pressure holders High flow rate and recovery Modular and scalable to six units The membrane surface per device is 50 cm² Contains all necessary accessories for running two devices with a laboratory pump and a size 16 pump head (modules include a filtrate tube, size 16 peristaltic tubing, a flow restrictor, and fittings) 3) Measurement unit: 2 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: PES Concentrator 3K MWCO 2) Characteristics: 3K MWCO Disposable Ready-to-use The crossflow cassette filters and concentrates samples from 100 mL to 3 mL Thin channel flip-flow recirculation path No need for pressure holders High flow rate and recovery Modular and scalable to six units The membrane surface per device is 50 cm² Contains all necessary accessories for running two devices with a laboratory pump and a size 16 pump head (modules include a filtrate tube, size 16 peristaltic tubing, a flow restrictor, and fittings) 3) Measurement unit: 2 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: PES Concentrator 50K MWCO 2) Characteristics: 50K MWCO Disposable Ready-to-use The crossflow cassette filters and concentrates samples from 100 mL to 3 mL Thin channel flip-flow recirculation path No need for pressure holders High flow rate and recovery Modular and scalable to six units The membrane surface per device is 50 cm² Contains all necessary accessories for running two devices with a laboratory pump and a size 16 pump head (modules include a filtrate tube, size 16 peristaltic tubing, a flow restrictor, and fittings) 3) Measurement unit: 2 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Screw neck bottles, 1 L, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Ring-shaped reinforcement in the bottles' shoulder, shows the bottles' nominal content Volume – 1 L Thread – GL 45 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Screw neck bottles, 100 mL, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Ring-shaped reinforcement in the bottles' shoulder, shows the bottles' nominal content Volume – 100 mL Thread – GL 45 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Screw neck bottles, 2 L, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Ring-shaped reinforcement in the bottles' shoulder, shows the bottles' nominal content Volume – 2 L Thread – GL 45 3) Measurement unit: 1 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Screw neck bottles, 250 mL, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Ring-shaped reinforcement in the bottles' shoulder, shows the bottles' nominal content Volume – 250 mL Thread – GL 45 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Screw neck bottles, 50 mL, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Volume – 50 mL Thread – GL 32 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Screw neck bottles, 500 mL, with cap 2) Characteristics: Made of glass Autoclavable at 121°C Ring-shaped reinforcement in the bottles' shoulder, shows the bottles' nominal content Volume – 500 mL Thread – GL 45 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Berzelius beaker set 50, 100, 250, 500, 1000 mL 2) Characteristics: Contains one unit of each of the following capacities: 50, 100, 250, 500 and 1000 mL Graduated beakers with spout Made of borosilicate glass LBG 3.3. 3) Measurement unit: 1 set of 5 pcs 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Flat sieves with 1 mm Ø mesh size, 200 mm inner diameter 2) Characteristics: Mesh diameter size – 1 mm Inner diameter – 200 mm Height – 60 mm Made of Stainless steel 18/10 Suitable for separating powders, granulates and other bulk goods Easy to clean, dishwasher safe 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Slide holder for 15 slides 2) Characteristics: Color – white Made from chemical-resistant polyethylene Holding capacity – 15 slides Solid, strong construction with rubber feet for bench top stability 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Glass spectrophotometer cuvette with lid (0.35 mL) 2) Characteristics: Made of optical glass (LB OG) for use within 320 – 2500 nm Light path – 1 mm Internal width – 10 mm Volume – 0.35 mL 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Quartz spectrophotometer cuvette with lid (0.35 mL) 2) Characteristics: Made of quartz (LB Q) for use within 200 – 2500 nm (includes Ultraviolet) Light path – 1 mm Internal width – 10 mm Volume – 0.35 mL 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel forceps with fine tips 2) Characteristics: Made of stainless steel With fine tips 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 1) Product name: Stainless steel forceps with bent fine tips 2) Characteristics: Made of stainless steel With fine tips, bent 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel forceps for membranes and cover glasses 2) Characteristics: Made of stainless steel For membranes and cover glasses 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel forceps with straight end 2) Characteristics: Made of stainless steel With straight end 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel micro spatula, 150x4 mm 2) Characteristics: Made of stainless steel Length – 150 mm Width of spatula – 4 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with spoon, 120x15 mm 2) Characteristics: Made of stainless steel Length – 120 mm Width of spatula – 15 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with spoon, 150x18 mm 2) Characteristics: Made of stainless steel Length – 150 mm Width of spatula – 18 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with spoon, 175x17 mm 2) Characteristics: Made of stainless steel Length – 175 mm Width of spatula – 17 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with spoon, 210x20 mm 2) Characteristics: Made of stainless steel Length – 210 mm Width of spatula –20 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Chattaway stainless steel spatula, 200x7 mm 2) Characteristics: Made of stainless steel Length – 200 mm Width of spatula –7 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with double curved end, 150x11 mm 2) Characteristics: Made of stainless steel Length – 150 mm Width of spatula –11 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Stainless steel spatula with double curved end, 210x11 mm 2) Characteristics: Made of stainless steel Length – 210 mm Width of spatula –11 mm 3) Measurement unit: 5 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cuvette holder 2) Characteristics: For 12 cuvettes (path length 10 mm) Hight – 15 mm Length – 170 mm Width – 50 mm 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Empty tipbox with blue tip-tray for 1000 µL pipette tips, compatible with BRAND pipette tips 2) Characteristics: For 1000 µL pipette tips Compatible with BRAND pipette tips Blue tip-tray 3) Measurement unit: 1 pcs 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Empty tipbox with yellow tip-tray for 200 µL pipette tips, compatible with BRAND pipette tips 2) Characteristics: For 200 µL pipette tips Compatible with BRAND pipette tips Yellow tip-tray 3) Measurement unit: 1 pcs 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: TLC separation chamber 2) Characteristics: Made of glass With ground glass plate Versatile for use as a slide storage box, TLC separation chamber, etc. Length – 150 mm Width – 50 mm Hight – 150 mm 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: TLC sprayer 2) Characteristics: Versatile spray unit with gas cartridge for spraying reagents and other aqueous liquids Capacity – 170 mL Constant spray pressure approx. 4.4 bar Eco-friendly propellant mixture of dimethyl ether, propane and isobutane Sufficient for approx. 500 ml liquid Must include propellant cartridge (55 g or 94 mL), glass reservoir bottle 170 mL, suction tube and screw cap 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Electronic battery compatible with Transferpette pipettes from BrandTech 2) Characteristics: Must be compatible with BrandTech single or multichannel Transferpette pipettes 3.6V 730mAH 3) Measurement unit: 1 pcs 4) Requested quantity: 10 |  |  |  |
|  | 1) Product name: Electronic battery compatible with Transferpette pipettes from BrandTech 2) Characteristics: Must be compatible with BrandTech single or multichannel Transferpette pipettes 3.6V 730mAH 3) Measurement unit: 1 pcs 4) Requested quantity: 10 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 100 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,10 Stopper 14/23 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 1000 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,40 Stopper 24/29 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 200 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,15 Stopper 14/23 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 25 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,04 Stopper 10/19 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 5 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,04 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 50 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of ± 0,06 Stopper 12/21 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Volumetric flask class A with plastic stopper, 500 mL 2) Characteristics: Made of borosilicate glass Designed according to ISO 1042, DIN 12664 and calibrated to contain (TC, In) Accuracy (mL) of± 0,25 Stopper 19/26 3) Measurement unit: 2 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Wide-neck Erlenmeyer flasks 100 mL, glass, autoclavable (1 pack of 10 pcs) 2) Characteristics: According to DIN ISO 24450 Outer flask diameter 42 mm Outer neck diameter 31 mm 3) Measurement unit: 1 pack of 10 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Wide-neck Erlenmeyer flasks 50 mL, glass, autoclavable (1 pack of 10 pcs) 2) Characteristics: According to DIN ISO 24450 Outer flask diameter 51 mm Outer neck diameter 34 mm 3) Measurement unit: 1 pack of 10 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Refrigerator and freezer, 295 l, Class A+, H 185.3 cm, white 2) Characteristics: Color white Reversible doors Noise level: 42 dB Static cooling system Annual energy consumption 297kWh Autonomy (without electrical power) 18h Refrigerator net volume 205 L Number of shelves: 3 Number of doors: 4 Automated defrosting system Shelves material: secure glass Freezer net volume : 90 Number of compartments: 3 Manual defrosting system Fast freezing function Audio signal for open door Height 185.3 cm Width 59.5 cm Depth 60 cm Weight 61 kg 3) Measurement unit: pcs 4) Requested quantity:1 |  |  |  |
|  | 1) Product name: Pipette tips 0.5-20 µL, standard, type 22 (1 pack of 1000 pcs) 2) Characteristics: Bag of non-sterile, loose pipette tips compatible with Bran pipette Made of PP, autoclavabile High quality PP types, free from DiHEMDA and oleamide, Colorless, Length 46 mm CE marked in accordance with IVD Directive 98/79 EC Produced without plasticizer, cadmium-free colour pigments Free of DNA, RNase, Endotoxins and ATP 3) Measurement unit: 1000 units/pack 5) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Pipette tips 100-1000 µL, type 26, blue (10 packs of 500 pcs) 2) Characteristics: Bag of non-sterile, loose pipette tips compatible with Bran pipette Made of PP, autoclavabile High quality PP types, free from DiHEMDA and oleamide, Color blue, Length 70 mm CE marked in accordance with IVD Directive 98/79 EC Produced without plasticizer, cadmium-free colour pigments Free of DNA, RNase, Endotoxins and ATP 3) Measurement unit: 10 packs of 500 pcs 4) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Pipette tips 1-200 µL, premium line, sterile (10 racks of 96 pcs) 2) Characteristics: 1 - 200 μl universal type sterile in rack, natural color, autoclavable High quality tips with graduation lines of 10, 50 and 100 µl serving as a visual reference to minimize errors during sample aspiration. Human DNA-free and certified free from DNase, RNase and PCR inhibitors. 3) Measurement unit:10x96 pcs/rack 4) Requested quantity: 10 racks of 96 pcs |  |  |  |
|  | 1) Product name: Pipette tips 2-200 µL, yellow (10 packs of 1000 pcs) 2) Characteristics: Loose tips in bags, Gilson type, A, yellow, autoclavable 3) Measurement unit:10 packs of 1000 pcs 4) Requested quantity:3 |  |  |  |
|  | 1) Product name: Pipette tips 5-300 µL, type 25 (1 pack of 1000 pcs) 2) Characteristics: Bag of non-sterile, loose pipette tips compatible with Brand pipette Made of PP, autoclavabile High quality PP types, free from DiHEMDA and oleamide, Colorless, Length 53 mm CE marked in accordance with IVD Directive 98/79 EC Produced without plasticizer, cadmium-free colour pigments Free of DNA, RNase, Endotoxins and ATP 3) Measurement unit: 1 pack of 1000 pcs 4) Requested quantity: 20 |  |  |  |
|  | 1) Product name: Pipette tips 5 mL 2) Characteristics: Loose tips in bags, Gilson type, natural color, autoclavabile, compatible with HTL Discovery Comfort pipette 3) Measurement unit:250 tips/pack 4) Requested quantity: 15 |  |  |  |
|  | 1) Product name: Nitrile disposable gloves, S (10 pack of 100 pcs) 2) Characteristics: Powder-free, non sterile, 100% latex protein free, Ambidextrous, size S According to 89/686/CEE, EN 420, EN 374 1/2/3 and EN 388 standards 3) Measurement unit: 1set of 10 boxes of 100pcs 4) Requested quantity: 2 sets |  |  |  |
|  | 1) Product name: Nitrile gloves, M, powder-free (10 boxes of 100 pcs) 2) Characteristics: Powder-free, non sterile, 100% latex protein free, Ambidextrous, size M According to 89/686/CEE, EN 420, EN 374 1/2/3 and EN 388 standards 3) Measurement unit:1 set of 10 boxes of 100 pcs 4) Requested quantity: 2 sets |  |  |  |
|  | 1) Product name: Centrifuge tubes, with skirt, sterile (50 mL) (450 pcs) 2) Characteristics: Centrifuge tube, sterile, 50 mL, 30 x 115 mm, conical (V) bottom, support skirt, w/ graduations, I.D. field 3) Measurement unit: 450 pcs 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Centrifuge tubes, without vertical lip, sterile (15 mL) 2) Characteristics: Centrifuge tubes without rim, 15 ml, sterile in a bag Made of PP Highly transparent, cap made of PE With printed black graduations and white labelling area. Supplied with lid fitted. Certified nuclease-, endotoxin- and metal-free (<1 ppm), not cytotoxic. Temperature stable from -80 to +122 °C. Suitable to be centrifuged in fixed angle rotor up to 17,000 x g Outer diameter 17 mm Height 120 mm 3) Measurement unit: 500 pieces/pack 4) Requested quantity: 5 |  |  |  |
|  | 1) Product name: Centrifuge tubes, without vertical lip, sterile (50 mL) 2) Characteristics: Centrifuge tubes without rim, 50 ml, sterile in a bag With printed black graduations and white labelling area. Supplied with lid fitted. Certified nuclease-, endotoxin- and metal-free (<1 ppm), not cytotoxic. Temperature stable from -80 to +122 °C. Can be centrifuged in fixed angle rotor up to 20,000 x g Outer diameter 29 mm Height 115 mm 3) Measurement unit: 500 pieces/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Reaction vials with lid lock (1.5 mL) 2) Characteristics: Made of PP, autoclavable Additional lid lock for extra tightness, prevents unintentional opening of the lid. The wide lid rim allows easy handling using only one hand. Centrifugability: 30,000 rpm Graduated with frosted labelling space Manufactured from high quality medical grade polypropylene Produced under clean-room conditions to ISO 14 644-1 Class 8 Free of RNase, DNase, DNA and endotoxins 3) Measurement unit: 1000 pieces/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Reaction vials with lid lock (2 mL) 2) Characteristics: Made of PP, autoclavable Additional lid lock for extra tightness, prevents unintentional opening of the lid. The wide lid rim allows easy handling using only one hand. Centrifugability: 30,000 rpm Graduated with frosted labelling space Manufactured from high quality medical grade polypropylene Produced under clean-room conditions to ISO 14 644-1 Class 8 Free of RNase, DNase, DNA and endotoxins Outer diameter 12.8 mL Height 40 mm 3) Measurement unit: 500 pieces/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: MINI Tank Blotting System 2) Characteristics: For the gentle transfer of proteins and nucleic acids Features: Electro tank blotting system for blotting of up to 4 gels in parallel Optimised hinged cassettes - for most easy handling and rapid blot set up Delivery must include MINI buffer tank, safety lid with cables and 1 MINI blotting module for max. 4 gels, incl. 4 cassettes and 16 fibre blotting mats. Module compatible with the vertical electrophoresis system ROTIPHORESE®PROclamp Blotting area: maximum 4 gels of 10 × 10 cm 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: PCR Purification Kit 2) Characteristics: The kit must be used for purification of DNA fragments from 25 bp to 20 kb with recovery rates up to 100%. Sample type: dsDNA Efficiency: 90 to 100% recoveries in the range of 100 bp to 10 kb Suitable for the removal of primers, dNTPs, unincorporated labeled nucleotides, enzymes, and salts from PCR and other reaction mixtures. 3) Measurement unit: 250 preps 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Plasmid Miniprep Kit 2) Characteristics: The kit must recover up to 20 µg of high copy plasmid DNA per isolation procedure. Efficiency: high yields of up to 20 µg of high quality plasmid DNA High Throughput Compatibility: Compatible (Vacuum manifold) Sample type: Bacteria 3) Measurement unit: 250 preps 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: MINI Vertical Dual Plate Electrophoresis Unit standard 2) Characteristics: Vertical Double Electrophoresis Units with Clamping System General features: For running of up to 4 gels in parallel Extensive range of accessories available incl. glass plates with fixed spacers and blotting modules Colour coded spacers and combs in up to 5 thicknesses available – 0,5 up to 2 mm Compatible with multi-channel pipettors (combs are marked *) Modular system: Tanks are compatible with different electrophoresis modules, keeping costs and space to a minimum. Electrophoresis chamber with accessories – ready for use Compatible with all major brands of 10 × 10 cm to 8 × 10 cm precast gels With fast-clamping technique Up to 20 samples per gel Effective buffer cooling independent of running water or chillers Delivery must include 1 vertical electrophoresis chamber incl. buffer tank, safety lid and 2 cables (4 mm plugs), 1 running module, 1 dummy plate for run of one gel only, 1 gel cooling pack, 2 glass plates with fixed spacers (thickness 1 mm), 2 notched glass plates, 2 combs (thickness 1 mm, 12 wells), 1 gel opener, 1 gel casting module for rapid and leak-proof gel casting. Dimensions: 10x10 cm Max. number of samples: 80 Buffer volume: 100/300 mL Dimensions: 13x19x15 cm Suitable for agarose, polyacrylamide gel 3) Measurement unit: pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 96-well black microplate for fluorescence clear bottom (1 pack of 48 pcs) 2) Characteristics: Flat bottom clear, black polystyrene plate (Ideal for fluorescent assays), matrix active group TC-treated, pkg of (individually wrapped), sterile, lid Bottoms are 60% thinner than conventional polystyrene plates, resulting in lower background fluorescence and enabling readings down to 340 nm Opaque walls to prevent well-to-well crosstalk Optically clear flat well bottom permits direct microscopic viewing Can be used for both top and bottom reading instruments Flat bottoms with 360 μL total volume Recommended working volumes of 75 to 200 μL Nonreversable lids with condensation rings to reduce contamination Sterilized by gamma radiation and certified nonpyrogenic Individually wrapped Sterile polystyrene plates, with lid, round clear wells Surface area 0.32 cm2 (cell growth area) Suitable for fluorescent/luminescent assays 3) Measurement unit: 1 pack of 48 pcs, individually wrapped 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: microtest plates, black 2) Characteristics: Made of PS. Black plates for low background and reduced light scatter during applications in fluorescence microscopy. Alphanumerically marked cavities with flat bottom "Chimney top" well design reduces the risk of contamination SBS dimensions Free of RNase, DNase and human-DNAi 96 Wells plate with capacity of 0.35 mL, color black 3) Measurement unit:100 pieces/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: 96 well white plates 2) Characteristics: 96 well plate, polystyrene, High Bind, white flat bottom wells, non-sterile, white, 100/cs White microplates enhance luminescent signals and have low background luminescence and fluorescence Designed to reduce well-to-well crosstalk High binding surface is capable of binding medium (>10kD) and large biomolecules that possess ionic groups and/or hydrophobic regions Binding capacity is approximately 400 to 500ng IgG/cm2 Ideal for immunoassays Without lids, lids supplied separately Total volume is 360μL; recommended working volume of 75 to 200μL Conforms to standard microplate footprint and dimensions 96 wells White polystyrene plate, nonsterile, no lid Matrix active group: high bind surface treatment 3) Measurement unit: 100 pieces/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Quartz Microplates, 96 well (1 pcs) 2) Characteristics: Provides for chemical resistance, UV transmission, microscopic observation and thermal stability. 96 wells Even well base, less than the wavelength of light Each well completely liquid guaranteed Color clear Well diameter 6.6 mm Depth 12.58 mm Material: synthetic quartz glass Well volume 300 µL 3) Measurement unit: 1 pcs 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Syringe filters 0.22 um, CA, sterile, 25 mm diameter (1 pack with 50 pcs) 2) Characteristics: Membrane made of Cellulose Acetate (CA) inherently hydrophilic. Low protein retention. High mechanical strength. Housing: Polypropylene (PP), fuchsia color. Designed with connectors: FLL/MSL. Sterile reference is individually packed and sterilized by Gamma Radiation Outer diameter 25 mm, sterile Pore 0.22 µm 3) Measurement unit: 1 pack with 50 pcs 4) Requested quantity: 10 |  |  |  |
|  | 1) Product name: Syringe filters 0.45 um, CA, sterile, 25 mm diameter (1 pack with 50 pcs) 2) Characteristics: Membrane made of Cellulose Acetate (CA) inherently hydrophilic. Low protein retention. High mechanical strength. Housing: Polypropylene (PP), fuchsia color. Designed with connectors: FLL/MSL. Sterile reference is individually packed and sterilized by Gamma Radiation Outer diameter 25 mm, sterile Pore 0.45 µm 3) Measurement unit: 1 pack with 50 pcs 4) Requested quantity: 5 |  |  |  |

# Category 6. Consumables for laboratory use (according with the detailed list)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 1) Product name: Indicator tape for sterilization 2) Characteristics: Self-adhesive tape with impregnated paper turning to color when exposed to sterilization process Tape turns from white to brown when exposed at 120°C for 20 minutes, at 125°C for 10 minutes or at 135°C for 5 minutes 55 m x 19 mm roll 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Bottle-top filters with PES-membrane, 0.2 µm, 500 mL 2) Characteristics: Pore size – 0.2 µm Membrane diameter – 75 mm Funnel volume – 500 mL Filter with PES membrane Screw – 45 mm or GL 45 A leakproof screw cap Blue color coding Graduated filter upper section Quick connector tube adapter for tubes with internal Ø 6–8 mm Leakproof screwed connection for vacuum filtration Filtration units are non-pyrogenic and non-cytotoxic 3) Measurement unit: 12 pcs/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Bottle-top filters with PES-membrane, 0.45 µm, 500 mL 2) Characteristics: Pore size – 0.45 µm Membrane diameter – 75 mm Funnel volume – 500 mL Filter with PES membrane Screw – 45 mm or GL 45 A leakproof screw cap Blue color coding Graduated filter upper section Quick connector tube adapter for tubes with internal Ø 6–8 mm Leakproof screwed connection for vacuum filtration Filtration units are non-pyrogenic and non-cytotoxic 3) Measurement unit: 12 pcs/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Cap inserts for cryogenic vials, different colors 2) Characteristics: Assorted colors Suitable for 1.2 to 5 ml cryogenic vials For color-coding and easier identification of samples 3) Measurement unit: 500 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cell culture bottles with vent (filter) screw cap, sterile, 25 cm2 2) Characteristics: Vent (filter) type stopper Growth area – 25 cm2 Volume – 50 mL Made of USP VI crystal class virgin polystyrene in a 100000-grade clean room Treated surface Flat base which is free from striation to maximize the usable growth area Polyethylene cap with vented cap with a 0.22-micron hydrophobic membrane to allow gas exchange minimizing the risk of cross contamination Angled neck design to improve access of pipettes and cell scrapers Engraved graduation on both sides and writing area next to the neck Sterilized by gamma irradiation and certified free of DNase, RNase and pyrogens Packed in resealable bags 3) Measurement unit: 200 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cell culture bottles with vent (filter) screw cap, sterile, 75 cm2 2) Characteristics: Vent (filter) type stopper Growth area – 75 cm2 Volume – 250 mL Made of USP VI crystal class virgin polystyrene in a 100000-grade clean room Treated surface Flat base which is free from striation to maximize the usable growth area Polyethylene cap with vented cap with a 0.22-micron hydrophobic membrane to allow gas exchange minimizing the risk of cross contamination Angled neck design to improve access of pipettes and cell scrapers Engraved graduation on both sides and writing area next to the neck Sterilized by gamma irradiation and certified free of DNase, RNase and pyrogens Packed in resealable bags 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: TC-surface treated cell culture plate, sterile, 96 wells, with cover plate 2) Characteristics: 96 wells cell culture plates TC-surface treated Sterile Translucent With cover Made of PS Multiwell plates for adherent cell cultures F-bottom Alphanumerical well coding Centrifugable to max. 4800 x g Free of detectable DNase, RNase and human DNA Free of detectable endotoxins, free of cytotoxic substances 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Treated surface culture plate, sterile, 12 wells, with cover plate 2) Characteristics: 12 wells culture plates Working volume – 076-1.14 mL Growth area – 3.85 cm2 Surface treated Sterile Translucent With lids included Flat bottom Packed in individual bags Made of P USP VI crystal class virgin polystyrene in a 100000-grade clean room Uniform volume and growth surface in all wells Smooth well surface which is free from striation to maximize the usable growth area Wells are labeled with alphanumeric code for easy identification Sterilized by gamma irradiation and certified free of DNAse, RNAse and pyrogens 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Treated surface culture plate, sterile, 24 wells, with cover plate 2) Characteristics: 24 wells culture plates Working volume – 0.38-0.57 mL Growth area – 1.93 cm2 Surface treated Sterile Translucent With lids included Flat bottom Packed in individual bags Made of P USP VI crystal class virgin polystyrene in a 100000-grade clean room Uniform volume and growth surface in all wells Smooth well surface which is free from striation to maximize the usable growth area Wells are labeled with alphanumeric code for easy identification Sterilized by gamma irradiation and certified free of DNAse, RNAse and pyrogens 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Treated surface culture plate, sterile, 48 wells, with cover plate 2) Characteristics: 48 wells culture plates Working volume – 0.19-0.29 mL Growth area – 0.83 cm2 Surface treated Sterile Translucent With lids included Flat bottom Packed in individual bags Made of P USP VI crystal class virgin polystyrene in a 100000-grade clean room Uniform volume and growth surface in all wells Smooth well surface which is free from striation to maximize the usable growth area Wells are labeled with alphanumeric code for easy identification Sterilized by gamma irradiation and certified free of DNAse, RNAse and pyrogens 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Treated surface culture plate, sterile, 6 wells, with cover plate 2) Characteristics: 6 wells culture plates Working volume – 1.9-2.9 mL Growth area – 9.6 cm2 Surface treated Sterile Translucent With lids included Flat bottom Packed in individual bags Made of P USP VI crystal class virgin polystyrene in a 100000-grade clean room Uniform volume and growth surface in all wells Smooth well surface which is free from striation to maximize the usable growth area Wells are labeled with alphanumeric code for easy identification Sterilized by gamma irradiation and certified free of DNAse, RNAse and pyrogens 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cell scraper, 250 mm, sterile 2) Characteristics: Blade width – 2 mm Total length – 250 mm Two blade positions: “scraper” and “lifter” Made of thermoplastic elastomer (TPE) Flexible ABS handle, specially designed to collect cells without damaging Sterilized by gamma irradiation and certified free of DNAse, RNAse and pyrogens Packaged in individual bags 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cell strainer 100 µm for 50 mL centrifuge tube, sterile 2) Characteristics: Blade width – 2 mm Mesh size – 100 µm Color – yellow Made of PP Filter membrane made of PET Fits all standard 50 ml tubes With handle and mantle surface for improved aseptic handling Venting slot for rapid filtration, no overflowing of the cell strainer Individually packed in blister pack For the filtration of cell suspensions and primary cell isolates, e.g. after organ digestion, for use in primary cell extraction or cell preparation for flow cytometry 3) Measurement unit: 50 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Nylon syringe filter. non-sterile, 0.2 µm 2) Characteristics: Pore size – 0.2 µm Membrane diameter – 25 mm Housing diameter – 30 mm Housing material – PP Autoclavable at 121°C PA (nylon) membrane for the filtration of medium polar organic-aqueous liquids 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Cryogenic vials, with outer thread, sterile, autoclavable, 2 mL, freestanding 2) Characteristics: Volume – 2 mL Type – Lip seal + silicone seal Lock type – Lip seal Outer diameter – 12.5 mm Height – 48 mL Temperature-stable up to -196 °C 1¼ turn for quick cap opening and closing High transparency, sample readily visible Round inside bottom, contents empty completely Gamma-sterilized RNase/DNase/DNA-free and pyrogen-free Suitable for storage of cell cultures, serums, blood components, bacteriological cultures and other liquids at low temperatures in refrigerators or nitrogen vessels. 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Filter paper, medium filtering, 90 mm diameter 2) Characteristics: Diameter – 90 mm Type – disk Made of superior Vellum paper made of pure cellulose and low ash (max 0.06%) Surface weight – 85 g/m² Pore size – 10 – 12 μm Thickness – 200 μm 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Inoculation loops, disposable, sterile, 1 µL, 10 pcs/zip-lock pack 2) Characteristics: Loop volume – 1 µL Made of HIPS plastic Hexagonal shaft with stripes Sterile White 10 pcs/zip-lock pack 3) Measurement unit: 100 packs of 10 loops 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Inoculation loops, disposable, sterile, 10 µL, 10 pcs/zip-lock pack 2) Characteristics: Loop volume – 10 µL Made of HIPS plastic Hexagonal shaft with stripes Sterile Dark blue 10 pcs/zip-lock pack 3) Measurement unit: 100 packs of 10 loops 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: L-shaped bacterial cell spreaders 2) Characteristics: Total length – 146 mm Width – 38 mm Diameter – 4 mm With smooth surface, prevents scratching for agar surfaces 3) Measurement unit: 500 pcs/pack 4) Requested quantity: 3 |  |  |  |
|  | 1) Product name: Membrane filters 0.20 µm for bottle top filter, cellulose nitrate, sterile 2) Characteristics: Pore size – 0.20 µm Diameter – 47 mm Sterile Made of Cellulose nitrate, hydrophilic High stability and flexibility, low extractable substances content Max. usage temperature 80°C 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Membrane filters 0.45 µm for bottle top filter, cellulose nitrate, sterile 2) Characteristics: Pore size – 0.20 µm Diameter – 47 mm Sterile Gridded Made of Cellulose nitrate, hydrophilic 3) Measurement unit: 200 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Microscope slide-grids, disposable 2) Characteristics: Pore size – 0.20 µm Suitable for use with dry or suspended specimens Made of polyester with an acrylic adhesive Easy-to adhere stickers having side-by-side twin grids (20 mm x 20 mm each with 1 mm line spacing) for microscope slides 3) Measurement unit: 1 pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Microtitration plates, 96 wells, sterile, with cover plate 2) Characteristics: Plate with 96 wells Cover plates included Translucent Sterile Made of PS F-profile (flat-shaped base) Volume – 0.39 mL Condensation rings in the lid reduce cross-contamination Alphanumeric numbering Gamma-sterilized Non-pyrogenic 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 5 |  |  |  |
|  | 1) Product name: Parafilm M sealing film 2) Characteristics: Size – 38 m x 10 cm Sealing film made of polyolefins and paraffin waxes Service temperature from –45 ºC up to +50°C (melting point +60°C). Resistant to saline solutions, most common inorganic acids and bases used in laboratory and some organics solvents (methanol, ethanol and 2-propanol) 3) Measurement unit: 12 rolls/pack 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: Polystyrene weighing pans, 89x89x24 mm 2) Characteristics: Size – 89x89x24 mm Volume – 100 mL Square shape 3) Measurement unit: 500 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Reagent reservoir for multichannel pipettes, autoclavable, 60 mL, non-sterile, with lid 2) Characteristics: Volume – 60 mL Reagent reservoirs for use with multichannel pipettes Made of translucent polypropylene Autoclavable Non-sterile With lid included 3) Measurement unit: 10 pcs/pack 4) Requested quantity: 5 |  |  |  |
|  | 1) Product name: PCR strips of 8 tubes 2) Characteristics: 8-tube PCR strips Volume – 200 µL each tube from the strip Numbering 1 to 8 for sample differentiation Colorless Thin-walled for optimal heat transfer between thermocycler and sample Suitable for most standard thermocyclers Containers with flat lids, suitable for real-time PCR Free of DNase, RNase and human DNA 3) Measurement unit: 125 strips/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Sealing film for cell culture plates, sterile 2) Characteristics: Sterile Dimensions – 142.9x82.6 mm Gas-permeable, for even air and CO2 exchange Pierceable Temperature stability - -20 to +80°C Made of Cellulose (plastic), self-adhesive Hydrophobic sealing film for cell culture and deep-well plates Minimizes evaporation and cross-contaminations Perforated ends for simple and fast adhesion Not cytotoxic 3) Measurement unit: 50 pcs/pack 4) Requested quantity: 4 |  |  |  |
|  | 1) Product name: Individual PCR vials, 0.2 mL, autoclavable 2) Characteristics: Colorless Volume – 0.2 mL Autoclavable at 121°C Made of PP Thin-walled for optimal heat transfer between thermocycler and sample Suitable for all current thermocouples Containers with flat lids, suitable for real-time PCR Free of DNase, RNase and human DNA 3) Measurement unit: 1000 pcs/pack 4) Requested quantity: 2 |  |  |  |
|  | 1) Product name: TLC saturation pads (20cmx20cm) 2) Characteristics: Dimensions – 20x20 cm Placement of pads with solvent in the development chamber helps to assure rapid and uniform saturation of the atmosphere with solvent vapor. Pads improve reproducibility and reduce "edge effects" 3) Measurement unit: 100 pcs/pack 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: TLC ready-to-use foils / plates with standard silica gel 2) Characteristics: Dimensions – 5x7.5 cm Layer thickness – 0.2 mm Made of Aluminum foil, silica gel 60 layer, with fluorescence indicator Specific surface: approx. 500 m2/g Medium pore width: 60 Å Specific pore volume: 0,75 ml/g Particle size: 5–17 µm 3) Measurement unit: 20 pcs/pack 4) Requested quantity: 1 |  |  |  |

# Category 7. Culture media for microorganisms and organisms, for ecotoxicological studies (according with the detailed list)

| 1. Item No | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 1) Product name: Acidithiobacillus ferrooxidans (Temple and Colmer 1951) Kelly and Wood 2000 2) Characteristics: Synonyms: Thiobacillus ferrooxidans Temple and Colmer 1951 Isolated from mine drainage water Collection no.: DSM 11477 3) Measurements unit: actively growing culture 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Aspergillus niger van Tieghem 2) Characteristics: Isolated from soil; mineral oil contaminated Collection no.: DSM 11167 3) Measurement unit: freeze dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Alivibrio fisheri 2) Characteristics: Synonyms: Photobacterium fischeri (Beijerinck 1889) Reichelt and Baumann 1973 Vibrio fischeri (Beijerinck 1889) Lehmann and Neumann 1896 Collection no.: CIP 105356, NRRL B-11177, DSM 7151 3) Measurement unit: freeze dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Bacillus cereus Frankland and Frankland 1887 2) Characteristics: Collection no. F 4810/72, NCTC 11143, DSM 4312 3) Measurement unit: freeze dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Dulbecco’s Modified Eagle’s Medium/High Modified powder 2) Characteristics: With 4500 mg/L dextrose, with 4.0 mM L-glutamine, with 110 mg/L sodium pyruvate, without sodium bicarbonate, powder Components: sodium pyruvate: yes L-glutamine: yes phenol red: yes glucose: high Suitable for cell culture/mammalian: suitable 3) Measurement unit: 1 pack for 50 L 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Enterococcus faecium 2) Characteristics: Full name: Enterococcus faecium (Orla-Jensen 1919) Schleifer and Kilpper-Bälz 1984 Isolated from human feces of healthy volunteer Collection no: DSM 25389 3) Measurement unit: Freeze Dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Enterobacter cloacae 2) Characteristics: Full name: Enterobacter cloacae (Jordan 1890) Hormaeche and Edwards 1960 Isolated from odour closure device in hospital Collection no.: DSM 106614 3) Measurement unit: Freeze Dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Fetal Bovine Serum, qualified, E.U.-approved, South America origin 2) Characteristics: Fetal Bovine Serum, E.U. Approved (South American Origin), 500mL, Liquid, –5 to –30°C Storage Temperature, Fetal, Cattle/Bovine Species, Standard (Sterile-Filtered) Serum Treatment, Dry Ice, Virus and Mycoplasma Tested Source: Cattle/Bovine 3) Measurement unit: 500 mL 4) Requested quantity: 5 |  |  |  |
|  | 1) Product name: Klebsiella pneumoniae 2) Characteristics: Full name: Klebsiella pneumoniae (Schroeter 1886) Trevisan 1887 Synonyms: Klebsiella pneumoniae subsp. pneumoniae (Schroeter 1886) Ørskov 1984 Isolated from clinical material, wound swap Collection No: DSM 103696 3) Measurement unit: Freeze Dried 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Leptospirillum ferrooxidans 2) Characteristics: Full name: Leptospirillum ferrooxidans (ex Markosyan 1972) Hippe 2000 Isolated from copper deposits Collection no.: ATCC 29047, VKM B-1339, DSM 2705 3) Measurement unit: actively growing culture 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Mitomycin C 2) Characteristics: ≥98 %, for biochemistry Covalent linkage to DNA. Causes cross-linking of complementary DNA-strands and strand breakage. Molar mass (M) 334,30 g/mol Boiling point (bp) 534 °C Melting point (mp) >360 °C 3) Measurement unit: glass bottle of 2 mg 4) Requested quantity: 2 pcs |  |  |  |
|  | 1) Product name: Synechococcus rubescens 2) Characteristics: Phototropic, non-axenic Isolated from sediment collection no.: DSM 104176, CCY15108 3) Measurement unit: actively growing culture 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Trypsin 2) Characteristics: 5.000 USP-U/mg Molar mass (M) ~24,000 g/mol Serine protease isolated from porcine pancreas, lyophilised. 3) Measurement unit: glass bottle of 500 mg 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Automated colony counter with screen 2) Characteristics: Minimum size of detected colony: 0.1 mm Camera: 1 megapixel - zoom x 28 Counting on round Petri dishes of Ø 55 - 90 mm Counting on pour, surface, Spiral, circle mode plated dishes TECHNICAL SPECIFICATIONS All stainless steel body CMOS color camera Lens M12 Digital zoom x28 (enabling to read the letters on the disks) Resolution : 1 mega-pixels Counting time: 1000 colonies per second Minimum size of colony: 0.1 mm Lighting system: long-life white LEDs / Dark Field Lighting system: automatic 6 combinations, top and/or bottom light, white or black background Counting: automatic with manual control Results / data export: PDF report, jpeg, png, bmp, Excel™ LIMS connection USB connection Counting on Petri dishes (55 - 90 mm) Spiral® plating Automatic separation of confluent colonies Automatic elimination of counting grids Polygonal exclusion areas Software available in English (and Romanian, if possible) Power : 100-240V~ / 50-60Hz (compatible with Romanian power supply) Processor: (quad core recommended) Intel i7 (or AMD FX 6000 or superior) Operating system: Windows Frequency: 2.4 GHz (or superior) RAM:8 GB (or superior) Equipment: 2.0 USB port Screen: 1280 X 1024 pixels (or superior) Graphic card: AMD or NVIDIA (chipsets are not recommended) Guarantee (after registration): 3 year Software update (after registration): 3 year In compliance with CFR 21 part 11, EUCAST, CA-SFM, ISO 7218, ISO 4887-2 Manufactured under: ISO 9001 V2008, CE, WEEE 2002/96/EC, RoHS 2002/95/EC standards Delivered with : Software on CD-ROM Connection cables 2 control dishes User’s manual, IQ/OQ/PQ guidelines 3) Measurement unit: pcs. 4) Requested quantity: 1 |  |  |  |
|  | 1) Product name: Package of DNA synthesis 2) Characteristics: Get ready to use clones in any vector, commercial or customized Electronic sequence for gene synthesis Vector details (map) & cloning strategy Mail in an aliquot of your vector 4 μg (1 μg for low-copy plasmid) lyophilized plasmid DNA of insert in a standard vector PLUS 4 μg (1 μg for low-copy plasmid) lyophilized plasmid DNA of insert in your chosen vector sequence chromatograms, plasmid construct map, and QC data 3) Measurement unit: 1 sequence (20-1500 bp) 4) Requested quantity: 10 |  |  |  |
|  | 1) Product name: BCA protein assay kit 2) Producer: Merck 2) Characteristics: Kit suitable for 500 assays Suitable for determining protein concentration in the range of 20-2000 µg/ml in either a standard assay or microassay configuration Must include a BSA standard (2 mg/ml) for convenient and reliable preparation of standard curves 3) Measurement unit: kit for 500 assays 4) Requested quantity: 1 kit |  |  |  |