# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Acquisition of Vehicles	p 1 /…

Publication reference : 365/PUCCGZS/T3

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 1. Ground Vehicles

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| L1.1 | Off-road pick up van 1 pcs - Max engine power: 100-110KW - Engine displacement: max. 2400 ccm - Engine type: diesel, EURO VI or better - Transmission: 2WD with 4WD selection - Combined fuel consumption: max 7l/100km - Cabin type: 4 doors - AC device - Day lights - Central locking system with remote - Forward and backward parking sensors - Radio device - Electrical motor lifting of side windows forward and backward - Minimal safety equipment: ABS, EBD - Servo assisted steering wheel - Wheels: min 225/70 R17 - Vehicle length: min 5100mm - Vehicle width: min 1800mm - Vehicle height: min 1800mm |  |  |  |
| L1.2 | Stake body double cabin truck 1pcs - Gross mass of vehicle: max 3500kg - Max engine power: min 80KW - Engine displacement: 2000-2400 ccm - Engine type: diesel, EURO VI or better - Transmission: 2WD on rear wheels - Wheelbase: 3400 - 3450 mm - Cabin type: 4 doors - Number of seats 1+2+4 - AC device - Central locking system - Radio device - Electrical motor lifting of side windows - Minimal safety equipment: ABS, ESP, assistance for uphill start - Disc brakes on all wheels - Servo assisted steering wheel adjustable on two axis - Wheels: min 225/65 R16 - Superstructure type: cargo trunk with minimal length 2860mm and minimal width 2100mm |  |  |  |
| L1.3 | Backhoe-loader 1 pcs - Description and purpose of vehicle: Combined backhoe loader/trencher, with stiff chassis, with frontal load bucket and backward backhoe telescopic arm with trench digging bucket. - Operation weight: min 9.500kg - Engine power: 70-80 KW - Engine type: diesel, Stage III or better - Transmission: Automatic transmission, 4WD/2WD selective by operator - Size tyres: min 28” - Wheelbase: max 2300mm - Max width of machine: 2450mm - Maximal height of machine in transport mode: 3600mm - Ground clearance: min 0.35m - Top speed road: min 35 kmh - Hydraulic pump max flow: min 140l/min - Cabin protection from rolling over and falling objects in line with: ROPS-u and FOPS standards - Min two electronic keys with theft protection system - Forward and backward working lights - Orange warning rotation lights mounted on cabin, - Backward moving warning sound system - Backhoe bucket with teeth width: 600mm - Backhoe Max Dig Depth: min 5.7 meters - Backhoe Max Reach From Swivel: min 6850 mm - Backhoe installation for possibility of mounting of hydraulic hammer - Loader bucket ”6 in 1” with forks, capacity: min 1.1m3 - Loader clearance at max height: min 3250mm - Loader lifting capacity at max height: 3150kg Important notice: All ground vehicles must be delivered with all required equipment and accessories necessary by traffic legislation of Republic of Serbia as well documentation necessary for registration for traffic use. |  |  |  |

LOT 2. Professional drones with accessories

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| L2.1 | Two drones with accessories in total 1set consisting of: Unmanned Aerial Vehicle with RTK module - 2pcs - Type of aerial vehicle: multi rotor - Type of propulsion system: electric or gasoline or alcohol or similar - Max speed without wind: min 10 m/s - Max wind speed allowed for operation: min 9m/s - Flight autonomy: min 25 minutes - Capable to use GPS and GLONASS systems - Horizontal and vertical positioning accuracy, with RTK enabled: max 0.1m error - Capable of return to home and safe landing on autopilot. - Capable of detecting and preventing obstacles collision at least in forward direction. - Camera: min resolution 20Mpix, capable to record 4K video in H264 or similar format, with external SD card of min 128Gb included, with ND filter included, lens Field of View: min 80o, equipped with mechanical shutter capable for 1/2000s speed or faster - Gimbal stabilization: 3-axis (tilt, roll, yaw) - Capable to fly between programmed waypoints while performing photogrammetry. - Remote Controller included for each drone with LCD/LED, 5,5” FullHD resolution daylight viewable screen or better, with min. 5km range in unobstructed interference-free conditions. - 1 spare batteries for drones and 1 spare batteries for remote included for each drone - 4 spare propellers included for each drone - Drone operating temperature range: 0oC to 40oC or better Software for photogrammetric modelling - 1 PC licence - Stand-alone PC software product for photogrammetric processing of digital images and videos, to generate textured 3D models - Capable to generate, edit, classify and export dense point clouds, and to generate textured 3D models - Capable to produce georeferenced results in GeoTIFF format - Capable to perform processing based on recordings from drone and terrestrial cameras Laptop for drone management 2 pcs CPU: - Number of cores min 6 - Number of threads min 12 - Cache memory: min 12 MB - TDP: max 45W RAM memory: - min 32 GB DDR4 or better GPU: - Core speed: min 1GHz - Bus width: min 128b - RAM: min 4096MB GDDR5 or better - Directx version supported: min 12 HDD: - min 500 GB type M2 NVME or better Ports: - min 2 USB - min 1 HDMI or DisplayPort or similar Web camera: - included Screen: - 15.6”, FullHD or better Aux: - Wireless mouse, transport backpack Important notice: The UAV system set should contain software and equipment required for regular operation and user maintenance, including but not limited to battery chargers, cables, specialized tools and user serviceable parts. Important notice: The UAV system specific operation and user maintenance training, for 3 operators, will be organized by Contractor before Provisional acceptance is signed by Contracting Authority. Operators will receive training to safely utilize contracted UAV system for recording of landscape and buildings. Important notice: If the UAV is subject to certification or registration by Civil Aviation Directorate of the Republic of Serbia, Contractor will provide Contracting Authority support in the process. |  |  |  |