# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Solar Illumination	p 1 /…

Publication reference : 481/CityofKikinda/TD1

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Solar Illumination set: 280 sets consisting of: Luminaries, having the following characteristics: - LED lamp power min 30W - LED lamp colour (Kelvin) 6000K - 6500K - LED lamp lumen efficiency min 160 lm / W - Solar panel integrated on top of luminaries, mono crystalline type with all electronic necessary for recharging batteries - LiFePO4 battery type (integrated in luminaries or in the foundation of the pillar) or better in terms of energy density and longer cycle life should be offered. Battery with Cobalt alloys will not be considered acceptable due environmental issues. - Has a micro wave detector to activate the lighting when there is movement in its vicinity - The ability to automatic dimming levels to reduce power consumption when there is no movement (but no to less than 400 lumens) - Battery life operation time with weak charging on bad weather should be min 24 hours with dimming - Charging battery time on sunny day less than 8 hours - Aluminium alloy casing resistant to outdoor elements. - Installation at a height of min 5meters or higher if luminaries optics provide dissipation of light less than 120o to cover distances between two poles. Pillars, having the following characteristics: - Height: min 5m above ground (depending of luminaries offered) - Material: galvanized steel with the electrical grounding for protection of bystander against lightning bolts |  |  |  |
| 2 | Video surveillance set: 1 set consisting of: Camera: 5pcs - Supports 180° Panoramic View - Resolution: min 15Mpix - Interface type: IP camera - With automatic IC lights - Capable to operate in colour mode with illumination of low as 0.05lux or better - To have integrated H264 or equivalent codec and to be capable for streaming in 20fps - AI capable to classify objects by shape (e.g peoples, vehicles) - AI detection of objects crossing virtual lines - AI detection of stopped objects. - AI detection of overtime delay of an object in the area of interest - Tamper detection - Secured from unauthorized access Network video recorder: 1pc - To be installed in premises of City of Kikinda - HDD capable to hold 30 days of recording, to be “Hot swappable” and in RAID5 configuration - Maximum bandwidth during simultaneous recording, live viewing or reproduction greater than 800 Mbps - Capable to be remotely connected online - Capable to support AI functionality of cameras and Video Management Software (VIM) Video Management Software (VMS): 1 licence - Must support client server architecture configuration with support for 100+ cameras - To have unlimited number of client licenses - To have database optimized for operating with high resolution surveillance cameras - To support alarm based on artificial intelligence, digital camera I / O or I / O modules. - The VMS must provide the ability to manage alarms and the alarm escalation mechanism so that the operator's level 1 does not respond within a specified time, to automatically send the same alarm to a higher level. - VMS must provide the Mobile App for iOS and Android devices with push-alarm notifications (send and receive alarms even if the Mobile App is shut down on an iOS or Android device) - Capable to classify objects by shape (e.g peoples, vehicles) - Detection of objects crossing virtual lines - Detection of stopped objects. - Detection of overtime delay of an object in the area of interest - Tamper detection - License Plate Recognition support - Capable for intelligent search for one designated classified object (person or vehicle) on different locations and timelines Workstation server: 1 server - CPU: latest generation server processor similar to Intel Xeon class or equivalent - RAM: min 8Gb or better if required - Network interface: 2 Gigabit Ethernet RJ-45 - HDDs as required by system - DVD-RW – 1pcs - Support for 2 screens - High performance 24/7 FullHD monitor min size 43” - 2pcs - With USB keyboard, USB mouse, and all required cables/accessories On the section that covers the LED lighting is necessary to set up a system of video surveillance at 5 key points. The purpose of installing cameras is traffic, personal and property security. It is necessary to mount the camera at 5 critical locations where the public road crosses itself a bike lane. Each camera should cover an area of ​​180 degrees, so to see the arrival of cyclists crossroads of cycle paths and public road, the intersection itself, and outgoing cyclists on the other side. The whole area of ​​180 degrees should be covered with smart camera with analytics, which should signal the risk situation at a given intersection, as potentially irregular diversion of cars on the bike path and potential incidents at the crossroads on the principle of detecting objects (people and vehicles) in the area of ​​interest and detection of objects (people and vehicles) crossing the virtual line. Camera recording should be doubled, ie. it is done in-camera, and the video should be transferred to the server to record in premises of City of Kikinda, where it could be accessed cameras live, get live events exacerbated the intersections at a given time violations, and to perform fast searches by detection of objects (people and vehicles) in the area of ​​interest and detection of objects (people and vehicles) across the virtual line. Communication from the city center in Kikinda to the camera is necessary to be provided via wireless links. Important Notice: Video surveillance system will be part of public surveillance system. All technical designs, equipment and technical solutions offered must be compliant with Serbian legislation covering this issue. Depending on system to be offered/provided Tenderer is obligated to include in price all required other equipment and works necessary for video surveillance system to be operational. This will include costs of equipment and installation including for instance wireless links between cameras and NVR, connection with workstation, POE switches, all necessary technical designs and supplies and works required by system or Serbian legislative covering this issue (public surveillance system). At no circumstances method of connection should be GPRS link or similar which will induce large operation costs to Contracting Authority, nor will Contractor have right to ask Contracting Authority for additional payment for works and equipment necessary for system to be operational. One must have in mind that cameras need to be installed in remote area so that powering of cameras and communication equipment should also be considered. Site visit is obligatory for all potential tenderers and during this visit tenderers should envisage acceptable technical solution. |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |