# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title :  Acquisition of in field mobile laboratory

Publication reference : RORS 380/Politehnica University Timisoara / 24

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item No. | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | Unless otherwise specified, the requirements in this Technical Specifications are presented as minimum standard which the offered goods must meet. The offers that will not satisfy completely and correctly the minimum requirements mentioned above will be disqualified. When brands names/specifications referring to or describing products of a given brand or origin are used in this technical specifications, they are used in descriptive purposes only and should be interpreted as equivalent. The tenderer is obliged to provide supporting documentation (catalogues, guides, brochures, manuals, etc.) confirming that the offered specification meets the required conditions. |  |  |  |
|  | Component of the mobile laboratory: - Autovehicle - Laboratory equipment |  |  |  |
|  | - Autovehicle |  |  |  |
| 1.1 | Technical characteristics |  |  |  |
|  | Type of vehicles: N1 |  |  |  |
|  | Passenger cabin: seats for min 3, |  |  |  |
|  | Colour of vehicles: no extra charge |  |  |  |
|  | Body type: min 3 doors |  |  |  |
|  | Volume of trunk: min 400 liters |  |  |  |
|  | Driver position: left side |  |  |  |
|  | Engine petrol or diesel min Euro 6 european emission standards |  |  |  |
|  | Wheel: 4 rims with winter tyres |  |  |  |
|  | Transmission: manual/automatic |  |  |  |
|  | Required equipment: valid first aid; spare set of lightbulbs; reflective vest 2 pcs ; reflective triangle 2 pcs; pressurized pressure extinguisher with gauge; floor jack; spare wheel; lug wrench, towing equipment |  |  |  |
|  | Warranty period: min 2 years on mechanical and electronic circuits or 100.000 km |  |  |  |
|  | Delivery date: 90 days from the date of signature of the contract by both parties and according to Special conditions, art. 19.1 |  |  |  |
|  | Other conditions: the supplier is obliged to provide free service with the original parts for maintaining the vehicle in the warranty period according to warranty conditions, as well as the supply of surcharge parts for 5 years after warranty expiration. The tenderer will bear the cost of the technical revisions during the project implementation period (Dec. 2020). The tenderer is obliged to submit a list of authorized services in the territory of Romania. Year of production: not before 2019. The tenderer will supply compulsory liability insurance and CASCO insurance during the implementation period. |  |  |  |
|  | 2. Laboratory equipment |  |  |  |
|  | 2.1. Spectrophotometer 1 pcs Beam Height: 10 mm Data Logger: 2000 measured values (Result, Date, Time, Sample ID, User ID) Display Resolution: WVGA (800 pix x 480 pix) Display Size: 7 inch (17.8 cm) Includes: Includes: Adapter A for 1 in. round and 1 cm square cells, matched pair of 1 in. square glass sample cells, light shield, dust cover, printed basic user manual and benchtop power supply Interfaces: USB type A (2), USB type B, Ethernet, RFID module Light Source: Gas-filled Tungsten (visible) Max. operating humidity: 80 % Max. Storage Humidity: 80 % Operating Conditions: 10 - 40 °C Operating Mode: Transmittance (%), Absorbance and Concentration, Scanning Optical System: Reference beam, spectral Photometric Accuracy: 5 mAbs @ 0.0 - 0.5 Abs Photometric Accuracy 2: 1 % at 0.50 - 2.0 Abs Photometric Linearity: < 0.5 % - 2 Abs Photometric Linearity 2: ≤ 0.01 % at >2 Abs with neutral glass at 546 nm Photometric Measuring Range: ± 3.0 Abs (wavelength range 340 - 900 nm) Power Requirements 110-240 V AC, 50 Hz Sample Cell Compatibility: Rectangular: 10, 20, 30, 50 mm, 1 inch; round: 13 mm, 16 mm, 1 inch Scanning Speed: > 8 nm/S (in steps of 1 nm) Specific Technology: RFID for easy method update, sample ID and Certificate of Analysis Spectral Bandwidth: 5 nm ± 1 nm User Programs: 100 Warranty: 12 months Wavelength Accuracy: ± 1.5 nm (wavelength range 340 - 900 nm) Wavelength Calibration: Automatic Wavelength Range: 320 - 1100 nm Wavelength Reproducibility: ± 0.1 nm Wavelength Resolution: 1 nm Wavelength Selection: Automatic, based on method selection |  |  |  |
|  | 2.2. Cuvete kit Tests (min. 24 samples), for next elements: NO3-N (0,23-13,5 mg/l), NO3-N (5 - 35 mg/l), NH4-N (2 - 47 mg/l), P2O5 (3 – 80 mg/l), K2O (8 – 50 mg/l), Mg (0,5 – 50 mg/l), Ca (5-100 mg/l), SO4 (40-100 mg/l), Fe (0,2 – 6 mg/l), Cu (0,1 – 8 mg/l), Zn (0,2 – 6 mg/l), Mo (1 – 10 mg/l), Mn (0,2 – 5 mg/l), B (0,05 – 2.5 mg/l), Cl (1 – 70 mg/l). |  |  |  |
|  | 2.3. Drone 1 pcs. Sensor: 1” CMOS, Effective Pixels: 20 million Lens:FOV: about 77°, 35 mm Format Equivalent: 28 mm, Aperture: f/2.8–f/11, Shooting Range: 1 m to ∞ ISO Range: Video: 100-6400, Photo: 100-3200 auto) 100-12800 (manual) Shutter Speed: Electronic Shutter: 8–1/8000s Still Image Size: 5472×3648 Still Photography Modes:Single shot, Burst shooting: 3/5 frames, Auto Exposure Bracketing (AEB): 3/5 bracketed frames at 0.7 EV Bias Interval (JPEG: 2/3/5/7/10/15/20/30/60s RAW:5/7/10/15/20/30/60s) Video Resolution: 4K: 3840×2160 24/25/30p 2.7K: 2688x1512 24/25/30/48/50/60p FHD: 1920×1080 24/25/30/48/50/60/120p Max Video Bitrate: 100Mbps Color Mode: Dlog-M (10bit), support HDR video (HLG 10bit) Supported File System: FAT32 (≤ 32 GB) exFAT (> 32 GB) Photo Format: JPEG / DNG (RAW) Video Format: MP4 / MOV (MPEG-4 AVC/H.264, HEVC/H.265) Operating Temperature Range: -10°C to 40°C Operating Frequency: 2.400 - 2.483 GHz 5.725 - 5.850 GHz Transmission Power (EIRP): 2.400 - 2.483 GHz FCC：≤26 dBm, CE：≤20 dBm, SRRC：≤20 dBm, MIC：≤20 dBm, 5.725-5.850 GHz, FCC：≤26 dBm, CE：≤14 dBm, SRRC：≤26 dBm GNSS: GPS+GLONASS Hovering Accuracy Range: Vertical: ± 0.1 m (when vision positioning is active), ± 0.5 m (with GPS positioning), Horizontal: ± 0.3 m (when vision positioning is active), ± 1.5 m (with GPS positioning) Internal Storage: 8 GB Accessory: flight time extension kit and Hard Case |  |  |  |
|  | 2.4. pH meter 1 pcs Peak measuring electrode in combined structure and backup electrode Calibration kit with 3 solutions 500 ml /bottle (4,01/7,01/10,01) Measurement range : 0-16 pH Accuracy : 0.01 pH Resolution : 0.01 pH Calibration : automatic 2-point calibration Probe : specially - designed Probe length : 125 mm Power : 4000 measurements, 9-Volt-battery or independent source Delivery contents : pH basic unit Special insertion pH probe Storage solution |  |  |  |
|  | 2.5 Insect pheromone trap UV light Solar panel 18V/ 25W Auto Control DC 12 V 2 Litium Ion batteries Consumption 8 W Lamp LED 3W Effective area 200 – 250 m2 Fan speed 1500 rpm |  |  |  |
|  | 2.6. Voltage inverter for auto 1 pcs Input voltage 12 V DC Output voltage 220 V AC Power 1200 W, 2400 W-short time Exit USB port 5 V/1000 mA LED overload warning; thermal protection; overload protection |  |  |  |
|  | 2.7. GNSS RTK complete system (receiver, controller and accessories), 1 pcs Number of channels: 220 Receives signals to the following satellites: - GPS simultaneously L1 C/A, L1C, L2C, L5 - Glonass simultaneously L1 C /A, L1 P, L2 C/A (Glonass only M), L2 P, L3 - SBAS simultaneously L1 C / A, L5 - Galileo: simultaneously L1 BOC, E5A, E5B, E5AltBOC1 - BDS: B1, B2 - QZSS: L1 C / A, L1 SAIF, L2C, L5 RTK initialization time: <8 sec Automatic connection on startup with the controller - 3G mode Communication interface:- 8 and 5-pin I / O connectors, - USB 2.0 port, - 2.4GHz Bluetooth Class II with a range up to 50 m - 1GB internal memory and microSD memory card (supports up to 32GB) - GSM, GPRS internal modem - Serial port: CMR +, sCMRx, CMR, RTCM2.1, RTCM 2.2, RTCM 2.3, RTCM 3.0, RTCM 3.1, RTCM 3.2 input yes output - High-capacity Li-ion battery 5Ah that provided using the instrument 10h in static mode and between 6-10h in RTK mode Accuracy: - Static horizontal: 2.5 mm + 0.5ppm - Vertical static: 5 mm + 0.5 ppm - Horizontal RTK: 8 mm + 1 ppm - Vertical RTK: 15 mm + 1 ppm - Horizontal DGPS: 25 cm + 1 ppm - Vertical DGPS: 50 cm + 1 ppm - Resistant to immersion in water at a depth of 1 m or 100% humidity |  |  |  |
|  | The mobile laboratory shall be labelled according to Visual Identity Manual of the Interreg-IPA Cross-border Cooperation Romania-Serbia Programme, the instructions are available at the following web address www.romania-serbia.net/?page_id=212 The concrete elements of visibility that will be registered will be established by the beneficiary after the knowledge of the configuration |  |  |  |
|  | Technical documentation, the maintenance and use manual (in Romanian), the technical book (in Romanian) for the delivered vehicle and accessories; |  |  |  |
|  | The tenderer will place and install all laboratory equipments in the vehicle according to all requests issued by CA based on the purchased vehicle configuration. The tenderer is obliged to perform all actions required by Romanian national road authorities (e.g. Romanian Automotive Register) in order to achieve the product authorizations from transforming the vehicle from N1 to SG-Autolaborator (according to 2007/46/CE). Laboratory equipment will be delivered in according to Special conditions, art. 19.1 The tenderer is obliged to supply all legal documents demonstrating the product is approved and certified as mobile laboratory (“Autolaborator”), and will perform the car registration on behalf of CA (CA will be the owner). |  |  |  |