# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Stage with lighting and sound system	p 1 /…

Publication reference : 395/Novi Knezevac/TD7

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Stage with lighting and sound system 1set consisting of: Stage platform elements (podium) – 1set consisting of: - Mobile elements of the stage for outdoor use (Outdoor use), dimensions 2 x 1 m, in aluminum frame, load capacity 750 kg / module, black color in total quantity of 40pcs (80m2) - To be delivered with sufficient number of connectors for interconnection of elements for a strong and stable connection - To be delivered with sufficient set of aluminum legs with adjustable height in maximum no less than 1000 mm. - To be delivered with appropriate mobile modular stairs adjustable for heights in total quantity 1set. - To be delivered with front and side canvas skirts and system for easy montage demontage of skirts Roof and construction - 1set consisting of: - Appropriate mobile aluminum roof construction with minimum dimensions of 8x6m and min 5m in height. - Top covered with tarpaulin, PVC or similar material. - Covered back and sides with a canvas. Audio system – 1 set consisting of - Digital Mixing Console (minimum 40 input channels, 25 mix buses, and 16 mic preamps) to be delivered with extension module with 16 mic preamps and connecting cable and one stage box with 16-channel - 1 set. - Active Subwoofer (min 1200 Watt RMS sustainable and 2400 Watt peak, min 30 Hz to 120 kHz frequency response, size min 1 x 18" woofer, DSP controlled input section) – 4 pcs - Active sattelite speaker with RDNET (robust management network) controll and DSP (Digital signal processing) with following spcs:amplifier power min 1000W RMS, frequency response min 50 Hz to 20 kHz, woofer size min 12” – 4 pcs - Active 12" Stage Monitor with following specs: Digital power amplifier min 400W RMS sustainable, woofer size min 12” with 2.5” voice coil, frequency response min 50 Hz to 20 kHz – 4pcs - Drum Microphone Set with case and holders (1 bass drum, 1 bass drum/floor tom, 2 toms, 1 snare, 2 overhead/hi-hat, 4 gooseneck rim clamp)– 1set - Wireless microphone set (max operating range min 60m, battery autonomy min 10h, adjustable output level, electromagnetic frequency noise resistant) – 1set - Dynamic vocal microphone (frequency response range min 50 to 16,000 Hz, steel mesh grille) – 3pcs - Dynamic instrument microphone(frequency response range min 50 to 16,000 Hz, steel mesh grille) – 3pcs - Microphone stand – 7pcs - Signal cables - min 10m length XLR or appropriate for equipment – 30pcs - Sound speaker stand appropriate for offered equipment – 4pcs Lighting system – 1 set consisting of - RGBW LED Wash Moving Head - with following specs: min total LED power 175W, min 4 separate controllable sections, digitally controlled pan & tilt movements, DMX support, dimming function supported, strobe function supported) - 8 pcs - Wash light reflector - with following specifications: light color temperature 3000-3500K, minimum 100W total LED power, silent, flicker free dimming capability, compatible with wireless or DMX control, included interchangeable magnetic lenses alter the beam to desired angles, strobe function supported. – 6pcs - Lighting Control Software and USB Dongle - with following specifications: for outputting min 2 DMX universes, includes SMPTE time-code input and USB MIDI control, All controls are available on the program interface, free software for control. - 1pcs - RGBW LED Par - with following specifications: DMX control support, min 12x8W RGBW LED Par, Control Mode DMX/ Manual/ Sound, dimming function supported, strobe function supported – 8pcs - DMX cable - length min 3m – 20pcs - DMX cable - length min 10m – 5pcs - Wireless DMX transmission set – 1set Important Note: Schematic diagram of stage and list of all required elements need to be submitted with offer. Stage with lighting and sound system offered should have all required elements to be fully functional and with all required parts/ accessories. It should be capable for easy disassembly/ assembly and should have suitable transport packages/trunks/suitcases for transportation of equipment and accessories. In case that some specific tools not available on market are required for disassembly/assembly of stage they should be also included in offer. Tenderer will be obligated to train crew designated by Contracting Authority for assembly/disassembly of stage and equipment. |  |  |  |