# VOLUME 3

# TECHNICAL SPECIFICATIONS

TECHNICAL DESCRIPTION OF WORKS

Areas of illegal landfills from which mixed municipal and construction waste has been removed are to be regulated as a green area of ​​a protective character. Areas on which there were illegal landfills, it is necessary to biologically re-cultivate, by forming artificial meadows. Their establishment requires the application of all necessary agro-technical measures, in order for the soil to reach optimal properties, productive level and complete vegetation cover as soon as possible. Agro technical measures include improving the quality of the land after the analysis, complete agro technical preparation of the land for sowing and sowing. The Contracting Authority will perform the analysis of the land and based on it determine which elements will be added to the subject areas. Areas where illegal landfills were located may not be used for other purposes, e.g. agricultural or forestry production, recreation and the like. Changing the purpose of the area is possible only when the permanent stability of the plant community is proven. In the meantime, it is necessary to monitor the state of ecosystems and the environment in the environment where the illegal landfill was located. When choosing the species and the amount of seeds to be used per unit area, species with more modest requirements and wide ecological amplitude of distribution should be used.

The total area on which agro technical measures are applied is 7776 m2 Following works are to be performed:

1. Filling with humus 20 cm over the entire area of ​​7776 m2 (1555.2 m3 of humus)

2. Repair of soil structure (NPK, PK ...) fertilization 7776 m2

3. Rigging, plowing, disking and preparation of land for sowing 7776m2

4. Grass sowing 7776m2

5. Emplacement of 12 warning signs forbidding illegal waste dump.

In total 12 areas on 6 locations in Municipality of Zrenjanin are cleared from waste and are designated to be remediated. Locations are following:

| No | Location | GPS Coordinates | Number of area | Surface area |
| --- | --- | --- | --- | --- |
| 1. | Mužlja špic | 45°21'20.32"N 20°22'42.01"E | 1 | 27 m2 |
|  |  |  | 2 | 86 m2 |
| 2. | Neimar | 45°24'7.78"N 20°23'25.02"E | 1 | 534 m2 |
|  |  |  | 2 | 184 m2 |
|  |  |  | 3 | 77 m2 |
|  |  |  | 4 | 29 m2 |
|  |  |  | 5 | 79 m2 |
| 3. | Jankov Most | 45°27'51.09"N 20°26'29.91"E | 1 | 1370 m2 |
| 4. | Mihajlovo | 45°29'15.20"N 20°23'26.87"E | 1 | 539 m2 |
|  |  |  | 2 | 1450 m2 |
|  |  |  | 3 | 1750 m2 |
| 5. | Taraš | 45°28'5.05"N 20°12'39.58"E | 1 | 919 m2 |
| 6. | Tisa Put | 45°23'23.52"N 20°17'2.75"E | 1 | 732 m2 |

Tenderers are advised to examine the sites included on site using the given GPS coordinates and aerial photographs to reduce economic risks related to agro technical measures to improve soil quality after the analysis, complete agro technical preparation of land for sowing and sowing of agreed grass species.