# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Supply of Fire engine and equipment – Lot 1	p 1 /…

Publication reference : 324/DUMBRAVA/04

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

When brand names / specifications /regulations / standards (e.g. SR EN, STAS, ISO XXX etc.) referring to or describing products of a given brand or origin are used in the technical specifications they are used in descriptive purposes only or should be interpreted as equivalent.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Medium capacity fire engine – 1 piece Minimal technical specification: 1. General technical data: - Hydraulic folding cabin with two doors for 1+2 persons - Specific power: at least 18 HP/t - Maximum speed: at least 100 km/h (completely equipped) - Traction: 4 X 2 - Gears: manual 6+1 - Direct injection diesel engine, minimum power 220 HP, turbo-charged - Pollution norm: EURO 6 - Hydraulic assistance for direction - Multi-disc in oil bath braking system hydraulically assisted on front and rear axles - ABS - EBA (Emergency break assistance) - ASR (Anti slip regulator) - ESC (Electronic stability control) - 2 heated side mirrors - Driver seat with pneumatic suspension - Fuel tank minimum 120 liter, adblue minimum 30 liter - Electric voltage: 24 V - Battery capacity: 170 Ah |  |  |  |
|  | 2. Special compartment for firefighting - Fire pump installed in the rear of the frame - Low pressure operation: 2000 l/minute, 8-10 bar - High pressure operation: 200 l/minute, 35-40 bar - Automatic priming device - Storz type couplings - Water tank minimum 1000 liter - Foam tank minimum 100 liter - Tank material: glass reinforced plastic (GRP), which can be used for drinking water transportation too) - Compartments for accessories according to the Romanian legislation in the front and in the rear of the water tank and on the roof of the vehicle - Roof access through a ladder - Command panel in the rear of the vehicle (equipped at least with manovacuumeter, low presure manometer, indicators for water and foam level, indicator for engine temperature). - Optic and acoustic signaling according Romanian legislation (two red lights on the cabin, acoustic signaling at least 100 W) - Inscriptions according Romania legislation - Personalization according to the visibility guide of the Interreg IPA CBC Romania-Serbia Programme (http://www.romania-serbia.net/?page_id=212&lang=en_GB) - Color FLAM RED (RAL 3000) - the vehicle will be marked with homologated reflective tape, according to the regulations in force |  |  |  |
|  | All the equipment must follow the prescriptions of the Romanian legislation in the field of firefighting. The Contractor will deliver a full set of instruction manuals for vehicle in Romanian language. Warranty at least 1 year. The vehicle need to have RAR (Romanian Antivehicle Register) authorization and can be registered according the Romanian legislation. Fire fighter accessories and protective equipment for three persons included according to the Romanian legislation. Accessories according to Order no. 75 from 27th of June 2019: NO.; NAME; PCS.. 1; Discharge hose type C; 15. 2; Discharge hose type B; 10. 3; Discharge pipe type C; 4. 4; Rubber suction tube type A; 6. 5; distributor B-CBC; 2. 6; Portable hydrant + key; 1 set. 7; collector; 1. |  |  |  |
|  | 8; Strainer for suction tubes; 2. 9; Metal basket; 2. 10; Rope for strainer support; 2. 11; Shallow water ejector; 1. 12; Couplings reductions - type A-B - type B-C; 1 1. 13; Line mixer; 1. 14; Mechanic foam generator pipe; 2. 15; Anti-caloric protection suit (for private emergency situations services) - approach normal (shirt, trousers, hood, gloves, boots) - penetration heavy (chemise with hood and sealed boots and gloves); 1 1. |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | 16; Compressed air breathing apparatus; 2. 17; Compressed air spare cylinder; 2. 18; Rescue rope; 2. 19; Rescue small rope; 2. 20; Axe – pickaxes + cover; 1. 21; Hatchet + cover; 2. 22; hook; 1. 23; Crowbar; 1. 24; Mobile projector; 1. 25; Projector tripod; 1. 26; Cable reel 50 m; 1. 27; Crossing bridge over hose; 2. 28; Hose elbow; 1. |  |  |  |
| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
|  | 29; Bag for keys and bandages; 2. 30; Key for connection (hoses); 4. 31; Swaddle for hose - type B - type C; 4 4. 32; Window ladder; 1. 33; Sliding ladder; 1. 34; Portable extinguisher with powder P6; 2. 35; RAR (Romania Autovehicle Register) homologated first aid kit; 1. |  |  |  |

# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Supply of Fire engine and equipment – Lot 2	p 1 /…

Publication reference : 324/DUMBRAVA/04

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

When brand names / specifications /regulations / standards (e.g. SR EN, STAS, ISO XXX etc.) referring to or describing products of a given brand or origin are used in the technical specifications they are used in descriptive purposes only or should be interpreted as equivalent.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | Professional boots – 6 pairs Minimum requirements: - The boots are made and used according to the provisions of the harmonized European standard SR EN 15.090/2012 "Footwear for firefighters" (or similar), different sizes between 38 and 44 - Water resistent leather - Reindorced soles according to EN 12568/2010 - The sole will be resistant to high temperatures above 300 0C and it will keep its flexibility at temperatures below -20 0C - Quick release with zipper protected on the outside with leather - The model will be accompanied by the EC Examination certificate, EC declaration of conformity for at least the requirements below: - EN 15090: 2012 Footwear for firefighters * F2A (Type 2) * heat insulated HI3 * heat resistant outsole HRO; * Cold insulated CI - EN ISO 20345/2011 (or similar) : SRC (slip resistance), S2 (water penetration / absorbtion) |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 2. | Complete protection suit – 6 pieces Minimum requirements: - The equipment must meet the minimum requirements of the following standards: SR EN 469:2006 (EN 469:2005) "Protective clothing for firefighters. Performance requirements for fire protection clothing" (or similar) - The suit contains jacket and pant - The suit is made of materials with special properties used by fire-fighters, being made up of jacket and trousers, made in the range of sizes 44-62, sizes I, II, III, and has blue-navy (ultramarine) and/or beige color, with retro-reflective and fluorescent band. The band must comply with the requirements of SR EN 20471:2013 (or similar). - The finished product must meet the performance levels set out below, which constitute the minimum quality requirements for level 2 of SR EN 469:2006 (EN 469:2005) "Protective clothing for firefighters. Performance requirements for fire protection clothing" |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 3. | Safety helmet for firefighters – 6 pieces Minimum requirements: - The equipment must meet the minimum requirements of the following standards: SR EN 443, SR EN 14458, SR EN 166 (or similar) - Resistant to instance heat (1000 C⁰ for 10 seconds) - Resistant to mechanical shocks - Integrated adjustment system - Includes folding glasses, anti-scratch and anti-fog treatments - Reflectorized accessories |  |  |  |
| 4. | Protective gloves for firefighters – 6 pairs Minimum requirements: The gloves must meet the requirements of SR EN 659 + A1:2008 (EN 659:2003 + A1:2008) (or equivalent) - "Protective gloves for firefighters" based on the methods described in the related standards: SR EN 388:2004 (EN 388:2003) (or equivalent) - "Protective gloves against mechanical risks", SR EN 407:2005 (EN 407: 2004) (or equivalent) -"Protective gloves against thermal risks"(heat and fire). Different sizes (9-12) |  |  |  |

# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Supply of Fire engine and equipment – Lot 3	p 1 /…

Publication reference : 324/DUMBRAVA/04

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

When brand names / specifications /regulations / standards (e.g. SR EN, STAS, ISO XXX etc.) referring to or describing products of a given brand or origin are used in the technical specifications they are used in descriptive purposes only or should be interpreted as equivalent.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | Water pumps – 10 pieces Minimum requirements: Medium capacity water pumps for dirty water - Power: minimum 4 KW - Diesel engine minimum 250 cm3 - Water flow: minimum 35 mc/h - Max depth: suction 7 m Accessories: - 3 '' connection: 2 pcs - 3 '' necklaces: 3 pcs - Sieve: 1 piece – Spark plug wrench - 50m discharge hose - 5m suction hose The Contractor will deliver a full set of instruction manuals for vehicle in Romanian language. |  |  |  |