# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Fire engine and equipment	p 1 /…

Publication reference : RORS-324/2021/T5/A

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

- For all goods from the technical specification, the Bidder is obliged to in the column "Offered" enter the technical characteristics of the offered goods, as a confirmation that the offered good fulfills required technical characteristics and to complete the technical specification in this way sign and certify and submit with the offer, as its integral part.

- The goods in question must be new, undamaged, correct and functional.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | One fire engine The detailed specification is attach on the bottom of this document - FIRE VEHICLE - TANK (1 PC.) Pleas fill in the detailed Specifications Offered and the remarks in the attached document, and also fill out Columns 3-4 based on the attached document. (For better understanding and overview ) |  |  |  |
| 2 | Water pump - medium capacity 10 pieces Minimum requirements: Purpose: Water pump Fuel: Petrol Pump type: Self-priming Pressure: min. 3 bar Weight: max. 50 kg: Work autonomy: min. 2.5 h Fuel consumption: max. 1.7 l / h Fuel tank capacity: min.3 L Engine type: 4-stroke, single-cylinder: Engine capacity: min. 162 cm3 Ignition system: Transistor magnetic Cooling system: Air Power: min. 3.6 (4.89) / 3600 kW (PS) / rpm Ignition: via cable Pump type: Self-priming Oil tank capacity: min. 0.5 l Flow: min. 700 l / min Accessories: - Sieve: 1 piece – Spark plug wrench - 50m discharge hose - 5m suction hose The Contractor will deliver a full set of instruction manuals for pump on Serbian language. |  |  |  |
| 3 | Professional boots - 6 pieces Minimum requirements: Purpose: Means of personal protection at work in: rescuing people from fire, protecting property from fire, fighting fires in buildings and outdoors. Face: Natural and synthetic rubber, rubber or other artificial material, bending resistant Sole: Pressed, trough, hot contact resistant (HRO), bending resistant Available sizes: 35-47 Standards or equivalent: EN 15090: 2012, F2 P HI3 SRA EN 50321: 1999, class 0 EN 12568 |  |  |  |
| 4 | Complete protection equipment - 6 pieces Minimum requirements: Purpose: For short-term exposure to open flames Material: NOMEX, or equivalent Material weight: max. 220 g / m2 In accordance with or equivalent: 89/686 / EEC - Appendix II EN 340 EN 15614: 2007, A1 + A2 EN 11 612: 2008 (code A1 + A2, code B1, code C1) |  |  |  |
| 5 | Safety helmet - 6 pieces Minimum requirements: - The equipment must meet the minimum requirements of the following standards: SR EN 443, SR EN 14458, SR EN 166 (or similar) - Resistant to instance heat (1000 C⁰ for 10 seconds) - Resistant to mechanical shocks - Integrated adjustment system - Includes folding glasses, anti-scratch and anti-fog treatments - Reflectorized accessories |  |  |  |
| 6 | Gloves - 6 pieces Minimum requirements: The gloves must meet the requirements of SR EN 659 + A1:2008 (EN 659:2003 + A1:2008) (or equivalent) - "Protective gloves for firefighters" based on the methods described in the related standards: SR EN 388:2004 (EN 388:2003) (or equivalent) - "Protective gloves against mechanical risks", SR EN 407:2005 (EN 407: 2004) (or equivalent) -"Protective gloves against thermal risks"(heat and fire). Different sizes (8-11) |  |  |  |

Item Number 1 Specification

FIRE VEHICLE - TANK (1 PC.)

1. PURPOSE:

The fire and rescue vehicle for extinguishing fires with water and foam, according to its construction and equipment, belongs to the group of tank trucks which serves for the supply of water to fire engine vehicles, but also for firefighting interventions. The vehicle enables the extinguishing of fires of solid flammable substances (class "A" fires), fires of highly flammable liquids ("B" class fires) and gases ("C" class fires). The vehicle is equipped with fire extinguishers, pump, water and foam tanks and fire and rescue equipment.

TECHNICAL REQUIREMENTS:

2.	TECHNICAL REQUIREMENTS FOR UNDERCARRIAGE

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Brand and type of undercarriage and country of origin | The undercarriage is delivered with built-in original factory parts, homologated, year of production min. 2021 |  |  |  |

2.1 DIMENSIONS AND MASSES

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Total permissible weight | max. 16000 kg |  |  |  |
| Load capacity - front axle | min. 7500 kg |  |  |  |
| Load capacity - rear axle | min. 11500 kg |  |  |  |
| Wheelbase | min. 3570 mm |  |  |  |

2. 2	ENGINE AND GEARBOX

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Diesel engine with turbocharger | In-line diesel engine, min. 6 cylinders |  |  |  |
| Exhaust emissions | Complied with the requirements in the Republic of Serbia, min. Euro 5 standard |  |  |  |
| Maximum engine power | min. 184 kW (250 HP) |  |  |  |
| Maximum torque | min. 1050 Nm |  |  |  |
| Transmission | automatic or semi-automatic, min. 12 speeds |  |  |  |
| Selection of transmission mode | For emergency interventions |  |  |  |
| Power take-off - PTO with flange | In accordance with the conditions for driving the fire pump via the cardan shaft |  |  |  |

2.3	RELIABILITY SYSTEM

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Propulsion formula | 4x2 |  |  |  |
| Front axle | Not driven, cranked, steered, not liftable |  |  |  |
| Rear axle | HY axle without drive shaft, straight, not steered, not liftable |  |  |  |
| Front axle suspension | Parabolic leaf springs, telescopic shock absorbers and stabilizers |  |  |  |
| Rear axle suspension | Parabolic leaf springs, telescopic shock absorbers and stabilizers |  |  |  |

2.4	BRAKING SYSTEM

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Brakes, front and rear axle | Disc brakes on front and rear axles |  |  |  |
| Characteristics | - EBS - ABS |  |  |  |
| Engine brake | min. 170 kW |  |  |  |
| Parking brake | On all wheels |  |  |  |
| Parking brake characteristics | Electronic, with automatic release on departure and automatic activation when removing the key from the lock |  |  |  |
| Handbrake | Yes |  |  |  |

2.5	ELECTRICAL SYSTEM AND LIGHTS

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Voltage | 24 V |  |  |  |
| Batteries | Two batteries with a capacity of min. 180 Ah, maintenance-free |  |  |  |
| Alternator | Yes |  |  |  |
| The lights | - LED position side lights - Tail lights - LED daylight - Turning light |  |  |  |
| Audible warning when reversing | Yes |  |  |  |

2.6	CABIN

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Type | Driver's cab and minimum 2 crew members (1 + 2). |  |  |  |
| Material | Cab made of galvanized sheet steel and painted. The cabin floor is lined with non-slip material. |  |  |  |
| Cabin color | Red, RAL 3000 |  |  |  |
| The door | 2 (two) with central locking, min. 4 keys |  |  |  |
| Seats | - Air-backed driver's seat - Driver's seat armrests - Driver seat belt height adjustment - All seats with seat belts |  |  |  |
| Mirrors | - external electrically adjustable mirror with heaters - edge mirror, electrically adjustable, heated - front mirror, manually adjustable |  |  |  |
| Other | - Air conditioning - Front bumper, painted white - Windshield, safety, tinted |  |  |  |

2.7	FUEL TANK and AdBlue

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Volume | min. 150 l |  |  |  |
| AdBlue tank | min. 35 l |  |  |  |

2.8	Accessories and associated tools

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| 2 connectors in the vehicle cab | - 1 x 12 V - 1 x 24 V |  |  |  |
| Tempomat (cruise control, CC) | Yes |  |  |  |
| Electronic Stability Program (ESP) | Yes |  |  |  |
| Drive wheel slip control (ASR) | Yes |  |  |  |
| 2 pull-out rings on the front of the vehicle | Yes |  |  |  |
| Vehicle crane | min. 10000 kg |  |  |  |
| Handy tool | Yes |  |  |  |
| A warning triangle | Yes |  |  |  |
| First aid | Yes |  |  |  |
| Fluorescent vest | Yes |  |  |  |
| Glass cleaning set | Yes |  |  |  |
| Bumper, steel, 3-piece | Yes |  |  |  |
| Metal grilles for headlights | Yes |  |  |  |

3.	UPGRADE

3.1 AUXILIARY FRAME

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Installing the auxiliary frame | Attaching the paws to the main chassis so that a detachable connection is achievable |  |  |  |
| U profile | Yes |  |  |  |
| Material | Carbon steel sheet |  |  |  |
| Reinforcement in characteristic places - transverse connection | Yes |  |  |  |
| The purpose | Even load transfer on the undercarriage |  |  |  |
| The connection of the auxiliary frame with the suspension is made in accordance with the recommendations of the chassis manufacturer | YES |  |  |  |

3.2 WATER TANK

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Volume | min. 5000 l |  |  |  |
| Positioning in relation to the chassis or auxiliary chassis | Transversely |  |  |  |
| Relying | Elastic supports |  |  |  |
| Material | Stainless steel (inox) |  |  |  |
| Barriers which prevents the water ripples | Yes |  |  |  |
| Inspection opening | diameter min. 500 mm |  |  |  |
| Overflow system with vacuum vent | Yes |  |  |  |
| Electric water level meter | Yes |  |  |  |
| Hydrant charging connection | Storc coupling type B |  |  |  |

3.3 FOAM TANK

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Volume | min. 300 l |  |  |  |
| Material | Stainless steel (inox) |  |  |  |
| Foam pouring hole | Yes |  |  |  |
| Breathing vent (AT - vent) | Yes |  |  |  |
| Electric foam level meter | Yes |  |  |  |
| Connections for the installation of foam and water mixing systems | Yes |  |  |  |
| Foam supply connection from an external source | Storc coupling type D |  |  |  |

3.4 BODYWORK CONSTRUCTION

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Roof superstructure/upgrade | Bounded by a protective fence - stainless steel shutter |  |  |  |
| Walking surface of the superstructure/ upgrade roof | Lined with non-slip aluminum sheet |  |  |  |
| Material of storage | Aluminum |  |  |  |
| Box profiles | Yes |  |  |  |
| AL rolling shutters with the possibility of locking | min. 5 pcs. |  |  |  |
| Equipment holders | Yes |  |  |  |
| Roof climber upgrade | Positioned in the rear storage |  |  |  |
| Climber tread | Non-slip surface |  |  |  |
| Holding handle and climbing aid | Positioned along the ladder itself |  |  |  |
| Winch - winch for self-extraction | Positioned at the front of the vehicle |  |  |  |
| Pulling force of the self-extracting winch | min. 5500 kg |  |  |  |
| Winch voltage | 24 V |  |  |  |

3.5 FIRE PUMP

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| brand | Specify |  |  |  |
| Type | Centrifugal |  |  |  |
| Label | Specify |  |  |  |
| Propulsion of the pump | On cardan shaft |  |  |  |
| Pump material | Aluminum |  |  |  |
| Pump axle material | Stainless steel |  |  |  |
| Vacuum device | Yes |  |  |  |
| Pump capacity | normal: min. 2000 l / min at 10 bar, 3 m suction head high: min.250 l / min at 40 bar |  |  |  |
| Standard | EN 1028 |  |  |  |
| Possibility of simultaneous work in medium and high mode | Yes |  |  |  |
| Ability to work when moving the vehicle at a lower speed | Yes |  |  |  |
| A pre-filter is installed on the suction line towards the pump | Yes |  |  |  |
| Automatic overheat protection | Yes |  |  |  |
| Foam mixing system | Manual from 0% to 6% |  |  |  |

3.6 INSTRUMENT CONTROL PANEL

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Waterproof | Yes |  |  |  |
| Manual commands | Yes |  |  |  |
| Positioning | Rear pump storage |  |  |  |
| Commands it possess | - medium pressure winch command - command for switching on the high pressure winch - command for switching on the vacuum device - return line - valve for releasing water from the tank into the pump - premixer - medium pressure manometer - high pressure manometer - manovakumetar - working hours counter - water level indicator in the tank - foam level indicator in the tank - command to switch the pump drive on / off - STOP switch. |  |  |  |

3.7 WATER AND FOAM INSTALATION

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Material | Stainless steel |  |  |  |
| Protection | Painting |  |  |  |
| Connections | - 2 outlets with 75mm diameter non-return valves on the rear of the vehicle (left and right of the pump), - 1 connection with 75 mm diameter butterfly valve for monitor - top, - One 32.5 mm diameter connection for medium pressure winch - One 25 mm diameter connection for high pressure winch. - 1 connection for water suction with a diameter of 110 mm - One 25 mm foam pull connector - water and foam mixer in the ratio of 0% 6% |  |  |  |
| Water and foam installation elements (elbows, forks, couplings) are made of stainless materials. | Yes |  |  |  |
| All connections of the installation are with standard couplings for fire hoses type "Storz" made of forged aluminum alloy | Yes |  |  |  |

3.8 HOSE REELS

3.8.1 QUICK INTERVENTION HOSE REEL - MEDIUM PRESSURE

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Position | Rear left storage |  |  |  |
| Hose material | Rubber or fabric (synthetics) |  |  |  |
| Hose length | min. 25 m |  |  |  |
| Diameter | 32.5 mm |  |  |  |
| Nozzle capacity min. 100 - 400 liters / min at 10 bar | Yes |  |  |  |
| Adjusting the shape and flow of the jet | Yes |  |  |  |
| Hose reel winding - electro | Yes |  |  |  |
| Hose reel winding - manual | Yes |  |  |  |
| Brake | Yes |  |  |  |

3.8.2 QUICK INTERVENTION HOSE REEL - HIGH PRESSURE

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Position | Rear right storage |  |  |  |
| Hose material | Rubber or fabric (synthetics) |  |  |  |
| Hose length | min. 60 m |  |  |  |
| Diameter | 25 mm |  |  |  |
| Nozzle capacity min. 200 liters / min at 40 bar | Yes |  |  |  |
| Adjusting the shape and flow of the jet | Yes |  |  |  |
| Foam adapters | Yes |  |  |  |
| Hose reel winding - electro | Yes |  |  |  |
| Hose reel winding - manual | Yes |  |  |  |
| Brake | Yes |  |  |  |

3.9 WATER JET

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| The purpose | Directing extinguishing agents - water or foam |  |  |  |
| Positioning | Firetruck roof superstructure/upgrade (front part) |  |  |  |
| Plate valve | Yes |  |  |  |
| Adjustable flow nozzle and jet shape | Yes |  |  |  |
| Characteristics | - capacity: min. 1200 l/ min at 10 bar - water jet range: min. 50 m - foam jet range: min. 45 m - management: manually - horizontal movement: 360 ° -vertical movement: from -15 ° to + 75 ° |  |  |  |

3.10 LIGHT SOUND SIGNALING

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Blue rotating light and electro howling siren | On the roof of the vehicle cabin |  |  |  |
| The equipment storage boxes and the pump area are illuminated. | Yes |  |  |  |
| Two flashing blue lights are installed on the rear of the vehicle. | Yes |  |  |  |
| On the sides of the vehicles in the roof gallery are placed 3 pcs. white LED lights to illuminate work environments. | Yes |  |  |  |

3.11 VEHICLE COLOR AND BRANDING

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Vehicle color | RAL 3000 (flame red) |  |  |  |
| Undercarriage | Black |  |  |  |
| Fenders and bumpers | White |  |  |  |
| Branding | Customer's logo on the cabin door. |  |  |  |

4. FIRE AND RESCUE EQUIPMENT

| No. | Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- | --- |
| 1. | Suction hose Ø 110 mm | 4 |  |  |  |
| 2. | Suction basket | 1 |  |  |  |
| 3. | Hydrant extension | 1 |  |  |  |
| 4. | Hydrant extension key | 1 |  |  |  |
| 5. | Divider B / 2C | 2 |  |  |  |
| 6. | Discharge hose B | 6 |  |  |  |
| 7. | Discharge hose C | 12 |  |  |  |
| 8. | Medium foam nozzle | 2 |  |  |  |
| 9. | Heavy foam nozzle | 2 |  |  |  |
| 10. | Universal (turbo rotomag) nozzle with regulation of flow and shape of the jet from 100 to 500 l / min | 4 |  |  |  |
| 11. | Pressure hose supports | 10 |  |  |  |
| 12. | Pressure hose holders | 10 |  |  |  |
| 13. | B / C adapter | 6 |  |  |  |
| 14. | Fire extinguisher S9 | 6 |  |  |  |
| 15. | Insulating apparatus with spare bottles in the same quantity. | 3 |  |  |  |
| 16. | Broom | 2 |  |  |  |
| 17. | Ax | 2 |  |  |  |
| 18. | Backpack | 3 |  |  |  |
| 19. | Pickaxe | 2 |  |  |  |
| 20. | Spade | 2 |  |  |  |
| 21. | Shovel | 2 |  |  |  |
| 22. | ABC key | 4 |  |  |  |
| 23. | Three-part ladder stretcher | 1 |  |  |  |

5. TECHNICAL DOCUMENTATION, TRAINING AND WARRANTY

5.1 TECHNICAL DOCUMENTATION

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Instructions for handling and maintenance of the complete vehicle in Serbian. | 1 printed copy |  |  |  |
| Workshop manual for the maintenance of all assemblies and devices on the vehicle for the complete vehicle. | 1 printed copy |  |  |  |
| Catalog of spare parts in electronic form. | 1 disc (CD) copy |  |  |  |
| All necessary accompanying documentation and confirmation from the performed technical inspection so that the vehicle can be registered for the intended purpose (with certificates). | Yes |  |  |  |

5.2 TRAINING

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| One-day training for staff who will operate the vehicle. | Yes |  |  |  |
| Training of technical and supervisory staff for maintenance of all assemblies and devices on the vehicle. | Yes |  |  |  |

5.3 WARRANTY

| Description | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committees notes |
| --- | --- | --- | --- | --- |
| Warranty period for the entire vehicle minimum | 24 months |  |  |  |
| Entry into force of the warranty period | From the moment of signing the qualitative - quantitative record |  |  |  |