# VOLUME 3

# TECHNICAL SPECIFICATIONS

The location in question is located in the construction area of ​​Majdan, Novi  Kneževac

At the address Maršala Tita 55, on cat. plot 293 KO Majdan, there is a family apartment, while the auxiliary building was very ruined. In the extension to the north, the plots of k.p. 294, 304/2 and 305/3 KO Majdan, where the archeological site is located, and listed plots have the property of a cultural asset.

Reconstruction and conversion of the residential building into a tourist info center is planned. Pannonian-type residential building, at the address Maršala Tita 55, on the first floor. plots 293 k.o. Majdan, Novi Kneževac, is located longitudinally in relation to the plot, placed on the western edge plots towards the neighbor, while the shorter side and gable lie on the street regulation line. It represents a traditional type of construction in this area. The auxiliary building was placed longitudinally in the extension of the residential building to the north. The archeological site is located on k.p. 294, 304/2 and 305/3 k.o. Majdan, Novi Kneževac, on the borders of Serbia, Hungary and Romania.

The archeological site dates from 11-15. century, includes the findings of churches from the 12th century, the Gothic church, masonry tomb, a series of grave sites, three furnaces, a wall around the courtyard and a series of movable finds: coins, jewelry, utensils. Within cadastral parcels number 294, 304/2 and 305/3 KO Majdan, investor of the Municipality of Novi Kneževac, the preliminary design E - 22 / 2019-2 - IDP for conservation and reconstruction was made archaeological site, while this project E - 22 / 2019-2 - IDP plans the reconstruction and conversion of a residential building into a tourist info center. These two projects are unique functional whole. It is planned to arrange the ground floor space, make a transparent fence around this space, lighting, construction of paths, paving, landscaping of the plots, parking. Conservation and reconstruction of part of the archeological find is planned: church, fence wall,

tombs and two or three furnaces. It is planned to convert the residential building into a tourist info center. Pedestrian and car access to the facility is provided from the street side from Maršala Tita Street.

The existing residential building with a gross area of ​​120 m2 is oriented in the northeast direction. southwest, the shorter side lies on the street regulation line, and the longer side lies on the border with the adjacent plot no.291. The roof is gabled, covered with tiles. The newly designed solution plans a total reconstruction of the existing facility, replacement dilapidated building, while maintaining the existing horizontal, and with minimal correction height gauge, in order to convert the attic into a living space. Object retains the existing horizontal dimension, while the vertical dimension will be increased by 60 cm to allow for loft landscaping.

RESIDENTIAL BUILDING

Functional solution:

The existing building has several small rooms. From the anteroom, you enter from the side, on one side

into a small street room, and into the kitchen toward the courtyard. The right is accessed in one smaller, and from it in a larger street room. From the kitchen you enter the pantry from which you climb the wooden ladder to the attic.

Separately, you enter the porch from the yard, and from there into the hallway from which the bathroom and the room open with two beds. The building is dilapidated, dilapidated, completely non-functional, damp and of low height room, 1.80 m to 2.14 m.

The total reconstruction is intended to preserve the external appearance of the building in its original form. He plans reconstruction of the existing building, replacement of the dilapidated building. By total reconstruction the object retains the existing horizontal dimension, while the vertical dimension will be corrected to allow for loft landscaping.

The new purpose of the space will be tourist education. A small one would be located within the building info desk, souvenir shop with replicas of archeological findings, multifunctional space that would have the purpose of an exhibition space with a display of the history of sites, monasteries and others objects through drawings, video presentations and replicas of archaeological finds: bricks, jewelry, coins and three-dimensional elements from objects. Within the space it would be possible organizing lectures, workshops etc. There would be a toilet (male, female and for people with disabilities) with po one cabin, as well as a smaller kitchenette. In the attic there would be a possibility of arrangement room for workshop participants. The arrangement of the attic is the subject of the second phase of works on the building.

Construction:

Existing condition

The existing building is built of mixed material, mostly of charge, it is the only one the northern part (room with bathroom and porch) was later extended by brick. The roof structure is classic wooden two-water, very dilapidated. The roof covering is a folded tile, sloping the roof is at the lower part 29, and 39 degrees at the higher part of the building. There are on the building horizontal and vertical galvanized sheet. The building is in a dilapidated, dilapidated condition with large cracks.

Total reconstruction

The complete dilapidated roof construction will be replaced with a new one, with a slope min.correct. The new slope is planned at 28, 36 degrees. The roof covering is being dismantled installs tiles with prior installation of parodiffusion roofing foil, battens and counter battens 3x5 cm. The existing wooden mezzanine structure is being demolished, as well as the gable walls towards street and yard.

Existing walls are being demolished, new walls will be made of brick, bearing 25 cm, partition 12 cm. Extensive walls will be reinforced with vertical and horizontal reinforced concrete cerclages. The new mezzanine construction will be Fert. Partition walls of the bathroom in attic are planned from moisture-resistant gypsum boards on a metal substructure in everything according to the manufacturer's instructions. Existing floors are being demolished, new ones are planned to be built of packed concrete floors d-10 cm over a gravel buffer, install Fimflex waterproofing tapes and thermally insulated, hard styrofoam d-5 cm. The final floors are made of ceramic tiles on the ground floor horizontal waterproofing of walls and floors. Facade carpentry is being replaced with new wooden ones look at the original.

Lighting and ventilation

Lighting is natural in all rooms.

Finishing and facades

Facade surfaces are plastered with cement mortar, which is attacked in the lower parts moisture. It is necessary to completely cover the facades, to insulate them with hard stone wool 10 cm thick and plastered with lime-based mortar, to allow the walls to breathe. The facade surface will be painted with parodiffuse silicate paint in the tone of your choice Designer MZZSK Subotica.

The wooden carpentry is worn out, dilapidated and requires replacement with a new one. New carpentry will be notified of well-dried three-layer laminated pine timber as to avoid warping and Exterior carpentry will be glazed with double thermopane glass 4 + 12 + 4mm, with standard fittings.

The roof covering will be pepper-rebate tiles all over, the lengths of the planned snow guards are horizontal vertical gutters of rectangular cross-section of appropriate dimensions, made of galvanized sheet metal d-0.55

Window sills, roof edgings are made of galvanized sheet metal 0.55 mm.

Interior treatment

The interior rooms are plastered (after the walls are covered with crumbling plaster) (at the walls of charges with a rabbit net), smoothed and painted with fasting paint. Sanitary facilities are lined sanitary ceramics to a height of h = 2m. There are floor coverings in the interior ceramic or granite slabs of the designer's choice a, the attic is laminate.

Net ground floor area - existing: 91.28 m2

Gross ground floor area - existing: 120.00 m2

Net ground floor area - new: 100.06 m2

Gross floor area-new: 120.00 m2

Net attic area - 68 m2

Gross attic area - 85 m2

Object location conditions

In terms of climate, the area in question has a continental climate consisting of harsh winters and hot summers with relatively unstable precipitation. Given such climatic conditions

The Rulebook on Energy Efficiency included the area of ​​Novi Kneževci in "zone B" as a place for which the external design temperature (heating period) is lower than - 15 Co. Great snow cover and pronounced constant winds are not characteristic of this area but are they appear occasionally and may be pronounced. Geological profile of the terrain characteristic for this area in the return periods of 50, 100 and 200 years the degree of seismicity is the lowest and is 8o per glass MSK-64.

Projected  installations

The facility is connected to the existing electricity and water supply network, to the existing one connection. The sewer is connected to a new two-part septic tank to be built in yard. Heating is planned by convectors or Norwegian radiators.

All works should be reported according to the applicable technical regulations and standards for such types of work made of quality material and skilled labor .__