# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Intervention Equipment	p 1 /…

Publication reference : RORS218/CityofZrenjanin/TD1

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | Truck tank -1 piece - Type of vehicle: Cargo vehicle with cistern for transportation of drinking water. Substructure (Vehicle): - Engine power: min 240 HP - Max engine torque: min 900Nm - Engine type: diesel, EURO VI or better - Batteries min 2 with min 170Ah maintenace free - Air Conditioner device included - Left side steering wheel with servo included - EBS, ABS systems - Power windows on both sides - Alarm for reverse movement included - Cabin capacity: min 3 seats Superstructure - Water reservoir type and capacity: min 8000 litres – max 10000 litres for drinking water with internal seawalls. - On top of water reservoir there should be access shaft min 500mm in diameter - Water reservoir needs to have overspill exhaust, water level measurement and connector for refill via hydrant. - On front of vehicle pipe should be mounted with minimum 6 left-right movable controllable water nozzles installed for street washing purpose. Controls installed in cabin of vehicle. Maximal water pressure in system for street washing is no less than 5 bars with maximal pump capacity no less than 800 litres per minute. Important Note: Vehicle should be equipped fully in line with Serbian Road Safety Legislation and vehicle should be road ready after delivery. Costs of all obligatory testing and inspections by State designated institutions (other than ones necessary for regular registration process) in according with Serbian legislative are obligation of supplier. |  |  |  |
| 2. | Minibus - 1 piece - Type of vehicle: Passenger transportation minibus/van with total of 21 seats including driver - Engine power: min 120KW - Engine Torque: min 360Nm - Engine type: diesel, EURO VI or equivalent/better - Vehicle total height: min 2.7 meters - Windows tinted in passenger area - Windows equipped with curtains in passenger area - AC device installed in ceiling of passenger area - Heating installed in passenger area - Passenger seats equipped with seatbelts Important Note: Vehicle should be equipped fully in line with Serbian Road Safety Legislation and vehicle should be road ready after delivery. Costs of all obligatory testing and inspections by State designated institutions (other than ones necessary for regular registration process) in according with Serbian legislative are obligation of supplier. |  |  |  |
| 3. | Stacker - 1 piece - Type: electrical stacker; - Lift height: 2m or higher; - Lift weight: 1t or more at 2m |  |  |  |
| 4. | Sanding Machine - 1 piece - Type: Equipment for mechanical sandbag filling - Compatible with CATERPILLAR BACKHOE LOADER MODEL 428F2 - Capacity: 6200 sandbags/8 hours or better |  |  |  |
| 5. | Chainsaw - 1 pieces - Gasoline engine; - Power: min 3kW/4 HP; - Chainsaw bar length: min 45cm; |  |  |  |
| 6. | Radio stations - 2 pieces - Type: AM/FM CB radio (Citizens band radio) - Minimum 8 channels; - Frequency: allowed to be used in Serbia without special permission |  |  |  |
| 7. | Handheld radio stations - 10 pieces - Minimum 8 channels; - Minimum range: up to 5km or more - Frequency an power: allowed to be used in Serbia without special permission - Rechargeable batteries |  |  |  |
| 8. | Handheld flashlights - 20 pieces - LED bulb; - Batery powered; - Power: min 3W; |  |  |  |
| 9. | Extension ladders - 1 piece - Material: Aluminium ladders. - Made from two or three pieces (segments). - Maximal length of extended ladders: 9m or more |  |  |  |
| 10. | Fire hose - 5 pieces - Material: high-strength polyester thread of thermoplastic elastomer, entwined around a rubber hose with coupling; - Internal diameter: Ø75mm; - Length: 15 m; |  |  |  |
| 11. | Masks - 20 pieces - Type: wide visibility angle protective face gas mask, for covering entire face, with filter |  |  |  |
| 12. | Handbarrow - 5 pieces - Type: Foldable rescue handbarrow with metal frame; - Length: min 2m; - Width: min 0.5m; |  |  |  |
| 13. | Foldable bed - 5 pieces - Material: metal frame, with padded PU mattress or similar - Size of folded bed: not over 100cm in any dimension - Size of unfolded bed for sleeping: Width 80cm, Length 190cm or greater |  |  |  |
| 14. | Life jackets - 5 pieces - Filled coloured lifejacket for water rescue (not inflatable) - With reflective stripes and whistle on string |  |  |  |
| 15. | Backpack sprayer - 5 pieces - Type: Backpack sprayer for outdoor vegetation fire fighting - Capacity: 20 - 25 litres; - Piston hand pump - Nozzle range: up to 10m |  |  |  |
| 16. | Uniforms - 20 sets Each set consisting of: - Hooded windbreaker: Type of windbreaker with hood, made from dark colour textile - Trousers: workpants made from textile, dark colour - Sweatshirt: made from dark colour textile at least 250g/m2 - Boots: type of protective work boots with steel protection plate in front or similar, dark colour |  |  |  |
| 17. | Drone - 1 piece - Weight: less than 1kg - Max service ceiling: min 4000m - Max flight time (no wind): at least 30min - Max hovering time (no wind): at least 30min - Max flight distance: at least 30km - Operating temperature range: -10oC to +40oC or better - Hovering accuracy range with satellite positioning +-0.5meters - Camera resolution: min 20MP - Video resolution supported: 4K at 60fps or better - Video transmission supported live on controller: 1080p at 30fps or better - Battery charger included - Controller included |  |  |  |
| 18. | Pickup vehicle - 1 piece - Type of vehicle: 4WD Pickup vehicle with closed passenger cabin and open rear cargo space - Engine power: min 110KW - Engine Torque: min 360Nm - Engine displacement: max. 2000 ccm - Engine type: diesel, EURO VI or better - Transmission: 4 WD (4 Wheel Drive) - Ground clearance: min 220mm - Cabin type: 4 doors, 5 seats - Carrying capacity: min 1000kg - AC device - Minimal safety equipment: min 6x airbag, ABS, Electronic stability control, Hill start assistance - Wheel size: min 16” Important Note: Vehicle should be equipped fully in line with Serbian Road Safety Legislation and vehicle should be road ready after delivery. Costs of all obligatory testing and inspections by State designated institutions (other than ones necessary for regular registration process) in according with Serbian legislative are obligation of supplier. |  |  |  |