# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Intervention Equipment	p 1 /…

Publication reference : RORS218/CityofKikinda/TD1

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | Water tanker truck -1 piece |  |  |  |
|  | CHASSIS (Vehicle) |  |  |  |
|  | Gross vehicle weight: min. 18.000 kg Engine: diesel engine with turbocompressor, minimum 6 cylinders Euro norm: min. Euro 5 Engine power: min. 130 kW Engine torque: min. 1000 Nm Gearbox: manual, minimum 9 +1 speed Drive:4x2 Service brake: ABS, ASR, ESP Compressor for breaking system: min.220 cm3, 1 cylinder Batteries: two batteries capacity min. 140 Ah Cabin capacity: min 3 seats Air conditioning included Audible signal for reversing movement included Radio set included Fuel tank volume: min. 200 L The chassis has to be delivered with installed original factory parts, homologated, year of manufacture min. 2022 |  |  |  |
|  | UPGRADE (Superstructure): |  |  |  |
|  | Capacity of water pump: min. 800 1/min RPM of water pump: max. 3000 min-1 Volume of water tank: min. 6.000 l On the top of the water tank there should be inspection opening, minimum diameter 500 m. Partitions to reduce water ripples while driving should be installed Ladder for access to the upper part of the superstructure should be positioned on the rear side of the tank. Water installation should consist from minimum 3 movable controllable water nozzles installed on the front of the vehicle Rear collector should be positioned on the rear side of the superstructure Equipped with hydrant wrench: min 1 piece Equipped with ABC clutch wrench: min 1 piece Equipped with hydrant extension B/2C: min 1 piece Equipped with pressure hose C: min 1 piece Equipped with water nozzle B: min 2 pieces |  |  |  |
|  | TRAINING Supplier should organize training for handling the superstructure and basic maintenance of the complete vehicle. Training should last one day and be organized for a three people minimum. |  |  |  |
|  | Important Note: Vehicle should be equipped fully in line with Serbian Road Safety Legislation and vehicle should be road ready after delivery. Costs of all obligatory testing and inspections by State designated institutions (other than ones necessary for regular registration process) in accordance with Serbian legislation are obligation of supplier. |  |  |  |
| 2. | Electrical sludge pump – 3 pieces - Electrical engine - Power supply: min 400 V - Three-phase power supply - Power of pump: min. 2,8 kW - Capacity: min. 1300 lit per minute |  |  |  |
| 3. | Chainsaw - 1 piece - Gasoline engine - Power: min 4kW/6 HP - Chainsaw bar length: min 30 cm |  |  |  |
| 4. | Firefighter backpack with engine power- 6 pieces - Thank capacity: min 12 l - Spray range: min 6 m - Engine power: min 1 kw |  |  |  |
| 5. | Breathing apparatus for respiratory protection- 4 pieces - Apparatus should be made from fire resistant materials - Contain full face mask - Contain pressure regulator - Number of Cylinder: min 2 - Capacity of cylinder: min 6.8 litres - Working pressure: min 300 Bar |  |  |  |
| 6. | Helmet with a visor-20 pieces - Material of helmet should be made of material resistant to high temperatures (min 100 ° C) - The visor should be made of materials that are resistant to scratches and fogging - Helmet should have integrated adjustment system |  |  |  |