# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply and installation of floating fountains for aeration of “Old Lake” surface waters in Kikinda 	p 1 /…

Publication reference: RORS 373/Cityof Kikinda/TD10

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Floating fountain aerator with aeration module (3 pcs.) General specification: - floating platform with installed water spray/jets pump, waterfall nozzle (flow stabilizer if necessary), aeration module/device, decorative lighting - minimum required depth of accumulation for setting up is 1m; - max. power of all installed devices is 1.6kW - single-phase or three-phase power connection - mobile anchoring system, quick setup and unfolding, system of cables and weights - fixing +/- 30cm from the projected position Water spray/jets pump: - submerged - power: max. 1100W - voltage: 230V/50Hz - made of material operable in water, corrosion resistant (stainless steel or equivalent) Water spray/jets pattern: - the water jets rise in a parabola to the outside and then descend - round horizontal shape; max. diameter 6,50m – min. 5,00m; - multiple water jets, with two groups of water sprays: 1st jets group: max. height 3,75m - min. height 2,50m 2nd jets group: max. height 1,50m - min. height 1,00m - flow stabilizer if necessary Aeration module/device: - one or more separate aeration modules installed on the floating platform - total power: max. 260W - total airflow: min. 10m³/ h Decorative light: - 4 (four) LED reflectors installed - white colour - power: max. 27W / per reflector - operable in water |  |  |  |
| 2. | Floating fountain aerator (2 pcs.) General specification: - floating platform with installed water spray/jets pump, waterfall nozzle (flow stabilizer if necessary), decorative lighting - minimum required depth of accumulation for setting up is 1m; - max. power 1.6kW - single-phase or three-phase power connection - mobile anchoring system, quick setup and unfolding, system of cables and weights, allows fixing +/- 30cm from the projected position Water spray pump: - submerged - power: max. 1100W - voltage: 230V/50Hz - made of material operable in water, corrosion resistant (stainless steel or equivalent) Water spray pattern: - multiple water jets, the water rises in a parabola to the outside and then descend - round horizontal shape, with max. diameter 3,3m; - one group of sprays with max. height of 2,5m and min. height 2m; - flow stabilizer if necessary Decorative light: - 2 (two) LED reflectors installed - white colour - power: max. 27W / per reflector - operable in water |  |  |  |