# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply and installation of floating fountains for aeration of “Old Lake” surface waters in Kikinda 	p 1 /…

Publication reference: RORS 373/Cityof Kikinda/TD11

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Floating fountain aerator (5 pcs.) General specification: - floating platform with installed water spray/jets pump, nozzle, decorative lighting and other accessories and/or attachments for proper fountain aerator operation - minimum required depth of accumulation for setting up is 0,9m; - max. power of all installed devices is 1.8kW - single-phase or three-phase power connection - mobile anchoring system, quick setup and unfolding, system of cables and weights - fixing +/- 30cm from the projected position - aerator flow rate: min. 100m3/h Water spray/jets pump: - submerged - power: max. 1.5kW - voltage: 230V - made of material operable in water, corrosion resistant (stainless steel or equivalent) Water spray/jets pattern: - the water jets rise in a parabola to the outside and then descend - multiple water jets - jets diameter / width: max. 6m - jets hight: min. 1.8m Decorative light: - LED reflectors installed (min. 3 pcs) - colour: white or orange - operable in water conditions (protection class IP 68) - angle of the light according to the fountain jets shape Note: installation of the fountain aerators should be performed according to the IDP – 1/2023 (positioning, anchoring, power supply and excavation routes for laying electrical installations, electrical equipment) |  |  |  |