# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Acquisition of AMBULANCE (Emergency medical vehicle – van with installed life support equipment)	p 1 /…

Publication reference : RORS 284 / Municipality of Jimbolia / 02

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | COMPULSORY REQUIREMENTS |  |  |  |
|  | I. GENERAL CONDITIONS |  |  |  |
|  | 1. DESTINATION |  |  |  |
|  | 1.1. The ambulance shall be designed and equipped for the purpose of providing qualified first aid, emergency pre-hospital medical care, patient transport and monitoring, within and outside localities, on developed and undeveloped roads, during all types of weather. |  |  |  |
|  | 1.2. The ambulance shall comply with an intense use, with frequent accelerations and brakes, including on carriageways with extremely steep ramps and slopes. |  |  |  |
|  | 2. TYPE-APPROVAL |  |  |  |
|  | 2.1. The vehicle must be type-approved by the R.A.R. (Romanian Automotive Register) - R.A. For travel on public roads and shall observe the provisions of the Romanian Government Ordinance No. 43/1997 on roads status, as further amended and supplemented. |  |  |  |
|  | 2.2. All equipment mounted on-board the vehicle must be type-approved according to European standards current at the time of the delivery and must hold all permits required for the commissioning thereof. |  |  |  |
|  | 2.3. Category of vehicle: M1 |  |  |  |
|  | 2.4. Type of body: SC Ambulance. |  |  |  |
|  | 2.5. The vehicle must hold a CE Certificate of Conformity, according to Directive EC 46/2007 (for the vehicle is ambulance is fitted on). |  |  |  |
|  | 2.6. The ambulance shall comply with the requirements of the SR EN 1789:2007+:2014 standard, deemed to be a minimum. |  |  |  |
|  | 2.7 The results of tests, carried out according to the SR EN 1789:2007+A2:2014 standard, for the tendered ambulance variant, shall be confirmed by submitting the full test reports (detailed tests, final certificate), certificates issued by notified bodies, upon submission of the tender |  |  |  |
|  | 3. DELIVERY |  |  |  |
|  | 3.1The vehicle shall be delivered within a maximum of 120 days from the date of execution of the supply agreement |  |  |  |
|  | 3.2 The vehicle shall be delivered at the headquarters of the contracting authority. Carriage expenses shall be included in the tender value |  |  |  |
|  | The tenderer shall submit an Affidavit for meeting the criteria under 3.1 and 3.2 |  |  |  |
|  | 4. WARRANTY PERIOD |  |  |  |
|  | 4.1. The supplier shall grant a 24-month warranty period |  |  |  |
|  | 4.2 The warranty is provided for the vehicle and all equipment installed on-board same |  |  |  |
|  | 4.3 During the warranty period, the supplier shall provide, at their own expense, the replacement of all broken parts, the related labour, and shall incur all carriage/travel costs |  |  |  |
|  | 4.4 During the warranty period, the supplier shall not cover the cost of the ambulance’s compulsory inspections. Such costs shall be incurred by the beneficiary |  |  |  |
|  | 4.5. The warranty shall act as the commitment, undertaken by the supplier or manufacturer towards the Municipality of Jimbolia, of not claiming additional repair and/or replacement costs for certain parts / systems / equipment purchased, should same fail to comply with the technical requirements undertaken or if, during the warranty period provided, any hidden flaws of the delivered products are identified. The tenderer shall submit an Affidavit for the purpose of meeting the requirements provided under items 4.1, 4.2, 4.3, 4.5. |  |  |  |
|  | 5. STAFF TRAINING |  |  |  |
|  | 5.1 The supplier shall carry out the training of staff regarding the method of use and maintenance of the delivered equipment |  |  |  |
|  | II. TECHNICAL REQUIREMENTS |  |  |  |
|  | 1. CONSTRUCTION FEATURES |  |  |  |
|  | 1.1. SIZE, WEIGHT AND VOLUME FEATURES |  |  |  |
|  | 1.1.1. Length: 6000 mm. max |  |  |  |
|  | 1.1.2. Height: 3000 mm max. (measured at net weight, without flexible antenna), according to item 4.1.2. of SR EN 1789:2007+A2:2014 |  |  |  |
|  | 1.1.3. Width: 2050 mm max. (measured without exterior mirrors) |  |  |  |
|  | 1.2. ENGINE AND RELATED INSTALLATIONS |  |  |  |
|  | 1.2.1. Pollution rate: according to regulations current in the E.U. at the time of the delivery |  |  |  |
|  | 1.2.2. Fuel type: diesel. |  |  |  |
|  | 1.2.3. Max. power (DIN) 100 kW minimum. |  |  |  |
|  | 1.2.4. Cylinder capacity: 2000 cubic centimetres minimum |  |  |  |
|  | 1.2.5. Fuel tank: 70 litres minimum. |  |  |  |
|  | 1.2.6. Metallic protection beneath the power train, at least in the area of the oil sump. |  |  |  |
|  | 1.3. DRIVE |  |  |  |
|  | 1.3.1. Drive type: minimum 4x2 (may be front or rear drive) |  |  |  |
|  | 1.3.2. Gearbox: automatic or manual, 5+l (reverse) gears minimum. |  |  |  |
|  | 1.4. WHEELS |  |  |  |
|  | 1.4.1. Tires installed on steel rims (including spare wheel). |  |  |  |
|  | 1.5. STEERING SYSTEM |  |  |  |
|  | 1.5.1. Servo-assisted. |  |  |  |
|  | 1.5.2. Steering wheel with adjustable column, on one direction minimum. |  |  |  |
|  | 1.6. BRAKE ASSEMBLY |  |  |  |
|  | 1.6.1. Servo-assisted. |  |  |  |
|  | 1.6.2. Anti-lock braking system (ABS or equivalent) and a vehicle stability control system. |  |  |  |
|  | 1.7. PERFORMANCE |  |  |  |
|  | 1.7.1. Maximum speed: minimum 140 km/h. |  |  |  |
|  | 1.7.2. The ambulance, at its maximum admitted load, must be able to reach 0 to 80 km/h in max 35 seconds (according to SR EN 1789:2007+A2:2014, item 4.2.1). |  |  |  |
|  | 1.7.3 ASR-type or equivalent anti-slip system. |  |  |  |
|  | 1.8. STRUCTURE |  |  |  |
|  | 1.8.1.The ambulance shall consist of a single van-type piece, with integrated cabin. Any sizes provided by the basic vehicle’s manufacturer, in relation to openings (doors, windows), must not be reduced. |  |  |  |
|  | 1.8.2. A divider wall - partition wall - shall separate the driver compartment (cabin) from the patient compartment. The divider wall shall be provided with a sliding window. The window shall allow direct visual contact with the driver. It shall be protected against accidental opening and shall be provided with an opaque shutter or other devices preventing the driver from being disturbed by the light in patient compartment. |  |  |  |
|  | 1.8.3. The cabin shall be equipped with the following: |  |  |  |
|  | A windscreen de-icing/de-fogging system, operating whenever the ambulance is either at a standstill, or travelling, with the engine on. |  |  |  |
|  | An external windscreen washing system. |  |  |  |
|  | Ventilation and air-conditioning system. |  |  |  |
|  | Two sunshades. |  |  |  |
|  | A passenger handle, located in the adjacency of the lower corner of the windscreen, and another handle above the access door. |  |  |  |
|  | Airbags - driver side, at minimum |  |  |  |
|  | Attendant seat, or a bench for two attendants, provided with seatbelts with a 3-point fastener, and headrests. |  |  |  |
|  | Electrically-adjustable and heated rear-view mirrors. |  |  |  |
|  | CD player or USB radio, and 2 speakers, at minimum |  |  |  |
|  | Rechargeable torch (at 12V), detachable from its holder, also provided with charger, installed inside the cab. |  |  |  |
|  | 1.8.4. Number of seats in cabin: 2 or 3 seats (driver included). |  |  |  |
|  | 1.8.5. The patient compartment shall be provided with openings (doors, windows) with the following features: |  |  |  |
|  | A minimum of 2 exits must be provided: |  |  |  |
|  | - one at the back (doors or tailgate). In the case of a tailgate, the distance between the ground and the tailgate, in open position, must be of 1800 mm minimum (from ground level to the lowest point of the fully opened door, at the permissible laden mass of the vehicle). If provided with doors, the minimum sizes of the opening must be 900 x 900 mm minimum. |  |  |  |
|  | - one to the side (door) at the patient compartment. The sizes provided by the manufacturer cannot be reduced. |  |  |  |
|  | Patient compartment doors must be maintainable in open position. |  |  |  |
|  | All openings shall be provided with waterproof sealing. |  |  |  |
|  | The angle provided for loading of the stretcher shall be 16 degrees maximum. |  |  |  |
|  | Ambulance doors shall be provided with central locking. |  |  |  |
|  | Openings must consider the sizes the stretcher. |  |  |  |
|  | The patient compartment must be provided with a minimum of two windows towards the outside: either one on each side, or one on the side and one at the rear. At least one of the side windows shall provide a sliding opening. |  |  |  |
|  | Windows must be positioned as such, or shielded so as to ensure the privacy of the patient, and the upper 1/3 of the windows shall allow a view towards the outside. |  |  |  |
|  | 1.8.6. General features of the patient compartment: |  |  |  |
|  | The patient compartment must be designed and built so as to provide the necessary space for the medical equipment required in the list of equipment. |  |  |  |
|  | The ceiling, interior walls and doors of the patient compartment must be fully covered with washable material. |  |  |  |
|  | The interior of the compartment shall be padded with washable material, able to withstand the effects of cleaning and disinfecting agents. |  |  |  |
|  | The wall sections, except for windowpanes, located above the stretcher level (including cabinets and drawer sides), must be made of washable material, able to withstand the effects of cleaning and disinfecting agents. |  |  |  |
|  | Surface edges must be designed or sealed against liquid infiltrations. If the flooring prevents the draining of fluids, one or more drains with plugs must be provided. Flooring edges, at the level of walls, shall be rounded upon ascending the wall. |  |  |  |
|  | Drawers shall be built using round edges. Drawers must be secured against accidental opening. |  |  |  |
|  | Shelves and drawers must be labelled with the maximum safety weight (according to SR EN 1789:2007+A2:2014). |  |  |  |
|  | 2 handles must be positioned next to the patient compartment: |  |  |  |
|  | One, placed on the divider wall next to the side door |  |  |  |
|  | A second one, placed on the side wall, next to the rear doors. |  |  |  |
|  | 1.8.7. When viewed from the rear door, the patient compartment must observe the following requirements: |  |  |  |
|  | The left-hand side wall (driver’s side) and the divider wall shall be used for the storage of medical equipment or frames, and of portable medical equipment chargers, such as the portable secretion vacuum and the oxygen provision system: cylinders and flow-meter humidifier. A minimum of 2 drawers must be provided for the storage of medical supplies. |  |  |  |
|  | On the right-hand side wall, 3 seats shall be mounted, to be used by patients and provided with seatbelts with a 3-point fastener, and another seat, mounted on the divider wall, with the back against the direction of travel (the option with 2 seats on the right-hand side wall and 2 seats on the divider wall is also accepted) |  |  |  |
|  | The medical compartment ceiling shall be used for mounting the drip frame. |  |  |  |
|  | The stretcher frame shall be placed to the left-hand side of the patient compartment, at a height allowing the loading / unloading of the stretcher with ease. |  |  |  |
|  | All elements of the stretcher’s non-mobile bracket shall be made of metallic material - stainless steel, providing an appropriate corrosive protection, so as to withstand the effects of cleaning and disinfecting agents, used for this purpose, throughout the ambulance’s normal life cycle. |  |  |  |
|  | Non-mobile oxygen cylinders shall be placed in a well-defined location of the patient compartment, preferably in an area allowing the easy replacement thereof, being shielded by a cabinet or a safety wall. |  |  |  |
|  | The portable oxygen cylinder shall be provided with a specifically-designed anchoring location, inside the medical compartment, allowing safe anchoring and easy access from the outside. The cylinder shall be provided with a safety and transport cover. |  |  |  |
|  | The flooring’s surface must provide appropriate adherence, even if wet; it must be resistant and easy to clean. |  |  |  |
|  | The inside of the patient compartment, fully equipped, shall be designed so as to minimise any risk of injury. |  |  |  |
|  | 1.8.8. The patient compartment shall be provided with a ventilation system, independent from the cabin ventilation system, capable of providing a minimum of 20 replacement of indoor air volume/ hour, whenever the vehicle is at a standstill. |  |  |  |
|  | The system shall operate independently from the patient compartment ventilation installation and shall be capable both of introducing fresh air and evacuating air in the aforementioned minimum amount. |  |  |  |
|  | 1.8.9. The patient compartment shall also be provided with a ventilation - air-conditioning system. |  |  |  |
|  | 1.8.10. In addition to the driver compartment heating, the patient compartment shall also be provided with an adjustable, independent air heating system. |  |  |  |
|  | 1.8.11. The independent heating system may be operable whenever the engine is on or off. It shall be provided with a thermoregulator, so that any temperature variations, compared to the set temperature, would not exceed ± 5°C. The system shall be connected to the engine’s pre-heating devices, installed by the basic vehicle’s manufacturer. The configuration of the ancillary heating installation shall be designed so as not to allow fuel or fumes to enter the patient compartment, even in the case of failures of such installation. |  |  |  |
|  | 1.8.12. The configurations of the ventilation and heating systems shall prevent exhaust gases from entering the patient compartment. |  |  |  |
|  | 1.8.13 The lighting in the patient compartment (natural balanced light) shall be provided as follows: |  |  |  |
|  | Patient area: 100 Ix minimum; |  |  |  |
|  | Surrounding areas: 30 Ix minimum. |  |  |  |
|  | 1.8.14. The noise level, inside the patient compartment, depending on the travel speed, shall comply with current European regulations (according to SR EN 1789:2007+A2:2014, chapter 4.5.7). |  |  |  |
|  | 1.8.15. The patient compartment shall be provided with a supporting system (frame) for drips, with the possibility of supporting 2 drips placed vertically, in such a way so as to avoid the swinging thereof and make full use of the vehicle’s height above the stretcher assembly. The frame system shall have a capacity of 5 kg minimum and shall be able to support three liquid bags, independent from one another (according to the SR EN l 789:2007+A2:2014 standard, chapter 4.5.8). |  |  |  |
|  | 1.8.16. The equipment stabilising / anchoring systems, located in the patient compartment (SR EN 1789:2007+A2:2014), must observe the following requirements: |  |  |  |
|  | Without any exceptions, all persons and material, such as, medical devices, the equipment and items normally present in an ambulance, must be anchored so as not to become a projective if subjected to a 10g (gravitational) force, both horizontally (front, rear, transversally), and vertically. |  |  |  |
|  | The distance travelled by persons or material, whenever subjected to such forces, may not pose a threat to the safety of any persons in the ambulance. |  |  |  |
|  | If subjected to such forces: |  |  |  |
|  | - no material shall have sharp edges posing a threat to the safety of any persons in the ambulance; |  |  |  |
|  | - the maximum travel distance of a stretcher or of any other attached component, and of the anchoring system, shall not exceed 150 mm. |  |  |  |
|  | 1.8.17. Number of attendant seats. |  |  |  |
|  | Minimum number: 3 (three) excluding the patient on the stretcher, (2 seats positioned to the right-hand side of the stretcher, and 1 seat installed on the divider wall, with the back against the direction of travel). (the option with 2 seats on the right-hand side of the stretcher and 2 seats on the divider wall, with the back against the direction of travel is also accepted) |  |  |  |
|  | All seats shall be provided with seatbelts with a 3-point fastener |  |  |  |
|  | 1.9. FIRE SAFETY |  |  |  |
|  | 1.9.1. All materials used for the inside of the vehicle must comply with the requirements of SR EN 13501- 1:2007+A1 :2009. |  |  |  |
|  | 1.10. ELECTRICAL INSTALLATION |  |  |  |
|  | 1.10.1. Voltage of the electrical installation: 12 V. |  |  |  |
|  | 1.10.2. Maintenance-free battery pack, with a 12 V rated voltage and capacity of 90 Ah min.: 1 pc. (according to SR EN 1789:2007+A2:2014). The battery’s structure and all its connections shall be designed so as to prevent the occurrence of an accidental (inadvertent) short-circuit. |  |  |  |
|  | 1.10.3. Alternator: minimum power of 1500 W / 12 V (according to SR EN 1789:2007+A2:2014). |  |  |  |
|  | 1.10.4. The batter pack and the alternator must produce sufficient energy to cover the necessary energy for the entire electrical equipment of the vehicle and specific additional systems. |  |  |  |
|  | 1.10.5. General circuit-breaker cutting off all consumers of the vehicles. |  |  |  |
|  | 1.10.6. Inlets shall be provided for the powering of consumers, as follows: |  |  |  |
|  | A 12 V inlet for the medical devices in the patient compartment - minimum of 2 pcs. |  |  |  |
|  | A 12 V inlet for driver cab - minimum of 1 pc. |  |  |  |
|  | 1.10.7. The electrical installation shall meet the following requirements: |  |  |  |
|  | All circuits inside the patient compartment shall be provided with automatic fuses and/or separate circuit-breakers, provided in the structure. |  |  |  |
|  | Fuses or circuit-breakers must be appropriately marked, and the function of every circuit shall be easily identifiable. Wirings shall be marked at distances of 1 m max.; |  |  |  |
|  | There shall be a minimum of two circuits, so that a fault in one of the circuits would not power off all lights or medical devices connected. |  |  |  |
|  | Wiring must be able to withstand more than the maximum load of fuses or circuit-breakers. |  |  |  |
|  | Wirings and conduits must be able to withstand vibrations. |  |  |  |
|  | Cables shall not cross the area where gaseous substances are used. |  |  |  |
|  | 1.10.8. The electrical system of the ambulance must contain at least three separate sub-systems, as follows: |  |  |  |
|  | The basic systems for the un-equipped vehicle. |  |  |  |
|  | The medical device supply system. |  |  |  |
|  | The patient compartment supply system. |  |  |  |
|  | Apart from the basic system, the ambulance’s frame shall not be used as part of any supply system (as “earth” for the other electrical circuits added pursuant to equipping the vehicle as an ambulance). |  |  |  |
|  | 1.11. LIGHTING-SIGNALLING INSTALLATION |  |  |  |
|  | 1.11.1. Daytime running lights (low beams intended for daytime travel, as detailed in Directive 2008/89/EC, other than low beams (passing beam), far beams (high beam) or fog lights, provided in Directive 79/756/EEC) |  |  |  |
|  | 1.11.2. The rear of the ambulance shall be provided with at least one LED lamp, of 10 W minimum, directed towards the ground, below a 45-degrees angle. Operation shall be carried out by buttons located in the driver’s cab and at the rear of the ambulance. The lamp shall be located on the upper side of the vehicle if the ambulance is provided with rear doors or, alternatively, below the tailgate. |  |  |  |
|  | 1.12. VISUAL AND SOUND WARNING SYSTEM |  |  |  |
|  | 1.12.1. The ambulance shall be equipped with both a visual, and a sound warning system. |  |  |  |
|  | 1.12.2. The visual and sound warning system must meet the following requirements: |  |  |  |
|  | The system shall be designed so that the sound system would not be operable unless the visual system is operational. |  |  |  |
|  | The different elements of the visual warning system shall be powered via a general circuit breaker connecting the warning system to the vehicle’s electrical system (connection upon the system being powered on). |  |  |  |
|  | The warning system shall be operational even if the engine is off. |  |  |  |
|  | Light signals shall observe the technical requirements provided in R 65 EEE - UN (certificates of conformity with the regulation R 65 EEE - UN shall be provided). |  |  |  |
|  | 1.12.3. The visual warning system shall consist of: |  |  |  |
|  | Two blue LED beacon lights, anchored on the roof, one above the driver cabin, to the left-hand side, and the other, at the rear right-hand side of the ambulance. Both of them shall be visible when viewed from the front, rear and sides of the ambulance. |  |  |  |
|  | 1.12.4. Sound warning system, with the following features: |  |  |  |
|  | Speaker placed in the engine compartment. |  |  |  |
|  | The siren shall be operable from the driver compartment and is provided with a general ON/OFF button. The siren’s power shall be a minimum of 100 W. |  |  |  |
|  | 1.13. MEDICAL DEVICES AND EQUIPMENT |  |  |  |
|  | 1.13.1. Provision of medical devices |  |  |  |
|  | The A2 ambulance shall be designed and built in order to embed the equipment mentioned (including the requirements needing to be met) in (The Equipment List) |  |  |  |
|  | 1.13.2. Storage of medical equipment |  |  |  |
|  | All equipment necessary for standard procedures must be stored in a specific location and anchored using an anchoring system. |  |  |  |
|  | The entire equipment shall be stored safely, via an anchoring system, in order to prevent any impact or injuries during the vehicle’s movement. |  |  |  |
|  | 1.14. PAINTING AND INSCRIPTION |  |  |  |
|  | 1.14.1. The ambulance shall be painted in white. No wrapping allowed. |  |  |  |
|  | 1.14.2. The ambulance shall be marked with reflective I fluorescent tape all around. |  |  |  |
|  | 1.14.3. The ambulance shall be provided with specific inscriptions, as well as any inscriptions required by the beneficiary. |  |  |  |
|  | 1.15. FACILITIES |  |  |  |
|  | 1.15.1. R.A.R.-approved medical kit |  |  |  |
|  | 1.15.2. Type-approved car-type fire extinguisher, with anchoring support, sized for this vehicle category - 2 pcs.; |  |  |  |
|  | 1.15.3. Rubber mats for the inside of the cabin. |  |  |  |
|  | 1.15.4. Reflective vest - 2 pcs. |  |  |  |
|  | 1.15.5. Reflective triangles - 2 pcs. |  |  |  |
|  | 1.15.6. Wheel wrench. |  |  |  |
|  | 1.15.7. Jack allowing a change of tires with the ambulance fully loaded. |  |  |  |
|  | 1.15.8. Seatbelt cutter - 1 pc. |  |  |  |
|  | 1.15.9. Seatbelt cutter, combined with hammer for breaking of emergency exit windows - 1 pc. |  |  |  |
|  | 1.15.10. Tool kit - 1 complete (10x11 , 13x15, 17x19 wrenches, flathead screwdriver, cross head screwdriver , 1 pc. each) or equivalent, allowing the use on the tendered product. |  |  |  |
|  | LIFE SUPPORT EQUIPMENT for Equipping A2 Ambulances |  |  |  |
|  | I. GENERAL REQUIREMENTS |  |  |  |
|  | 1.Provision of devices |  |  |  |
|  | 1.1. The A2 ambulance shall be designed and built in order to embed the equipment mentioned in the “EQUIPMENT LIST” paragraph and to provide the following levels of therapeutic interventions: Equipment for providing assisted transport to a patient. |  |  |  |
|  | 2. Storage equipment |  |  |  |
|  | 2.1. All equipment necessary for standard procedures must be stored in a specific location and anchored using an anchoring system. |  |  |  |
|  | 2.2. The entire equipment shall be stored safely, via an anchoring system, in order to prevent any impact or injuries during the vehicle’s movement. |  |  |  |
|  | 3. Requirements for devices |  |  |  |
|  | 3.1. General requirements |  |  |  |
|  | Equipment shall be designed, both for use while the ambulance is travelling, and for the use thereof in the field. |  |  |  |
|  | If a piece of equipment is designated as "portable" (save for any patient transportation equipment), it must comply with EN 60601-1-X: 1990 and shall need to: |  |  |  |
|  | be transportable by one person |  |  |  |
|  | have its own power source (where applicable) and be rechargeable inside the vehicle, both while travelling, and during a standstill. |  |  |  |
|  | be usable outside the vehicle. |  |  |  |
|  | • Temperature |  |  |  |
|  | - In the absence of other inscriptions on the device, it must be capable of operating over a temperature range between 0°C and 40°C. |  |  |  |
|  | - In the absence of other inscriptions on the device, the equipment must operate for a minimum of 20 minutes when placed in an environment with a temperature of - 5 °C, after being stored at a temperature of 20°C. |  |  |  |
|  | - In the absence of other inscriptions on the device, it must be able to operate when restored to ambient temperature (20°C) after being stored at temperatures ranging between -30°C and +70°C |  |  |  |
|  | • Anchoring of the equipment |  |  |  |
|  | It shall be anchored inside the vehicle. |  |  |  |
|  | The anchoring system must withstand 10 g accelerations on the longitudinal, transversal and vertical axes |  |  |  |
|  | Electrical terminals and inlets may not be part of the system for anchoring such device. |  |  |  |
|  | • Electrical safety |  |  |  |
|  | All equipment must be selected and installed so as not to cause any malfunction to the electricity-supply equipment. |  |  |  |
|  | • Electromagnetic compatibility |  |  |  |
|  | The EN 60601-1-2 requirements shall apply, both for the ambulance, and for the commissioned equipment. |  |  |  |
|  | • Maintenance |  |  |  |
|  | The manufacturer shall provide Use and Maintenance Manuals in Romanian. |  |  |  |
|  | II. EQUIPMENT LIST |  |  |  |
|  | 1. EQUIPMENT FOR PATIENT IMMOBILISATION AND HANDLING |  |  |  |
|  | • Main stretcher, with its own transportation system (trolley + detachable barrow) - 1 pc. It must comply with the requirements in the EN 1865-1:2010+A1:2015 standard |  |  |  |
|  | Side handles |  |  |  |
|  | Wheel brakes |  |  |  |
|  | Safety belts, including over the patient’s shoulders |  |  |  |
|  | Drip frame |  |  |  |
|  | Made of easily maintainable material |  |  |  |
|  | Minimum stretcher load - 200 kg |  |  |  |
|  | • Collapsible patient transportation chair |  |  |  |
|  | Patient fastening seatbelts |  |  |  |
|  | Ambulance fastening system |  |  |  |
|  | Brakes for a minimum of 2 wheels |  |  |  |
|  | • Transfer hood - 1 pc. |  |  |  |
|  | • Non-mobile oxygen installation |  |  |  |
|  | Oxygen cylinders: 2 pcs. of 10 litres minimum each (made of steel) |  |  |  |
|  | Reducing valve provided with pressure gauge for each cylinder. Quick connection for the flowmeter, located on the left-hand side wall Humidifier with possibility for connecting to a quick connection (DIN standard) |  |  |  |
|  | Flowmeter (with the possibility of connecting to a quick connection - DIN standard), with a maximum capacity of at least 15 l / minute, check stop valve, tubing and face mask - 1 pc. |  |  |  |
|  | • Portable oxygen |  |  |  |
|  | 1 5-litre cylinder (made of steel), anchored in the ambulance |  |  |  |
|  | Reducing valve with flowmeter, with a maximum capacity of at least 15 l / minute, check stop valve, tubing and face mask , plus quick connection for the ventilator. – 1 pc. |  |  |  |
|  | Protection and transport bag / cover for the oxygen cylinder- 1 pc. |  |  |  |
|  | • Adult / child ventilation balloon - 2 pc. (1 pc. for adults + 1 1 pc. for children / new-born). |  |  |  |
|  | • Oropharyngeal airway kit for adults and children |  |  |  |
|  | • Bag valve mask - 1 pc |  |  |  |
|  | • Portable secretion vacuum |  |  |  |
|  | Minimum negative pressure of 500 mmHg |  |  |  |
|  | Adjustable vacuum pressure |  |  |  |
|  | Minimum capacity of the collection container - 1000 ml |  |  |  |
|  | Holder with charger |  |  |  |
|  | • Drip frame - 1 pc. |  |  |  |
|  | Minimum 2 drip bags on the ambulance ceiling, no swinging |  |  |  |
|  | 2. Sanitary material |  |  |  |
|  | • Blankets - 2 pcs |  |  |  |
|  | • Emergency backpack with multiple storage areas – 1 pc. |  |  |  |
|  | • Single-use stretcher sheet - 1 roll |  |  |  |
|  | • Yankauer vacuum probe - 2 pc |  |  |  |
|  | • Renal tray - 1 pc |  |  |  |
|  | • Vomiting tray - 10 pcs |  |  |  |
|  | • Reusable male bedpan - 1 pc |  |  |  |
|  | • Reusable female bedpan - 1 pc |  |  |  |
|  | • Black plastic rubbish bags - 10 pcs |  |  |  |
|  | • Non-sterile examination gloves - 100 pcs |  |  |  |
|  | • Sharps and cuts waste container - 1 pc (approved variant) |  |  |  |
|  | • Disinfection biocidal material used for the cleaning and disinfection of surfaces in the medical compartment - 5 litres. (for the tendered product, the endorsement by the Ministry of Health - the National Commission for Marketing of Biocidal Products shall be attached) |  |  |  |

Clarifications on the tender submission method:

The technical proposal shall include:

- Technical Proposal Form (according to the form made available by the Contracting Authority), including all information required;

- The technical documentation supporting the requirements in the tender book. The documentation shall be numbered, so that, for every specification, the page number of the marking shall be recorded in the counterpart.

Tenderers shall draw up the technical documentation in an organized manner, so that it may provide an easy check of compliance with the requirements/specifications provided under this section, and with the provisions of the Tender Book and any documents appended thereto, acting as integral part of the award documentation.

- Technical specifications indicating a particular origin, source, production, a factory or trademark, an invention patent, a manufacturing license are listed in the tender book for the sole purpose of an easy identification of the product type and are not aimed at favouring or eliminating any economic operators or certain specific products, and such specifications shall be deemed to be provided with the note “OR EQUIVALENT”.   All the standards required in the technical specifications must be met, specifying that more current standards (if they exist) than those specified are recommended.

- The tenderer shall tick the YES or NO boxes with regard to meeting or not meeting, respectively, the specific technical requirement. If such feature is met, indication shall be provided of the document submitted and the page in the attached documentation (technical specification, user manual, technical sheet/manual) issued by the manufacturer, certifying the conformity with the declared provisions.

- No commercial brochures or Internet screenshots are allowed in support of technical components.

- The requirements in this tender book shall be deemed minimal and compulsory to be met (all technical features must be met). To this end, the basic tender submitted, with deviations from the provisions of the tender book, shall be considered only to the extent that the technical proposal entails the provision of a quality level higher than that of the listed requirements.

- Any technical features exceeding or above those required by the contracting authority shall be listed in the comments column and indication shall be provided of the page number in the technical documentation where the feature is listed.

- If, throughout the progress of the contract, it is found that elements of the technical proposal are non-compliant (inferior to or inconsistent with the requirements provided in the tender book), the contracting authority reserves the right to terminate the contract for convenience, or to claim a suspension in the supply of products until the situation detected has been remedied.

- Tendered products shall be new and not used.

- Warranty

All products must be covered by warranty for at least the requested period (24 months). The warranty period shall begin to lapse from the date of commissioning the vehicle and execution of the acceptance and commissioning protocol.

The warranty must cover all costs arising from the remedy of faults throughout the warranty period, including, but not limited to:

- fault diagnosis, included staff expenses;

- repair of all faulty elements or supply of new elements;

- replacement of faulty parts;

- setup in initial state;

- testing to provide a correct operation;